3GPP TSG-RAN WG2 Meeting #128                                   R2-2409713

Orlando, USA, 18th Nov. – 22th Nov, 2024

Agenda Item	: 8.4.4

Source	: LG Electronics Inc.

Title	: Discussion on LP-WUS for RRC_CONNECTED mode

Document for	: Discussion and Decision

Introduction

During the recent RAN2#126 [1], RAN2#127 [2], and RAN2#127bis [3] meetings, progress has been made in RAN2 regarding LP-WUS operation methods to trigger PDCCH monitoring identified by RAN1, as outlined below.

[RAN2#126 meeting agreement] For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. [RAN2#127 meeting agreement] For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1. [RAN2#127bis meeting agreement] For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

During this time, RAN1 not only reached an agreement on LP-WUS operations in the RRC_CONNECTED state but also left several FFS items, such as simultaneous configuration among options.

This document will present not only our additional opinions regarding LP-WUS operation as a continuation of the discussions from previous meetings but also our viewpoints on the items that remain FFS from both RAN1 and RAN2.

Discussion

Procedures to allow UE MR PDCCH monitoring triggered by LP-WUS

Option 1-2

In RAN2#127bis [3] meeting, for Option1-2, RAN2 agreed to start a new timer when LP-WUS is detected, and the duration during which the new timer is running is the C-DRX active time. It was left as FFS whether to start the legacy drx-onDurationTimer or not if the new timer is configured for Option 1-2.

In our understanding, starting the drx-onDurationTimer if the new timer is configured for Option1-2 is unnecessary. This is because, during the Active Time when the new timer is running, all operations such as PDCCH monitoring and CSI/L1-RSRP reporting can be performed. Therefore, there seems to be no reason to additionally start the drx-onDurationTimer in Option1-2. Moreover, starting the drx-onDurationTimer without any specific reason would only lead to unnecessary power consumption.

So, we believe that it is desirable to allow starting only a new timer in Option 1-2.

Proposal 1. The drx-onDurationTimer is disabled if the new timer is configured for Option 1-2.

LP-WUS indication

At the RAN1#116bis [4] and RAN1#118 [5] meetings, RAN1 reached the following agreement regarding the LP-WUS information to trigger PDCCH monitoring of UEs in RRC_CONNECTED. However, based solely on the agreement below, it is unclear whether the LP-WUS includes cases of not triggering PDCCH monitoring and detection misses in addition to triggering PDCCH monitoring.

RAN1#116bis Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs, at least consider the following： - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 2: A codepoint value corresponding to one or part of UE identity, e.g., C-RNTI - Option 3: A codepoint value corresponding to [one or more] UEs - Option 4: Multiple codepoint values with each corresponding to [one or more] UE(s) - Option 5: Multiple bit blocks with each corresponding to [one or more] UE(s) - Combination of above options are not precluded. - FFS how to carry LP-WUS information, e.g, by encoded bits (with/without CRC) and/or by OOK sequence selection for ‘ON-OFF’ pattern for OOK symbols of LP-WUS. - FFS how to carry LP-WUS information by overlaid OFDM sequences. - It doesn’t preclude considering the configuration where a single candidate overlaid OFDM sequence is used - FFS details of LP-WUS information to trigger PDCCH monitoring (e.g. whether above is applicable to one or more serving cells) RAN1#118 Agreement Regarding the LP-WUS information to trigger PDCCH monitoring of RRC connected UEs (for non-CA case), select at least one from the following - Option 1: A bitmap with each bit corresponding to [one or more] UEs - Option 3: A codepoint value corresponding to [one or more] UEs - FFS details for extension of option 1 and/or 3 when UE is configured with CA

According to the current specification, the legacy DCP operation functions as follows: when the UE detects a value of 1, the corresponding timer is started; when the UE detects a value of 0, the corresponding timer is not started; and if the UE detects nothing, the DCP is considered missing. In this case, the drx-onDurationTimer corresponding to the missing DCP is always started.

The basic understanding of LP-WUS operation is that "when an LP-WUS indication is received, the timer starts." It is clear that the LP-WUS indication includes a wake-up indication, but it is unclear whether it also includes a not wake-up indication. In our view, it is appropriate for the LP-WUS indication to include only the wake-up indication. This is because, LP-WUS operation is intented to be enabled in environments where data transmission and reception are infrequent. So, the LP-WUS indication would be rare. In such situation, if the LP-WUS indication were to include a not wake-up indication, this would require the NW to send many indications. This would result in a waste of NW resources and inefficient operation.

Therefore, it would be more efficient for the LP-WUS indication to include only the wake-up indication.

Proposal 2. LP-WUS should include only the information about starting the timer when detected.

An additional aspect of LP-WUS operation worth discussing is the behavior when the LP-WUS indication is missed.

If we agree to start the timer even when the LP-WUS indication is missed, it could lead to unnecessary power consumption without significant benefits. This contradicts the primary goal of introducing LP-WUS, which is to save power.

On the other hand, if we agree not to start the timer even when the LP-WUS indication is missed, the NW can simply resend the LP-WUS indication on the next LP-WUS monitoring occasion. While this may cause some data latency, the impact would be marginal.

Therefore, we think that it is better not to start the timer if the UE does not receive the LP-WUS indication on the LP-WUS monitoring occasion. Also, the operation of LP-WUS should not differ between Option 1-1 and Option 1-2.

Proposal 3. If the UE does not receive the LP-WUS indication, the timer does not start.

Simultaneous Configuration of Option 1-1 and Option 1-2

In the RAN1#118 [5] meeting, whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE is left as FFS.

RAN1#118 Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

According to the RAN2#126 agreement [1], for Option 1-1, if an LP-WUS indication is received on an LP-WUS monitoring occasion that is located at a configured time offset before the start of drx-onDurationTimer, the drx-onDurationTimer starts, and PDCCH monitoring is performed while drx-onDurationTimer is running.

In contrast, according to the RAN2#127 [2], RAN2#127bis [3] agreement, for Option 1-2, if an LP-WUS indication is received during an LP-WUS monitoring occasion configured by the LP-WUS configuration, which is independent of the DRX configuration, the new timer starts, and PDCCH monitoring is performed while the new timer is running.

The figure below shows an example of the operation of Option1-1 and Option1-2 in the case of receiving DL data.

In our understanding, the main difference between Option 1-1 and Option 1-2 is which timer is started to ensure that the UE is in Active Time when the LP-WUS is detected. Additionally, we assume that the LP-WUS configuration for Option 1-1 and Option 1-2 is configured separately, meaning that the LP-WUS configurations for each two options are independent.

If both Option 1-1 and Option 1-2 are configured on the UE, the UE will monitor LP-WUS monitoring occasions configured by the LP-WUS configurations of both Option 1-1 and Option 1-2. In this case, if the LP-WUS is received on the LP-WUS monitoring occasion corresponding to Option 1-1, the drx-onDurationTimer will be started. If the LP-WUS is received on the LP-WUS monitoring occasion corresponding to Option 1-2, the new timer will be started. In other words, which timer to start depends on which LP-WUS monitoring occasion the UE receives the LP-WUS. This will unnecessarily add complexity to the UE's operation.

Moreover, there may be instances where the UE receives LP-WUS on overlapping LP-WUS monitoring occasions. This occasion is configured by the LP-WUS configurations of both Option 1-1 and Option 1-2. In such cases, it has not yet been determined whether the UE will start both timers or start only one of the timers.

If the UE starts both timers, it will result in unnecessary operation of the shorter duration timer. This is because the Active Time triggered by the longer duration timer can cover Active time triggered by the shorter duration timer.

If the UE starts only one of the two timers, a rule will be needed to synchronize between the NW and the UE on which timer to start. Introducing a new rule will complicate the specification.

If the simultaneous configuration of Option 1-1 and Option 1-2 provides significant advantages, it would be worth supporting despite the added complexity. However, from our perspective, there are no substantial benefits, as similar performance can be achieved when either of the two options is applied individually.

For a UE configured with Option 1-1, the disadvantage compared to the simultaneous configuration of Option 1-1 and Option 1-2 is the increased data latency. This is because the length of the DRX cycle is longer than that of the LP-WUS cycle. However, this drawback can be mitigated by configuring the DRX cycle length to be as short as the LP-WUS cycle length in Option1-2.

For a UE configured with Option 1-2, the disadvantage compared to the simultaneous configuration of Option 1-1 and Option 1-2 is the absence of the drx-onDurationTimer. However, since all operations such as PDCCH monitoring and CSI/L1-RSRP reporting can already be performed during the Active Time initiated by the new timer, there seems to be no need to additionally start the drx-onDurationTimer additionally.

Therefore, we believe that simultaneous configuration of Option1-1 and Option1-2 is not needed.

Proposal 4. Simultaneous configuration of Option 1-1 and Option 1-2 is not supported.

Conclusion

Based on the above discussion, we made the following proposals.

[Option1-2]

Proposal 1. The drx-onDurationTimer is disabled if the new timer is configured for Option 1-2.

[LP-WUS indication]

Proposal 2. LP-WUS should include only the information about starting the timer when detected.

Proposal 3. If the UE does not receive the LP-WUS indication, the timer does not start.

[Simultaneous Configuration of Option1-1 and Option1-2]

Proposal 4. Simultaneous configuration of Option 1-1 and Option 1-2 is not supported.

References

- R2_126 MIMOevo MUSIM LPWUS (Erlin) 2024-05-24 1115.docx

- R2_127 MIMOevo MUSIM LPWUS (Erlin) 2024-08-23 0900.docx

- R2_127bis MIMOevo MUSIM LPWUS (Erlin) 2024-10-17 1710.docx

- Chair notes RAN1#116bis eom2.docx

- Chair notes RAN1#118 eom1.docx

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- TR 38.869 v18.0.0, Study on low-power wake up signal and receiver for NR