3GPP TSG-RAN WG2 Meeting #128	R2-2409720

Orlando, U.S.A, Novermber 18th – 22nd, 2024

Source:	CATT

Title:	Analysis on LP-WUS for RRC_CONNECTED Mode

Agenda Item:	8.4.4

Document for:	Discussion and Decision

Introduction

In previous RAN1 meetings and RAN2 meetings, great progress was made for LP-WUS in RRC_CONNECTED mode. And in RAN1#118bis meeting, both Option 1-1 and Option 1-2 were agreed for LP-WUS in RRC_CONNECTED mode [1].

Agreement Confirm following working assumption Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

Based on the agreements, we will continue to analyze the LP-WUS monitoring procedure in RRC_CONNECTED mode for Option 1-1 and Option 1-2 and our proposals are provided.

Discussion

2.1	Common issues for Option 1-1 and Option 1-2

In RAN1#118bis meeting, the following agreements regarding Option 1-2 were agreed:

Agreement - For LP-WUS CONNECTED mode operation, Option 1-2 can be configured separately from Option 1-1 - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

It can be seen that it is still FFS on whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE based on RAN1 agreement. In our understanding, the UE behaviours are complex if both Option 1-1 and Option 1-2 are configured simultaneously. For Option 1-1, the UE will start drx-onDurationTimer when it is waked up by LP-WUS, while for Option 1-2, the UE will be triggered by LP-WUS to start a timer duration which UE monitors PDCCH. If these two options are combined, one possible procedure is that the UE performs LP-WUS monitoring outside at least legacy C-DRX Active time according to the LP-WUS. And the UE also monitors LP-WUS which is located before drx-onDurationTimer. When this LP-WUS indicates the UE to start drx-onDurationTimer, the drx-onDurationTimer will be started like Option 1-1. Otherwise, the UE continues to monitor LP-WUS during the legacy drx-onDurationTimer. Apparently, when both Option 1-1 and Option 1-2 simultaneously configured, the procedure will be quite complex and no further benefit compared with separate Option 1-1 or Option 1-2. Therefore, it is proposed that:

Proposal 1: It is NOT supported to configure both Option 1-1 and Option 1-2 simultaneously for the same UE.

    - Dual DRX group

In order to reduce the power consumption when both FR1 and FR2 cells are configured via Carrier Aggregation, a secondary DRX group was introduced with a separate drx-InactivityTimer and drx-onDurationTimer configured for the FR2 cells enabling FR2 to go to sleep more quickly. However, in order to reduce the impacts to RAN1 and keep the simplicity of DRX group mechanism, dual DRX group is not configured simutanously with DCP.

As RAN2 has assumed that the solutions/ operations introduced for DCP mechanism is taken as baseline, the LP-WUS should not be configured simultaneously with dual DRX group to reduce the specification complexity.

Proposal 2: In Rel-19, LP-WUS should not be configured with dual DRX groups simultaneously for both Option 1-1 and Option 1-2.

    - Short DRX cycle

For DCP, if short DRX cycle is supported, it may require a huge overhead in the DL which may reduce the power saving gain. In RAN1 replied LS [2], it indicated RAN1 assumed that DCP was not applicable for the short DRX cycle if both long and short DRX cycles are configured for the UE. Finally, it was agreed that DCP was only applicable to long DRX cycle.

For LP-WUS, it can simply follow the same mechanism of DCP as above.

Proposal 3: Short DRX cycle is not applicable for LP-WUS for both Option 1-1 and Option 1-2.

    - DC/CA

For LP-WUS working for DC/CA, RAN1 has made the following agreements in RAN1#118bis meeting:

Agreement LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

Based on RAN1 discussion, it can be seen RAN1 will continue to discuss the cell(s) where PDCCH monitor triggered by a LP-WUS is applicable. For DCP, it is only configured on the SpCell [3]. It is more preferred that the same principle can be applied for LP-WUS. But considering RAN1 will continue to discuss this issue, RAN2 can leave this to RAN1 for further progress.

Proposal 4: RAN2 waits for more inputs from RAN1 for LP-WUS with DC/CA for both Option 1-1 and Option 1-2.

2.2	Option 1-2

Regarding Option 1-2, RAN1 and RAN2 made the following agreements:

RAN1#118 meeting: Agreement For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18 - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration - LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS

RAN2#127bis meeting: For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

Based on the agreements above, we continue to discuss the remaining issues for option 1-2.

- 2.2.1	New timer triggered by LP-WUS to monitor PDCCH

Based on the updated agreements in RAN2, it can be seen that one new timer will be defined for PDCCH monitoring triggered by LP-WUS and the UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

In order to facilitate subsequent discussion for LP-WUS in RRC_CONECTED mode, we think one terminology can be used to define this new timer. Considering the function of the new timer is similar to that of the drx-onDurationTimer, we think lp-onDurationTimer can be used for this timer.

Proposal 5: The new timer in Option 1-2 can be named as lp-onDurationTimer.

It is specified in MAC that:

1> if a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission or a Long DRX Command MAC CE is received: 2> stop drx-onDurationTimer for each DRX group; 2> stop drx-InactivityTimer for each DRX group.

The similar mechanism can be applied to the lp-onDurationTimer. That is when a DRX Command MAC CE indicated by PDCCH addressed to C-RNTI or CS-RNTI, or by a configured downlink assignment for unicast transmission or a Long DRX Command MAC CE is received, the UE stops lp-onDurationTimer triggered by LP-WUS.

Proposal 6: When the UE receives (long) DRX Command MAC CE, the UE stops the lp-onDurationTimer in Option 1-2.

- 2.2.2	Whether to start drx-onDurationTimer

It has been agreed in RAN1 for Option 1-2 that:

- For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not

According to the agreement, the UE behavior can be clarified as:

Based on configuration,

- the periodic CSI/L1-RSRP only can be reported when the UE is indicated to wake-up which is friendly to power saving but the channel status may not be reported promptly especially when the UE keeps monitoring LP-WUS; or

- the UE will report CSI/L1-RSRP periodically not matter whether it is indicated to wake-up or not which reduces the power saving gain but guarantees that the network is awareness of the updated channel status.

Based on the agreements, it can be seen that whether periodic CSI/L1-RSRP can be reported is independent of the drx-onDurationTimer which results in CSI mask may not be configured in Option 1-2. In legacy DRX, CSI mask is an efficient way for the network to configure physical resources for CSI reporting regardless of the UE activities. By configuring CSI mask and the long DRX cycle, the network knows that the UE only reports CSI during the drx-onDurationTime and will not report CSI outside of drx-onDurationTimer, and then the network can reuse the physical resources for CSI reporting out of drx-onDurationTimer.

Based on the clarification above, it may bring resource allocation issues for periodic CSI/L1-RSRP reporting. Considering RAN1 may continue to discuss this, RAN2 can wait for RAN1 further input to decide whether to start drx-onDurationTimer in Option 1-2.

Proposal 7: RAN2 waits more inputs from RAN1 to decide whether to start the drx-onDurationTimer in Option 1-2.

Conclusion

In this document, we analyse procedure and activation/deactivation of LP-WUS in RRC_CONNECTED state, and we have the following proposls:

Common issues for Option 1-1 and Option 1-2:

Proposal 1: It is NOT supported to configure both Option 1-1 and Option 1-2 simultaneously for the same UE.

Proposal 2: In Rel-19, LP-WUS should not be configured with dual DRX groups simultaneously for both Option 1-1 and Option 1-2.

Proposal 3: Short DRX cycle is not applicable for LP-WUS for both Option 1-1 and Option 1-2.

Proposal 4: RAN2 waits for more inputs from RAN1 for LP-WUS with DC/CA for both Option 1-1 and Option 1-2.

Specific issues for Option 1-2:

Proposal 5: The new timer in Option 1-2 can be named as lp-onDurationTimer.

Proposal 6: When the UE receives (long) DRX Command MAC CE, the UE stops the lp-onDurationTimer in Option 1-2.

Proposal 7: RAN2 waits more inputs from RAN1 to decide whether to start the drx-onDurationTimer in Option 1-2.

Reference

- R2-2409502, RAN2#127bis Meeting Report;

- R1-1911475, LS on PDCCH-based Power Saving Signal/Channel carrying indication of UE wakeup before DRX ON, CATT;

- TS 38.300, V18.3.0, NR and NG-RAN Overall Description, Release 18.