3GPP TSG-RAN WG2 Meeting #128	R2-2409730

Orlando, USA, November 2024

Agenda Item:	8.13.2

Source:	OPPO

Title:	Discovery and relay (re)selection for multi-hop U2N relay

Document for:	Discussion, Agreement

Introduction

The objective of multi-hop U2N relay is as follows, this paper will discuss the discovery and relay (re)selection for multi-hop U2N relay.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays. - Relay discovery and (re)selection [RAN2] - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3] - Impact on SRAP and QoS handling for multi-hop [RAN2] - Control plane procedures [RAN2, RAN3]

Discussion

Multi-hop U2N Discovery

For the discovery transmission at the intermediate relay UE, whether Uu and PC5 threshold conditions should be defined is discussed.

For Uu threshold condition, in R17 single hop relay, the upper bound of Uu quality is defined for the remote UE to make sure cell centre UE (can access the network directly) doesn’t access the network via the relay.

In R17 U2N Relay, the upper bound of Uu quality is defined for the remote UE to make sure the cell centre UE doesn’t access the network via the relay.

In RAN2 #127 meeting, it was concluded that for the last relay, both the lower bound and upper bound of Uu threshold is needed.

If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds).

It is concluded that the last relay can perform discovery transmission if the Uu measurement of the serving cell is below an upper bound.

It is unreasonable that the upper bound of Uu RSRP restricts discovery transmission as the last relay but not the intermediate relay, i.e., to allow the intermediate relay closer to the network than the last relay UE.

It is not reasonable to allow the case that the intermediate relay UE is closer to the network than the last relay UE.

In the WID, it is clarified the motivation of multi-hop relay is to extend the communication range limitation of single-hop relay.

However, single hop U2N relays that have been developed in Rel-17 and Rel-18 have limited applicability because of the range limitation of a single hop sidelink relay. Thus, additional sidelink relay enhancements are needed to support some remaining (and new) requirements for Proximity Services captured in TS 22.278 and TS 22.261, e.g. multi-hop support for UE-to-Network Relay.

Therefore, the principle in R17 should be followed in the multi-hop case, i.e., cell centre UE should not access the network via other UE(s) to ensure communication efficiency and coverage extension gain.

For the discovery transmission by the intermediate relay UE, an upper bound of Uu channel quality is needed.

Then for the PC5 threshold condition, for the PC5 link quality with the last relay, on the one hand, the link quality needs to be checked to ensure the E2E multi-hop relay link quality, on the other hand, after the intermediate relay has selected/connected to a parent relay, the link quality is evaluated by the intermediate relay UE, and if the link quality is bad, the intermediate relay UE should trigger relay reselection procedure.

RAN2 to discuss for both Model-A and Model-B discovery, the intermediate relay UE evaluates PC5 link quality with the parent relay UE based on relay (re)selection criteria defined for intermediate relay UE.

For the PC5 link between the remote UE and the intermediate relay UE, it can be evaluated by the remote UE based on relay (re)selection criteria.

The PC5 link quality between the remote UE and the intermediate relay UE is checked by the remote UE based on relay (re)selection criteria.

Therefore, there is no need for a double evaluation of the PC5 link quality at the intermediate relay upon Model-B solicitation message reception.

For Model-B Solicitation message transmission by the intermediate relay, no PC5 threshold configuration is needed.

There is also a discussion about whether the last relay needs to check the SD-RSRP of the received Model-B solicitation message to decide whether to respond. As discussed in Proposal 2, the link quality between the intermediate relay and the last relay is evaluated by the intermediate relay via relay (re)selection, i.e., if the link quality between the intermediate relay and the last relay is bad, the intermediate relay should perform relay reselection to another last relay. Therefore, as in single-hop U2N Relay, there is no PC5 condition to be checked at the last relay UE, i.e., the PC5 threshold configuration is not needed for Model-B response at the last relay UE.

For Model-B Response transmission by the last relay, no PC5 threshold configuration is needed.

Relay (re)selection

For the relay selection and reselection trigger condition, the following trigger condition is defined in R17 single-hop U2N relay and R18 U2U relay case:

The U2N Remote UE triggers U2N Relay selection in following cases: - Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold; - Indicated by upper layer of the U2N Remote UE. The U2N Remote UE may trigger U2N Relay reselection in following cases: - PC5 signal strength of current U2N Relay UE is below a (pre)configured signal strength threshold; - Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by U2N Relay UE via PC5-RRC signalling; - When U2N Remote UE receives a PC5-S link release message from U2N Relay UE; - When U2N Remote UE detects PC5 RLF; - Indicated by upper layer. The U2U Remote UE may trigger U2U Relay selection in the following cases: - When the SL-RSRP or SD-RSRP between U2U Remote UEs is below a (pre)configured signal strength threshold; - When U2U Remote UE receives an indication to trigger U2U relay selection from the upper layer of the UE. The U2U Remote UE may trigger U2U Relay reselection in the following cases: - When the SL-RSRP or SD-RSRP of the current U2U Relay UE is below a (pre)configured signal strength threshold; - When U2U Remote UE receives an indication from the upper layer due to detecting PC5 RLF; - When L2 U2U Remote UE receives an indication from the upper layer due to receiving the PC5 RLF indication from the L2 U2U Relay UE; - When U2U Remote UE receives a PC5-S link release message from U2U Relay UE; - When U2U Remote UE receives an indication to trigger U2U relay reselection from the upper layer of the UE.

For multi-hop relay selection, it should be the same as single-hop relay, i.e., indicated by the upper layer and direct link quality is below a configured threshold.

For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE.

For Relay reselection, in R17 single-hop U2N Relay and R18 U2U Relay, the trigger condition can be summarized into four aspects:

- Upper layer triggered;

- Link quality with the current relay below a configured threshold;

- The link between the remote UE and the relay UE breaks, e.g., either RLF or PC5-S release;

- The link between the relay UE and other side (network or the peer remote UE) breaks, e.g., RLF, cell change, RRC failure.

In R17 single-hop U2N Relay and R18 U2U Relay, the following 4 aspects are considered as relay reselection trigger condition:

- Upper layer triggered;

- Link quality with the current relay below a configured threshold;

- The link between the remote UE and the relay UE breaks, e.g., either RLF or PC5-S release;

- The link between the relay UE and the other side (network or the peer remote UE) breaks, e.g., RLF, cell change, RRC failure.

In R19 multi-hop U2N Relay, we can follow the spirit and consider relay reselection triggers from the four aspects. For the upper layer trigger, link quality degradation with the current direct connected relay, and the link between the remote UE and direct connected relay breaks (RLF or PC5-S release), it is similar for single-hop U2N Relay and multi-hop U2N Relay, so the R17 U2N Relay trigger condition can be reused.

For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition:

- PC5 signal strength of current direct connected multi-hop U2N Relay UE is below a (pre)configured signal strength threshold;

- When U2N Remote UE receives a PC5-S link release message from current direct connected multi-hop U2N Relay UE;

- When U2N Remote UE detects PC5 RLF with the current direct connected multi-hop U2N Relay UE;

- Indicated by upper layer.

For the indication of the other link problem, firstly for the Uu link between the relay UE and network, when Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure happens, it is reasonable for the remote UE to trigger relay reselection since group handover is not supported.

Considering group handover is not supported, it is reasonable that multi-hop U2N Remote UE triggers relay reselection when cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by the multi-hop U2N Relay UE(s) as in R17 single hop case.

And for the PC5 link between two multi-hop U2N Relay UEs, similar to Uu link, when the PC5 radio link between 2 multi-hop U2N Relay UEs fails, the multi-hop U2N Relay UE which is closer to the remote UE should indicate such information to the remote UE so that the remote UE can know the multi-hop relay link is broken.

The PC5 radio link failure between two multi-hop U2N Relay UEs may happen, and the indication from multi-hop U2N Relay UE to the remote UE is helpful for the remote UE to know the multi-hop relay link is broken and relay reselection is needed.

The unclear part is how for the multi-hop U2N Relay UE to indicate such information to the remote UE considering the legacy Notification message is a per-hop PC5 RRC message. So the indication can be hop-by-hop which needs RRC message (Notification message) forwarding between the UEs. This depends on the control plane design which can be further discussed by RAN2.

How to indicate cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure and PC5 RLF from the multi-hop U2N Relay UE to the Remote UE depends on control plane design.

Multi-hop U2N Remote UE triggers relay reselection upon PC5-RRC signaling indicating (FFS on the detailed PC5-RRC signaling design):

- cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the multi-hop U2N Relay UE(s) and network;

- PC5 RLF between the multi-hop U2N Relay UE(s);

For the relay (re)selection at the intermediate relay UE, there is a discussion on whether the relay (re)selection condition is required.

Firstly, the intermediate relay UE also acts as remote UE considering it needs to rely on the last relay for Paging/SIB monitoring in IDLE/INACTIVE case, and for its own data forwarding in CONNECTED case, i.e., the intermediate relay can perform relay reselection as a remote UE, which is concluded by SA2 in TS 23.304.

##### 6.4.3.8.3 Relay reselection and mobility support […] The relay reselection procedure can be used by a 5G ProSe Intermediate UE-to-Network Relay, taking the role of the 5G ProSe Remote UE in the procedure.

The intermediate relay UE also acts as remote UE for Paging/SIB and data forwarding, it performs relay reselection same as remote UE.

Then, in case the link is broken, i.e., not workable, there is no need to keep the link between the intermediate relay UE and the serving relay, i.e., the intermediate relay can decide to release/switch to another link.

There is no need to keep the link between the intermediate relay UE and its serving relay if the link is broken (RLF, link release, RRC failure), the intermediate relay can decide to release/switch to another link in such cases.

Therefore, it is needed for the intermediate relay UE to perform relay reselection, the same trigger condition of relay (re)selection defined for the remote UE can be used.

RAN2 to discuss intermediate relay UE can perform relay (re)selection with the same trigger condition defined for the remote UE.

Conclusion

We have the following observations:

Observation 1	In R17 U2N Relay, the upper bound of Uu quality is defined for the remote UE to make sure the cell centre UE doesn’t access the network via the relay.

Observation 2	It is concluded that the last relay can perform discovery transmission if the Uu measurement of the serving cell is below an upper bound.

Observation 3	It is not reasonable to allow the case that the intermediate relay UE is closer to the network than the last relay UE.

Observation 4	The PC5 link quality between the remote UE and the intermediate relay UE is checked by the remote UE based on relay (re)selection criteria.

Observation 5	In R17 single-hop U2N Relay and R18 U2U Relay, the following 4 aspects are considered as relay reselection trigger condition:

-	Upper layer triggered;

-	Link quality with the current relay below a configured threshold;

-	The link between the remote UE and the relay UE breaks, e.g., either RLF or PC5-S release;

-	The link between the relay UE and the other side (network or the peer remote UE) breaks, e.g., RLF, cell change, RRC failure.

Observation 6	Considering group handover is not supported, it is reasonable that multi-hop U2N Remote UE triggers relay reselection when cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by the multi-hop U2N Relay UE(s) as in R17 single hop case.

Observation 7	The PC5 radio link failure between two multi-hop U2N Relay UEs may happen, and the indication from multi-hop U2N Relay UE to the remote UE is helpful for the remote UE to know the multi-hop relay link is broken and relay reselection is needed.

Observation 8	How to indicate cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure and PC5 RLF from the multi-hop U2N Relay UE to the Remote UE depends on control plane design.

Observation 9	The intermediate relay UE also acts as remote UE for Paging/SIB and data forwarding, it performs relay reselection same as remote UE.

Observation 10	There is no need to keep the link between the intermediate relay UE and its serving relay if the link is broken (RLF, link release, RRC failure), the intermediate relay can decide to release/switch to another link in such cases.

We have the following proposals:

Proposal 1	For the discovery transmission by the intermediate relay UE, an upper bound of Uu channel quality is needed.

Proposal 2	RAN2 to discuss for both Model-A and Model-B discovery, the intermediate relay UE evaluates PC5 link quality with the parent relay UE based on relay (re)selection criteria defined for intermediate relay UE.

Proposal 3	For Model-B Solicitation message transmission by the intermediate relay, no PC5 threshold configuration is needed.

Proposal 4	For Model-B Response transmission by the last relay, no PC5 threshold configuration is needed.

Proposal 5	For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE.

Proposal 6	For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition:

-	PC5 signal strength of current direct connected multi-hop U2N Relay UE is below a (pre)configured signal strength threshold;

-	When U2N Remote UE receives a PC5-S link release message from current direct connected multi-hop U2N Relay UE;

-	When U2N Remote UE detects PC5 RLF with the current direct connected multi-hop U2N Relay UE;

-	Indicated by upper layer.

Proposal 7	Multi-hop U2N Remote UE triggers relay reselection upon PC5-RRC signaling indicating (FFS on the detailed PC5-RRC signaling design):

-	cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the multi-hop U2N Relay UE(s) and network;

-	PC5 RLF between the multi-hop U2N Relay UE(s);

Proposal 8	RAN2 to discuss intermediate relay UE can perform relay (re)selection with the same trigger condition defined for the remote UE.