# 3GPP TSG-RAN WG2 Meeting #128	                               R2-2409738
Orlando, US, 18 – 22 Nov, 2024

Agenda Item	: 8.2.4 (FS_Ambient_IoWT_solutions)

Source	: LG Electronics Inc.

Title	: Discussion on random access aspects for Ambient IoT

Document for	: Discussion and Decision

Introduction

In this contribution, we show our views on the random access procedure for A-IOT.

Discussion

  - Overall random-access procedure

It was agreed that the A-IOT paging message triggers the random access procedure for at least one A-IOT device. Then, the reader indicates the Msg1 resource indication to the device in order to indicate the Msg1 resource in time domain. Note that, in RAN1, it was agreed that a R2D transmission triggers the random access procedure. In addition, this R2D message indicates the time domain resource for Msg1 transmission.

During the A-IOT paging round, an A-IOT device can perform only one random access procedure for one access occasion. If the contention resolution for an A-IOT device is not completed during the random access procedure, the A-IOT device performs the re-access procedure. Otherwise, if the contention resolution for an A-IOT device is completed, the A-IOT device does not perform the re-access procedure.

The main purpose of the re-access procedure is to perform the random access procedure for the A-IOT devices that have failed the random access procedure. Thus, as agreed before, an A-IOT device may perform the re-access procedure when the subsequent A-IOT paging message is received if the A-IOT device does not successfully complete the random access procedure in the previous A-IOT paging round.

Considering above explanation, we propose to introduce a new message to indicate the time domain resource for Msg1 transmission. In addition, since RAN2 do not capture the terminology of the paging round in TR, we propose to capture the definition of the paging round in TR.

Proposal 1. A new message is introduced to indicate the time domain resource for Msg1 transmission.

Proposal 2. The definition of paging round should be captured in TR as follows

- Paging round: The paging round is initiated by A-IOT paging message. One paging round consists one or multiple access occasions. Within one paging round, one A-IOT device has an opportunity to perform the random access procedure for one access occasion.

  - Need of additional indication for re-access procedure

In RAN2#127bis meeting, the following agreement was made.

5 R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message. FFS additional indications to differentiate if needed in paging message. FFS if other message can be used.

During [POST127][033] email discussion, it was proposed that the reader extends the access occasions within one paging round when the reader wants to extend the access occasions. We think that there are two motivations for the extension of the access occasion within one paging round. The first motivation is that if collision ratio is high, the reader extends the access occasions to relax the collision. The second motivation is that the re-access procedure is performed within the extended access occasions. The following figure shows an example for extension of the access occasions within one paging round.

However, the A-IOT paging message can cover both motivations. For relaxation of the collision, if the reader determines that the collision ratio is high, the reader can terminate the current paging round and the reader indicates new paging round by indicating the large number of the access occasions.

In addition, as agreed before, the A-IOT paging message is used to indicate the access and re-access procedure for both A-IOT devices that the random access failure happens and the A-IOT devices that the previous A-IOT paging message is missed. However, the indication for extension of access occasions indicates the re-access procedure only for the A-IOT devices that the random access failure happens.

In summary, we think that the A-IOT paging message can cover the both motivations of the indication for extension of access occasions. Thus, the additional indication for the same purposes is not needed.

Proposal 3. The additional indication is not introduced for extension of the access occasions within a paging round.

  - Success/failure indication

In previous RAN2 meetings, it was discussed when to perform the re-access procedure, and the following agreements were made for re-access procedure.

In RAN2#125bis meeting 6 Handling of contention resolution failure and access failure at the device will be studied in RAN2, including failure detection and re-access. FFS details In RAN2#127bis meeting 2 Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose. FFS for following D2R data.

The current agreements say that the re-access procedure is performed when the random access procedure is failed, e.g., contention resolution failure or reception of R2D failure feedback indication for Msg3 transmission. However, even after completion of the random access procedure, the re-access procedure is performed when the data transmission, i.e., command procedure, is failed based on the R2D failure feedback indication.

For example, as agreed before, the reader can perform the combination of the inventory and command procedure. It means that even if the random access is completed, the reader can transmit the command message to the device. In this case, if the reader does not receive the D2R response message, the reader can indicate the failure feedback indication to the A-IOT device.

If the A-IOT device receives the failure feedback indication, we think that the A-IOT device should perform the same behaviour as Msg3 transmission failure. Thus, we propose that the A-IOT device always performs the re-access procedure when the failure feedback indication is received.

Proposal 4. The A-IOT device always performs the re-access procedure when the failure feedback indication is received even after the contention resolution is completed.

Next, we need to discuss when to transmit the success feedback indication. From the reader point of view, if the reader has a subsequent message, there is no reason to transmit the success feedback indication. This is because the transmission of the subsequent message implies that the previous D2R message is successfully received. Thus, the reader may need to transmit the success feedback indication only when there is no more subsequent message to the A-IOT device. Then, the A-IOT device considers that all procedures are successfully completed when the success feedback is indicated.

Proposal 5. The success feedback indication is transmitted only when there is no more subsequent message to the A-IOT device. Then, the A-IOT device considers that all procedures are successfully completed when the success feedback is indicated.

Even if the success/failure feedback indication is introduced, RAN2 need to discuss what is a A-IOT device behaviour when the success/failure feedback indication is not received after transmitting the D2R response message. In this case, the A-IOT device does not know whether the D2R response message is successfully transmitted or not.

For above cases, two options can be considered as follows.

- Option 1. The absent of the success/failure feedback indication is considered as successful transmission of D2R message, and the A-IOT device does not need to perform the re-access procedure.

- Option 2. The absent of the success/failure feedback message is considered as not successful transmission of D2R message, and the A-IOT device needs to perform the re-access procedure.

The option 1 requires that the reader should transmit the dedicated A-IOT paging message for an A-IOT device if the reader considers that all procedures are not successfully completed for the A-IOT device due to reception failure of the D2R message.

The option 2 may cause the congestion during the re-access procedure by unnecessarily performing the random access procedure even if the A-IOT device already successfully performs all procedures.

In our view, from the service perspective, both options are workable. However, the option 2 has an impact on the performance of the re-access procedure for all A-IOT devices requiring the re-access procedure, we slightly prefer to go option 1.

Proposal 6. The A-IOT device considers that the D2R transmission is successfully transmitted if there is no success/failure feedback indication.

  - Down-selection/unified design of the random access procedure

In RAN2#127bis, the following agreement was made for unified design for random access procedure

For all use cases, device determines the random access type from the paging message. FFS whether it is explicitly or implicitly. FFS whether we will down selection for all RA types (2step, 3step, CFRA) or try to unify the design for 2step, 3step

Before discussing the down-selection of random access procedure, we need to look into each CBRA procedure.

As shown above figure, the main difference between the 3-step and 2-step CBRA procedure is when the A-IOT device transmits the A-IOT device ID. In 3-step CBRA, the A-IOT device transmits the A-IOT device ID in Msg3 and in 2-step CBRA, the A-IOT device transmits the A-IOT device ID in Msg1. Considering that the main purpose of the CBRA procedure is the contention resolution, whether to transmit the A-IOT device ID in Msg1 or Msg3 is not important.

In addition, since the characteristic of the A-IOT device is low complexity, supporting both CBRA procedures for A-IOT is questionable. Thus, we think that it would be better to down-select only one CBRA procedure for A-IOT.

Observation 1. Since there is not much difference between the 3-step and 2-step CBRA procedure, it would be better to down-select only one CBRA procedure for A-IOT to simplify the random access procedure in A-IOT.

The 3-step CBRA may be used when the high congestion is expected, and the 2-step CBRA is used when the high congestion is not expected. This is because the transmission of the A-IOT device ID may cause the signalling overhead when the A-IOT device fails the random access procedure, i.e., the contention resolution failure.

However, in our view, since the reader can indicate the proper number of access occasions, the high congestion can be avoided by the reader implementation. In addition, since RAN2 may need to consider the unified solution for all random access procedures for the simplification of the random access procedure, it would be better to down-select the 2-step CBRA because the procedure of the 2-step CBRA is very similar to CFRA.

Proposal 7. Only 2-step CBRA procedure should be introduced for A-IOT.

If the above proposal is agreeable, the next discussion is how to design a unified solution between the CBRA and CFRA.

As per current agreements, there are two different points between the 2-step CBRA and CFRA. The first point is whether to transmit the random ID in Msg1 or not. The second point is the presence of the Msg2 for transmission of the random ID.

For the first point, in RAN2#127bis meeting, it was agreed that AS ID is used for the purpose of D2R scheduling and R2D reception. To allocate the AS ID for CFRA, two options were proposed to assign the AS ID. The first option is that the AS ID is included in the A-IOT paging message, and the second option is that the random ID transmitted in Msg1 is used for AS ID even in CFRA case. Among the two options, we think that the second option is better than the first option because the A-IOT paging message should be as small as possible to increase the decoding success probability for the A-IOT paging message. Thus, we think that the random ID is contained in Msg1 even in CFRA.

Proposal 8. For CFRA, a random ID is contained in Msg1 for the purpose of D2R scheduling and R2D reception.

For the second point, we do not think that Msg2 is needed for CFRA case. Since there is no contention in the access occasion indicated for the CFRA, the contention resolution is not needed by receiving the Msg2 with the same random ID. It means that the A-IOT device considers that the contention resolution is successfully completed after transmitting the Msg1 containing the A-IOT device ID and random ID.

Proposal 9. For CFRA, the A-IOT device considers that the contention resolution is successfully completed after transmitting the Msg1 containing the A-IOT device ID and random ID.

For the above proposals, we show our proposals for the unified design in random access as follows.

  - Further consideration of the selection of the access occasion

    - CBRA

In RAN2#127bis meeting, it was agreed that the A-IOT device randomly selects one access occasion within the access occasions provided by reader to perform the random access procedure. However, even if the A-IOT device can randomly select one access occasion, we think that the energy level of the A-IOT device should be considered.

Since the A-IOT device has small energy storage capability, an A-IOT device may be in a condition that it doesn’t have enough energy to monitor PRDCH for all access occasions. In this case, the A-IOT device may not try to perform the random access procedure due to out-of-energy if the A-IOT device selects the last part of the access occasion due to monitoring the PRDCH for a long time.

In addition, since the reader does not have information on whether an A-IOT device has enough energy or not, the reader may not control the prioritization of the A-IOT device having not enough energy to perform the random access procedure first. Thus, we think that how to select an access occasion based on the A-IOT device’s energy level should be discussed.

It may be argued that the CN can trigger the dedicated A-IOT paging procedure for the A-IOT devices that the access procedure fails due to out-of-energy because the CN knows which A-IOT device does not perform the access procedure. However, since the energy harvesting should be performed for a long time, e.g., a hundred milliseconds or several seconds, it would have an impact on the QoS satisfaction and efficiency.

Proposal 10. How to select an access occasion based on the A-IOT device’s energy level should be studied.

If above proposal is agreeable, how to consider the A-IOT device’s energy level to select the access occasion should be discussed. We think that the simple way is to divide the access occasions into two groups. One group is to use the A-IOT devices having not enough energy and the other group is used to use the A-IOT devices having enough energy. Using this group information, the A-IOT device can select a group based on the A-IOT device’s energy level. Then, the A-IOT device selects one access occasion associated with the selected group. Note that the following figure shows an example for dividing access occasion into two groups.

Proposal 11. The access occasions are divided into two groups, and the A-IOT device selects one group based on the A-IOT device’s energy level.

    - CFRA

In RAN2#125bis meeting, it was agreed that we will study the support for access triggering for a single device, group of devices, or all devices. For the single device case, the RA procedure only for a single A-IOT device would not need CBRA procedure, and the CFRA procedure would be simple and appropriate because the reader requests the performing random access procedure only for a single A-IOT device.

Proposal 12. For random access procedure from a single A-IOT device, the CFRA is used as a baseline.

In addition, in the RAN2#127bis meeting, the following agreements were made for containing multiple A-IOT device IDs in A-IOT paging message

3 From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs. RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs. 4 If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

As stated above, A-IOT paging message can include multiple A-IOT device IDs. In addition, the reader can indicate whether to perform the CFRA or CBRA.

For CBRA case with multiple A-IOT device IDs, it is clear that the A-IOT device randomly selects one access occasion within the access occasion provided by reader. However, for CFRA case with multiple A-IOT device IDs, how to assign the dedicated access occasion for each A-IOT device should be discussed.

We think that the straightforward solution is that if the A-IOT paging message includes multiple A-IOT device IDs, the CFRA can be used based on the order of the A-IOT device IDs. For example, if the reader transmits the A-IOT paging message containing the first A-IOT device ID and the second A-IOT device ID, only first A-IOT device and the second A-IOT device perform the RA procedure. In this case, if the first and second A-IOT devices select an access occasion according to the order of the A-IOT device ID, the A-IOT device can perform the CFRA procedure. The following figure shows an example for this case.

Proposal 13. When the A-IOT paging message contains multiple A-IOT device IDs, the A-IOT device selects the access occasion according to the order of the A-IOT device ID and each A-IOT device perform the CFRA procedure.

Conclusion

Based on the above discussion, we made following proposals and observations.

Proposal 1. A new message is introduced to indicate the time domain resource for Msg1 transmission.

Proposal 2. The definition of paging round should be captured in TR as follows

- Paging round: The paging round is initiated by A-IOT paging message. One paging round consists one or multiple access occasions. Within one paging round, one A-IOT device has an opportunity to perform the random access procedure for one access occasion.

Proposal 3. The additional indication is not introduced for extension of the access occasions within a paging round.

Proposal 4. The A-IOT device always performs the re-access procedure when the failure feedback indication is received even after the contention resolution is completed.

Proposal 5. The success feedback indication is transmitted only when there is no more subsequent message to the A-IOT device. Then, the A-IOT device considers that all procedures are successfully completed when the success feedback is indicated.

Proposal 6. The A-IOT device considers that the D2R transmission is successfully transmitted if there is no success/failure feedback indication.

Observation 1. Since there is not much difference between the 3-step and 2-step CBRA procedure, it would be better to down-select only one CBRA procedure for A-IOT to simplify the random access procedure in A-IOT.

Proposal 7. Only 2-step CBRA procedure should be introduced for A-IOT.

Proposal 8. For CFRA, a random ID is contained in Msg1 for the purpose of D2R scheduling and R2D reception.

Proposal 9. For CFRA, the A-IOT device considers that the contention resolution is successfully completed after transmitting the Msg1 containing the A-IOT device ID and random ID.

Proposal 10. How to select an access occasion based on the A-IOT device’s energy level should be studied.

Proposal 11. The access occasions are divided into two groups, and the A-IOT device selects one group based on the A-IOT device’s energy level.

Proposal 12. For random access procedure from a single A-IOT device, the CFRA is used as a baseline.

Proposal 13. When the A-IOT paging message contains multiple A-IOT device IDs, the A-IOT device selects the access occasion according to the order of the A-IOT device ID and each A-IOT device perform the CFRA procedure.