3GPP TSG-RAN WG2 Meeting #128	R2-2409769

Orlando, USA, 18th – 22nd Nov. 2024

Source:	vivo

Title:	Remaining issues on LCP enhancements for XR

Agenda Item:	8.7.4.1

Document for:	Discussion and Decision

- Introduction

In RAN2#127 [1], there were the following agreements regarding LCP enhancements:

RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

In RAN2#127bis [2], there were the following agreements on LCP enhancements:

Agreements on LCP enhancements - As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation - Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. - We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

In this contribution, the remaining issues of LCH priority enhancements are discussed based on the above agreements.

- Discussion

Regarding the FFS below:

FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation

If there are remaining resources after the first round of radio resource allocation based on Bj’s of all LCHs including delay-critical LCH, it means that all LCHs, including low priority LCHs, have already got the minimum required radio resources according to their QoS requirements (i.e. Bj) respectively in the first round of LCP procedure. In this condition, the motivation to restore the priority of delay-critical LCH from additional priority to the original priority to ensure the fairness for high/low priority LCHs is weak.

- When there is the second round of LCP procedure, all LCHs, including the low priority ones, have already been allocated with the minimum required resources according to the respective Bj’s, i.e. in this condition, the required QoS for services over low priority LCHs is expected to be ensured via the first round of LCP procedure.

If the UE is required to adjust the delay-critical LCH’s priority from the additional one to the original one for the second round of radio resource allocation, it means that there is another round of ordering LCHs considering the LCH priority restoration of the delay-critical LCH. This will increase the complexity of the UE but the expected benefit is marginal considering all LCHs have already got the required minimum amount of radio resources in the first round of radio resource allocation.

- If LCH priority fallback for delay-critical LCH is supported for the 2nd round of LCP procedure, the LCHs must be reordered by UE for resource allocation after the LCP priority fallback of the delay-critical LCH.

In RAN2#126, RAN2 has already agreed to control the UE complexity:

The solution should consider impact on UE complexity (as already indicated in SI objective description)

In RAN2#127bis, RAN2 reminded to consider the UE complexity and the tight timeline of LCP procedure in the UE side:

- FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation

- RAN2 agreed and confirmed to restrict the UE complexity for LCP enhancement.

With this, we think once the additional priority is determined for an LCH, the additional priority should be applied to all data of the LCH, including both delay critical data and non-delay critical data.

- The additional priority determined for a delay-critical LCH before the first round of LCP procedure is also used for the second round of LCP procedure, i.e. no LCH priority fallback for delay-critical LCH is supported for the second round of LCP procedure.

For a first LCH with delay-critical data, the gNB may issue UL grants to speed up the delay-critical data transmissions. After receiving the UL grants, the UE may prioritize the resource allocation for the first LCH according to the additional priority of this LCH. However, when the UE has a large amount of delay-critical data for the first LCH, after the UE has allocated radio resource up to Bj for the first LCH, there are still remaining delay-critical data for the first LCH. According to the current LCP procedure, the UE will allocate the remaining resource for the second LCH of lower priority. After radio resource allocation for the second LCH, there may be no or no enough remaining capacity to transmit all the remaining delay-critical data of the first LCH.

In another instance, when there can be certain scheduling restriction, e.g. due to measurement gap (MG) overlapping with the PDB/PSDB window of a frame burst, the actual available time period(s) for transmission of the frame burst after DSR triggering could be further reduced due to the overlapping between the MG and the remaining time period. In this case, even when the UE gets enough UL grants providing enough capacity for the delay-critical data of a LCH, the UE may not be able to include all delay-critical data of this LCH into the corresponding MAC PDUs due to the restriction of the Bj of this LCH and some capacity of the UL grants intending for the delay-critical data of this LCH are allocated to transmit the non-delay-critical data of other LCHs instead according to the existing multiplexing mechanism.

- For MAC PDU construction, the LCH with delay-critical data may not be able to be allocated enough radio resource due to its buckets (Bj) exhausted.

To speed up the delay-critical data transmission, the UE should be temporarily allowed to allocate radio resources exceeding the rate limit (Bj limit).

- In addition to LCH priority adaptation, RAN2 should also consider temporarily allowing UE to raise the rate limit (e.g. Bj) for delay-critical LCH.

Yet there is another issue on whether or how to handle the additional priority impact on the intra-UE prioritization. It is well known that intra-UE prioritization was specified to prioritize the transmission of URLLC service which is specific for industry wireless. It is doubtful whether it is a reasonable assumption that one UE could have concurrent URLLC service and XR service. Furthermore, we need also to think about whether the gNB can ensure desired intra-UE prioritization procedure without specification impact even if one UE “could” have the concurrent XR service and the URLLC service. When one UE “occasionally” has concurrent URLLC service and XR service, the gNB should be able to ensure that both the additional LCH priority and the original LCH priority for delay-critical LCHs carrying XR service data should be lower than that of the URLLC service data. With this assumption, the URLLC service data transmission would not be blocked by the additional LCH priority of XR data due to the URLLC service data always has higher priority than the XR service data no matter whether the original LCH priority or additional priority is applied for the LCH carrying the XR service data.

- Intra-UE prioritization was initially for ensuring URLLC data transmission while it is less likely that one UE has concurrent URLLC service and XR service.

- Even if one XR UE has concurrent URLLC service and XR service, by proper additional LCH priority configuration, gNB can ensure that URLLC service data always has higher priority than the XR service data, regardless of whether the additional LCH priority or the original LCH priority is applied for the XR data.

Based on the above assumption, one concluded that no special enhancement is required for the intra-UE prioritization procedure due to introducing the additional LCH priority:

- No enhancement is needed for intra-UE prioritization procedure due to introducing the additional LCH priority.

- Conclusion

In this contribution, we have discussed the LCP enhancements and potential solutions. Based on the discussions, there are the following observations:

Observation 1	When there is the second round of LCP procedure, all LCHs, including the low priority ones, have already been allocated with the minimum required resources according to the respective Bj’s, i.e. in this condition, the required QoS for services over low priority LCHs is expected to be ensured via the first round of LCP procedure.

Observation 2	If LCH priority fallback for delay-critical LCH is supported for the 2nd round of LCP procedure, the LCHs must be reordered by UE for resource allocation after the LCP priority fallback of the delay-critical LCH.

Observation 3	RAN2 agreed and confirmed to restrict the UE complexity for LCP enhancement.

Observation 4	For MAC PDU construction, the LCH with delay-critical data may not be able to be allocated enough radio resource due to its buckets (Bj) exhausted.

Observation 5	Intra-UE prioritization was initially for ensuring URLLC data transmission while it is less likely that one UE has concurrent URLLC service and XR service.

Observation 6	Even if one XR UE has concurrent URLLC service and XR service, by proper additional LCH priority configuration, gNB can ensure that URLLC service data always has higher priority than the XR service data, regardless of whether the additional LCH priority or the original LCH priority is applied for the XR data.

Based on the above discussions and the observations, we have the following proposals:

Proposal 1	The additional priority determined for a delay-critical LCH before the first round of LCP procedure is also used for the second round of LCP procedure, i.e. no LCH priority fallback for delay-critical LCH is supported for the second round of LCP procedure.

Proposal 2	In addition to LCH priority adaptation, RAN2 should also consider temporarily allowing UE to raise the rate limit (e.g. Bj) for delay-critical LCH.

Proposal 3	No enhancement is needed for intra-UE prioritization procedure due to introducing the additional LCH priority.

- References

- RAN2-127, Chair notes

- RAN2-127bis, Chair notes