3GPP TSG-RAN WG2 Meeting #128	R2-2409770

Orlando, USA, 18th – 22nd Nov. 2024

Source:	vivo

Title:	Remaining issues on DSR enhancements for XR

Agenda Item:	8.7.4.2

Document for:	Discussion and Decision

- Introduction

In RAN2#127bis [1], there are following agreements regarding DSR enhancements:

Agreements on DSR enhancements - We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In this contribution, the remaining issues on DSR are discussed with respect to multiple DSR reporting thresholds, comprising restriction on DSR triggering threshold, DSR triggering, DSR cancellation and DSR format based on the conclusions from the previous meetings.

- Discussion

  - DSR triggering/reporting threshold(s) configuration

In RAN2#127bis, whether the restriction on DSR triggering threshold configuration for an LCG configured with multiple DSR reporting thresholds was discussed. According to the contributions in the meeting, the solutions can be categorized into the following 3 options for DSR triggering threshold configuration for one LCG configured with multiple DSR reporting thresholds:

- Option 1: DSR triggering threshold should be set to the maximum DSR reporting threshold.

- Option 2: No restriction for DSR triggering threshold configuration.

- Option 3: DSR triggering threshold can be set to any one of the configured DSR reporting thresholds.

Among these 3 options, it can be observed that Option 1 and Option 3 are the special cases for Option 2, and Option 1 and Option 3 could be achieved if Option 2 is introduced.

- With Option 2, the gNB has flexibility to configure DSR triggering threshold to any desired value, i.e. Option 1 and Option 3 could also be achieved by Option 2.

As usual, smart gNB implementation can be assumed and the gNB can determine a proper DSR triggering threshold configuration for one LCG configured with multiple reporting thresholds, i.e. it is not necessary to restrict the gNB configuration on the reporting thresholds for one LCG configured with multiple thresholds, i.e. the reporting thresholds could be either higher or lower than the trigger threhsold.

- DSR reporting threshold(s) for one LCG are configured by gNB, wherein the DSR reporting threshold(s) can be equal to, higher or lower than the DSR triggering threshold.

In the following, we will analyse the corresponding impacts of setting the triggering threshold to be lower than largest reporting threshold on DSR procedure, including the DSR triggering, cancellation etc.

  - DSR triggering

According to the current MAC specification, as below, DSR triggering in R18 is based on the remainingTimeThreshold configured by network.

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG: 1> if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and 1> if there is no DSR pending for the logical channel: 2> trigger a DSR for the logical channel.

- In R18, DSR is triggered for an LCH when any buffered data (of the LCH), which has not been reported as data volume in DSR MAC CE, has become to delay-critical data.

When multiple DSR reporting thresholds are configured for an LCG and there is no restriction on the DSR triggering threshold configuration, some reporting threshold(s) may be lower than the triggering threshold while some other reporting threshold(s) may be higher than the triggering threshold. Besides, RAN2 has agreed that we do not change the definition of delay-critical data. In this way, some delay status pairs may indicate the data volumes for non-delay-critical data (i.e. with remaining time higher than the trigger threshold) while some other delay status pairs may indicate the data volumes for delay-critical data (i.e. with remaining time lower than the trigger threshold). Then there is one issue on whether DSR should be triggered when some data which has been reported as non-delay-critical data volume becomes delay-critical data later. This issue was mentioned but not sufficiently discussed in RAN2#127bis.

- RAN2 needs to address the issue that whether a DSR should be triggered when any data already reported in the DSR MAC CE as non-delay-critical data volume becomes delay-critical data later on.

When some data has been reported as non-delay-critical data via a certain portion/pair for a LCG in a DSR MAC CE, the gNB can derive the remaining time for the non-delay-critical data for each portion/pair for reporting non-delay-critical data volume. The gNB can determine whether the non-delay-critical data for the LCG has become delay-critical data or not and performs scheduling accordingly without subsequent DSR from the UE. In such a sense, subsequent DSR triggering, due to that any data which has already been reported as non-delay-critical data volume in a DSR becomes delay-critical data, is meaningless.

- Reasonable gNB implementation should be able to derive whether the data reported as non-delay-critical data volume has become delay-critical data or not.

Based on the above discussion, we can conclude:

- No DSR triggering is allowed due to that a data reported as non-delay critical data volume previously has become delay-critical data later on.

- The DSR triggering condition specified in R18 MAC specification should be reused, i.e., no specification change is required for DSR triggering condition.

  - DSR cancellation

In R18, the DSR for an LCG can only be cancelled if all delay-critical data of the LCG is discarded or transmitted or the DSR MAC CE comprising delay status of the LCG has been transmitted. There is no motivation to change these principles for R19 DSR enhancement, i.e. the DSR cancellation rules for R18 DSR should be reused. In other words, for an LCG configured with multiple reporting thresholds, the DSR for the LCG is allowed to the reported only when there is at least one delay status pair for delay-critical data report among the multiple data status pairs for the LCG.

- R18 DSR cancellation principles are reused for R19 DSR enhancement, i.e. R19 DSR can be cancelled in any of the following conditions:

  - All delay-critical data has been discarded;

  - All delay-critical data has been transmitted;

  - The delay status of all delay-critical data has been reported.

  - DSR format

In RAN2#127bis, there are the following agreements regarding enhanced DSR format:

- A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG.

It is not clear yet how the one-bit indication is placed in the enhanced DSR format. There are two options:

- Option A: one bitmap per LCG to indicate the presence of the pairs/portions

For this option, for an LCG configured with multiple reporting thresholds, a pair bitmap is used to indicate the presence of pairs when any pair of this LCG is reported, wherein the pair bitmap length is equal to the maximum number of pairs configured for this LCG and each bit of the pair bitmap corresponds to one pair. For an LCG configured with single reporting threshold or only configured with DSR triggering threshold, no pair bitmap is needed. Figure 1 shows one example for R19 DSR format of Option A, wherein LCG 7, 5 and 4 are configured with multiple reporting thresholds and each of them can have more than one pair to be reported. Please be noted that LCGi pair bitmap does not present if LCGi has no delay status to be reported.

Figure 1 Example of R19 DSR format with one pair presence bitmap per LCG

- Option B: One extension bit per pair to indicate the presence of next pair of one LCG.

For this option, the R bit in each pair of remaining time/buffer size in legacy DSR format (see Figure 6.1.3.72-1 below) is redefined as extension bit to indicate if the next pair corresponds to the same LCG as the current pair or not. Figure 2 shows one example for R19 DSR format of Option B, wherein the extension bit value 1 or 0 indicates the next pair and the current pair belongs to the same or different LCG respectively. In this example, LCG 7, 5 and 4 are configured with multiple DSR reporting thresholds and respectively have 2 pairs, 2 pairs and 1 pair of remaining time/buffer sizes to be reported. By redefining the R bits, R19 DSR format has a relatively smaller size than that according to Option A as Option A requires a dedicated pair bitmap per LCG.

Figure 6.1.3.72-1: DSR MAC CE

Figure 2 Example R19 DSR format with one extension bit per pair

- RAN2 to down-select one of the following options for R19 DSR format design:

  - Option A: One bitmap per LCG to indicate the presence of the pairs;

  - Option B: One extension bit (e.g. redefining R bit in R18 DSR) per pair to indicate the presence of the next pair of the same LCG.

Then for a UE configured with multiple reporting thresholds for at least one LCG, two DSR formats are available: one is legacy DSR format and the other is R19 DSR format. When any LCG has multiple pairs to be reported, R19 DSR format should be adopted regardless the R19 DSR format is designed based on pair bitmap per LCG or extension bit per pair.

- When any LCG configured with multiple pairs of remaining time/buffer sizes has multiple pair remaining time/buffer sizes to be reported, new DSR format in R19 should be used.

However, when each LCG has at most one pair to be reported, there is an issue to select legacy DSR format or R19 DSR format. For R19 DSR format designed based on pair bitmap (Option A), when one LCG configured with multiple reporting thresholds actually has single pair to be reported upon DSR generation, the corresponding pair bitmap should be included in the R19 DSR, which results in that in this case DSR size based on R19 DSR format is relatively larger than that based on legacy DSR. For R19 DSR format designed based on pair extension bit (Option A), the DSR size based on R19 DSR format and legacy DSR format is equal.

No matter what option is adopted, when each LCG has at most one pair of remaining time/buffer size to be reported upon DSR generation, there is no loss to allow the UE to always use the legacy DSR format:

- When each LCG has at most one pair of remaining time/buffer size to be reported, legacy DSR format should be selected.

- Conclusion

In this contribution, we have discussed the DSR enhancements and potential solutions. Based on the discussions, there are the following observations:

Observation 1	With Option 2, the gNB has flexibility to configure DSR triggering threshold to any desired value, i.e. Option 1 and Option 3 could also be achieved by Option 2.

Observation 2	In R18, DSR is triggered for an LCH when any buffered data (of the LCH), which has not been reported as data volume in DSR MAC CE, has become to delay-critical data.

Observation 3	RAN2 needs to address the issue that whether a DSR should be triggered when any data already reported in the DSR MAC CE as non-delay-critical data volume becomes delay-critical data later on.

Observation 4	Reasonable gNB implementation should be able to derive whether the data reported as non-delay-critical data volume has become delay-critical data or not.

Based on the above discussions and the observations, we have the following proposals:

Proposal 1	DSR reporting threshold(s) for one LCG are configured by gNB, wherein the DSR reporting threshold(s) can be equal to, higher or lower than the DSR triggering threshold.

Proposal 2	No DSR triggering is allowed due to that a data reported as non-delay critical data volume previously has become delay-critical data later on.

Proposal 3	The DSR triggering condition specified in R18 MAC specification should be reused, i.e., no specification change is required for DSR triggering condition.

Proposal 4	R18 DSR cancellation principles are reused for R19 DSR enhancement, i.e. R19 DSR can be cancelled in any of the following conditions:

-	All delay-critical data has been discarded;

-	All delay-critical data has been transmitted;

-	The delay status of all delay-critical data has been reported.

Proposal 5	RAN2 to down-select one of the following options for R19 DSR format design:

-	Option A: One bitmap per LCG to indicate the presence of the pairs;

-	Option B: One extension bit (e.g. redefining R bit in R18 DSR) per pair to indicate the presence of the next pair of the same LCG.

Proposal 6	When any LCG configured with multiple pairs of remaining time/buffer sizes has multiple pair remaining time/buffer sizes to be reported, new DSR format in R19 should be used.

Proposal 7	When each LCG has at most one pair of remaining time/buffer size to be reported, legacy DSR format should be selected.

- References

- RAN2-127bis, Chair notes