3GPP TSG-RAN WG2 Meeting #128		R2-2409771

Orlando, USA, 18th - 22nd Nov. 2024

Source:	vivo

Title:	Discussion on RLC enhancement for XR

Agenda Item:	8.7.5

Document for:	Discussion and Decision

- Introduction

In RAN2#127bis, the following agreements were made for RLC enhancement [1]:

Agreements on RLC timely retransmissions - RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. - Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay Agreements on avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

In this paper, we will discuss the enhancements to ensure timely RLC retransmission and to avoid unnecessary retransmission.

- Discussion

  - Enhancements to ensure timely RLC retransmissions

- Approach 1: Autonomous retransmission

Autonomous retransmission is a kind of proactive retransmission, allowing the transmitting AM RLC entity to autonomously perform RLC retransmissions without waiting for the RLC NACK feedback. In this approach, the transmitting AM RLC entity may decide to perform blind retransmissions based on traffic needs (e.g., the delay budget of the RLC SDU) or the HARQ feedback from the physical layer (e.g., the number of HARQ retransmission failures).

Impact on system capacity

In XR, one of the common cases of late PDUs is congestion. Thus, the congestion scenario is a primary scenario that requires optimization. In a congested scenario, autonomous retransmission may have a negative impact on the system and worsen the congestion, as the transmitting RLC entity may perform unnecessary RLC retransmission, leading to the wastage of radio resources, even if the packet has already been successfully transmitted.

Observation 1: Autonomous retransmission may bring much unnecessary retransmission and make the congestion worse.

Impact on packet delay

As discussed, autonomous retransmission would lead to a significant waste of radio resources waste by unnecessarily retransmitting delay-critical packets. This is not only inefficient but also may revert the subsequent non-delay-critical packets into delay-critical packets due to lack of scheduling resources.

Observation 2: Autonomous retransmission would delay the transmission of the subsequent packets, would potentially lead more packets as delay-critical data from non-delay-critical data in congestion scenario.

Furthermore, if autonomous retransmission relies on HARQ feedback, the network can perform DL RLC autonomous retransmission accordingly. However, in UL scenarios, there is no HARQ feedback and the UE is unable to determine whether the HARQ transmission was successful or the network just gives up the HARQ retransmission when the NDI is toggled. As a result, it is not feasible for the UE to perform UL autonomous RLC retransmissions based on the HARQ feedback.

Observation 3: Autonomous retransmission based on UL HARQ feedback is not feasible according to the existing HARQ mechanism.

Considering the negative effects of autonomous retransmission on capacity and packet delay, as well as the relatively large impact on specifications. We think there is no need to support the autonomous retransmission in Rel-19.

Proposal 1: Autonomous retransmission is not supported in Rel-19.

- Approach 2: Polling enhancement

Compared to autonomous retransmission, polling enhancement can ensure timely retransmission while avoiding the resource waste of unnecessary retransmission, because RLC retransmission is triggered by NACK feedback from STATUS PDUs. Some companies believe that autonomous retransmission is faster than retransmission based on STATUS report because it initiates retransmission without waiting for the STATUS report. However, network implementation can adjust the threshold for applying polling enhancement to trigger the STATUS report earlier thus ensuring that retransmissions can be initiated on time.

Observation 4: Compared with autonomous retransmission, polling enhancement can ensure timely retransmission and avoid the resources waste of unnecessary retransmission in autonomous retransmission.

As for the details of polling enhancement, the triggering of polling mainly includes the following four cases in the current RLC specification [2] as below:

- Case 1: PDU_WITHOUT_POLL >= pollPDU or BYTE_WITHOUT_POLL >= pollByte

- Case 2: Buffers become empty

- Case 3: No data to transmit due to window stalling

Case 4: t-PollRetransmit expires

TS 38.322

5.3.3.2	Transmission of a AMD PDU

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte:

-	include a poll in the AMD PDU as described below.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or

-	if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

-	include a poll in the AMD PDU as described below.

…

5.3.3.4	Expiry of t-PollRetransmit

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

-	if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling):

-	consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or

-	consider any RLC SDU which has not been positively acknowledged for retransmission.

-	include a poll in an AMD PDU as described in clause 5.3.3.2.

TS 38.322

5.3.3.2	Transmission of a AMD PDU

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte:

-	include a poll in the AMD PDU as described below.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or

-	if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

-	include a poll in the AMD PDU as described below.

…

5.3.3.4	Expiry of t-PollRetransmit

Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

-	if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling):

-	consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or

-	consider any RLC SDU which has not been positively acknowledged for retransmission.

-	include a poll in an AMD PDU as described in clause 5.3.3.2.

In our view, there are mainly two alternatives for enhancing polling to ensure timely retransmission of delay-critical SDUs.

- Alt 1: The UE triggers polling when the remaining time of RLC SDU(s) falls below a threshold

- Alt 2: The network configures dedicated polling thresholds and polling retransmitting timer for delay-critical RLC SDUs

For Alt 1, a new type of polling trigger mechanism is introduced. When the remaining time of an RLC SDU falls below a threshold configured by the network, the UE will trigger polling to obtain a fast status report from the network regarding the status of the delay-critical RLC SDU. Additionally, for Alt 1, it may be necessary to introduce a prohibit timer for this polling trigger to avoid excessively frequent polling. This is particularly important in cases where there are many delay-critical RLC SDUs or when the UE does not receive timely status feedback from the network. Otherwise, the UE will continue to trigger polling until there are no more delay-critical RLC SDUs.

For Alt 2, the legacy polling mechanism is reused as much as possible (i.e., the case 1 and case 4). In Alt 2, dedicated polling thresholds and retransmission timer for delay-critical RLC SDUs are introduced. When there are delay-critical RLC SDUs, the UE will utilize these dedicated polling counters and retransmission timer for making polling triggering decisions. Once all the delay-critical RLC SDUs have been transmitted or discarded, the UE will revert to using the legacy polling thresholds and retransmission timer.

From our perspective, Alt 2 reuses the legacy polling mechanism as much as possible and is more flexible, since the threshold and timer values are configured by the network can ensure timely retransmission for delay-critical SDUs while also avoiding excessive signaling overhead due to polling.

Proposal 2: Dedicated polling thresholds and polling retransmission timer are introduced for delay-critical RLC SDUs.

Comparison﻿ between Solution 1 and Solution 2:

The pros and cons of the above solutions for ensuring timely RLC retransmission(s) are summarized in the following table:

Table 1: Pros and Cons of different approaches to ensure timely RLC retransmission(s)

| Approach description | Pros | Cons |
| --- | --- | --- |
| Autonomous retransmission | - Fast RLC retransmission can be achieved | - May introduce unnecessary retransmission and make the congestion worse - Unnecessary retransmission for delay-critical SDUs may make the following non-delay-critical SDUs become delay-critical SDUs and improve the packet delay - Retransmission based on the HARQ feedback is not feasible for UL - Large spec impact when introducing a new retransmission mechanism |
| Polling enhancement Alt 1: UE triggers polling when the remaining time of RLC SDU(s) is below a threshold | - Fast RLC retransmission can be achieved - Unnecessary retransmission will not happen | - New polling triggering mechanism needs to be introduced. - Prohibit timer may need to be introduced to avoid frequent polling. |
| Polling enhancement Alt 2: Network configures dedicated polling thresholds and polling retransmission timer for delay-critical RLC SDUs | - Fast RLC retransmission can be achieved - Unnecessary retransmission will not happen - Flexible since the counters threshold and timer value can be configured by the network by considering the delay and the signaling overhead for polling | - Dedicated polling counters and timer need to be introduced for delay-critical RLC SDUs. |

  - Enhancements to avoid unnecessary retransmissions

In the last RAN2 meeting, combined approach for avoiding unnecessary retransmission is agreed:

RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard

In our view, the following issues need to be discussed regarding the combined approach:

Issue #1: How the Tx side determines which SDUs are outdated

The first issue concerns how the Tx side decides which RLC SDUs are outdated and should no longer be retransmitted. According to TS 38.323, PDCP first performs SDU discard based on whether the discardTimer or discardTimerForLowImportance has expired for a PDCP SDU. If the corresponding PDCP Data PDU has already been submitted to lower layers, then the transmitting PDCP entity will send a discard indication to RLC.

TS 38.323

5.3	SDU discard

When the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP Data PDU.

When the discardTimer or discardTimerForLowImportance expires for a PDCP SDU, the transmitting PDCP entity shall:

-	if pdu-SetDiscard is configured:

-	discard all PDCP SDUs belonging to the PDU Set to which the PDCP SDU belongs along with the corresponding PDCP Data PDUs;

NOTE 1:	PDCP SDUs subsequently received from upper layers are also discarded if they belong to the PDU Set.

-	else:

-	discard the PDCP SDU along with the corresponding PDCP Data PDU.

If the corresponding PDCP Data PDU has already been submitted to lower layers, the discard is indicated to lower layers.

In our view, the discard indication from PDCP to RLC can be reused by the RLC Tx side to identify outdated SDUs.

Proposal 3: The Tx side of RLC determines outdated SDU(s) based on the discard indication from the Tx side of PDCP.

Issue #2: How the local timer at the Rx side functions

The second issue concerns the operation of the local timer at the Rx side. Document R2-2407986 [3] has discussed two approaches for the Rx timer:

- Approach 1: per RLC SDU timer

- Approach 2: per RLC entity timer, similar to the t-Reassembly timer

For approach 1, each RLC SDU will maintain its own timer, which may start under the following two cases as illustrated in Figure 1:

- Case 1:  A segment of the corresponding RLC SDU is received, but not all the bytes of the RLC SDU have been received.

- Case 2: An RLC SDU or an RLC SDU segment with a higher SN than that of the concerned RLC SDU is received.

Figure 1. Per RLC SDU Rx timer starting occasion

When the Rx timer for the corresponding RLC SDU expires, the Rx side of the RLC entity treats the corresponding RLC SDU as ACK and updates the Rx window. Subsequently, the Rx side informs the Tx side by including fake ACK in the STATUS report, in accordance with legacy behavior. The Rx timer for the corresponding RLC SDU stops when the RLC SDU is fully received.

For approach 2, the Rx timer is maintained on a per RLC entity basis, which means that multiple RLC SDUs may be associated with a single timer, and only one timer is active for the RLC entity at any given time. The conditions for starting and stopping the new Rx timer can be the same as those for t-Reassembly. The only difference between the new Rx timer and t-Reassembly lies in the actions taken when the timer expires. When t-Reassembly expires, it triggers a STATUS report; however, when the new Rx timer expires, it results in the abandonment of SDUs and an update of the Rx window.

Both approaches can work well, and RAN2 should evaluate the pros and cons of each approach and select the most suitable method for determining which SDUs are to be abandoned at the Rx side.

Proposal 4: RAN2 discusses the following two options for local timer at the Rx side to decide the abandoned SDU(s).

- Option 1: Per RLC SDU timer

- Option 2: Per RLC entity timer, similar to the t-Reassembly timer

Issue #3: Whether special handling for PDCP control PDU is needed

In the last RAN2 meeting, the handling of PDCP control PDUs on the Rx side was discussed for both the combined approach and the Rx-initiated approach. It was noted that the RLC entity's receiving side is unable to distinguish between PDCP control PDUs and PDCP data PDUs. Consequently, when the local timer at Rx side mentioned in Issue #2 expires, there is a risk that RLC may inadvertently abandon PDCP control PDUs, making the PDCP layer cannot receive the PDCP control PDUs.

PDCP control PDUs may carry crucial compression/decompression information, which affects the processing of subsequent PDCP data PDUs. Therefore, their loss could potentially disrupt the normal reception of PDCP data PDUs.

In case where PDCP is configured with outOfOrderDelivery, since there is no requirement of in-sequence delivery, the compression mechanism may not be used and there may not be much impact for discarding of PDCP control PDU at the RLC level. However, for PDCP configurations requiring in-sequence delivery, it is essential to prevent such discard at the RLC layer.

To resolve this issue, one solution is to configure a relatively long Rx timer at the RLC's Rx side (for example, setting the Rx timer longer than the t-Reordering timer in PDCP) to minimize the chances of discarding PDCP control PDUs before the PDCP layer decides to abandon them.

Proposal 5: The local timer at the Rx side should be configured with a sufficiently long value (for instance, at least longer than the t-Reordering timer in PDCP) to prevent the early discard of PDCP control PDUs within the RLC layer.

Issue #4: Whether indication is sent from Tx to Rx when the Tx decides the outdated SDU(s)

As discussed in Issue #3 above, it is recommended that the Rx timer be set to a relatively long value (e.g., exceeding the length of the t-Reordering timer in PDCP) to prevent the unintentional discard of PDCP control PDUs by RLC. However, such a long Rx timer length might not align with the traffic PDB/PSDB requirements and could potentially delay the transmission of subsequent packets on the Tx side. This delay may occur if the Tx side has already stopped (re)transmission for the outdated SDU(s) but does not receive a STATUS report from Rx side for a long time and the transmitting window stalled.

Therefore, it is essential that when the Tx side determines that SDU(s) are outdated, especially for the service with critical latency requirements, it should send an indication to the Rx side. This would enable the Rx to halt any active Rx timers, issue a STATUS report to Tx, and prompt Tx to update its transmission window in a timely manner. This process is critical to ensure the prompt transmission of subsequent packets.

Proposal 6: Indication about obsolete SDUs from Tx side to Rx side is needed for “combined” approach for avoiding unnecessary RLC retransmissions. Timer running at Rx side should stop based on the indication from Tx side.

- Conclusion

In this contribution, we discuss the two directions of RLC AM enhancements, and we have the following observations and proposals:

Enhancements to ensure timely RLC retransmission(s)

Observation 1: Autonomous retransmission may bring much unnecessary retransmission and make the congestion worse.

Observation 2: Autonomous retransmission would delay the transmission of the subsequent packets, would potentially lead more packets as delay-critical data from non-delay-critical data in congestion scenario.

Observation 3: Autonomous retransmission based on UL HARQ feedback is not feasible according to the existing HARQ mechanism.

Observation 4: Compared with autonomous retransmission, polling enhancement can ensure timely retransmission and avoid the resources waste of unnecessary retransmission in autonomous retransmission.

Proposal 1: Autonomous retransmission is not supported in Rel-19.

Proposal 2: Dedicated polling thresholds and polling retransmission timer are introduced for delay-critical RLC SDUs.

Enhancements to avoid unnecessary retransmissions

Proposal 3: The Tx side of RLC determines outdated SDU(s) based on the discard indication from the Tx side of PDCP.

Proposal 4: RAN2 discusses the following two options for local timer at the Rx side to decide the abandoned SDU(s).

- Option 1: Per RLC SDU timer

- Option 2: Per RLC entity timer, similar to the t-Reassembly timer

Proposal 5: The local timer at the Rx side should be configured with a sufficiently long value (for instance, at least longer than the t-Reordering timer in PDCP) to prevent the early discard of PDCP control PDUs within the RLC layer.

Proposal 6: Indication about obsolete SDUs from Tx side to Rx side is needed for “combined” approach for avoiding unnecessary RLC retransmissions. Timer running at Rx side should stop based on the indication from Tx side.

- References

- R2_127bis_ChairNotes.

- TS 38.322 v18.1.0, Radio Link Control (RLC) protocol specification (Release 18).

- R2-2407986, Discussion on RLC AM enhancements. Huawei, HiSilicon.