3GPP TSG-RAN WG2 Meeting #128	R2-2409793

Orlando, FL, US, 18 –22 Nov 2024

Agenda Item:	8.11.2

Source:	NEC

Title:	Selection of SBFD Cell

Document for:	Discussion, Decision

# Introduction

In RAN2#127bis meeting, the following agreements were reached for the random access procedure for SBFD operation:

- RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact.

- From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

- Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

- For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

- The following two RACH configuration options are considered for SBFD based random access:

Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed.

Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration

- For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters.

- The RACH configuration for SBFD is transmitted via SIB1.

- FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

This paper intends to discuss the cell (re)selection aspects for SBFD cells.

# Discussion

## Cell Selection and Reselection of SBFD Cell

In the last RAN2 meeting, it is agreed that the RACH configuration for SBFD is transmitted via SIB1. This means that the UE can identify if the cell is a SBFD cell via reading its SIB1.

In a practical deployment, we believe that both SBFD cell and non-SBFD cell should be able to co-exist within the mobile network. In order to take the full advantage of Rel-19 SBFD feature, Rel-19 SBFD-aware UEs should stay at SBFD cell(s) as much as possible, otherwise, the SBFD feature will be rarely utilized.

To support Rel-19 SBFD-aware UEs to stay at SBFD cell(s), the legacy cell selection and reselection process needs to be enhanced. As one of the example, a priority can be configured for the SBFD cell(s).

Proposal-1: Rel-19 SBFD-aware UEs should camp on SBFD cell(s) as much as possible.

Proposal-2: Introduce priority based SBFD cell camping for Rel-19 SBFD-aware UEs during its cell selection and reselection process.

Pre Rel-19 UE or non SBFD-aware Rel-19 UE may be better to stay in the non-SBFD cells during cell selection and reselection process. Allowing these UEs to camp on SBFD cells will downgrade the whole cell throughput and may impact the UE performance hereafter.

RAN2 can discuss if it is practical to bar these UEs by SBFD cells to prevent them from camping on SBFD cells.

Proposal-3: RAN2 to discuss if a barring mechanism is introduced to prevent Pre Rel-19 UE or non SBFD-aware Rel-19 UE from camping on SBFD cells.

## Consideration on handover

When SBFD-aware Rel-19 UE is served by SBFD cell, it would be beneficial for the UE to move to another target SBFD cell during handover cell, which can maximize the scheduling gain for that UE for uplink. RAN2 can discuss if and how the SBFD-aware Rel-19 UE prioritize the selection of a SBFD cell as the handover target cell during handover preparation.

Proposal-4: RAN2 to discuss if and how the SBFD-aware Rel-19 UE prioritize the selection of a SBFD cell as the handover target cell during handover preparation.

# Conclusion and Proposal

We have the following proposals:

Proposal-1: Rel-19 SBFD-aware UEs should camp on SBFD cell(s) as much as possible.

Proposal-2: Introduce priority based SBFD cell camping for Rel-19 SBFD-aware UEs during its cell selection and reselection process.

Proposal-3: RAN2 to discuss if a barring mechanism is introduced to prevent Pre Rel-19 UE or non SBFD-aware Rel-19 UE from camping on SBFD cells.

Proposal-4: RAN2 to discuss if and how the SBFD-aware Rel-19 UE prioritize the selection of a SBFD cell as the handover target cell during handover preparation.

# Reference

- RAN1#118bis meeting minutes

- R2-2409089 Other Aspects of SBFD