3GPP TSG-RAN WG2 Meeting #128	R2-2409796

Orlando, FL, US, 18 –22 Nov 2024

Agenda Item:	8.13.3

Source:	NEC

Title:	CP and SRAP for Multi-hop Relay

Document for:	Discussion, Decision

# Introduction

In RAN2#127 meeting, the following agreements were reached for Control Plane Aspects for Multi-hop Relay:

The following connections are assumed as a baseline to be needed:

-	From last Relay UE perspective:

	A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and

	A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED.

-	From intermediate relay UE perspective (including first relay UE):

	A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and

	An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED.

FFS what RRC states are supported for the intermediate relay UE.

-	From U2N Remote UE perspective:

	A direct (non-relayed) PC5 connection with Intermediate Relay UE, and

	An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED.This paper intends to discuss the RAN2 impact of the random access procedures for SBFD operation.

The multi-hop CP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

The multi-hop UP protocol stack is end-to-end for Uu-PDCP and above and hop-by-hop for SRAP and below (as in Rel-17/18).

There was a post meeting email discussion on Control Plane Aspects for Multi-hop Relay: [POST127][402][Relay] Multi-hop relay control plane.

This paper discusses the open issues that were not resolved so far on CP aspects for Multi-hop Relay.

# Discussion

## General Connection establishment

The multi-hop indirect path comprises more relay hops on top of the Rel-17 U2N relay (i.e., remote UE -> first relay UE ->Intermediate relay UE-> last relay UE -> gNB).

There are two approaches discussed during the post meeting email discussion on the connection establishment, approach-1 and approach-2.

Approach-1: Each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED. Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED.

Approach-2: A relay UE in RRC_IDLE/RRC_INACTIVE (other than the Last Relay UE) is not required to trigger its own RRC Connection upon RRC connection establishment of the U2N Remote UE.

As explained, the main difference in the procedure with approach 1 is that a relay UE (other than the Last Relay) in RRC_IDLE/RRC_INACTIVE is not required to trigger an RRC connection as a result of the remote UE’s RRC connection. And for the procedure itself and the configuration aspects, it is still for further study.

The first relay UE and the Intermediate relay UE are connected to the gNB via the last relay UE. Given the U2N relay scheme, the first relay UE and the Intermediate relay UE can be also under the control of the gNB of the last relay UE. Therefore, for the control of the first relay UE and the Intermediate relay UE, the RRC connection shall be established at the last relay UE.

A first relay UE in RRC_IDLE/RRC_INACTIVE may be selected by the remote UE due to not knowing the RRC status of the first relay UE. Then, the first RRC message received from the remote UE will trigger the first relay UE to enter RRC_CONNECTED when the remote UE and the gNB establish the RRC connection. Meanwhile, we believe, a RRC Connected intermediate UE is in a better position to be controlled by the gNB to offer traffic relay service.

Proposal 1:	Approach-1 is adopted for the connection establishment for Multi-hop relay, i.e., the first RRC message, for the RRC connection establishing between the remote UE and the gNB, received from the remote UE will trigger all the relay UEs (including the first relay UE, intermediate relay UE and last relay UE) to enter RRC_CONNECTED.

## SRAP Data forwarding

Based on the existing methods of U2N relay and U2U relay, the connection between the remote UE and the gNB can be divided into two parts:

-	The U2U-like connection between the remote UE and the last relay UE, and

-	The connection between the last relay UE and the gNB serving it.

Therefore, the data forwarding can also be divided into two parts accordingly.

Based on the U2U-like connection, the U2U-like SRAP method, such as the local ID pair of the remote UE and the first relay UE, can be used for the data forwarding between the remote UE and the first relay UE. Additionally, the U2N-like SRAP method, wherein only the local ID of the remote UE is used, can be used for the data forwarding between the last relay UE and the gNB. For example, the last relay UE will remove the local ID of the remote UE form the SRAP PDU when it forwards the data to the gNB. And in the opposite direction, the last relay UE will add the local ID of the remote UE into the SRAP PDU.

Proposal-2:	The U2U-like SRAP method, such as the local ID pair of the remote UE and the first relay UE, can be used for the data forwarding between the remote UE and the first relay UE.

Proposal-3:	The U2N-like SRAP method, wherein only the local ID of the remote UE is used, can be used for the data forwarding between the last relay UE and the gNB.

## Configurations of SRAP and QoS

Considering the U2N relay, the SRAP configurations, especially those for the first relay UE, are provided by the gNB.

Similarly, the QoS is under the control of the gNB. Thus, the QoS parameters of the first relay UE are also configured by the gNB.

Proposal 4:	The SRAP configurations, especially those for the first relay UE, are provided by the gNB.

Proposal 5:	The QoS parameters, especially those for the first relay UE, are configured by the gNB.

## Paging and System Information

As specified for Rel17 U2N relay, the relay UE monitors paging and system information on behalf of the remote UE. The remote UE needs to distribute some information to the relay UE beforehand, in order to help the relay UE to perform such monitoring, which may include the UE identity and the request system information. In Rel-17 U2N relay, the relay UE will basically relay the receiving paging or system information. The abovementioned information distribution and SI/paging forwarding is based on PC5-RRC signalling. In our opinion, the similar principles can apply to multi-hop case, which means a dedicated PC5-RRC connection is required between remote UE and intermediate U2N relay UEs. The only thing different is that there are multiple Relay UE in the multi-hop path. Hence we assume that there may be multiple PC5-RRC connections along with the multi-hop path as long as multiple intermediate U2N relay UEs exist.

Proposal 6:	For paging and SI forwarding, the similar principles of Rel-17 U2N relay mechanism can apply to multi-hop case.

Proposal 7:	For paging and SI forwarding, there are multiple PC5-RRC connections along with the multi-hop path (i.e., among the remote UE and multiple intermediate U2N relay UEs).

## Link failure handling

In Rel17 U2N relay, the remote UE triggers the RRC re-establishment procedure when detecting PC5 RLF and it can suspend RLM if it is connected to gNB via relay UE. This basic principle of Rel-17 U2N relay PC5 failure handling may be applicable to multi-hop case.

Proposal 8:	For PC5 Link failure handling of Remote UE, this basic principle of Rel-17 U2N relay PC5 failure handling applies to multi-hop case.

In addition, in Rel17 U2N relay, the radio link failure of relay UE will trigger the notification message transfer or PC5 link release. For the case of multi-hop, we believe, any PC5 radio link failure of intermediate relay UE(s) can lead to the suspension of the multi-hop path.

Proposal 9:	Any PC5 radio link failure of intermediate relay UE(s) can lead to the suspension of the multi-hop path.

# Conclusion and Proposal

We have the following proposals:

Proposal 1:	Approach-1 is adopted for the connection establishment for Multi-hop relay, i.e., the first RRC message, for the RRC connection establishing between the remote UE and the gNB, received from the remote UE will trigger all the relay UEs (including the first relay UE, intermediate relay UE and last relay UE) to enter RRC_CONNECTED.

Proposal-2:	The U2U-like SRAP method, such as the local ID pair of the remote UE and the first relay UE, can be used for the data forwarding between the remote UE and the first relay UE.

Proposal-3:	The U2N-like SRAP method, wherein only the local ID of the remote UE is used, can be used for the data forwarding between the last relay UE and the gNB.

Proposal 4:	The SRAP configurations, especially those for the first relay UE, are provided by the gNB.

Proposal 5:	The QoS parameters, especially those for the first relay UE, are configured by the gNB.

Proposal 6:	For paging and SI forwarding, the similar principles of Rel-17 U2N relay mechanism can apply to multi-hop case.

Proposal 7:	For paging and SI forwarding, there are multiple PC5-RRC connections along with the multi-hop path (i.e., among the remote UE and multiple intermediate U2N relay UEs).

Proposal 8:	For PC5 Link failure handling of Remote UE, this basic principle of Rel-17 U2N relay PC5 failure handling applies to multi-hop case.

Proposal 9:	Any PC5 radio link failure of intermediate relay UE(s) can lead to the suspension of the multi-hop path.

# Reference

- RAN2#127 meeting minutes

- Report of [POST127][402][Relay] Multi-hop relay control plane