3GPP RAN WG2 Meeting #128 	R2-2409803

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	8.6.4

Source:	InterDigital, Inc.

Title:	Conditional LTM.

Document for:	Discussion

# Introduction

The Rel-19 WI on mobility enhancements contains the following measurement related objectives.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In RAN2#127bis, the following progress was made:

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

# Discussion

In terms of what trigger(s) to use, it was agreed in the previous meeting that Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

While LTM3/5 may be suitable for some scenarios they may not be good enough for triggering cell switch without carefully considering parameters such as TTT and hysteresis. For triggering of cell switch, a more suitable approach at least for some scenarios might be cell based conditions, in which case it may be better to re-use the L3 events. In other words, it is worth considering allowing a conditional LTM to be associated with either a beam based event (e.g. e.g. LTMx) or a cell based events (e.g. event Ax), or perhaps both (both need to be met).

Proposal 1: A conditional LTM may be configured to be associated with

- a beam-based condition (event LTM3/5)

- a cell based condition (event A3/5)

- both (both need to be met).

The previous meeting agreements imply conditional LTM is used to trigger at least the cell switch case.

It was also agreed to support explicit triggering of early UL sync (using PDCCH order for TA acquisition) and early DL sync (Rel-18 Early candidate TCI State activation/deactivation).

For event triggered reporting, it has been agreed to consider the following scenarios:

  - Select the candidate beam/cell to trigger early synchronization.

  - Select the target beam/cell and trigger LTM cell switch procedure.

For the conditional case, similar scenarios should also be considered. The conditional LTM trigger could be a trigger to execute LTM cell switch itself, or it could be a trigger for performing early synchronization (e.g. activate TCI on target, trigger a RA for TA acquisition).

Essentially, it should be possible to configure a condition (e.g. beam-based, cell-based threshold) and an action (cell switch, UL sync, DL sync).

Proposal 2: A condition may be configured to trigger

- An LTM cell switch (reconfiguration)

- a DL early sync (TCI state activation)

- an UL early sync (RA for TA acquisition).

One of the main advantages of LTM in R18 is the reduction of interruption due to support of RACH-less, and it was agreed in the previous meeting to support RACH-less conditional LTM, and at least PDCCH ordered early TA acquisition.

For the case of PDCCH ordered RACH for TA acquisition, one scenario that we assume to be supported is the TA is provided by the source cell in a MAC CE, similar to R18 (either the existing cell switch MAC CE, with new indicator, or a new MAC CE). The main difference compared to R18 is that the resulting RACH-less LTM cell switch does not take place immediately, but rather only after the condition is met.

While this can provide some improvement compared to explicitly triggered cell switch, there is still some dependency on signalling from the source to enable the cell switch (e.g. an event triggered MAC CE to report measurements, and a MAC CE to provide TA, before the conditional LTM can occur)

The advantage of enabling conditional early UL sync would be that the TA value can be obtained early (ahead of cell switching) reducing the amount of time needed during cell switch itself. It also allows the RACH to be performed without relying on sending event triggered report to the source and receiving PDCCH order – the early sync and the cell switch could therefore be performed completely based on UE conditions and not rely on being enabled by the source just before cell switch.

For this approach, the conditional trigger for performing RACH would need to be set such that the condition is met earlier than the condition for performing cell switch.

If conditional uplink sync is supported, then the UE will trigger a RACH to the target when the condition is met. In this case, it would make sense to obtain the TA by RAR.

Proposal 3: For conditional trigger to perform RACH for UL sync, RAR containing TA is received from the target.

As long as the UE has a valid TA for a particular target meeting the condition, then RACH-less conditional LTM could be performed. However, it was not yet agreed how UE determines the TA validity. It was also not discussed what actions UE takes if it has/does not have a valid TA.

This means that the TA needs to be maintained for a certain period of time between it being received and the LTM being executed.

This would apply whether the TA is received from the source or from the target.

Proposal 4: Introduce TA validity timer, which is started upon TA reception and causes the TA to be invalid upon expiry.

If a conditional LTM is configured to allow RACH-less cell switch, there are several options depending on the TA validity. The conditional LTM configuration might only be valid while there is a valid TA. Alternatively, the conditional LTM could use RACH-less if a valid TA is stored, or use RACH-based LTM otherwise – and in this case we need to decide whether the condition for executing RACH-less or RACH-based is the same or not.

It seems more straightforward to configure whether RACH-less is enabled or not by RRC. That is, for a RACH-less conditional LTM, it is only to be executed if the condition is met, and a valid TA is stored. It would then be up to the network to configure a corresponding RACH-based conditional LTM, if necessary.

Proposal 5: RACH-less conditional LTM is enabled by RRC, and is executed only if the condition is met AND the UE has a valid TA.

Proposal 6: It is up to the network to configure a separate RACH-based conditional LTM, if necessary, for the same candidate (i.e. no dual RACH-less/RACH-based LTM for the same conditional event).

It was agreed that for RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

For the DG based approach to work, network needs to know when UE has performed cell switch and can receive DCI on the PDCCH of the new cell. The potential approaches are to either indicate something to the source (e.g. an event triggered measurement report in MAC CE, or using a dedicated SR) or to indicate something to the target (E.g. send a dedicated SR on the target). Without any indication to source or target, there is no way to reliably schedule a UE on the target after cell switching and therefore DG-based cannot be supported.

Proposal 7: For DG-based RACH-less conditional LTM, select from the following options

- Indicate to source when conditional LTM is performed (e.g. in MAC CE or dedicated SR)

- Indicate to target when conditional LTM is performed (E.g. in dedicated SR)

- DG-based RACH-less conditional LTM is not supported.

Even for the CG-based approach, reserving a CG in advance implies longer term resource reservation than compared to explicit RACH-less LTM. It could be beneficial if there is some indication shortly before the cell switch occurs, so that the CG can be reserved at the target.

One option is that the CG is reserved for the duration of TA validity time – so, e.g. the UE first reports an event triggered MAC CE report that the candidate cell measurement is above a threshold (event LTM4). Then the source issues a PDCCH order for the UE to send a PRACH to the target. After this the source sends a MAC CE containing the TA, and this enables RACH-less conditional LTM, with the CG being available for the duration of TA validity.

In case of conditional early UL sync, the TA value may come from a RAR on the target, but the same principle could apply.

However, for the case of UE based TA measurement, the target may not know when the conditional LTM will occur. In this case the CG resource may need to be reserved for a long time unless there is some pre-notification. Similar to the DG case, a dedicated SR might be needed first in order to activate the CG. Otherwise, the UE may need to report completion of the UE based TA measurement to the source, so that the source can “enable” conditional LTM for a time duration (E.g. using a TA validity timer), to limit how long the target needs to reserve the CG resources.

Proposal 8: For CG-based RACH-less conditional LTM, discuss which of the following options need to be supported:

- CG can be considered as valid for the duration of TA validity (for TA provided by source or target)

- CG can be considered as valid after dedicated SR transmission to target (at least for the case of UE based TA measurement)

- For the case of UE based TA measurement, UE reports TA measurement completion to the source, to enable the CG.

# Conclusion

In this paper we make some suggestions on conditional LTM as follows:

Proposal 1: A conditional LTM may be configured to be associated with

- a beam-based condition (event LTM3/5)

- a cell based condition (event A3/5)

- both (both need to be met).

Proposal 2: A condition may be configured to trigger

- An LTM cell switch (reconfiguration)

- a DL early sync (TCI state activation)

- an UL early sync (RA for TA acquisition).

Proposal 3: For conditional trigger to perform RACH for UL sync, RAR containing TA is received from the target.

Proposal 4: Introduce TA validity timer, which is started upon TA reception and causes the TA to be invalid upon expiry.

Proposal 5: RACH-less conditional LTM is enabled by RRC, and is executed only if the condition is met AND the UE has a valid TA.

Proposal 6: It is up to the network to configure a separate RACH-based conditional LTM, if necessary, for the same candidate (i.e. no dual RACH-less/RACH-based LTM for the same conditional event).

Proposal 7: For DG-based RACH-less conditional LTM, select from the following options

- Indicate to source when conditional LTM is performed (e.g. in MAC CE or dedicated SR)

- Indicate to target when conditional LTM is performed (E.g. in dedicated SR)

- DG-based RACH-less conditional LTM is not supported.

Proposal 8: For CG-based RACH-less conditional LTM, discuss which of the following options need to be supported:

- CG can be considered as valid for the duration of TA validity (for TA provided by source or target)

- CG can be considered as valid after dedicated SR transmission to target (at least for the case of UE based TA measurement)

- For the case of UE based TA measurement, UE reports TA measurement completion to the source, to enable the CG.

# References

- RP-242356,   “Revised Work Item: NR mobility enhancements Phase 4”