3GPP TSG-RAN WG2 Meeting #128                                    R2-2409835

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	8.1.2.3

Source: 	Fujitsu

Title: 	Discussion on LCM for Positioning Use Case

Document for:	Discussion

- Introduction

In this contribution, we would like to give our views on the LCM for positioning use cases. As agreed in RANP meeting, both prioritized NW-side and UE-side use cases will be discussed, including the following:

- NW-side models:

  - gNB-side model (Case 3a)

  - LMF-side model (Case 3b)

- UE-side model:

  - UE-side model (Case 1)

- Discussion

  - NW-side model (Case 3a/3b)

In RAN2 #125bis meeting [2], the following agreements were given for positioning NW-side model LCM:

Agreements: For POS, RAN2 assumes gNB or LMF could perform performance monitoring for case 3a and LMF is responsible for the performance monitoring for case 3b and waits for any further inputs from other WGs. For POS, RAN2 assumes that NRPPa is used for the signaling between gNB and LMF for case 3a and 3b and the detailed signaling design is up to RAN3.

According to the above agreements, for the performance monitoring for case 3a, either gNB or LMF can derive monitoring metrics or make preliminary decisions. gNB and LMF are both NW entities, but LMF is superior to gNB during LCM procedure, so the following options are available for Case 3a monitoring decision making.

- gNB makes both the preliminary and final monitoring decisions and reports to LMF.

- gNB makes the preliminary monitoring decision while LMF makes the final monitoring decision/confirmation.

- LMF makes the preliminary and final monitoring decisions.

Further study of the details of the above three options depends on further RAN1 progress.

For case 3b, LMF can derive the performance monitoring metrics and make the monitoring decisions. Some assistance information from gNB/PRU/UE may be necessary for case 3b metrics calculation. The details depend on RAN1 progress.

Potential NRPPa signaling design for decision making, monitoring assistance information between gNB and LMF need to involve RAN3.

Proposal 1 Both gNB and LMF can make monitoring decision for case 3a. The details depend on further RAN1 progress.

Proposal 2 LMF can make monitoring decision for case 3b. RAN2 wait for further RAN1 progress on potential assistance information/data for metrics deriving and decision making of case 3b.

For NW-side models, applicable conditions are not necessary since the NW entities will be fixed after deployment. Even for case 3a, the gNB-side applicability will not change and no need to report to LMF even if functionality identification may be necessary from gNB to LMF.

For NW-side models, additional conditions can also be used to ensure the consistency between training and inference. However, NW can be aware of the full information of the training details since the training is performed at NW-side, so NW can maintain the information by implementation and no need to inform UE. For case 3a, gNB can report the related conditions or information (e.g., SRS configuration) via Transformation in NRPPa to LMF as legacy way, so no extra specification support is expected.

Proposal 3 For the NW-side models of positioning use cases, no extra specification effort is needed for NW-side additional condition used at least for consistency between model training and inference.

  - UE-side model (Case 1)

Applicable Conditions and Additional Conditions

In the RAN2 127bis meeting, the following agreements were given:

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

A bunch of terminologies have been proposed for the RAN1 and RAN2 discussion, e.g., applicable condition, additional condition, and condition. To avoid further confusion, it is necessary to clarify these terminologies as soon as possible.

For the signaling and framework to report applicable conditions, Capability and Assistance Data defined in LPP can be used for reactive and proactive reporting. Details can be discussed after the clarification of the definitions of applicable conditions.

Proposal 4 In step 4, both LPP provide capabilities, and LPP assistance data messages can be used to report applicability for proactive and reactive reporting.

Similar to our parallel discussion for beam management, we believe the inference configuration can be signaled via either step 3 or step 5 so that more flexibility can be achieved. UE may start the AI/ML inference once the applicability is determined, without waiting for further signaling.

Proposal 5 NW may signal the inference configuration in step 5 or step 3.

In legacy LPP framework, UE may send unsolicited capability information upon NW request, however, the content to be reported can be decided by UE itself if no explicit configuration is given. In AI/ML, we suggest following this mechanism as a baseline.

Proposal 6 UE may decide the contents to be reported via step 4 if no explicit NW configuration is given.

Monitoring and LCM

In RAN2 #125bis meeting, the following agreements were given:

Agreements: 1 For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). 2 “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19.

For positing case 1, both UE and LMF can be used to calculate the monitoring metrics according to RAN1’s current discussion. The feasibility of the two options is still under investigation in RAN1, so RAN2 may wait for further RAN1 progress.

According to the current LPP signaling and framework design, even if UE can make decision of the preliminary monitoring results, a final confirmation from LMF is preferred.

Proposal 7 For detailed design of functionality management, RAN2 wait for RAN1 progress on UE/LMF monitoring options of positioning case 1.

Proposal 8 For positioning case 1, even if UE can calculate the monitoring metrics and make preliminary decisions of the monitoring results, final confirmation from LMF is preferred.

There are two types of monitoring proposed in RAN1, functionality and model monitoring. In RAN1 discussion, many companies prefer to have functionality-based LCM including functionality monitoring. However, the details have not been touched in RAN1. From RAN2 point of view, it is better to wait for further RAN1 conclusions on the relationship between functionality and model monitoring. For instance, if multiple models are defined under one functionality, RAN2 needs to study signaling and framework for some details, e.g., functionality failure definition, historical monitoring data recording for specific models under the same functionality, etc.

Observation 1 For UE-side positioning use case 1, model specific information may need to be reported to NW for functionality monitoring if multiple models are defined under one functionality.

Proposal 9 For positioning case 1, RAN2 study the signaling and procedure for model specific information reporting from UE to NW when multiple models are defined under one functionality.

- Conclusion

Proposal 1 Both gNB and LMF can make monitoring decision for case 3a, the details depend on further RAN1 progress.

Proposal 2 LMF can make monitoring decision for case 3b, RAN2 wait for further RAN1 progress on potential assistance information/data for metrics deriving and decision making of case 3b.

Proposal 3 For the NW-side models of positioning use cases, no extra specification effort needed for NW-side additional condition used at least for consistency between model training and inference.

Proposal 4 In step 4, both LPP provide capabilities, and LPP assistance data messages can be used to report applicability for proactive and reactive reporting.

Proposal 5 NW may signal the inference configuration in step 5 or step 3.

Proposal 6 UE may decide the contents to be reported via step 4 if no explicit NW configuration is given.

Proposal 7 For detailed design of functionality management, RAN2 wait for RAN1 progress on UE/LMF monitoring options of positioning case 1.

Proposal 8 For positioning case 1, even if UE can calculate the monitoring metrics and make preliminary decisions of the monitoring results, final confirmation from LMF is preferred.

Observation 1 For UE-side positioning use case 1, model specific information may need to be reported to NW for functionality monitoring if multiple models are defined under one functionality.

Proposal 9 For positioning case 1, RAN2 study the signaling and procedure for model specific information reporting from UE to NW when multiple models are defined under one functionality.

- References

[1] RAN2 meeting Chair Note, 3GPP TSG RAN WG2 Meeting #127bis, 2024.

[2] RAN2 meeting Chair Note, 3GPP TSG RAN WG2 Meeting #125bis, 2024.