3GPP TSG-RAN WG2 Meeting #128	R2-2409838

Orlando, USA, Nov. 18th –22nd, 2024

Agenda item:	8.2.3

Source: 	Fujitsu

Title: 	Discussions on AIoT Paging

Document for:	Discussion and decision

1. Introduction

According to the objective specified in the New SID [1], RAN2 needs to study and decide whether Paging is needed for an Ambient IoT compact protocol stack and lightweight signaling procedure to enable DO-DTT and DT data transmission and study the Paging mechanism for an Ambient IoT, if needed.

At previous RAN2 meetings, the following agreements on the Paging for an Ambient IoT are reached [2] [3][4][5]:

Agreement

- Legacy paging message for device will not be supported.

- Legacy paging occasion and legacy DRX for the device is not supported.  This doesn’t preclude solutions that address device monitoring (taking into account discussions from RAN1 as well).

- RAN2 assumes that the device will not support tracking/RAN area update procedure.

- For the case of reaching single or group of devices, an identifier may be required to identify the device/group of devices in the trigger message. FFS pending the details from SA2

Agreements 1 RAN2 will study the following cases for AIoT paging message: - a message containing an ID of a single A-IoT device. - a message containing a group ID that maps to multiple A-IoT devices. - a message that does not contain an ID, i.e., addressed for all devices that can receive the AIoT message. - a message containing multiple IDs of A-IoT devices. Need to confirm the need for this use case based on SA2 discussion. What device ID and group ID and scenarios is depending on SA2 discussion. 2 AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message). FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion. 3 From RAN2 perspective, we assume the device can receive as long as there is enough energy. We will wait for RAN1 further progress on device monitoring details.

Agreements:

- Support mechanism for reader to be able to trigger multiple/subsequent AIoT paging messages that are associated with the same request from the CN. Study how to avoid duplicated response from devices

Agreements on AIoT paging

- The A-IoT paging message can include information to avoid duplicate response from the device to a reader.  This information should be short and simple.  FFS how to indicate.  Wait for further information and requirements from other WGs and make this decision in normative phase.

- Based on this information the device determines whether to skip sending the response to paging.

- From RAN2 perspective it is feasible to support paging multiple device IDs, however depending on TB size it may not be possible to include multiple device IDs.    RAN2 will determine the need for this after considering aspects related to RAN2 multiplexing and other WGs.

- If multiple device IDs in single paging is supported, if A-IoT paging message contains multiple device IDs, reader can configure either contention free RA or contention-based RA

This contribution will provide more consideration on the Ambient IoT Paging.

2. Discussion

Based on RAN2 discussions, the overall AS procedures is formulated in draft TR38.769 as:

- Step A: A-IoT paging. Based on the service request, the reader sends the A-IoT paging message indicating device(s) that need to respond.

- Step B: D2R data transmission. Triggered A-IoT device(s) perform the device ID transmission via the A-IoT random access procedure or without using the A-IoT random access procedure.

- Step C1: Possible R2D data transmission (e.g. for sending the command).

- Step C2: Possible D2R data transmission (e.g. the corresponding response to command).

Figure 6.2.1-1 Overall AS procedures between A-IoT device and reader [6]

Step B related aspects are discussed in our companion contribution [7].

Step A related aspects are discussed in this contribution.

Multiple/subsequent A-IoT paging

RAN2 has agreed to support the mechanism for reader to trigger multiple/subsequent A-IoT paging messages associated with the same request from the CN. The A-IoT paging messages can include information to avoid duplicate response from the device to a reader and the device determines whether to skip sending the response to the A-IoT paging based on this information.

For multiple/subsequent A-IoT paging messages, the following cases are observed:

- Case 1: multiple/subsequent A-IoT paging messages are used for re-access

In this case, same A-IoT upper layer information included in the all A-IoT paging messages should be considered. However, whether to study the possibility that the A-IoT upper layer information will not be included in the subsequent A-IoT paging messages needs discussion.

- Case 2: multiple/subsequent A-IoT paging messages are used considering energy harvesting

- Case 3: multiple/subsequent A-IoT paging messages are used for grouping of intended A-IoT devices

In this case, it is better not to include the A-IoT upper layer information. Instead, AS grouping information is used to indicate the intended A-IoT devices for response. For example, different IDs for A-IoT device/group will be included in different A-IoT paging messages.

In our understanding, Case 1 is studied in this release. So, the possibility that the A-IoT upper layer information will not be included in the subsequent A-IoT paging messages needs discussion. However, based on discussions and agreements, it is not clear whether Case 2 or Case 3 is included or not. Case 2 should be studied in WI if RAN1 makes some progress. Considering the timeline, multiple/subsequent A-IoT paging messages are not used for AS grouping of intended A-IoT devices.

Proposal 1: Multiple/subsequent A-IoT paging messages are not used for AS grouping of intended A-IoT devices.

Proposal 2: RAN2 to discuss whether to study the following case:

- the A-IoT upper layer information is included in the first/initial A-IoT paging message, but not included in the subsequent A-IoT paging messages.

Content of A-IoT paging

The overall AS procedures with different steps shown above will be used for different use cases. For example, the AS procedure with step A and step B can be used to support inventory-only use case. While, the use case of inventory and command, or use case of command-only can be supported by the procedure with step A, step B, step C1 and step C2. Therefore, it is better to indicate paging cause/type in the A-IoT paging message.

Observation 1: it is better to indicate paging cause/type in the A-IoT paging message.

The issue is whether AS information to indicate the paging type is needed or not. Considering the case in Proposal 2, i.e. the A-IoT upper layer information is not included in the subsequent A-IoT paging messages, it is beneficial to include the AS information indicating the paging type in the (subsequent) A-IoT paging messages.

Proposal 3: Paging type, e.g., for inventory-only, command-only or inventory and command is included in the (subsequent) A-IoT paging message.

RAN2 has agreed that the AIoT paging message can contain an ID of a single device, a group ID mapping to multiple devices or does not contain the ID to trigger all devices. It means that different ID may be included in the message. It is useful to include an indication to identify the ID type. One possibility is to include an explicit indication to notify if device ID or group ID is used in the AIoT paging message. Alternatively, if device ID or group ID is used, it can be indicated implicitly. For example, device ID can be differentiated from group ID according to the component/structure of the A-IoT device ID. So, it is beneficial to include the ID type in the A-IoT paging messages.

Observation 2: it is beneficial to indicate ID type, e.g., device ID or group ID in the A-IoT paging message.

Since there is no conclusion on whether the command can be included in the paging message, the information on whether to monitor or receive subsequent R2D transmission including the command needs to be indicated in the A-IoT paging message, if the command is not included in the A-IoT paging message. Some information about response to the command can be included in the A-IoT paging no matter if the command is included in the A-IoT paging or not. The benefit to include the information is that the UE may discard the random number, or the temporary identifier earlier (AS ID), e.g., if no response is needed. If the command is not so critical and the network can send this command without expecting any response, then the response to the command may be not needed. From this perspective, indication if any or no response to the command is included in the A-IoT paging. Besides, if the response is required, the paging message can also indicate if the response can be delayed. If the response cannot be delayed, the triggered A-IoT device with low battery can take some actions.

Observation 3: For the use case of “command-only”, it is beneficial to include following information in the A-IoT Paging message:

- Indication if any response to the command is needed.

- Indication if the response can be delayed when the response is needed.

Based on these observations, it is beneficial to include additional information in the A-IoT Paging message. Considering the timeline, RAN2 further study this during WI phase.

Proposal 4: Whether and what additional information can be contained in the A-IoT paging message will be discussed in WI phase, e.g.:

- ID type, e.g., device ID or group ID

- if any response to the command is needed for the use case of “command-only”

- if the response can be delayed when the response is needed for the use case of “command-only”

3. Conclusion

In this contribution, we had some discussions on the Ambient IoT Paging and made the following observations and proposals:

Multiple/subsequent A-IoT paging

Proposal 1: Multiple/subsequent A-IoT paging messages are not used for AS grouping of intended A-IoT devices.

Proposal 2: RAN2 to discuss whether to study the following case:

- the A-IoT upper layer information is included in the first/initial A-IoT paging message, but not included in the subsequent A-IoT paging messages.

Content of A-IoT paging

Observation 1: it is better to indicate paging cause/type in the A-IoT paging message.

Proposal 3: Paging type, e.g., for inventory-only, command-only or inventory and command is included in the (subsequent) A-IoT paging message.

Observation 2: it is beneficial to indicate ID type, e.g., device ID or group ID in the A-IoT paging message.

Observation 3: For the use case of “command-only”, it is beneficial to include following information in the A-IoT Paging message:

- Indication if any response to the command is needed.

- Indication if the response can be delayed when the response is needed.

Proposal 4: Whether and what additional information can be contained in the A-IoT paging message will be discussed in WI phase, e.g.:

- ID type, e.g., device ID or group ID

- if any response to the command is needed for the use case of “command-only”

- if the response can be delayed when the response is needed for the use case of “command-only”

Reference

- RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#103, March 18-21, 2024

- RAN2#125bis Chair Notes, RAN2 Chair (InterDigital), RAN2#125bis

- RAN2#126 Chair Notes, RAN2 Chair (InterDigital), RAN2#126

- RAN2#127 Chair Notes, RAN2 Chair (InterDigital), RAN2#127

- RAN2#127bis Chair Notes, RAN2 Chair (InterDigital), RAN2#127bis

- R2-240xxxx, “TP for TR 38.769 update”, Huawei, RAN2#128

- R2-2409839, “Discussions on AIoT Random Access”, Fujitsu, RAN2#127