3GPP TSG-RAN WG2 Meeting #128                                                        R2-2409857

Orlando, USA, 18th – 22nd Nov, 2024

Source:	CATT

Title:	Consideration on XR-specific RLC Enhancement

Agenda Item:	8.7.5

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the issues on timely RLC retransmissions, avoiding unnecessary retransmission for XR and the latest LS from SA2, our views are also given in the following part.

Discussion

Timely RLC retransmissions for XR

In RAN2#127bis meeting, the agreements on timely retransmissions on RLC layer were as follows [1]. Two options were proposed for further study. More details are discussed for these options in this section.

RAN2 confirm that existing mechanisms are insufficient to resolve the timely RLC retransmission problem and RLC enhancements for timely RLC retransmission are investigated in Rel-19. Exclude enhanced status reporting. Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay

On autonomous retransmission

In this option, RLC retransmission is not performed based on RLC status report but depends on blind retransmission. During this procedure, the RLC will retransmit the RLC SDU which may be already successfully received at Rx side, which will result in resource waste and impact on system capacity considering the data volume for XR service is indeed more than legacy service. Besides, in congested scenario, this behavior will also deteriorate the congestion.

If still going to this way, the specific condition to trigger the blind retransmission should be defined, such as based on the remaining time or the failed retransmissions number from HARQ feedback.

For the triggering condition of failed retransmissions number from HARQ feedback, the cross-layer interaction at UE will be introduced, and the HARQ feedback for UL sometimes is not applied or not corrected, so this triggering condition is not desired. For the method based on the remaining time, Tx side could perform retransmission when the remaining time is lower than a threshold, which seems reasonable and feasible.

Proposal 1: If RLC autonomous retransmission is supported, the triggering condition could be further studied based on the remaining time.

Retransmission based on enhanced polling

5.3.3.2 Transmission of a AMD PDU Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below. Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); - include a poll in the AMD PDU as described below. NOTE: Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation. <--skip--> Upon submission of an AMD PDU including a poll to lower layer, the transmitting side of an AM RLC entity shall: - set POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer; - if t-PollRetransmit is not running: - start t-PollRetransmit. - else: - restart t-PollRetransmit. 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

Based on above procedure for polling[2], it shows that the number of PDU_WITHOUT_POLL, BYTE_WITHOUT_POLL could be used to trigger a poll in an AMD PDU. For UL XR traffic, the value associated to above two parameters could be configured to be short one by network, which could trigger PDCP status report more quickly. The short value for pollPDU and pollByte absolutely have the same function as a timer, thus is not necessary to introduce a new timer to trigger the polling. Obviously, the overhead used by polling and the following status report are acceptable, the RLC retransmission triggered by this type of status report could give Rx more opportunity to receive the not-acknowledged RLC SDU or segment and try to receive it successfully under a limited delay budget, which also avoids the resource waste compared to the blind retransmission method without considering the latest ACK/NACK information at Rx Side.

Moreover, the condition to trigger polling could be at the moment when the data become delay critical. In this condition, most of the RLC SDU or segment with NACK indication needs be retransmitted as soon as possible, it is a reasonable condition to trigger the polling. Considering the polling transmission and associated status report receiving will have extra time consumption, so extra delay requirement is needed compared with RLC autonomous retransmission, this method could be applied in the case that the delay could guarantee at least one round of polling transmission and associated status report receiving. In our view, this delay threshold to trigger polling is not difficult to be determined and set by network, further, it could be applied in the congestion scenario and with an early retransmission compared with autonomous retransmission method, which is beneficial to help network to receive the RLC SDU or segment successful in UL congestion scenario. If Rx still could not receive the ACK from network within extremely short remaining time (such as 1-5ms, which could not be used for polling), the RLC autonomous retransmission could be applied.

Proposal 2: If enhanced Polling is used for RLC timely retransmission, the below two options could be used for down-selection:

- Option 1: the network can configure a new set of shorter value for pollPDU and pollByte to trigger the polling for timely RLC retransmission.

- Option 2: the UE can use the delay criticality of the AMD PDU to trigger the polling for timely RLC retransmission.

Proposal 3: Both RLC autonomous retransmission and enhanced Polling could be supported and applied to different remaining delay case if configured.

Avoid unnecessary retransmission

In RAN2#127 meeting, the agreement on avoiding unnecessary retransmission on RLC layer is as follows, a combined approach is agreed. More details on this approach are given here.

Agreements on avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

The Rx side behavior

In last meeting, a local timer in Rx side is introduced to abandon the obsolete RLC SDU or segment. Considering that legacy RLC layer is not aware of t-Reordering timer status at PDCP layer which could be used to abandon the PDCP SDU, to avoid the cross layers impact, it is preferable to introduce new timer at RLC layer to avoid unnecessary retransmission.

Proposal 4: At Rx side introduce a local timer at RLC layer for avoiding unnecessary retransmission.

For the timer at RLC layer, one option is to introduce a timer per RLC SDU, the UE will abandon the RLC SDU based on the timer for each RLC SDU. Another option is introduce a timer per RLC entity, the timer is similarly to the t-Reordering timer. For first option, Rx side needs to initiate and maintain so many timers that it will increase UE complexity and storage, the second option is aligned to the legacy t-Reordering timer to advance the Rx Window, it is simple and feasible. Hence, the local timer to abandon the obsolete RLC SDU or segment is preferable to be a timer per RLC entity. This timer could be started when RLC delivers a RLC SDU with SN higher than RX_Next to PDCP layer, i.e., a gap is generated at RX side. When the timer is expired, the UE will discard all segments with SN less than the SN recorded for this timer and advance the Window.

Proposal 5: A new timer per RLC entity is introduced for Rx initiated approach to avoid unnecessary retransmission, it could be performed similar to the t-Reordering timer in PDCP layer.

Proposal 6: At Rx side, the new timer can be started when RLC delivers a RLC SDU with SN higher than RX_Next to PDCP layer, i.e., a gap is generated at RX side.

Proposal 7: At Rx side, when the new timer expired, the Rx Side will discard all segments with SN less than the SN recorded for this timer and advance the Window, the other associated details are FFS.

Once the timer is expired, the RX side may discard the corresponding RLC SDU or segment and advance its window, so it is better to transmit a status report to Tx side to ensure the windows at Tx side and Rx side not out of sync, in order to reduce the impact to legacy Tx procedure, the discarded SDU or segment could be considered as ACK in status report, then the Tx side will update its variables as legacy procedure.

Proposal 8: The RX side will trigger a status report if the RX discards outdated PDU based on local timer and the window is advanced, which is also restricted by t-StatusProhibit.

Proposal 9: The SN for the discarded RLC SDU or segment can be reported in RLC status report PDU and set as ACK.

The Tx side behavoir

For this combined Rx and Tx approach, it is agreed that Tx side stops to retransmit an outdated SDUs. It is still not clear how the Tx side discard the outdated SDU, one option is based on the Tx own indication from PDCP layer or a new timer at Rx side, another option is based on the discard indication from Rx side, such as the fake ACK in Rx status report, RAN2 needs to further discuss and clarify which option is selected.

Proposal 10: RAN2 discuss which option is selected for Tx side discarding outdated SDUs:

- Option 1: Based on the Tx own indication from PDCP layer or a new timer at RLC layer;

- Option 2: Based on the discard indication from Rx side, such as the fake ACK in Rx status report.

For the Tx window, RAN2 emphasize that the RLC receiving window always advances to any given RLC SN before the transmitting window does. So, the TX side will not advance its window if Tx side discard the outdated SDUs based on its own determination, anyway, it will update TX_Next_Ack and advance the Tx window with SNs in [TX_Next_Ack, TX_Next_Ack + AM_Window_Size) based on the status report from Rx side.

Proposal 11: The Tx side will follow the legacy procedure to update the TX_Next_Ack and advance the window, i.e., based on the received status report, no further spec impact.

Discussion on LS

SA2 sent a LS S2-2410999 to RAN2[3], SA2 defined the content ratio as the ratio of PDUs of a PDU Set that are needed at the UE to be able to reconstruct the original content. The LS also said, if NG-RAN supports PDU Set content ratio awareness and has received content ratio information for a QoS flow, then NG-RAN may consider content ratio information as follows:

-	If NG-RAN needs to discard PDUs of the QoS flow during congestion, then NG-RAN may use the content ratio information for discarding.

Based on above information from S2 LS, SA2 would like to ask the following questions:

Question 1: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC AM mode based on the above principles. Please also provide feedback on applicability of RLC AM for XR services, also considering the RAN-level enhancements of RLC Acknowledged Mode (RLC AM) with small packet delay budget as per RP-240791. With respect to RLC UM, RAN indicated in R2-2405781 that "RAN2 does not think it is possible for NG-RAN to determine reliably whether a PDU has been successfully delivered over an unacknowledged-mode data bearer." After sending the previous LS in S2-2405625, SA2 clarified the principles for potential use of content ratio information for discarding of PDUs during congestion as listed above. Question 2: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC UM based on the above principles.

For question 1, the RLC AM mode with ACK/NACK indication can be used to compute how many DL PDU is transmitted successfully, so the gNB can discard the DL PDU when the number of PDU with ACK is achieved to content ratio. Hence, from the view of technique, it is feasible to use the content ratio information for discarding of DL PDUs during congestion for RLC AM.

Besides, in the current RAN2 XR discussion, the TX/RX could discard the outdated RLC PDU based on the delay budget information or timer, and RAN2 also discuss to enhance the retransmission of RLC SDU with small remaining delay for UL PDU. The method on retransmission enhancement (such as autonomous retransmission, or enhanced polling) seems independent with the content ratio information based DL PDU discarding proposed by SA2.

Proposal 12: From RAN2 perspective, it is feasible to use the content ratio information for discarding of DL PDUs during congestion for RLC AM.

For question 2, for RLC UM, it is difficult for gNB to compute how many DL PDU is transmitted successfully, the computing result on content ratio maybe not correct, so it is not feasible to use the content ratio information for discarding of DL PDUs during congestion for RLC UM.

Proposal 13: It is not feasible to use the content ratio information for discarding of DL PDUs during congestion for RLC UM.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: If RLC autonomous retransmission is supported, the triggering condition could be further studied based on the remaining time.

Proposal 2: If enhanced Polling is used for RLC timely retransmission, the below two options could be used for down-selection:

- Option 1: the network can configure a new set of shorter value for pollPDU and pollByte to trigger the polling for timely RLC retransmission.

- Option 2: the UE can use the delay criticality of the AMD PDU to trigger the polling for timely RLC retransmission.

Proposal 3: Both RLC autonomous retransmission and enhanced Polling could be supported and applied to different remaining delay case if configured.

Proposal 4: At Rx side introduce a local timer at RLC layer for avoiding unnecessary retransmission.

Proposal 5: A new timer per RLC entity is introduced for Rx initiated approach to avoid unnecessary retransmission, it could be performed similar to the t-Reordering timer in PDCP layer.

Proposal 6: At Rx side, the new timer can be started when RLC delivers a RLC SDU with SN higher than RX_Next to PDCP layer, i.e., a gap is generated at RX side.

Proposal 7: At Rx side, when the new timer expired, the Rx Side will discard all segments with SN less than the SN recorded for this timer and advance the Window, the other associated details are FFS.

Proposal 8: The RX side will trigger a status report if the RX discards outdated PDU based on local timer and the window is advanced, which is also restricted by t-StatusProhibit.

Proposal 9: The SN for the discarded RLC SDU or segment can be reported in RLC status report PDU and set as ACK.

Proposal 10: RAN2 discuss which option is selected for Tx side discarding outdated SDUs:

- Option 1: Based on the Tx own indication from PDCP layer or a new timer at RLC layer;

- Option 2: Based on the discard indication from Rx side, such as the fake ACK in Rx status report.

Proposal 11: The Tx side will follow the legacy procedure to update the TX_Next_Ack and advance the window, i.e., based on the received status report, no further spec impact.

Proposal 12: From RAN2 perspective, it is feasible to use the content ratio information for discarding of DL PDUs during congestion for RLC AM.

Proposal 13: It is not feasible to use the content ratio information for discarding of DL PDUs during congestion for RLC UM.

Reference

- RAN2#127bis meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei

- TS 38.322 Radio Link Control (RLC) protocol specification

- R2-2409525 Reply LS on Application-Layer FEC Awareness at RAN (S2-2410999; contact: Qualcomm)