3GPP TSG-RAN WG2 Meeting #128                                                        R2-2409859

Orlando, USA, 18th – 22nd November 2024

Source:	CATT

Title:	Discussion on multi-hop discovery and (re)selection

Agenda Item:	8.13.2

Document for:	Discussion and Decision

Introduction

After RAN2#127 meeting, one long post email discussion for multi-hop relay discovery and (re)selection[1] was carried out. Based on progress, we focus on the below selected issues from CATT’s view for further clarification:

- Whether the upper bound for the intermediate Relay UE is needed in case the intermediate Relay UE is in coverage?

- Discovery message forwarding condition at the intermediate relay UE for discovery model A

- PC5 AS conditions for discovery at the last relay UE for discovery model B

- (Re)selection criteria for remote UE

- Whether to support cross-path topologies and cross group issue handling

Discussion

Whether the upper bound for the intermediate Relay UE is needed in case the intermediate Relay UE is in coverage

As the proponent companies noted, the upper Uu threshold is used to restrict a cell-center UE to act as an intermediate relay UE.  We use the below schematic diagram of the scene for example. The green area is controlled by legacy relay UE’s upper and lower Uu thresholds. The yellow area is the outer area while in the coverage of cell. The intention of upper Uu threshold used for the intermediate relay UEs is to restrict the intermediate relay in the yellow area (not into the green area). On the technical point of view, it is reasonable to introduce one upper bound for the intermediate relay UE in case the intermediate relay UE is in coverage. But we doubt the possibility for the intermediate relay UE in the yellow part area is very small.

Figure 1. Schematic diagram of the scene1

Besides there will be another scene showing that the Uu bound for the intermediate relay UE is not needed.

Figure 2. Schematic diagram of the scene2

In the above scene2, PLMN Y doesn’t support RAN sharing, Last relay UE1 and Last relay UE2 are belong to PLMN Y while Remote UE belongs to PLMN X, the remote UE can’t access to PLMN Y through Last relay UE1 by single hop relay but it can access to PLMN X through First relay UE by multi hop relay. In case we introduce the Uu upper bound for the intermediate relay UE (i.e. Last relay UE1 and Last relay UE2), the remote UE can’t access the gNB(PLMN X) through the good path(Remote UE->Last relay UE1->First relay UE->PLMN X), instead access the gNB through the bad path(Remote UE->Last relay UE2->First relay UE->PLMN X)). This is the side effect of introducing the Uu upper bound for the in coverage intermediate relay UE.

All in all, we prefer not to introduce an upper bound for the intermediate relay UE in Rel-19.

Proposal 1: A upper bound is not required for the intermediate Relay UE.

Discovery message forwarding condition at the intermediate relay UE for discovery model A

In the summary contribution, the below initial summary of discovery announcement message initialization/forwarding condition for the intermediate relay UE was raised:

[EASY] Proposal 2: RAN2 understands that the intermediate Relay UE can initiate the announcement message when receiving announcement message or, at least, when the intermediate Relay UE has the information for the announcement message provided by the last Relay UE.

[EASY] Proposal 3: The discovery announcement message can be forwarded at the intermediate Relay UE when the SD-RSRP or SL-RSRP between the last/parent intermediate Relay UE and itself is above a configured threshold.

Further clarification can be discussed in RAN2.

Since according to SA2’s agreed CR, it stated that the intermediate U2N relay had already been authenticated and established a connection to the network before serving the remote UE (for discovery or connection). With a small scope offline discussion, there are below three different understandings for above SA2’s conclusion:

- Understanding1(PC5-S connection should be established as condition): For discovery model A, before the intermediate relay UE initiate the discovery message, the intermediate relay UE shall choose its parent relay UE and establish the PC5-S connection with the parent relay UE. Then the intermediate relay UE can further initiate the discovery message.

- Understanding2(Once receive the discovery message, start to establish the PC5-S connection, but don’t need to wait the PC5-S connection establishment done): For discovery model A, the intermediate relay UE shall choose its parent relay UE, then start the PC5-S connection establishment procedure. Without waiting the PC5-S connection establishment procedure finish, the intermediate relay UE can further initiate the discovery message.

- Understanding3(Don’t need to establish PC5-S connection, receive and forward it directly): For discovery model A, the intermediate relay UE shall not choose its parent relay UE, the intermediate relay UE just handle the discovery message received and forward it.

The above different understandings will affect RAN2’s further discussion, initial discussion for alignment with the above three understandings are necessary in RAN2 firstly, besides, a LS to SA2 for better understanding as soon as possible.

Proposal 2: RAN2 discuss the understanding for SA2’s conclusion on discovery message initialization condition with the below options. After that, RAN2 send a LS to SA2 for alignment:

- Option 1: PC5-S connection should be established as condition.

- Option 2: Once receive the discovery message, start to establish the PC5-S connection, but don’t need to wait the PC5-S connection establishment done.

- Option 3: Don’t need to establish PC5-S connection, receive and forward it directly.

For Rel-19 multi-hop U2N relay discovery model A, there is no need to consider the AS condition for announcing discovery message. Since in the LS from SA2 to SA3[4], it recorded that the intermediate U2N relay had already been authenticated and established a connection to the network before serving the remote UE (for discovery or connection). Before the intermediate relay UE initiates the announcing discovery message, the PC5-S link between the intermediate relay UE and its adjacent’ relay UE should be established in advance. This is also one agreed CR to support the above description:

#### 6.4.3.x	Layer-2 link management over PC5 reference point for 5G ProSe multi-hop UE-to-Network Relay

##### 6.4.3.x.1	Layer-2 Link establishment and management (based on Model A discovery)

Each of the 5G ProSe Intermediate UE-to-Network Relay establishes a Layer-2 Link with its immediate parent Intermediate UE-to-Network Relay or the 5G ProSe UE-to-Network Relay, before it can serve the 5G ProSe Remote UE, or a child Intermediate UE-to-Network Relay. RSC based configuration for the 5G ProSe Intermediate UE-to-Network Relay indicates whether it should establish the Layer-2 Link with a parent relay before it can participate in the discover operation.

Hence, we propose:

Proposal 3: For multi-hop relay discovery Model A, there is no AS channel condition should be considered for intermediate relay UE when initiating the discovery message.

PC5 AS conditions for discovery at the last relay UE for discovery model B

For multi-hop U2N relay in Rel-19, one link is PC5 link and the other link is Uu link for last relay UE. In the last RAN2 meeting, the below conclusion had been agreed for Uu link:

If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds).

Here we further discuss that whether any PC5 AS conditions for discovery at the last relay UE is needed or not. It seems reasonable to discuss the relay discovery for multi-hop U2N relay with an analogy to Rel-17 U2N relay design for the last relay UE. In Rel-17 U2N relay, for relay discovery related discussion, there is no PC5 AS condition captured yet for discovery message initiating/forwarding condition. Hence, we propose:

Proposal 4: For multi-hop relay discovery Model B, there is no AS criterion is needed for the last relay UE to forwards the response message (similar as Rel-17 U2N relay discovery).

(Re)selection criteria for remote UE

In LTE, no other AS-related criteria, except radio link quality were specified for relay UE (re)selection. In Rel-17 U2N relay, serving cell ID and PLMN ID are considered as AS criteria. It is up to SA2 to decide how to include L2/L3 relay support in discovery message. Whether L2/L3 relay support can be used as additional criteria for relay (re)selection can be left to SA2. Besides, relay load is not considered as a (re)selection criterion in Rel-17.

For Rel-19 multi-hop U2N relay, we need to discuss that whether any additional AS criteria for relay (re)selection is needed. The most important issue that we raise to discuss is that whether to introduce the hops number as additional AS criteria. In the latest SA2’s TR for Rel-19, the Solution#1 for model A and Solution#2 and #7 for model B are used as basis for normative work. Both solutions had recorded that the hops number will be considered as criteria for multi-hop U2N relay (re)selection. Considering the hop related information had been adopt by SA2 into discovery message, hence we propose:

Proposal 5: It is up to SA2 to decide how to include hop related information in discovery message. Whether hop related information can be used as additional criteria for relay (re)selection can be left to SA2.

Proposal 6: For relay selection and reselection, when remote UE has multiple suitable intermediate relay UE candidates which meet all AS-layer & higher layer criteria and remote UE need to select one intermediate relay UE by itself, it is up to UE implementation to choose one intermediate relay UE.

Whether to support cross-path topologies and cross group issue handling

In Rel-19 multi-hop U2N relay, since the additional hop of relay, there is one scenario issue that needs to be clarified: Whether to support cross-path topologies, e.g., whether (physically) different remote UE can select each (physically) different last relay UE via one (physically same) intermediate relay UE.

The sketch is listed as below:

Figure 3. Sketch for cross-path topologies

In the above sketch, the remote UE1 connects to gNB through First relay UE and Last relay UE1; the remote UE X connects to gNB through First relay UE and Last relay UE X. One single first relay UE has two paths to the gNB, the two legs are both indirect path, which is not a supported in Rel-18 multi-path relay. Additionally, the multi-path and multi-hop L2 U2N Relay combination is not in the scope of RAN2 WID. Besides the scope of the WID, the multi-path and multi-hop combination also makes the scenario complex, for example, whether the different legs can connect to different cells or even gNBs, the impact to bearer configurations and how to handle the path switch etc. Considering the scope, complexity and limited TU, it is suggested to not consider cross-path topologies in Rel-19 for L2 multi-hop U2N Relay.

Proposal 7: RAN2 doesn't support cross-path topologies e.g., whether (physically) different remote UE can select each (physically) different last relay UE via one (physically same) intermediate relay UE in Rel-19 for L2 multi-hop U2N relay.

With the above proposal, the next question needs to be further discussed is how to achieve this limitation. To exclude the cross-path topologies, it better to be excluded at the relay discovery stage, which is more of SA2 scope, so it can be up to SA2 to enforce.

Proposal 8: RAN2 confirms that it is up to SA2 to exclude the cross-path topologies. A LS is raised to SA2 for further information and coordination.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: A upper bound is not required for the intermediate Relay UE.

Proposal 2: RAN2 discuss the understanding for SA2’s conclusion on discovery message initialization condition with the below options. After that, RAN2 send a LS to SA2 for alignment:

- Option 1: PC5-S connection should be established as condition.

- Option 2: Once receive the discovery message, start to establish the PC5-S connection, but don’t need to wait the PC5-S connection establishment done.

- Option 3: Don’t need to establish PC5-S connection, receive and forward it directly.

Proposal 3: For multi-hop relay discovery Model A, there is no AS channel condition should be considered for intermediate relay UE when initiating the discovery message.

Proposal 4: For multi-hop relay discovery Model B, there is no AS criterion is needed for the last relay UE to forwards the response message (similar as Rel-17 U2N relay discovery).

Proposal 5: It is up to SA2 to decide how to include hop related information in discovery message. Whether hop related information can be used as additional criteria for relay (re)selection can be left to SA2.

Proposal 6: For relay selection and reselection, when remote UE has multiple suitable intermediate relay UE candidates which meet all AS-layer & higher layer criteria and remote UE need to select one intermediate relay UE by itself, it is up to UE implementation to choose one intermediate relay UE.

Proposal 7: RAN2 doesn't support cross-path topologies e.g., whether (physically) different remote UE can select each (physically) different last relay UE via one (physically same) intermediate relay UE in Rel-19 for L2 multi-hop U2N relay.

Proposal 8: RAN2 confirms that it is up to SA2 to exclude the cross-path topologies. A LS is raised to SA2 for further information and coordination.

Reference

- [Post127][401][Relay] Multi-hop relay discovery and (re)selection (LG)

- R2-2102242 Report of 3GPP TSG RAN2#111-e meeting, Online

- S2-2409396 5G ProSe multi-hop UE-to-Network Relay connection management Qualcomm Incorporated, AT&T, LG Electronics, FirstNet, China Telecom, Ericsson