3GPP TSG-RAN WG2 Meeting #128                                           R2-2409868

Orlando, US, Nov 11th – 12th, 2024

Agenda Item:	8.3.2.1

Source:	Mediatek Inc.

Title:	Simulation Results for AIML RRM Prediction and Remaining Issues

Document for:	Discussion, Decision

# Introduction

In RAN2 #126 meeting, the agreement on the use cases and the corresponding priority are given as follows.

| Case number | Prioritization | Evaluation scenario combination | Study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

In this contribution, we present the simulation results for RRM prediction for the above 4 marked cases. Observations from the simulation results are provided for each use case. Additionally, we clarify the derivation of intermediate KPI, especially on how to derive the “average”, where companies have different understandings in the email discussion [1].

# Discussion

## Simulation results and observations

### Case 2: Intra-frequency temporal domain case B

In this case, we examine the AI approaches for reducing measurement overhead in the temporal domain. For a given measurement reduction ratio (MRRT), RAN2 allows companies to consider their own observation and prediction patterns. The following observation/prediction pattern is chosen in our simulation results. This pattern uniformly allocates the observation and prediction time slots, minimizing the distance between the last observation in OW and the last prediction in PW.

Figure 1: observation and prediction pattern for (a) MRRT = 0.5 (b) MRRT = 0.8

We consider two different cases: sliding and non-sliding, with varying MRRT and OW lengths. For the sliding case, the L1 filter and L3 filter measurements are updated every sampling period, i.e., 40ms. For the non-sliding case, they are updated every measurement period, i.e., 200ms. The PW is then determined based on the update frequency and MRRT as shown in Figure 1. For the sliding case, when MRRT = 0.5, PW = 40ms (contain one predicted measurement). When MRRT = 0.8, PW = 160ms (contain 4 predicted measurements). For the non-sliding case, when MRRT = 0.5, PW = 200ms (contain one predicted measurement), and when MRRT = 0.8, PW = 800ms (contain 4 predicted measurements). We consider sub-use case 2 with cluster-based approach where both AI model input and output are L3 cell level RSRP of 21 cells. The approach is selected because the sub-use case 2 can achieve the highest accuracy among all sub-use cases and the cluster-based approach generally outperforms the cell-based approaches in our simulation result. Detailed AI model parameters and settings are given in the below table. The general simulation setting can be found in the Appendix.

| Setting/Parameters | Value |
| --- | --- |
| AI model | CNN |
| Flop | 1.26M |
| Model parameters | 304KB |
| Cluster approaches setting | Input: 21 cells, output: 21 cells |
| AI input/output | Input: L3 cell level RSRP, output: L3 cell level RSRP |

Table 1:AI model parameters and settings for intra-frequency temporal domain case B

In the following simulation result, we examine the impact of different OW lengths, MRRT, UE speed, and sliding/non-sliding cases by comparing the result with non-AI sample and hold approaches, which directly replace the predicted measurement with the last observation measurement.

| Intra-freq: FR1, Sliding | Average L3 Cell RSRP diff (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | MRRT = 0.5, PW=40ms (1 point) |  | MRRT = 0.8, PW=160ms (4 points) |  |
|  | OW = 40ms (1 point) | OW =1600ms (20 points) | OW = 40ms (1 point) | OW=1600ms (8 points) |
| Sample and hold (90km/hr) | 0.198 | 0.198 | 0.482 | 0.482 |
| AI (90km/hr) | 0.198 | 0.08 | 0.482 | 0.384 |
| Sample and hold (30km/hr) | 0.128 | 0.128 | 0.305 | 0.305 |
| AI (30km/hr) | 0.127 | 0.064 | 0.305 | 0.253 |

Table 2:  intra-frequency temporal domain case B with sliding case

| Intra-freq: FR1, Non-sliding | Average L3 Cell RSRP diff (dB) |  |  |  |
| --- | --- | --- | --- | --- |
|  | MRRT = 0.5, PW=200ms (1 point) |  | MRRT = 0.8, PW=800ms (4 points) |  |
|  | OW = 200ms (1 point) | OW =2000ms (5 points) | OW = 200ms (1 point) | OW=2000ms (2 points) |
| Sample and hold (90km/hr) | 0.986 | 0.986 | 1.958 | 1.958 |
| AI (90km/hr) | 0.986 | 0.908 | 1.969 | 1.959 |
| Sample and hold (30km/hr) | 0.63 | 0.63 | 1.277 | 1.277 |
| AI (30km/hr) | 0.633 | 0.58 | 1.279 | 1.278 |

Table 3:  intra-frequency temporal domain case B with non-sliding case

Our result shows that with a short OW, especially in cases containing only 1 observed measurement, there is almost no gain compared with non-AI cases. In contrast, when OW contains more observed measurements, the AI outperforms non-AI cases. This is reasonable since more AI input samples enable AI model to learn better and provide more accurate predictions. Additionally, we notice that the performance in the sliding case is generally better than in the non-sliding case. One reason is that the interval between two samples is larger in the non-sliding case, leading to greater channel variation between samples. Another reason is that, for a similar OW length, there are more observed measurements in the sliding case.

From our result, we can also observe that higher accuracy is achieved at lower UE speed and lower MRRT. However, it should be noticed that for intra-frequency temporal domain case B, the non-AI approach, i.e., sample and hold, provides good performance, even when compared with AI approaches. This is because, in the observation/ prediction pattern given in Figure 1, the PW is relatively short in terms of sample point (at most 4 points in MRRT=0.8 case). To fully exploit the advantage of AI approaches, we may need to consider the case with more long-term predictions; otherwise, the improvement from AI is limited.

Observation 1: For intra-frequency temporal domain case B, the prediction accuracy is affected by OW length, MRRT, and UE speed. Specifically,

- higher accuracy can be achieved with longer OW

- higher accuracy can be achieved with lower MRRT

- higher accuracy can be achieved with lower UE speed

Observation 2: For intra-frequency temporal domain case B, the benefit of AI is limited compared to simple non-AI approaches, e.g., sample and hold. Particularly, when PW is short, non-AI approaches already provide very good performance, thus, the margin for AI enhancement is limited.

### Case 3: Inter-frequency prediction

In this section, we examine the simulation result of co-located inter-frequency prediction, where 4GHz and 2GHz cells are deployed at the same place. The AI approach is applied to predict the measurement result of 4GHz cells based on the observation of the co-located 2Ghz cell. Both cell-based and cluster-based approaches are considered in our simulation results. Detailed AI model parameters and settings are given in the following table.

| Setting/Parameters | Value |
| --- | --- |
| AI model | CNN |
| AI input/output | Input: 2GHz measurement, output: 4GHz measurement Input: L3 cell level RSRP, output: L3 cell level RSRP |
| Cell-specific approaches | Input: 1 cell, output: 1 cell, total 21 models |
| Cluster approaches setting | Input: 21 cells, output: 21 cells |
| Flop | Cluster-based: ~3M. Cell-based: 0.14*21 = ~2.9M |
| Model parameters | Cluster-based: ~201KB, Cell-based:27.8*21= ~585KB |

Table 4: AI model parameters and settings for inter-frequency prediction

To consider the AI enhancement, we compare the AI results with a non-AI approach that assumes the measurement result of 4GHz cells can be derived by the result of co-located 2GHz cell minus the path loss difference, i.e., the path loss of 4GHz – the path loss of 2GHz. In Table 5, we show that sub-use case 2, i.e., both input and output of AI model are L3 cell level RSRP, can achieve higher accuracy compared to other sub-use cases. The cluster-based approach which considers all 21 cells has significant improvement compared to the cell-based approach and non-AI approach.

| Inter-freq: FR1, 30km/hr | AI Input/ output | Average L3 Cell RSRP diff (dB) |
| --- | --- | --- |
| Non-AI: 4GHz measurement = 2 GHz measurement – path loss difference | NA | 2.34 |
| AI: Cluster-based | L3 to L3 | 1.64 |
|  | L1 to L3 | 1.93 |
|  | L1 to L1 | 1.97 |
| AI: Cell-based | L3 to L3 | 2.29 |
|  | L1 to L3 | 3.26 |
|  | L1 to L1 | 2.53 |

Table 5: Simulation results for inter-frequency prediction with different sub-use cases (30km/hr)

In Table 6, we consider the AI performance at different UE speeds. Simulation results show that the UE speed has no significant impact on the predicted accuracy.

| Inter-freq: FR1, L3 to L3 | UE speed |  |  |  |
| --- | --- | --- | --- | --- |
|  | 30km/hr | 60km/hr | 90km/hr | 120km/hr |
| Non-AI | 2.34 | 2.30 | 2.31 | 2.21 |
| AI: Cluster-based | 1.64 | 1.55 | 1.49 | 1.40 |

Table 6: Simulation results for inter-frequency prediction with different UE speeds

Observation 3: In frequency domain prediction cases, cell-based approaches achieve limited gain compared to the non-AI approach without the help of neighbor cell measurement. The cluster-based approach outperforms cell-based approaches across all sub-use cases.

Observation 4: For the inter-frequency prediction in the co-located deployment, the UE speed has almost no impact on the prediction accuracy.

### Case 4: Intra-frequency temporal domain case A

For intra-frequency temporal domain case A, we use continuous historical measurement results to predict continuous future measurement results without considering any measurement reduction. The following is the default setting in our simulation if no further specification.

| Setting/Parameters | Value |
| --- | --- |
| AI model | CNN |
| Cell-specific approaches | Input: 1 cell, output: 1 cell, total 21 models. |
| UE speed | 60km/hr |
| Sliding/non-sliding | Sliding |
| OW/PW | OW=160ms, PW=640ms |
| AI input/output (sub-use cases) | Sub-use case 2, input: L3 cell RSRP; output: L3 cell RSRP |

Table 7: Default settings for intra-frequency temporal domain case A

We first examine the results of 3 different sub-use cases as shown in the following table. Under the same setting, the sub-use case 2, i.e., L3 cell to L3 cell, can achieve the highest prediction accuracy.

| Intra-freq: FR2 | Average L3 Cell RSRP diff (dB) | AI Complexity: FLOPs(M) / Parameter size (K Bytes) |
| --- | --- | --- |
| Sub-use case 1: L1 beam to L1 beam | 2.49 | 16.06M/359.2KB |
| Sub-use case 2: L3 cell to L3 cell | 0.86 | 0.46M/96.3KB |
| Sub-use case 3: L1 beam to L3 cell | 1.94 | 14.79M/104.2KB |

Table 8: Simulation result for intra-frequency temporal domain case A with different sub-use cases.

In the next table, we consider different OW/PW rates (1/4, 1, 4). Since the OW and PW length directly affect the number of samples that can be used as AI model input and the number of samples needed to predict, a lower OW/PW rate would increase the prediction difficulty and vice versa. Our results also verify this observation.

| Intra-freq: FR2 | Average L3 Cell RSRP diff (dB) | AI Complexity: FLOPs(M) / Parameter size (K Bytes) |
| --- | --- | --- |
| OW/PW=1/4: OW= 160, PW=640 | 0.86 | 0.46M/96.3KB |
| OW/PW=1: OW=400, PW=400 | 0.40 | 0.30M/123.9KB |
| OW/PW=4: OW=640, PW=160 | 0.16 | 0.37M/151.4KB |

Table 9: Intra-frequency temporal domain case A with different OW/PW ratios

According to the email discussion [2], the L1 and L3 filter measurements can be updated every sampling period, e.g., 20*4=80ms, or every measurement period, e.g., 400ms. The former refers to the case with sliding filtering, while the latter refers to the case with non-sliding filtering. The following table compares the result of these two cases under the same OW and PW length (note that, the number of measurements in OW and PW are different for these two cases). Our result show that under the same OW and PW length, the sliding case can achieve higher prediction accuracy.

| Intra-freq: FR2 | Average L3 Cell RSRP diff (dB) | AI Complexity: FLOPs(M) / Parameter size (K Bytes) |
| --- | --- | --- |
| Sliding filtering | 1.94 | 0.46M/96.3KB |
| Non-sliding filtering | 3.38 | 0.60M/124.4KB |

Table 10: Intra-frequency temporal domain case A with sliding and non-sliding cases.

Next, we examine the AI improvement under different UE speeds by comparing the result with the non-AI approach, i.e., sample and hold, which is described in section 2.1. For both 60km/hr and 120km/hr, the AI outperforms the non-AI approach and can achieve less than 1dB RSRP difference. The performance gain in the more challenging case, i.e., 120km/hr, is slightly larger than in the 60km/hr case.

| Intra-freq: FR2 | Average L3 Cell RSRP diff (dB) | AI improvement | AI Complexity: FLOPs(M) / Parameter size (K Bytes) |
| --- | --- | --- | --- |
| Non-AI: sample and hold (60km/hr) | 1.07 | 1.07-0.86 =0.21(dB), 0.21/1.07=19.6% | NA |
| AI (60km/hr) | 0.86 |  | 0.46M/96.3KB |
| Non-AI: sample and hold (120km/hr) | 1.23 | 1.23-0.93=0.3 (dB) 0.3/1.23=24.3% | NA |
| AI (120km/hr) | 0.93 |  | 0.46M/96.3KB |

Table 11: AI enhancement in intra-frequency temporal domain case A with different UE speeds.

Observation 5: For intra-frequency temporal domain case A, the sub-use case 2, i.e., the L3 cell to L3 cell prediction, exhibits the smallest cell-level RSRP difference compared to other sub-use cases.

Observation 6: For intra-frequency temporal domain case A, a higher OW/PW rate leads to higher prediction accuracy.

Observation 7: For intra-frequency temporal domain case A, under the same OW and PW length, the sliding filtering case has higher prediction accuracy compared to the non-sliding filtering case.

Observation 8: For intra-frequency temporal domain case A, AI can provide ~20% gain (in terms of L3 cell RSRP difference) compared to the non-AI approach, i.e., sample and hold. The improvement is more significant in the high UE speed case, e.g., 120km/hr, than in the low UE speed cases, e.g., 60km/hr.

### Case 6: Intra-frequency spatial domain prediction

In this case, AI is applied to reduce measurement overhead in the spatial domain. For MRRS=50%, only half of Tx beam RSRP is observed, the rest of the beams are predicted based on the observation. However, we notice that under the same MRRS, the observation beam selection will affect the prediction accuracy. In the following simulation, we consider three different observation beam patterns, e.g., uniform odd index, uniform even index, and upper half index. We also compare the result with the non-AI approach which directly derives L3 cell level RSRP based on the set of observation beams (i.e., only considering half of the beams). Our results show that the selection of observation beams significantly affects the performance of both AI and non-AI approaches. Once the selection of observation beams contains strong beams, the non-AI approach will perform well (even better than AI approaches).

| 30km/hr, MRRS=0.5, Sub-use case 1 (L1 to L1) | Average L3 Cell RSRP diff (dB) |  |  | AI Complexity: FLOPs(M) / Parameter size (K Bytes) |
| --- | --- | --- | --- | --- |
|  | Uniform Odd index | Uniform Even Index | Upper Half index |  |
| AI (CNN) | 0.52 | 0.48 | 0.99 | 9.789M/164.9KB |
| Non-AI | 0.18 | 0.18 | 1.19 | NA |

Table 12: Intra-frequency spatial domain prediction with different observation beam selection.

Observation 9: The selection of observed beam patterns can significantly affect the prediction accuracy of the spatial domain prediction.

Proposal 1: RAN2 capture the above observations in the TR.

## Further clarification about the intermediate KPIs

In the email discussion [1], we noticed that companies have different understandings about how to calculate the intermediate KPI, i.e., average RSRP difference. One key issue is “When we derive the average RSRP difference, do we consider the serving cell prediction only, or do we consider the neighbouring cells prediction, too”. To clarify the definition of average RSRP difference, we raise the following question and provide the corresponding options.

Question: When we derive the average L3 cell-level RSRP difference, do we consider neighbouring cells prediction? What is the scope of “average”?

- Option 1: We consider neighbouring cells prediction, the “average” refers to the average over all predicted cells including both serving and neighbouring cells.

- Option 2: We only consider serving cell prediction, the “average” refers to the average over the serving cell.

- Other?

Since UE needs to measure not only the serving cell but also neighbour cell to trigger the measurement report, e.g., A3 event, we think the RRM prediction should contain neighbour cells prediction. Additionally, we think there is no need to separately calculate RSRP difference for the serving cell and other neighbouring cells as our results indicate that the AI model prediction accuracy is similar for both serving and neighbouring cell predictions. From our point of view, we should take all neighbouring cells into account, since in a practical situation, when an MO is configured by NW, UE must measure all cells in the frequency layer identified in the MO. We understand there is another view that we can only predict those neighbouring cells with strong RSRP in the historical measurement. But for simplicity, we think predicting all cells can be the baseline.

Proposal 2: Companies can report how to derive the average L3 cell-level RSRP difference. Specifically, do we consider neighbor cells and what is the scope of the “average”?

- Option 1: We consider neighbor cells prediction, the “average” refers to the average of all cells including the serving cell and neighboring cells.

- Option 2: We only consider serving cell prediction, the “average” refers to the average of the serving cell.

- Other option

Proposal 3: RAN2 consider “average” to be the average of all cells, including the serving cell and all other neighbor cells as the baseline method to derive the average L3 cell-level RSRP difference.

# Conclusion

Observations:

Simulation results: Intra-frequency temporal domain case B

Observation 1: For intra-frequency temporal domain case B, the prediction accuracy is affected by OW length, MRRT, and UE speed. Specifically,

- higher accuracy can be achieved with longer OW

- higher accuracy can be achieved with lower MRRT

- higher accuracy can be achieved with lower UE speed

Observation 2: For intra-frequency temporal domain case B, the benefit of AI is limited compared to simple non-AI approaches, e.g., sample and hold. Particularly, when PW is short, non-AI approaches already provide very good performance, thus, the margin for AI enhancement is limited.

Simulation results: Inter-frequency prediction

Observation 3: In frequency domain prediction cases, cell-based approaches achieve limited gain compared to the non-AI approach without the help of neighbor cell measurement. The cluster-based approach outperforms cell-based approaches across all sub-use cases.

Observation 4: For the inter-frequency prediction in the co-located deployment, the UE speed has almost no impact on the prediction accuracy.

Simulation result: Intra-frequency temporal domain case A

Observation 5: For intra-frequency temporal domain case A, the sub-use case 2, i.e., the L3 cell to L3 cell prediction, exhibits the smallest cell-level RSRP difference compared to other sub-use cases.

Observation 6: For intra-frequency temporal domain case A, a higher OW/PW rate leads to higher prediction accuracy.

Observation 7: For intra-frequency temporal domain case A, under the same OW and PW length, the sliding filtering case has higher prediction accuracy compared to the non-sliding filtering case.

Observation 8: For intra-frequency temporal domain case A, AI can provide ~20% gain (in terms of L3 cell RSRP difference) compared to the non-AI approach, i.e., sample and hold. The improvement is more significant in the high UE speed case, e.g., 120km/hr, than in the low UE speed cases, e.g., 60km/hr.

Simulation result: Intra-frequency spatial domain prediction

Observation 9: The selection of observed beam patterns can significantly affect the prediction accuracy of the spatial domain prediction.

Proposals:

Observation of simulation results

Proposal 1: RAN2 capture the above observations in the TR.

Clarification of average RSRP difference

Proposal 2: Companies can report how to derive the average L3 cell-level RSRP difference. Specifically, do we consider neighbor cells and what is the scope of the “average”?

- Option 1: We consider neighbor cells prediction, the “average” refers to the average of all cells including the serving cell and neighboring cells.

- Option 2: We only consider serving cell prediction, the “average” refers to the average of the serving cell.

- Other option

Proposal 3: RAN2 consider “average” to be the average of all cells, including the serving cell and all other neighbor cells as the baseline method to derive the average L3 cell-level RSRP difference.

# Reference

[1] 	[POST127bis][016][AI Mob] Simulation results (Mediatek)

[2]	R2-2407849 summary of [POST127][030][AI mobility] RRM simulation assumptions - Phase 1

# Appendix: Simulation Setting

| parameters | Value for FR1 | Value for FR2 |
| --- | --- | --- |
| Frequency Range | FR1@{4GHz,30KHz} as central frequency for intra-frequency scenario FR1@{2GHz,30KHz} as another frequency for inter-frequency scenario | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 2-tier model (7 sites, 3 sectors/cells per site) | 2-tier model (7 sites, 3 sectors/cells per site) |
| Channel model | UMa With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and LOSsoft; Inter-frequency correlation model is optional. | Umi With distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901, fast fading and LOSsoft; |
| System BW | 20MHz | 80MHz |
| UE speed | 30,90 km/h for study targeting measurement reduction 60,120 km/h for study targeting HO performance improvement | 30,,90 km/h for study targeting measurement reduction 60,120 km/h for study targeting HO performance improvement |
| UE distribution | 100% outdoor | 100% outdoor |
| BS Antenna Configuration | (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ | (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| BS Antenna pattern | 3-sector antenna radiation pattern, 8 dBi | TR 38.802 Table A.2.1-6, |
| UE Antenna Configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) | Antenna setup and port layouts at UE: (1, 4, 2, 1, 1, 1, 1) |
| UE Antenna pattern | Omni-direction | TR 38.802 Table A.2.1-8, |
| Tx/Rx beam | Tx: 1, Rx:1 | Tx:32, Rx:4 |
| BS Tx Power | 44dBm | 40 dBm |
| Maximum UE Tx Power | 23dbm | 23 dBm |
| BS receiver Noise Figure | 5db | 7 dB |
| UE receiver Noise Figure | 9dB | 10 dB |
| Inter site distance | 500m | 200 m |
| BS Antenna height | 25m | 10m |
| UE Antenna height | 1.5m | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 with coarse update | Procedure A in TR38.901 with coarse update |
| UE trajectory model | Option 1 | Option 1 |
| UE trajectory boundary processing model | Option 2: circle-bouncing model, | Option 2: circle-bouncing model, |
| Sampling period | 40ms | 20ms *4 = 80ms |