3GPP TSG-RAN WG2 Meeting #127bis	R2-2409873

Orlando, US, Nov. 18th – 22nd, 2024

Agenda Item:	8.6.2

Source: 	OPPO

Title:  	Discussion on open issues for inter-CU LTM

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to provide our considerations on open issues for inter-CU LTM.

Discussion

Security

RAN2 sent an LS to SA3 on security handling for inter-CU LTM in non-DC cases and 4 options were listed in the LS for further SA3 considerations. In the reply LS[1], SA3 shown their preference on Option4, i.e., after every inter-CU LTM cell switch execution with PDCP anchor change, the UE is provided with the NCC via RRC signalling to be used for key derivation at the next inter-CU LTM cell switch. They also informed that further SA3 work is required for solutions that can avoid RRC configuration/singling between cell switches, e.g., NCC in MAC or NCC preconfigured during LTM preparation phase.

In our opinion, it can be efficient to just follow SA3 preference, based on which further progress can be made in RAN2 for inter-CU LTM. Although the Option4 may involve additional RRC signalling interactions, it is not frequent since it is only required after inter-CU LTM execution. Furthermore, optimization can be considered for RRC signalling avoidance, e.g., RRC signalling with NCC value is not required for horizontal key derivation.

After every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used for key derivation at the next inter-CU LTM cell switch.

LTM fast failure recovery

In last RAN2 meeting, it was agreed that the intra-CU LTM candidate cell configuration can be applied for fast recovery after intra-CU LTM failure.

- The Rel18 handling on failure is reused in R19 if the UE selects an intra-CU LTM candidate cell after intra-CU LTM failure; for other cases, e.g. inter-CU LTM failure, the failure handling is FFS (related to SA3’s inputs).

To fully leverage the advantage of preconfigured LTM candidate cell configurations, it is beneficial to extend the supported scenarios of LTM failure recovery to RLF and inter-CU LTM failure.

LTM fast recovery for RLF and inter-CU LTM failure is supported in R19.

The UE performs cell reselection after RLF or LTM cell switch failure (in this case, the UE reverts back to the source cell configuration), if the selected cell is an LTM candidate cell, it can be either inter-CU or intra-CU compared to the source cell. Whether the LTM candidate cell configuration can be applied for recovery depends on whether the configuration is valid for next attempt and the concern mainly comes from security key update.

For intra-CU LTM candidate cell configuration, security key update is not required and the source cell security information can be reused. For inter-CU LTM candidate cell configuration, the NCC for security key update info is also available by going with security mechanism preferred by SA3. Therefore, it is proposed that both intra-CU LTM and inter-CU LTM configurations can be applied for fast failure recovery.

Both intra-CU LTM and inter-CU configurations can be applied for fast failure recovery

For R18 LTM fast recovery, the UE should perform PDCP continuation for SRB(s) after LTM cell switch failure to avoid keystream reuse issue. For instance, if the COUNT value “N” is used for transmitting the RRCReconfigurationComplete message during the first LTM attempt, the COUNT value should be continued from “N+1” for the second LTM attempt (i.e., LTM failure recovery) due to consecutive LTM attempts w/o security key update.

For R19 LTM fast recovery, the two LTM attempts can be inter-CU or intra-CU as summarized in the table below, which may result in different UE behaviour upon T304 expires.

|  | First LTM attempt | LTM fast recovery |
| --- | --- | --- |
| Case1 | Intra-CU LTM | Intra-CU LTM |
| Case2 | Intra-CU LTM | Inter-CU LTM |
| Case3 | Inter-CU LTM | Intra-CU LTM |
| Case4 | Inter-CU LTM | Inter-CU LTM |

- Case1/Case2: If the two LTM attempts are both intra-CU LTM as in Case1, the same behaviour as R18 should be followed upon T304 expires, i.e., the UE revert back to the source configuration except for the PDCP state variables for SRB(s). If an inter-CU LTM candidate cell is selected for fast recovery as in Case2, the security key update and PDCP re-establishment shall be performed, during which procedure the TX_NEXT for SRB(s) will be reset to the initial value. For Case2, the PDCP continuation for SRBs is not required upon the failure of the first LTM attempt while it also doesn’t matter if the same behaviour as for case1 is performed upon T304 expires. Since the type of LTM candidate cell selected for fast recovery is not predictable, the PDCP continuation should be executed after intra-CU LTM failure in order to support the possible occurrence of Case1.

- Case3/Case4: If the first attempt is inter-CU LTM, the UE will perform PDCP re-establishment and security key update during LTM execution. If an intra-CU LTM candidate cell is selected for fast failure recovery as in Case3, the security key of the source cell should be applied which is different from the one used for the first LTM attempt and the PDCP state variables for SRB(s) need to continue from the status of source. If an inter-CU LTM candidate cell is selected for fast failure recovery as in Case4, a new security key which is different from the one used for the source cell and the first LTM attempt will be applied. For both cases, there is no keystream reuse issue even if the UE revert back to the source cell configuration including the PDCP state variables for SRBs upon T304 expires.

For LTM fast recovery after intra-CU LTM failure, the UE revert back to the source cell configuration except for the PDCP state variables for SRB(s) upon T304 expires.

For LTM fast recovery after inter-CU LTM failure, the UE revert back to the source cell configuration upon T304 expires.

Coexistence issue

For coexistence of various types of LTM configurations, the agreement was made as follows in last RAN2 meeting.

11. RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

For above co-existence cases, the LTM configuration in one node might be impacted by the LTM execution at the other node as analysed below.

- The SCG can be released during MCG LTM execution. If SCG is released, the SCG LTM configuration (either inter-SN or inter-SN) should also be released.

- The execution of inter-CU LTM at one node may require a configuration update at the other node, which would impact the validity of the intra-CU LTM configuration at the other node. For example, there is a case that the bearer type change is performed during inter-CU LTM, e.g., from MCG bearer to SCG bearer, but the candidate cells for intra-CU LTM are prepared based on the context that it is an MCG bearer. The bearer type change is not expected during intra-CU LTM as discussed in R18, and if intra-CU LTM is executed after inter-CU LTM, it will lead to error bearer type change. To handle the issue, it is preferred to rely on the NW to guarantee the validity of LTM configuration, e.g., the LTM configuration can be released or updated by explicit NW signalling.

If the SCG is released at MCG LTM execution, the SCG LTM configuration should be released.

- Obervation1  The intra-CU LTM configuration configured in one node (i.e., MN or SN) might be invalid due to the execution of inter-CU LTM at the other node (i.e., SN or MN).

It is up to the NW to guarantee the validity of the LTM configuration for case that the serving cell configuration is updated due to the LTM execution at the other NW node (MN or SN).

LTM configuration handling

As agreed in previous RAN2 meeting, how to handle the LTM configuration after inter-CU LTM cell switch was discussed and the following agreement was achieved. In this section, we would like to further discuss the LTM configuration handling in other cases.

Handling of candidate configuration after inter-CU LTM cell switch: - As in Rel-18 LTM, the UE keeps its LTM candidate cell configurations after at least a inter-CU LTM cell switch procedure where the UE is not configured with DC, unless these are explicitly released by the network.

For R18 LTM, the handlings of LTM configuration are summarized as in the table below.

| RRC procedure | RRCRelease/go to IDLE | MR-DC release | Re-establishment (Case1: attemptLTM-Switch not configured Case2: attemptLTM-Switch configured but LTM candidate cell is not selected during T311 | PCell/PSCell change/addition (HO/CHO /CPA/CPC/SCPAC) | MCG/SCG failure |
| --- | --- | --- | --- | --- | --- |
| R18 MCG LTM config | Autonomous release | \ | Autonomous release | Keep | Keep |
| R18 SCG LTM config | Autonomous release | Autonomous release | Autonomous release | Keep | Keep |

For reception of RRCRelease/going to IDLE, we understand the same behaviour can be simply followed for R19 LTM configuration, i.e., both inter-CU MCG LTM configurations and inter-CU SCG LTM configurations are autonomously released for the two cases.

For MR-DC release, the inter-CU SCG LTM configuration also requires to be released due to the detachment from the NW node (i.e., SN) to trigger the SCG LTM procedure. Compared to NW-triggered release, UE autonomous release is preferred as in R18 for signalling efficiency. Companies may have concern on the feasibility of UE autonomous release in this case since both inter-CU MCG LTM and inter-CU SCG LTM are configured in MN format and the UE may not be able to distinguish the inter-CU SCG LTM configurations. The case also exists for inter-SN CPC and the location of reconfigurationwithsync IE is used for distinction when autonomously releasing the inter-SN CPC candidates. The same approach can be followed for inter-CU SCG LTM.

For RRC re-establishment, it is still not clear how the R19 LTM configuration can be applied for fast recovery as discussed in section 2.2. The handling of LTM configuration for this case can be studied when further progress is made for R19 LTM failure recovery.

The R19 inter-CU MCG LTM configuration is autonomously released for the following cases

  - Reception of RRCRelease.

  - Go to IDLE.

The R19 inter-CU SCG LTM configuration is autonomously released for the following cases

  - Reception of RRCRelease.

  - Go to IDLE.

  - MR-DC release.

For PCell change/PSCell change or addition as well as MCG/SCG failure, the LTM configuration can be kept unless explicitly released by the NW, i.e., it is up to the NW to determine how to handle the LTM configurations for these cases. In our understanding, the same principle can be followed for R19 LTM.

The R19 inter-CU LTM configurations are kept for the following cases unless explicitly released by the NW.

  - Normal and conditional PCell change/PSCell change/addition.

  - MCG failure/SCG failure

LTM execution

As agreed in last RAN2 meeting, the new Rel-19 ID and R18 ltm-NoRestID are applied for determination of L2 reset operations.

- For non-DC case, if the new Rel-19 IDs of the serving cell and the target cell have same values, the UE compares the ltm-ServingCellNoResetID and ltm-NoResetID and performs the corresponding L2 reset operation as defined in Rel-18.

The agreement was made for non-DC case. While for DC case, L2 reset operation relying on the two IDs may not be enough. To be specific, there can be radio bearer type change during LTM execution for DC case and the UE needs to perform L2 handling for the involved bearer even if there is no change of Rel-19 ID or ltm-NoResetID. The L2 operations for each case are shown as the table below which is summarized according to Table A-1 in TS37.340.

| Type of change | RLC | PDCP |
| --- | --- | --- |
| Termination point change w/o bearer type change | Re-establishment | Re-establishment |
| Termination point change w/ bearer type change | Release/establishment/re-establishment | Re-establishment |
| No termination point change w/ bearer type change | Release/establishment | PDCP data recovery |

For DC case, bearer type change and termination point change should be considered for L2 handling during LTM execution.

For a radio bearer (i.e., SRB or DRB) with termination point change, the UE performs PDCP re-establishment and RLC establishment (if the RLC bearer maintains).

For a radio bearer (i.e., SRB or DRB) with bearer type change, the UE perform RLC release/establishment and PDCP data recovery (if it is an AM DRB and there is RLC bearer release).

SCG handling

For SCG handling at inter-CU MCG LTM execution, the following agreement was made in RAN2#127:

Inter-CU MCG LTM:

- SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

It is still open whether the coexistence of inter-CU MCG LTM with SCG release and inter-CU MCG LTM with SCG reconfiguration is supported, i.e., the SCG can be either released or reconfigured and these candidates can be simultaneously configured to a UE. In this case, the NW may firstly trigger an MCG LTM with SCG release, after which, the NW triggers an MCG LTM with SCG reconfigured. Since there is no SCG for the latter LTM execution, SCG addition needs to be performed.

To support subsequent LTM, the inter-CU MCG LTM candidate cell configuration for DC case should include both MCG part and SCG part if the SCG is expected to be unchanged/reconfigured. The UE always applies a complete RRC configuration if the target is configured with DC, thus there is no difference to perform SCG addition/SCG reconfiguration from UE side. Based on this understanding, it is proposed that the coexistence of inter-CU MCG LTM with SCG release and inter-CU MCG LTM with SCG reconfiguration can be supported.

Coexistence of inter-CU MCG LTM with SCG release and inter-CU MCG LTM with SCG reconfiguration can be supported.

SCG addition is supported during inter-CU MCG LTM execution.

Conclusion

Based on the discussion in section2, we have following observation proposals:

Proposal 1	After every inter-CU LTM cell switch execution that changes the PDCP anchor, the UE is provided with the NCC value via RRC signalling to be used for key derivation at the next inter-CU LTM cell switch.

Proposal 2	LTM fast recovery for RLF and inter-CU LTM failure is supported in R19.

Proposal 3	Both intra-CU LTM and inter-CU configurations can be applied for fast failure recovery

Proposal 4	For LTM fast recovery after intra-CU LTM failure, the UE revert back to the source cell configuration except for the PDCP state variables for SRB(s) upon T304 expires.

Proposal 5	For LTM fast recovery after inter-CU LTM failure, the UE revert back to the source cell configuration upon T304 expires.

Proposal 6	If the SCG is released at MCG LTM execution, the SCG LTM configuration should be released.

Obervation1  The intra-CU LTM configuration configured in one node (i.e., MN or SN) might be invalid due to the execution of inter-CU LTM at the other node (i.e., SN or MN).

Proposal 7	It is up to the NW to guarantee the validity of the LTM configuration for case that the serving cell configuration is updated due to the LTM execution at the other NW node (MN or SN).

Proposal 8	The R19 inter-CU MCG LTM configuration is autonomously released for the following cases

a.	Reception of RRCRelease.

b.	Go to IDLE.

Proposal 9	The R19 inter-CU SCG LTM configuration is autonomously released for the following cases

a.	Reception of RRCRelease.

b.	Go to IDLE.

c.	MR-DC release.

Proposal 10	The R19 inter-CU LTM configurations are kept for the following cases unless explicitly released by the NW.

a.	Normal and conditional PCell change/PSCell change/addition.

b.	MCG failure/SCG failure

Proposal 11	For DC case, bearer type change and termination point change should be considered for L2 handling during LTM execution.

Proposal 12	For a radio bearer (i.e., SRB or DRB) with termination point change, the UE performs PDCP re-establishment and RLC establishment (if the RLC bearer maintains).

Proposal 13	For a radio bearer (i.e., SRB or DRB) with bearer type change, the UE perform RLC release/establishment and PDCP data recovery (if it is an AM DRB and there is RLC bearer release).

Proposal 14	Coexistence of inter-CU MCG LTM with SCG release and inter-CU MCG LTM with SCG reconfiguration can be supported.

Proposal 15	SCG addition is supported during inter-CU MCG LTM execution.

Reference

[1] S3-244317 Reply LS on security key update of inter-CU SCG LTM