3GPP TSG-RAN WG2 Meeting #128	R2-2409883

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.4.4

Source:	Nokia, Nokia Shanghai Bell

Title:	LP-WUS in RRC_CONNECTED

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#127bis:

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

In RAN1#118bis, also made some progress for the connected mode as below:

Agreement - For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer - UE is configured with legacy C-DRX configurations as Rel-18 - UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP) - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification Agreement Confirm following working assumption Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions. Agreement For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported candidate value - UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional. - FFS: A single time offset is configured for a UE at a given time. Agreement LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable Agreement For LP-WUS CONNECTED mode operation, Option 1-2 can be configured separately from Option 1-1 - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Agreement Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective, - Opt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions - Introducing monitoring window is not precluded

We continue the discussion of LP-WUS in RRC_CONNECTED.

# 2	Discussion

## 2.1	Option 1-3

Option 1-1 and 1-2 are being actively discussed in RAN1 and RAN2, and the detailed procedures are gradually taking shape. However, option 1-3 has not yet been discussed.

Option 1-3 is understood that the UE monitors LP-WUS inside C-DRX active time. As concluded from the study [TR 38.869], the power saving gain is expected by reducing unnecessary PDCCH monitoring within C-DRX active time. However, there may be limited use of this option 1-3 considering the length of On duration [R2-2404380].

In our view, we already have some mechanism to end or reduce unnecessary PDCCH monitoring during active time, e.g., sending DRX command MAC CE or PDCCH skipping. Thus, it would be desirable to focus more on first two options, i.e., option 1-1 and option 1-2.

Proposal 1: Option 1-3 is excluded in RAN2.

## 2.2	Option 1-2

### 2.2.1	drx-onDurationTimer

In option 1-2, RAN2 agreed that LP-WUS monitoring is performed at least outside the legacy C-DRX Active Time. RAN2 further discussed if the UE monitors LP-WUS outside the ON-Duration with no conclusion made yet since it was considered to be subject to whether drx-onDurationTimer is maintained or not in option 1-2.

In DCP, the network can configure ps-TransmitOtherPeriodicCSI and ps-TransmitPeriodicL1-RSRP for report of periodic CSI/L1-RSRP:

ps-TransmitPeriodicL1-RSRP Indicates the UE to transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic L1-RSRP report(s) when the drx-onDurationTimer does not start. ps-TransmitOtherPeriodicCSI Indicates the UE to transmit periodic CSI report(s) other than L1-RSRP reports when the drx-onDurationTimer does not start (see TS 38.321 [3], clause 5.7). If the field is absent, the UE does not transmit periodic CSI report(s) other than L1-RSRP reports when the drx-onDurationTimer does not start.

As specified in TS 38.321, if these parameters are not configured with the value true, the UE shall not report CSI on the On-Duration where drx-onDurationTimer is not started. If drx-onDurationTimer started by wake-up signal, then the UE will report the CSI on On-Duration where drx-onDurationTimer is running or on Active Time.

Observation 1: For DCP, the parameters (ps-TransmitOtherPeriodicCSI and ps-TransmitPeriodicL1-RSRP) controls the report of the periodic CSI/L1-RSRP on the On-Durations where drx-onDurationTimer is not started. If drx-onDurationTimer is started by DCP indication, the CSI is reported during the DRX active time or On-Duration where drx-onDurationTimer is running.

The benefit of such configurability is that if UE keeps DCP monitoring for a long time (no wake-ups), UE would still provide reports for the CSI to the network. Therefore, RAN1 agreed to support the same configurability for option 1-1 (in RAN1#118bis) and option 1-2 (in RAN1#118). It is also noted by RAN1 that, for option 1-2, Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification.

In the current specification, CSI report can be restricted to On-Duration with the use of CSI mask. It is specified in TS 38.321 that the UE shall not report CSI if it is predicted well in advance that drx-onDurationTimer is not running. Note that having such condition, i.e., if it is predicted well in advance, is to allow for the report of the prepared CSI in cases where there was insufficient prior prediction that drx-onDurationTimer would not be running.

Observation 2: In the legacy, during C-DRX active time, CSI report can be restricted to On-Duration where drx-onDurationTimer is running.

In RAN2#127bis, it was argued that drx-onDurationTimer is unnecessary for option 1-2 because the UE monitors PDCCH based on other active timers, such as drx-InactivityTimer and drx-RetransmissionTimerDL/UL. From a PDCCH monitoring perspective, it may be okay not to have drx-onDurationTimer. However, given the above observations, drx-onDurationTimer remains necessary, as the current CSI reporting can depend on whether drx-onDurationTimer is running and this approach is decided to be followed in option 1-2 as well.

Proposal 2: In option 1-2, drx-onDurationTimer is maintained.

With proposal 2, it would make sense that drx-onDurationTimer starts while the UE is in Active Time and drx-onDurationTimer does not start while the UE is not in Active Time, i.e., while the UE monitors LP-WUS.

However, RAN2 may need to clarify whether or when drx-onDurationTimer starts while the newly introduced timer (tentatively drx-lpwusInactivityTimer) is running. Although RAN2 agreed that the UE is considered to be in Active Time while drx-lpwusInactivityTimer is running, this duration is a kind of transition period and drx-lpwusInactivityTimer could expire without the UE receiving any PDCCH. In this case, the expiration of drx-lpwusInactivityTimer might not coincide with the end of drx-onDurationTimer, raising a question of whether the UE should continue monitoring until the end of drx-onDurationTimer or stop drx-onDurationTimer together. To simplify the UE behavior, it might be beneficial to start drx-onDurationTimer only after the UE receives a PDCCH.

Proposal 3: In option 1-2, the UE resumes start of the drx-onDurationTimer only after receiving a PDCCH while a newly introduced timer (entatively named drx-lpwusInactivityTimer) is running.

### 2.2.2	New timer (drx-lpwusInactivityTimer)

In RAN1, time window containing multiple MOs is under discussion, which might impact the start time of the new timer (drx-lpwusInactivityTimer). For instance, drx-lpwusInactivityTimer could start either at the end of the time window, i.e., at the last MO, or immediately upon detection of the LP-WUS indication (without waiting until the end of time window).

In our understanding, the network can select any MO, even multiple MOs, within the time window. In this case, it would be safe to start the new timer at the end of time window to make sure that the active time is aligned between the UE and the network.

Proposal 4: In option 1-2, with the time window containing multiple MOs, the UE starts the new timer (tentatively named drx-lpwusInactivityTimer) at the last MOs of the window.

### 2.2.3	CG/SPS

It has not yet been discussed whether the MR keeps the SPS and CG activated while the UE monitors the LP-WUS.

If SPS and CG are kept activated, transmission or reception of MAC PDU on SPS/CG will start the corresponding timers, i.e., drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL, which might then keep the MR awake. To save more power, it might be better to deactivate SPS and CG. As it is possible to deactivate SPS and CG by PDCCH and activation/deactivation MAC CE, respectively, it seems not necessary to define additional deactivation mechanism such as the UE by itself deactivate the SPS or CG when starting LP-WUS monitoring.

Proposal 5: In option 1-2, when the UE starts LP-WUS monitoring, SPS and CG are kept as it is unless activated or deactivated by the network explicitly.

If SPS and CG are deactivated while monitoring LP-WUS, the gNB needs to send the LP-WUS to wake up the MR first and then activates the SPS/CG. This would introduce additional latency because the UE waits for receiving the activation command after starting the PDCCH monitoring. Moreover, the gNB may intend to rely on SPS/CG scheduling instead of dynamic scheduling in the MR, in which cases further PDCCH monitoring after SPS/CG activation would be needlessly consuming power.

Thus, RAN2 needs to discuss how to reduce unnecessary PDCCH monitoring when using SPS/CG. For instance, we can consider that the LP-WUS indicates SPS/CG activation.

Proposal 6: For option 1-2, RAN2 to discuss the LP-WUS impact activation of SPS and CG Type 2.

## 2.3	Configurability

It is FFS (in RAN1) whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE.

One may argue that option 1-1 and option 1-2 are mutually complementary because option 1-1 is to start drx-onDurationTimer based on the LP-WUS indication and option 1-2 is to start active time outside Active Time (other than On-Duration); hence they should be able to be configured simultaneously.

In our view, the two options differ in the gains they aim to achieve; Option 1 aims to reduce the power consumption by selectively monitoring On-Duration based on LP-WUS indication. Meanwhile, option 2 enables immediate PDCCH monitoring via LP-WUS indication, i.e., without waiting for On-Duration, which can further reduce the latency compared to starting to schedule based on the legacy On-Duration. In addition, the gain by configuring two options together is unclear as the new timer starts anyway (by considering the offset) in option 1-2, which may overlap with the On-Duration.

In addition, we are still in the discussion phase regarding how each option operates, so there is no analysis yet on what issues may arise when both options operate together, the level of complexity, or the specific configurations/UE behavior required. For instance, in option 1-2, the new timer will start an offset after LP-WUS indication, and in option 1-1, the MOs will be set an offset before On-Duration. Therefore, if both options are configured, questions arise about how to set the MOs, whether ON-Duration should also begin when the new timer already starts, and other related issues.

Given these considerations, there doesn’t seem to be sufficient rationale to support such flexibility of configuring both options together. Similarly, there seems to be no practical use case to switch between these two options.

Proposal 7: Not support configuring both of option 1-1 and option 1-2 at the same time.

Proposal 8: Not support switching between option 1-1 and option 1-2.

# 3	Conclusion

Proposal 1: Option 1-3 is excluded in RAN2.

Observation 1: For DCP, the parameters (ps-TransmitOtherPeriodicCSI and ps-TransmitPeriodicL1-RSRP) controls the report of the periodic CSI/L1-RSRP on the On-Durations where drx-onDurationTimer is not started. If drx-onDurationTimer is started by DCP indication, the CSI is reported during the DRX active time or On-Duration where drx-onDurationTimer is running.

Observation 2: In the legacy, during C-DRX active time, CSI report can be restricted to On-Duration where drx-onDurationTimer is running.

Proposal 2: In option 1-2, drx-onDurationTimer is maintained.

Proposal 3: In option 1-2, the UE resumes start of the drx-onDurationTimer only after receiving a PDCCH while a newly introduced timer (entatively named drx-lpwusInactivityTimer) is running.

Proposal 4: In option 1-2, with the time window containing multiple MOs, the UE starts the new timer (tentatively named drx-lpwusInactivityTimer) at the last MOs of the window.

Proposal 5: In option 1-2, when the UE starts LP-WUS monitoring, SPS and CG are kept as it is unless activated or deactivated by the network explicitly.

Proposal 6: For option 1-2, RAN2 to discuss the LP-WUS impact activation of SPS and CG Type 2.

Proposal 7: Not support configuring both of option 1-1 and option 1-2 at the same time.

Proposal 8: Not support switching between option 1-1 and option 1-2.