3GPP TSG-RAN WG2 Meeting #128                      R2-2409885

Orlando, USA, Nov.18th – Nov.22nd, 2024

Agenda Item:	8.2.4

Source: 	Transsion Holdings

Title:  	Discussion on Random Access for A-IoT

Document for:	Discussion and Decision

- Introduction

In the last RAN2#127b meeting, the A-IoT Random Access was discussed and the following agreements were made[1]:

Agreements for Random Access

- In case of D2R data transmission failure, device follows the reader instruction:

	it is supported to re-access in another opportunity controlled/provided by the reader (i.e. retry the random access); and

	reader can repeat the R2D “upper layer command” to trigger the device to re-send the same D2R “response” (i.e., device just follows the received R2D to transmit D2R).

2	Support optional explicit R2D failure/success feedback indication for at least MSG3 for re-access purpose.  FFS for following D2R data.

3	For 2step CBRA, if mgs2 is not received by the device, the device is not expected to autonomously re-access.  The re-access is always controlled by reader.

4	For 2step and 3step CBRA, The A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure.

5	R2D message is used to provide access occasion(s) which can be used for re-access purpose. One option to be captured in the TR is that this R2D message is AIoT paging message.   FFS additional indications to differentiate if needed in paging message.    FFS if other message can be used.

6	From RAN2 perspective, at least for TDMA the device can randomly select one access occasion for A-IoT Msg1 within the access occasions provided/assigned by the reader, as the baseline for CBRA.  FFS if this is applicable to FDMA.  Further enhancement option(s) can be considered after more RAN1 progress on TDMA/FDMA.

In this contribution, we continue to discuss the Random Access for A-IoT.

- Discussion

2.1 A-IoT contention-free access procedure

In last meeting the procedure of CFRA for AIoT device was discussed with which the device sends Device ID and/or any other upper layer data in MSG1, the reader may echo some information from MSG1 in MSG2. For the contention-free access procedure RAN2 will study single and multi-device case. For single and multi-device case, as the number of devices is already known by the reader, when trigger the device to access, the corresponding access configuration can also be assigned to each device to increase efficiency, the access configuration can include time and frequency configuration.

Proposal 1: The device id list and corresponding access configuration list can be included in the initial trigger message.

In last meeting, it has been agreed that for 2step and 3step CBRA, the A-IoT device performs re-access in another opportunity provided by the reader (i.e. retry the random access) at least in case of contention resolution failure. For contention-free access, although there is no contention between devices, the device may failed to access due to the poor signal. To increase the access efficiency, the re-access configuration can also be provided by the reader for contention-free access, the device can use the re-access configurtion to re-access if contention-free access failed without the reader re-initiated trigger.

- Proposal 2: For contention-free access, the devices can re-access in another opportunity provided by the reader without re-trigger.

- 3 Conclusions

In this contribution we discussed issues related to A-IoT Random Access and made the following proposals:

Proposal 1: The device id list and corresponding access configuration list can be included in the initial trigger message.

Proposal 2: For contention-free access, the devices can re-access in another opportunity provided by the reader without the reader re-trigger..

- 4 References

- “Draft_RAN2_127b_Meeting_Report_v1”