3GPP TSG-RAN WG2 Meeting #128	 R2-2409904

Orlando, USA, Nov 18-22, 2024

Agenda Item:	8.4.4

Source: 	NEC

Title:  	LP-WUS in RRC_CONNECTED

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#127b meeting, the following agreements are made:

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2.

In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring.

When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

In this contribution, we further discuss how to design LP-WUS applied in RRC_CONNCTED

- 2 Discussion

- 2.1 Option 1-2

For option 1-2, the baseline is that LP-WUS monitoring is performed outside of legacy C-DRX active time, and the UE will not monitor PDDCH within drx-onDurationTimer duration. Question is that in this case whether we still need to start drx-onDurationTimer or not since there is no PDCCH monitoring during this timer.

In our understanding, the legacy drx-onDurationTimer impact not only PDCCH monitoring behavior, but also other aspects like SRS transmission and CSI report. For example, if CSI masking (csi-Mask) is setup by RRC, the UE can only report CSI on PUCCH during drx-onDurationTimer is running. Therefore, to avoid other potential spec impact, it is suggested that drx-onDurationTimer can also be started for other potential UE behaviors, but not for PDCCH monitoring if only option 1-2 is configured.

Proposal-1: for the case of only option 1-2 is configured, the drx-onDurationTimer still need to be started (to avoid other spec impact, e.g., SRS/CSI), but when this timer is running the UE is not required to monitor PDCCH.

One more question is that in last meeting, several companies mentioned that whether a new drx-InactivityTimer is needed or not considering legacy drx-InactivityTimer could trigger short C-DRX cycle. However in our understanding, before this question, we need to think about whether the short C-DRX can be configured with LP-WUS simultaneously in the case of only option 1-2 is configured.

In option 1-2, we already agreed that drx-onDurationTimer, i.e., long C-DRX cycle, will not be used for PDCCH monitoring, instead, the UE trigger PDCCH monitoring during a new timer only when LP-WUS is detected or when other potential active timer (except for drx-onDurationTimer) is running. When it comes to short C-DRX cycle, in legacy behavior, if drx-InactivityTimer is expired or DRX Command MAC CE is received, short C-DRX cycle will be used, and drx-onDurationTimer will be started based on short C-DRX cycle. If we still reuse this legacy behavior, it seems a little weird that PDCCH monitoring is not followed based on long C-DRX cycle, but followed based on short C-DRX cycle. Therefore, it is suggested that the network does not configure LP-WUS with short C-DRX cycle simultaneously if it is the case of only option 1-2 is configured.

Proposal-2: for the case of only option 1-2 is configured, network does not configure LP-WUS with short C-DRX cycle simultaneously.

- 2.2 Combination solution

Agreement

Confirm following working assumption

Working Assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

- Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

- Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

- FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note: Above can be revisited considering RAN2 decisions.

In last RAN1 meeting, RAN1 confirmed that both option 1-1 and 1-2 are supported, now we have two clear solution, solution 1-1 is similar to DCP mechanism, and solution 1-2 is to monitor LP-WUS outside of active timer to trigger a new active timer for PDCCH monitoring. There is one FFS to consider whether/how to combine option 1-1 and option 1-2.

To our understanding, there is no problem to support a combination of optional 1-1 and 1-2, because no matter the active timer is drx-onDurationTimer or new active timer, there is no unnecessary power consumption due to the fact that both active timer should be control by LP-WUS triggering. Furthermore drx-onDurationTimer should better be configured as it reflect UE traffic mode which is determined by NW implementation, and new active timer triggered by LP-WUS could alleviate the latency issue. To us, we are still feeling surprised that the classic drx-onDurationTimer cannot be used for PDCCH monitoring if we go for the only option 1-2 solution, it seems like the C-DRX is not configured.

One issue to support this combination is how to configure LP-WUS mapping relationship between new active timer and drx-onDurationTimer. Some LP-WUS will trigger a new active timer, and some LP-WUS will trigger drx-onDurationTimer. Do we need two types of LP-WUS occasion configuration (i.e., part of LP-WUS is offset specific to drx-onDurationTimer, part of LP-WUS is located anywhere outside of C-DRX active time)? We think it makes things complicated and not needed. In fact we can just provide only one LP-WUS configuration, and the periodicity of C-DRX cycle is integer multiple of periodicity of LP-WUS. Then the closet LP-WUS to next drx-onDurationTimer will be used to trigger drx-onDurationTimer and other LP-WUS is used to trigger new active timer. Details about what is closet LP-WUS could be FFS. The whole picture is shown as Fig.1.

Fig.1 combination of option 1-1 and 1-2

Proposal-3: support both Option 1-1 and Option 1-2 simultaneously configured for the same UE, i.e., LP-WUS can be used to trigger both new active timer and legacy drx-onDurationTimer, when the new active timer and legacy drx-onDurationTimer is running, the UE monitors PDCCH.

- 3 Conclusions

Proposal-1: for the case of only option 1-2 is configured, the drx-onDurationTimer still need to be started (to avoid other spec impact, e.g., SRS/CSI), but when this timer is running the UE is not required to monitor PDCCH.

Proposal-2: for the case of only option 1-2 is configured, network does not configure LP-WUS with short C-DRX cycle simultaneously.

Proposal-3: support both Option 1-1 and Option 1-2 simultaneously configured for the same UE, i.e., LP-WUS can be used to trigger both new active timer and legacy drx-onDurationTimer, when the new active timer and legacy drx-onDurationTimer is running, the UE monitors PDCCH.

- 4 References