3GPP TSG-RAN WG2 Meeting #128	 R2-2409905

Orlando, USA, Nov 18-22, 2024

Agenda Item:	8.1.3

Source: 	NEC

Title:  	AIML PHY NW-side data collection

Document for:	Discussion and Decision

- 1 Introduction

In last RAN2#127b meeting, the following agreements are made:

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

Based on the achieved agreement, we further discuss remain issue related to report.

- 2 Discussion

- 2.1 Considerations on data collection report

For data collection report, RAN2 has agreed that on-demand reporting will be specified and periodical reporting will not be supported. And for event-based reporting, there is no consensus in RAN2.

For the details of on-demand reporting, the procedure of UEInformationRequest/UEInformationResponse is reused, but one question is that when does NW trigger on-demand reporting? In our understanding, since the data is logged by UE itself, the UE has the knowledge and could send some assistance information to NW for facilitating NW triggering, therefore, availability indication of logged data is introduced. One FFS is that what trigger/definition of availability indication is.

Back to last meeting, AS buffer event-based reporting is discussed, it means that when the logged data size reach to UE buffer size limitation, the UE will directly report the logged data to NW. However we didn’t make a consensus on this part since companies think on-demand reporting is enough. But when it comes to the triggering of availability indication, the same principle can be considered, it means when the logged data size reach to UE buffer size limitation, the UE could just send availability indication (instead of the logged data) to NW, then the following on-demand reporting can be triggered by NW based on UE availability indication. In our understanding, this is reasonable to consider such kind of combination between on-demand reporting and event-based reporting, reason is that only event-based reporting could make a large amount of data reported to NW without considering NW overhead, while only on-demand reporting is lack of UE assistance information which makes NW a little bit difficult to make data collection decision. Therefore it is promising to support AS buffer event-based availability indication, i.e., when the logged data size reach to UE buffer size limitation, the UE send the availability indication to NW, then the NW can facilitate on-demand data collection.

Proposal-1: support AS buffer event-based availability indication, i.e., when the logged data size reach to UE buffer size limitation, the UE send the availability indication to NW.

- 2.2 Release condition of data collection configuration

We agreed that immediate MDT is the baseline to support data collection for both gNB-side and OAM-side. Immediate MDT configuration is reusing the existing measurement framework, which means the configuration is provided by RRCReconfiguration. If the data collection is done, the NW could explicitly release the configuration. Besides explicit way, we think some mechanism from logged MDT can be referred, i.e., using maximum logging duration, or maximum data size required by NW could be considered to let UE release the configuration automatically, in this way, NW-UE signalling interaction could be reduced. For example, the NW could configure the UE to perform data collection for 12 hours (e.g., at night), when timer is expired, the data collection configuration could be released. Another example, the NW could configure the UE to perform data collection up to X MB data, and the configuration could be released once the UE report data up to X MB data.

Proposal-2: maximum logging duration or maximum data size can be considered to release data collection configuration, i.e., when the UE has already reported data up to max duration/size, the UE release the configuration.

- 3 Conclusions

Proposal-1: support AS buffer event-based availability indication, i.e., when the logged data size reach to UE buffer size limitation, the UE send the availability indication to NW.

Proposal-2: maximum logging duration or maximum data size can be considered to release data collection configuration, i.e., when the UE has already reported data up to max duration/size, the UE release the configuration.

- 4 References