3GPP TSG-RAN WG2 Meeting #128	R2-2409921

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

# Introduction

Recently in RAN2#127bis, the following agreement were made for the procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE.

- If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). - The LPWUS monitoring exit condition does not include MR measurements. - For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion. - For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping. - RAN2 inform this conclusion to SA2/CT1/RAN3. - This LS is approved in R2-2409225

In this contribution, we continue to discuss the related issues considering the progress in RAN1/2/4 so far.

# Discussion

## Configuration Aspects of LP-WUS

### Dedicated configuration in RRC Signalling

RAN2 has reached the agreement that the LP-WUS related configuration for IDLE/INACTIVE state is provided at least via system information and whether dedicated signaling for configuration is still FFS.

The LP-WUS related configuration should be cell specific since it is for IDLE/INACTIVE state. The configuration includes both configuration for LP-SS and LP-WUS thus covering both OOK-based and OFDM-based receivers respectively. Providing dedicated configuration in RRC signalling for example in RRC Release limits the scenarios in which LP-WUS can be used. For example, when an LP-WUS UE is provided with dedicated configuration and if the UE selects a new cell due to mobility, the new cell is not aware of the dedicated configuration of the LP-WUS for the UE and thus the UE still needs to rely on the system information from the cell. Thus, there is no need for dedicated configuration.

Proposal 1: Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

### Enabling of LP-WUS

If LP-WUS functionality is enabled for all the UEs supporting LP-WUS in a camped cell, for example, when a UE receives LP-WUS configuration via SIB from the camped cell, it can use LP-WUS functionality. It is simple but may lead to a lot of UEs using LP-WUS, which means the false paging probability increases and causes more transitions from LP-WUR to MR. This will reduce the power saving gain or even result in negative power saving gain considering the maximum number of subgroups for LP-WUS has been decided by RAN1 to be 32. Hence there can be UE/group specific control for enabling LP-WUS.

To reduce false paging probability, the network (CN or RAN) decides whether a LP-WUS capable UE is allowed to use the LP-WUS functionality based on the conditions in the network (e.g., radio resources) and/or the characteristics of the UEs (e.g., paging probability, shorter DRX cycle, etc.). For example, the CN uses NAS signaling to indicate whether a specific UE can use LP-WUS functionality, which is similar to Rel-17 CN-assigned subgrouping method. To minimize the impact for deployment, the UE by default can use LP-WUS functionality unless a specific indication to disable the LP-WUS functionality is received from CN, which implies that the absence of indication means UE is allowed to use LP-WUS functionality. In this case, if the CN does not implement this feature, LP-WUS can still be implemented as an AS solution; if CN prefers to have precise control, CN can implement the control indication in NAS signalling.

Proposal 2: The CN indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality by NAS signalling; the absence of indication means UE is allowed to use LP-WUS functionality.

### Configuration for different types of LP-WUR

RAN1 has been discussing to support both OOK-based and OFDM-based receivers for LP-WUS monitoring. Since the OFDM-based LP-WUR has stronger capability for LP-WUS signal reception and processing, it is possible for NW to configure a shorter LP-WUS to wake up only OFDM-based receiver to reduce the resource overhead. In this case, the UE with OFDM-based receiver can benefit from the low-complexity operation on LP-WUS, e.g., to cancel any OOK sequence/pattern detection or implement termination of LP-WUS procedure by early detection. The UE with OOK-based receiver will skip the current LP-WUS for obtaining more power saving gain. Thus, the NW should be able to support the scenario that only UEs with OFDM-based LP-WUR are paged, e.g., by sending a shorter LP-WUS that cannot be received by OOK-based receiver. This information can be implicitly indicated by the absence of LP-WUS configuration for OOK-based receiver.

Proposal 3: NW should configure whether only OFDM-based LP-WUR is supported in a cell.

### LP-WUS entry/exit condition

In RAN2#127, RAN2 has agreed that the thresholds of LP-WUS entry/exit condition should be separately configured for different types of receiver. Considering the LP-WUS cell coverage cannot reach the full coverage of legacy cell as specified in the WID [1], the feasibility of LP-WUS feature may be greatly affected if the entry/exit condition is absent in the LP-WUS configuration, i.e., to leave when to enter/exit LP-WUS mode to UE implementation, since UE will spend plenty of time and resource overhead to decide whether LP-WUS can be reliably used. Therefore, it is beneficial that the thresholds of LP-WUS entry/exit condition are always carried in the LP-WUS related configuration as a reference for UE to determine the LP-WUS coverage with more precision. Thus, we propose that the thresholds of LP-WUS entry/exit conditions for a specific type of receiver should be always configured if supported by the network.

Proposal 4a: Thresholds of LP-WUS entry/exit conditions for OOK-based receivers are always configured when OOK-based LP-WUS configuration is provided by the network.

Proposal 4b: Thresholds of LP-WUS entry/exit conditions for OFDM-based receivers are always configured when OFDM-based LP-WUS configuration is provided by the network.

RAN2 has also discussed on whether the entry condition can be also based on the measurement result of LP-WUR in RAN2#127 and RAN2#127bis, and made the following working assumption and agreements.

Working Assumption in RAN2#127 - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. Agreement in RAN2#127bis - If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). - The LPWUS monitoring exit condition does not include MR measurements.

UE usually performs RRM measurements by MR as per legacy behavior. Thus, the serving cell measurements by MR are always available and good enough. As the hardware capability of MR is better than that of LP-WUR, the measurements by LP-WUR do not provide any additional benefit for LP-WUS entry decision. On the other hand, using both MR and LP-WUS measurements can make the LP-WUS entry procedure complicated, introducing much specification impact. Hence, the measurement by LP-WUR is not needed for LP-WUS entry condition and it can be left to NW to decide whether the threshold based on LP-WUR is needed for UE to enter LP-WUS monitoring.

Proposal 5: For LP-WUS entry condition, the threshold based on MR is mandatorily configured, while the threshold based on LP-WUR is optional.

If the NW configures the LP-WUS entry thresholds for both MR and LP-WUR measurements, it should be up to UE implementation to obtain the measurement results on LP-WUR, i.e., the metrics and requirement of LR measurement should wait for RAN4’s discussion and there is no need to define in RAN2 spec the specific UE behavior on LP-WUR measurement for LP-WUS entry condition.

Proposal 6: If the LP-WUS entry threshold based on LP-WUR is configured, it is up to UE implementation how to obtain the LP-WUR measurement results used for LP-WUS entry condition.

## Issues for paging monitoring triggered by LP-WUS

The primary purpose of LP-WUS in RRC_IDLE/INACTIVE is to indicate paging monitoring. RAN1 made the following related agreements in RAN1#116:

RAN1#116 Agreements - For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not. - It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO - Conclusion For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

Observation 1: Both options of UE monitoring legacy paging DCI/paging and monitoring PEI after MR wakes up are supported in RRC_IDLE/INACTIVE.

LP-WUS subgroup design is one of the main issues to be considered for LP-WUS paging design and RAN2 has agreed in RAN2#127bis as below that Rel-17 PEI subgrouping method can be reused.

- For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion. - For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

### LP-WUS and PEI

When LP-WUS and PEI are both configured and used, there are two cases of potential subgrouping types possible for LP-WUS and PEI.

Case 1 - Same type of subgrouping method used for LP-WUS and PEI: In this case, both LP-WUS and PEI use either CN assigned subgrouping or UE_ID based subgrouping. For CN assigned subgrouping, CN can ensure proper subgrouping of the LP-WUS and PEI by implementation to reduce the power consumption. For UE_ID based subgrouping, it is straightforward to support it without CN control.

Case 2 - Different type of subgrouping method used for LP-WUS and PEI: In this case, CN assigned subgrouping is applied for LP-WUS and UE_ID based subgrouping is applied for PEI and vice-versa. We do not see any functional issues other than a slight impact on the power saving gain. Hence what type of subgrouping to apply for LP-WUS and PEI can be left to NW implementation.

Proposal 7: If LP-WUS and PEI are used together, it is up to NW to decide which types of subgrouping (i.e., CN assigned subgrouping or UE_ID based subgrouping) are applied.

### LP-WUS subgrouping

In practice, not all UEs in a subgroup are paged. When the UEs that are not paged in the subgroup wake-up (i.e., false alarm wake-up), power consumption for these UEs increases. If LP-WUS subgrouping is used together with the existing subgrouping for PEI, the false alarm wake-up for the UEs can be reduced. In this method, referred to as two-level subgrouping, UEs in the first level subgrouping monitor LP-WUS. And the UEs in the first level subgrouping are further distributed to different subgroups in the second level subgrouping.

The two-level UE_ID based subgrouping is shown in Figure 1. UE1 and UE2 are placed in the same group assigned with group ID 1 in LP-WUS as the first-level subgrouping. Then in the second-level subgrouping in PEI, UE1 and UE2 are placed in different groups. When UE1 is paged, UE2 does not need to monitor paging based on its PEI so that its paging false alarm can be further reduced.

Figure 1: Illustration of 2-level subgrouping

Observation 2: The paging false alarm can be further reduced if UEs in one LP-WUS subgroup can be further divided into different PEI subgroups.

To achieve the two-level subgrouping described above, when UE_ID based subgrouping is used for both LP-WUS and PEI, the UE_ID based subgrouping for LP-WUS should be independent of that for PEI subgrouping. The formula of UE_ID based subgrouping for PEI in TS 38.304 can be taken as base line for LP-WUS using LP-WUS related parameters:

SubgroupID_LP-WUS = (floor(UE_ID/(N*Ns)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS - subgroupsNumForUEID_LP-WUS),

In the latest RAN1#118bis, a Working Assumption was made that the maximum number of LP-WUS subgroups per PO is finalized to be 32, i.e., up to 5 bits are used in the UE ID for LP-WUS subgrouping.

RAN1# 128bis Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 32.

Assuming the number of UE_ID based subgroups for PEI is set to 8 (corresponding to 3 bits in the UE_ID) and the number of UE_ID based subgroups for LP-WUS is configured to 32 (corresponding to 5 bits in the UE_ID), the graphical representation of using above LP-WUS UE_ID based subgrouping formula for PEI and LP-WUS is shown in Figure 2.

Figure 2: Subgrouping effect in UE_ID based subgrouping formula

It can be observed that if LP-WUS applies the same UE_ID subgrouping formula structure as PEI, there will be an obvious overlap between the bits allocated for LP-WUS subgrouping and those designated for PEI subgrouping in the UE_ID. This overlap will lead to UEs with the same three least significant bits (LSBs) in the PEI and LP-WUS subgrouping bits (i.e., from bn to bn+2 regarding the above configuration) being assigned into the same PEI subgroup. In other words, once the UE monitors LP-WUS that indicates to wake up, it will definitely be indicated to monitor PO during subsequent PEI monitoring, meaning that no additional false alarm filtering can be achieved through PEI. In this scenario, UEs monitoring PEI would inevitably incur additional power consumption.

To address this issue, careful consideration must be given to the subgrouping formula and the potential overlap of bits in the UE_ID. Ensuring distinct subgrouping assignments is crucial for and minimizing unnecessary power consumption for UEs. The design of the formula should ensure that the bits used for LP-WUS subgrouping do not overlap with those used for PEI subgrouping as much as possible. One effective approach to achieve this is to introduce an offset K after N*Ns in the subgrouping formula as below. This adjustment can help avoid the conflicts between the two subgrouping schemes and optimize the overall design.

SubgroupID_LP-WUS = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS - subgroupsNumForUEID_LP-WUS)

The graphical representation of the updated formula for LP-WUS is illustrated in Figure 3. Similar to the function of N*Ns used in the formula, the effect of K is to ensure that the bits taken in UE_ID for LP-WUS subgrouping is decoupled with those for PEI, so that all the UEs within one LP-WUS subgroup can be optimally divided into different PEI subgroups. Of course, there exists exception. The scenario where two UEs are assigned to the same subgroup would only occur if the subgroup number indicated by bits bn​ to bn+2 of one UE's ID matches that represented by bits bn+3 to bn+7​ of another UE's ID. However, the possibility of this case is quite low regarding the generation of the UE_ID is entirely an independent and random event.

Figure 3: Subgrouping effect in the formula with offset K for LP-WUS

In Table 1, the numerical analysis on FAR is presented when different number of UEs in one PO apply LP-WUS subgrouping formula with/without introducing offset K. We assume the number of subgroups for PEI and LP-WUS is 8 and 32 respectively, the paging probability of UE is 1% and is independent, and an offset K equal to 8 is introduced in the formula. The paging false alarm rate for arbitrary UE comes from the probability that UE receives the invalid paging when the subgroup of this UE is indicated by LP-WUS to wake up to monitor legacy PO/PEI.

Table 1. Paging FAR for one UE monitoring LP-WUS & PEI with/without introducing offset K

| Number of UEs in one PO | Paging false alarm rate for one UE |  |
| --- | --- | --- |
|  | LP-WUS + PEI without offset K | LP-WUS + PEI with offset K |
| 96 | 66.3% | 10.4% |
| 128 | 74.6% | 13.9% |
| 256 | 87.1% | 26.6% |
| 512 | 93.3% | 46.6% |
| 1024 | 96.5% | 71.7% |

As seen from the numerical results, the UE can achieve a great reduction of paging false alarm rate when an offset K is used in formula to guarantee UEs to be divided into different subgroups of PEI and LP-WUS as much as possible.

Proposal 8: An offset K can be introduced in the UE_ID based subgrouping formula to avoid the potential subgrouping duplication and obtain the maximum power saving gain when monitoring LP-WUS and PEI.

When introducing offset K, another important aspect to consider is the interpretation of the K, specifically what K should represent in the subgrouping formula. If K depends on the number of subgroups configured for PEI, it implies that the network always needs to provide PEI configuration to configure/enable LP-WUS, which is too restricted for the NW. Also, if NW changes PEI configuration, UE has to re-calculate the UE_ID for LP-WUS if K depends on the PEI configuration which makes UE implementation more complicated. Hence LP-WUS subgrouping calculation should be decoupled from any PEI related configuration. From our perspective, K should denote the maximum number of subgroups that can be configured for PEI, which should be set as a fixed value of 8. This approach reserves the bits for configuring the maximum UE_ID based subgroups for PEI and ensures that the LP-WUS subgrouping determination does not depend on changes to PEI-related configurations. Consequently, even if the PEI function is not configured by the network, the UE can still obtain its corresponding subgroup ID by using fixed bits in UE ID without introducing additional complexity for conditional checks.

Observation 3: If the offset K is subgroupNumForUEID for PEI, i.e. the number of subgroups configured for PEI, it restricts NW configuration and makes UE implementation more complicated.

Proposal 9: The offset K introduced in the formula can be set to a fixed value of 8 which is the maximum number of subgroups can be configured for PEI.

In summary, we propose the following UE_ID subgrouping formula for LP-WUS:

Proposal 10: The UE_ID based subgrouping formula in TS 38.304 can be reused for LP-WUS subgrouping by introducing an offset K, to ensure that the LP-WUS subgrouping can use different bits than the PEI subgrouping in the truncated UE ID, i.e.,

SubgroupID = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS - subgroupsNumForUEID_LP-WUS),

where:

- N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state;

- Ns: number of paging occasions for a PF;

- UE_ID: 5G-S-TMSI mod X_LP-WUS, X_LP-WUS = 8192 * maximum number of LP-WUS subgroups;

- subgroupsNumForUEID_LP-WUS: number of subgroups for LP-WUS UE_ID based subgrouping in a PO, which is broadcasted in system information

- K: 8;

## Discussion on RAN1 LS

RAN1 has sent an LS [2] to RAN2 and RAN4 to check if there is any issue and specification impact in case the UE may not support LP-WUS reception on all the bands supported by the UE.

R1-2407559 LS on LP-WUS operation in IDLE/INACTIVE mode In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

There are two candidate deployments for LP-WUS, as showed in the Figure 4 below: one is LR-MR intra-band/frequency deployment, another one is LR-MR inter-band/frequency deployment. We further clarify the understanding of these two deployments and analyze the RAN2 impacts.

Figure 4: Example of deployments for LP-WUS

Deployment 1: LR-MR intra-band/frequency deployment

In this case, LR and MR are operating on the same band/carrier frequency, e.g. both on a lower frequency, for example 700MHz.

In such deployment, UE should camp on the cell with LP-WUS enabled/configured as much as possible so that UE can use LP-WUS function to obtain the power saving gain. Hence, the cells on which UE is able to use LP-WUS should be prioritized during cell reselection procedure, e.g. LP-WUS capable UEs may regard its supported frequency with LP-WUS enabled to be the higher priority, or leave it to NW implementation to set a new list of priority for cell reselection, which is specific for LP-WUS capable UEs.

One potential issue is that, it may lead to high load for band/carrier frequency with LP-WUS deployed, since the LP-WUS capable UEs have higher possibility to camp on this cell, it may cause congestion for access procedure. One solution is to fall back to original priority of cell reselection after the UE receives the paging targeted to itself, UE may perform the cell reselection and then initiates the RA in another cell, that means the cells on which UE is able to use LP-WUS are only prioritized when UE is monitoring LP-WUS and PO. Anyway, an optimized adjustment for priority of cell reselection can solve the load issue for access.

Observation 4: Cells on which UE is able to use LP-WUS should be prioritized during cell reselection procedure to obtain the power saving gain; an optimized adjustment for priority of cell reselection can solve the load issue for access.

Deployment 2: LR-MR inter-band/frequency deployment

In this case, LR and MR are operating on the different band/carrier frequency, e.g. the LR is operating on a lower frequency and MR is operating on a higher frequency. For example, the LR is operating on 700MHz, if the LP-WUS is detected, the UE turns on the MR which is operating on 2.6GHz.

This generally breaks the basic concept of serving cell model, the LR serving cell measurement and MR serving cell measurement become inter-frequency measurement even though both are serving cell measurement. Due to LR-MR inter-band/frequency deployment, the measurement results from LR and MR are independent, so that the LR measurement should be mandatory for LP-WUS monitoring entry condition. Similarly, for LP-WUS monitoring exit condition, the LR measurement result cannot reflect the MR quality, even if the LR is experiencing a good channel condition, the MR quality may already be very bad or even UE is out of previous MR coverage, especially considering the fully offloading case, i.e. no MR serving cell measurement, UE may lose the MR serving cell when it is monitoring LP-WUS. It requires to re-evaluate LP-WUS monitoring entry/exit condition, measurement fully offloading/relaxation cases, and may lead to reverting our previous conclusions.

Observation 5: If LR and MR are inter-band/frequency deployed, entry/exit conditions for LP-WUS monitoring and measurement fully offloading/relaxation cases need to be re-evaluated since the measurement results from LR and MR are independent.

For LP-WUS and PO monitoring, currently UE determines the LO based on the PO configuration. However, the PO configuration can be different for different cells, but UEs in different cells will be associated to the same cell LR belongs to. It is unclear whether the UE will use the PO configuration from the cell LR belongs to, or PO configuration from the cell MR belongs to, but finally it needs to be mapped to different serving cell which makes the configuration much more complicated.

Observation 6: If LR and MR are inter-band/frequency deployed, the configuration of mapping between LO and PO will be more complicated and needs further discussion.

For SI reception, currently it is supported to use LP-WUS to wake up all the UEs. If the SI is changed in cell 1, the LP-WUS can be used to wake up all the UEs monitoring the associated cell that LR monitors LP-WUS, but at the same time the UEs from other cells are also be waken up falsely since they are monitoring the same “LP-WUS” cell. To solve this issue, additional bits need to be added in LP-WUS to distinguish different cells monitored by MR, which introduces additional specification impacts.

Observation 7: If LR and MR are inter-band/frequency deployed, more bits in LP-WUS are needed to wake up all UEs in one cell for receiving updated SI without impacting UEs in other cells.

Based on the above analysis, we think that LR-MR intra-band/frequency deployment can be supported with less RAN2 impacts.

Proposal 11: The RAN2 impacts for supporting the different LP-WUS deployment:

| Deployment | RAN2 impacts |
| --- | --- |
| intra-band/frequency | Cell reselection: cells on which UE is able to use LP-WUS are prioritized during cell reselection procedure for LP-WUS capable UEs. |
| inter-band/frequency | RRM measurement: entry/exit conditions for LP-WUS monitoring and measurement fully offloading/relaxation cases need to be re-evaluated considering measurement results from LR and MR are independent; Paging procedure: a more complicated configuration of mapping between LO and PO needs to be provided by NW; SI reception: more bits in LP-WUS are needed to wake up all UEs in one cell for receiving updated SI without impacting UEs in other cells |

Proposal 12: From RAN2 perspective, LR-MR intra-band/frequency deployment can be supported with less RAN2 impacts.

# Conclusion

In this contribution, we discuss procedure and configuration of LP-WUS for IDLE/INACTIVE modes, and have the following proposals:

Configuration Aspects of LP-WUS

Proposal 1: Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

Proposal 2: The CN indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality by NAS signaling; the absence of indication means UE is allowed to use LP-WUS functionality.

Proposal 3: NW should configure whether only OFDM-based LP-WUR is supported in a cell.

Proposal 4a: Thresholds of LP-WUS entry/exit conditions for OOK-based receivers are always configured when OOK-based LP-WUS configuration is provided by the network.

Proposal 4b: Thresholds of LP-WUS entry/exit conditions for OFDM-based receivers are always configured when OFDM-based LP-WUS configuration is provided by the network.

Proposal 5: For LP-WUS entry condition, the threshold based on MR is mandatorily configured, while the threshold based on LP-WUR is optional.

Proposal 6: If the LP-WUS entry threshold based on LP-WUR is configured, it is up to UE implementation how to obtain the LP-WUR measurement results used for LP-WUS entry condition.

Issues for paging monitoring triggered by LP-WUS

Observation 1: Both options of UE monitoring legacy paging DCI/paging and monitoring PEI after MR wakes up are supported in RRC_IDLE/INACTIVE.

Proposal 7: If LP-WUS and PEI are used together, it is up to NW to decide which types of subgrouping (i.e., CN assigned subgrouping or UE_ID based subgrouping) are applied.

Observation 2: The paging false alarm can be further reduced if UEs in one LP-WUS subgroup can be further divided into different PEI subgroups.

Proposal 8: An offset K can be introduced in the UE_ID based subgrouping formula to avoid the potential subgrouping duplication and obtain the maximum power saving gain when monitoring LP-WUS and PEI.

Observation 3: If the offset K is subgroupNumForUEID for PEI, i.e. the number of subgroups configured for PEI, it restricts NW configuration and makes UE implementation more complicated.

Proposal 9: The offset K introduced in the formula can be set to a fixed value of 8 which is the maximum number of subgroups can be configured for PEI.

Proposal 10: The UE_ID based subgrouping formula in TS 38.304 can be reused for LP-WUS subgrouping by introducing an offset K, to ensure that the LP-WUS subgrouping can use different bits than the PEI subgrouping in the truncated UE ID, i.e.,

SubgroupID = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS - subgroupsNumForUEID_LP-WUS),

where:

- N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state;

- Ns: number of paging occasions for a PF;

- UE_ID: 5G-S-TMSI mod X_LP-WUS, X_LP-WUS = 8192 * maximum number of LP-WUS subgroups;

- subgroupsNumForUEID_LP-WUS: number of subgroups for LP-WUS UE_ID based subgrouping in a PO, which is broadcasted in system information

- K: 8;

Discussion on RAN1 LS

Observation 4: Cells on which UE is able to use LP-WUS should be prioritized during cell reselection procedure to obtain the power saving gain; an optimized adjustment for priority of cell reselection can solve the load issue for access.

Observation 5: If LR and MR are inter-band/frequency deployed, entry/exit conditions for LP-WUS monitoring and measurement fully offloading/relaxation cases need to be re-evaluated since the measurement results from LR and MR are independent.

Observation 6: If LR and MR are inter-band/frequency deployed, the configuration of mapping between LO and PO will be more complicated and needs further discussion.

Observation 7: If LR and MR are inter-band/frequency deployed, more bits in LP-WUS are needed to wake up all UEs in one cell for receiving updated SI without impacting UEs in other cells.

Proposal 11: The RAN2 impacts for supporting the different LP-WUS deployment:

| Deployment | RAN2 impacts |
| --- | --- |
| intra-band/frequency | Cell reselection: cells on which UE is able to use LP-WUS are prioritized during cell reselection procedure for LP-WUS capable UEs. |
| inter-band/frequency | RRM measurement: entry/exit conditions for LP-WUS monitoring and measurement fully offloading/relaxation cases need to be re-evaluated considering measurement results from LR and MR are independent; Paging procedure: a more complicated configuration of mapping between LO and PO needs to be provided by NW; SI reception: more bits in LP-WUS are needed to wake up all UEs in one cell for receiving updated SI without impacting UEs in other cells |

Proposal 12: From RAN2 perspective, LR-MR intra-band/frequency deployment can be supported with less RAN2 impacts.

# References

RP-241824 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

R1-2407559 LS on LP-WUS operation in IDLE/INACTIVE mode;