3GPP TSG-RAN WG2 Meeting #128	R2-2409934

Orlando, US, 18th – 22nd Nov, 2024

Agenda Item:	8.10.2

Source: 	China Unicom

Title:  	Discussion on MRO enhancement for CHO with candidate SCGs

Document for:	Agreement

Introduction

During the previous meeting, RAN2#127bis and RAN3#125bis reached agreements on MRO enhancement for CHO with candidate SCGs as referenced [1] [2].

Agreements - UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS. - Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

RLF report is enhanced to including - Identifier of candidate PCell(s) which met the configured CHO execution conditions when the RLF is encountered - Identifier of candidate PSCell(s) which met the configured CPAC execution conditions when the RLF is encountered - The Identifier of candidate PCell(s) or PSCell(s) that fulfilled execution conditions before the RLF is encountered.

It’s noted that cases addressed and included in the scope by RAN3 are outlined in [3] clause 2, with specifics provided in the Annex of this document. Portions “highlighted green” indicate cases to be addressed in RAN3, while “highlighted yellow” indicates cases within RAN3’s scope.

RAN2 and RAN3 have made agreements on RLF cases, however SCG failure case is not discussed yet. This document focuses on “highlighted green” portions and proposes further insights into the MRO discussion and analysis involving MCG with candidate SCGs.

Discussion

- 2.1	MRO enhancement for CHO with candidate SCGs

In alignment with both RAN2 and RAN3 agreements, the following table introduces a new classification of cases that will be given priority in discussions within both RAN2 and RAN3.

Table 1: Cases are preferable in both RAN2 and RAN3

| Scenario | Prerequisites | Failure cases |
| --- | --- | --- |
| No condition is fulfilled | Failure happens before CHO and/or CPAC execution. | 1a) RLF@srcPCell 1b) SCG failure |
| Only one condition is fulfilled | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. | 2a) RLF@srcPCell 2b) SCG failure |
| Only one condition is fulfilled | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. | 3a) RLF@srcPCell 3b) SCG failure |
| Only one condition is fulfilled | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. | 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |
| Both conditions are fulfilled in failure scenario | Both CHO and CPAC execution conditions are met. | 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. |
| Both conditions are fulfilled in sub-optimal scenario | Failure happens shortly after CHO and CPAC execution success | 8a) RLF@targetPCell 8b) SCG failure |

RAN2#127bis has discussed RLF cases and agreed that UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. And include the elapsed time between the point in time of the first fulfilled condition and RLF in the RLF report. In the case of only SCG failure cases occuring as listed above, e.g., 1b, 2b, 3b, 7b, 8b, 9b, it’s SCGFailureInformation instead of RLF report that will be generated. To figure out what happens when network finds that only SCG failure happens while configuring MCG with candidate SCGs, the same MRO enhancement should also be applied to SCGFailureInformation, so that MN can know what happened during the CHO and/or CPAC conditions were executed. So, it’s proposed as below:

Proposal 1: UE logs the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), the first met execution condition, and the elapsed time between the point in time of the first fulfilled condition and SCG failure in SCGFailureInformation.

In cases 9a, 9b, and 9c, only one condition is fulfilled at the same time. The network needs to ascertain whether the first condition (e.g., CHO or CPAC) has been satisfied more than once based on the analysis of RLF report or SCGFailureInformation. If so, the network only needs to adjust the configurations or conditions that were not satisfied initially, e.g. configure CHO only configurations instead of CHO with candidate SCGs configurations to the UE.

Proposal 2: For cases 9a, 9b, and 9c, UE should log the indication that the first execution condition has been fulfilled more than once in the RLF report and SCGFailureInformation.

Cases 8a and 8b are sub-optimal cases, and the reason may possibly be the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO) is too long or signal quality is bad. Thus, UE needs to report additional information to tell the network which condition is met firstly, whether the time gap between the two execution conditions are too long and the signal quality of PCells and PScells. Then the network can optimize the corresponding CHO or CPAC conditions. So, it’s proposed as below:

Proposal 3: For cases 8a and 8b, UE need to log the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), the first met execution condition, and measurement results of PCells and PSCells in SHR/SPR.

Conclusion

RAN2 is kindly asked to discuss the following observations and proposals:

Proposal 1: UE logs the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), the first met execution condition, and the elapsed time between the point in time of the first fulfilled condition and SCG failure in SCGFailureInformation.

Proposal 2: For cases 9a, 9b, and 9c, UE should log the indication that the first execution condition has been fulfilled more than once in the RLF report and SCGFailureInformation.

Proposal 3: For cases 8a and 8b, UE need to log the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), the first met execution condition, and measurement results of PCells and PSCells in SHR/SPR.

References

- (Mattias)_20240823_1000R2_127b_Maint_SONMDT_Mattias_20241017_1300

- RAN3_125bis_agenda_20241018_1430_EOM

- R3-244223	Failure scenarios on MRO for CHO with candidate SCGs Samsung, Cybercore, ZTE Discussion & Decision

- Annex: Failure scenarios (R3-244223)

| Cases | Figure: CHO with candidate SCG(s) is configured | Brief Description |
| --- | --- | --- |
| Case 1 |  | Failure happens before CHO and/or CPAC execution. 1a) RLF@srcPCell 1b) SCG failure 1c) MCG failure+SCG failure |
| Case 2 |  | CPC/CPA execution condition is met. Failure happens before CHO execution condition is met. 2a) RLF@srcPCell 2b) SCG failure 2c) MCG failure + SCG failure |
| Case 3 |  | CHO execution condition is met. No CHO only or CHO with SCG configuration. Failure happens before CPAC execution condition is met. 3a) RLF@srcPCell 3b) SCG failure 3c) MCG failure + SCG failure |
| Case 4 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. CHO execution to a candidate PCell fails. |
| Case 5 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. RLF shortly after CHO execution success |
| Case 6 |  | CHO execution condition is met. There is CHO only or CHO with SCG configuration. SCG failure shortly after CHO execution success |
| Case 7 |  | Both CHO and CPAC execution conditions are met. 7a) CHO execution to a candidate PCell failure. 7b) CPAC to a candidate PSCell failure. 7c) Execution to SCG failure and toMCG failure. |
| Case 8 |  | Failure happens shortly after CHO and CPAC execution success 8a) RLF@targetPCell 8b) SCG failure 8c) MCG failure + SCG failure |
| Case 9 |  | Failure due to the execution conditions for candidate PCell and the associated candidate PSCell cannot be met simultaneously. 9a) RLF@srcPCell 9b) SCG failure 9c) MCG failure + SCG failure |