# 3GPP TSG RAN WG2 Meeting #128      	                           R2-2409940
Orlando, US, 18th – 22th Nov., 2024

Agenda item:	8.5.2

Source:	Apple

Title:	Discussion on on-demand SSB for SCell

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

The Rel-19 WID of Network Energy Savings (WID RP-242354) was updated in RAN#105 [1], the WI objective on on-demand SSB (OD-SSB) SCell operation is copied below:

OD-SSB was discussed in RAN2#127 [2] and RAN2#127b [3]. And below agreements were made.

Agreements on OD-SSB in RAN2#127

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

OD-SSB transmission indication:

- RRC based OD-SSB transmission indication is used to indicate at least the initial activation/deactivation state of OD-SSB configuration. FFS on reconfiguration.

- New MAC-CE for OD-SSB transmission indication is introduced. We will not change legacy SCell activation/deactivation MAC CE. FFS if we need further optimization for scenario 2A.

Measurement on OD-SSB:

- Measurement based on OD-SSB in both case 1 and case 2 will be considered. (case 1 and case 2 defined in RAN1).

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

- RAN2 does not handle the issue raised in R2-2407414 in OD-SSB based measurements.

- RAN2 study how the UE to perform L3 measurement according to OD-SSB L3 RRM configuration.

Agreements on OD-SSB SCell in RAN2#127b

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this contribution, we continue to discuss RAN2 impacts of OD-SSB based on above agreements.

# 2 Discussion

RAN2#127 agreed to first study Scenario 2/2A and wait RAN1 conclusion on Scenario 3A/3B [2].

Agreements on OD-SSB

Scenarios on OD-SSB:

- RAN2 start the discussion from Scenario 2/2A and wait for RAN1 conclusion on Scenario 3A/3B.

We provide Figure.1 to illustrate Scenario 2/2A for ease of followed discussion.

Figure.1 Illustration of considered scenarios (scenario 2/2A) of OD-SSB

Meanwhile, RAN1 also agreed to support Case 1 and Case 2, which are illustrated in Figrue.2 for ease of followed discussion.

- Case 1: No always-on SSB on the SCell.

- Case 2: Always-on SSB is also periodically transmitted on the SCell besides OD-SSB.

Figure. 2 Illustration of Case 1 and Case 2 of OD-SSB

## 2.1 Further details of OD-SSB MAC CE

In RAN2#127b [3], below high-level agreements on OD-SSB MAC-CE format were made:

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

Some detailed design options on MAC-CE format were discussed during RAN2#127b [3]. However, as RAN1 is still discussing what information needs to be included in OD-SSB MAC-CE, no conclusion was made. The related RAN1 progress includes below agreements:

Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement (RAN1#118b) For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options

As can be observed, RAN1 agreed to include periodicity of OD-SSB in MAC-CE. Meanwhile RAN1 also agreed deactivation status of OD-SSB as one option (i.e. option 1) for further study, and other parameters were not precluded.

Observation 1: RAN1 agreed to include periodicity of OD-SSB in MAC-CE. RAN1 also agreed deactivation status of OD-SSB as one option (i.e. option 1) for further study, and other parameters were not precluded.

Then, we think that RAN2 can’t discuss detailed MAC-CE format until RAN1 make conclusion.

Proposal 1: RAN2 postpone the discussion on detailed MAC-CE format design until RAN1 make conclusion on all the required OD-SSB configurations in the MAC-CE.

## 2.2 L3 measurement on OD-SSB

L3 measurement on OD-SSB was discussed in RAN2#127b [3], but the discussion is controversial. In this section, we first discuss case 2 (i.e. whether always-on SSB and/or OD-SSB are measured), and then discuss L3 measurement framework. Finally, we discuss spec impact on how to support case 1.

### 2.2.1 How to support L3 measurement in case 2

In RAN2#127b [3], the following two options were identified, but no conclusion can be made due to divergent views.

- Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB

- Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

Meanwhile, we noticed that RAN4#112b also discussed this issue and identified the following 3 scenarios for study [4]:

Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies.

We think whether always-on SSB and/or OD-SSB are measured can be left to RAN4 because RAN4 need to first decide whether to specify new measurement requirements in OD-SSB case 2.

Proposal 2: RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

However, we have concern on Scenario 3 in RAN4 agreement (i.e. OD-SSB and always-on SSB are within different SSB carrier frequencies). If it is supported, it means one serving cell can have two SSBs in different frequencies with same PCI. Then, we believe it will introduce significant specification changes in RAN2 spec, including TS 38.331 [5] and TS 38.300 [6]. As follows, we provide our analysis on required spec changes:

- It challenges the basic UE assumption that SSBs in two MOs are regarded as different cells and definition of intra-frequency and inter-frequency SSB measurement captured in Section 9.2.4/B.2 of TS 38.300 [6].

  - As highlighted in NOTE2 and Section B.2, it is specified that the UE considers SSBs in two MOs as different cells, and one serving cell can be associated with at most a single SSB.

  - Following the above assumption, one serving cell has only one center frequency of the SSB as highlighted below. If scenario 3 is supported, it will introduce ambiguity whether the “center frequency of the SSB” is the frequency of always-on SSB or the frequency of OD-SSB.

From Section 9.2.4 of TS 38.300:

Intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements are defined as follows:

- SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the center frequency of the SSB of the serving cell and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same.

- SSB based inter-frequency measurement: a measurement is defined as an SSB based inter-frequency measurement provided the center frequency of the SSB of the serving cell and the center frequency of the SSB of the neighbour cell are different, or the subcarrier spacing of the two SSBs is different.

NOTE 2: For SSB based measurements, one measurement object corresponds to one SSB and the UE considers different SSBs as different cells.

NOTE 2a: If a UE is configured to perform serving cell measurements based on an NCD-SSB configured in its active BWP, this NCD-SSB is considered as the SSB of the serving cell in the definition of intra-frequency and inter-frequency measurements as above.

From Section B.2 of TS 38.300:

For a UE in RRC_CONNECTED, the BWPs configured by a serving cell may overlap in the frequency domain with the BWPs configured for other UEs by other cells within a carrier. Multiple SSBs may also be transmitted within the frequency span of a carrier used by the serving cell. However, from the UE perspective, each serving cell is associated to at most a single SSB. Figure B.2-1 below describes a scenario with multiple SSBs in a carrier, identifying two different cells (NCGI = 5, associated to SSB1, and NCGI = 6, associated to SSB3) with overlapping BWPs, and where RRM measurements can be configured to be performed by the UE on each of the available SSBs, i.e. SSB1, SSB2, SSB3 and SSB4.

- L3 RRM framework and configuration (Section 5.5.1/5.5.2): it challenges the basic L3 RRM framework which is per-frequency configured and per-cell reporting. Essentially, it requires either per-cell L3 measurement configuraiton with two ssbFrequency or one serving cell can be included in two different MOs.

- Performing measurement (Section 5.5.3): existing Section 5.5.3 uses loop structure to specify the UE to perform L3 measurements independently for each measId (and its associated single MO). However, it requires the UE to derive cell measurement result with SSB across different measId, and thereby the structure of procedure text needs to be changed.

- Measurement report triggering (Section 5.5.4): similar to Section 5.5.3, existing Section 5.5.4 uses loop structure to specify the UE to trigger measurement report independently for each measId based on L3 cell level measurement derived from one measId. It may also require change of whole structure of procedure text to allow the UE to determine whether to trigger event based on cell measurement result derived from SSB across different measId.

- Measurement reporting (Section 5.5.5): existing Section 5.5.5 specify the UE to use measId to identify the triggered measurement reporting. However, it is not clear whether one measId is sufficient if always-on SSB and OD-SSB of the same serving cell are configured in different MOs, which needs at least two measId.

Thus, we can summarize the required RAN2 specification impacts of Scenario 3 in Observation 2:

Observation 2: If OD-SSB and always-on SSB can have different SSB carrier frequencies for one serving cell, it will introduce significant spec changes in RAN2 spec, including TS 38.331 and TS 38.300:

- It challenges the basic UE assumption that SSBs in two MOs are regarded as different cells and definition of intra/inter-frequency SSB measurement captured in Section 9.2.4 of TS 38.300.

- It challenges the basic L3 RRM framework which is per-frequency configured. Essentially, it requires either per-cell configuration with two ssbFrequency, or one cell can be included in two different MOs.

- It challenges the basic loop structure of independent processing of L3 measurements for each measId in Section 5.5.3 / 5.5.4 / 5.5.5 of TS 38.331. Coupled processing across different measId is needed to derive L3 cell measurement result with SSBs across different measId.

We are not sure how much benefit Scenario 3 can bring, but it will introduce a lot of RAN2 specification changes. From RAN2 point of view, we think it should not be encouraged at late stage of 5G.

Proposal 3: In OD-SSB case 2, RAN2 agree that if OD-SSB and always-on SSB have different SSB carrier frequencies for one serving cell, it will introduce significant RAN2 specification changes in TS 38.300 and TS 38.331. Thus, it is not preferred at late stage of 5G.

To make progress, we propose to inform RAN4 Proposal 2 and Proposal 3.

Proposal 4: Send LS to inform RAN4 on Proposal 2 and Proposal 3.

### 2.2.2 L3 measurement framework

In RAN2#127b [3], the following two options were identified, but no conclusion can be made due to divergent views.

- Option1: Based on different measurement configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

We think the key issue is whether to follow legacy SMTC based L3 RRM framework:

- Option 1 keeps the legacy SMTC based L3 RRM framework.

  - Whether / how different measurement configurations (e.g. SMTC or MO) can be adapted when OD-SSB is transmitted can be further discussed as next step, which is a kind of stage 3 issue.

- Option 2 challenges the legacy SMTC based L3 RRM framework, i.e. based on OD-SSB transmission pattern rather than SMTC configuration

Thus, we first discuss whether to keep legacy SMTC based L3 RRM framework, and then discussed required specification changes.

#### 2.2.2.1 Whether follow legacy SMTC based L3 RRM framework

We prefer Option 1 because Option 2 will bring significant cross-WG specification impacts:

- RAN2 impacts: the UE autonomous OD-SSB measurement outside of SMTC will challenge the per-frequency L3 RRM framework and introduce significant RAN2 specification impacts.

  - Option 2 needs to at least modify below highlighted specification in Section 5.5.2.10 of TS 38.331 [5].

5.5.2.10	Reference signal measurement timing configuration

…omit unrelated part...

On the indicated ssbFrequency, the UE shall not consider SS/PBCH block transmission in subframes outside the SMTC occasion for RRM measurements based on SS/PBCH blocks and for RRM measurements based on CSI-RS except for SFTD measurement (see TS 38.133 [14], clause 9.3.8).

  - Option 2 essentially intends to change L3 RRM framework from per-frequency configuration to per-cell configuration. It will introduce at least the following spec changes in TS 38.331.

    - As Section 5.5 of 38.331 is drafted with assumption on per-frequency L3 measurement configuration, the loop structure is used to independently process measurements for each measId. Option 2 will need spec changes to support measurement with per-cell configuration.

    - In existing L3 framework, one MeasId links one per-frequency measurement configuration (i.e. MO) with one report configuration (i.e. ReportConfigNR). Then, it is not clear how to link per-cell measurement configuration with report configuration. Further spec changes are needed.

- RAN4 impacts: the UE autonomous OD-SSB measurement outside of SMTC will lead to significant RAN4 specification changes in TS 38.133 [7] because all L3 measurement requirements are specified with SMTC periodicity (e.g. SCell activation latency).

  - Please note that RAN4 has clearly indicated it in their LS to RAN2 (R4-2416911) [8].

However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation.

From section 5.1.1 of TS 38.215:

| Definition | SS reference signal received power (SS-RSRP) is defined as the linear average over the power contributions (in [W]) of the resource elements that carry secondary synchronization signals. The measurement time resource(s) for SS-RSRP are confined within SS/PBCH Block Measurement Time Configuration (SMTC) window duration. If SS-RSRP is used for L1-RSRP as configured by reporting configurations as defined in TS 38.214 [6], the measurement time resources(s) restriction by SMTC window duration is not applicable. For SS-RSRP determination demodulation reference signals for physical broadcast channel (PBCH) and, if indicated by higher layers, CSI reference signals in addition to secondary synchronization signals may be used. SS-RSRP using demodulation reference signal for PBCH or CSI reference signal shall be measured by linear averaging over the power contributions of the resource elements that carry corresponding reference signals taking into account power scaling for the reference signals as defined in TS 38.213 [5]. If SS-RSRP is not used for L1-RSRP, the additional use of CSI reference signals for SS-RSRP determination is not applicable. SS-RSRP shall be measured only among the reference signals corresponding to SS/PBCH blocks with the same SS/PBCH block index and the same physical-layer cell identity. If SS-RSRP is not used for L1-RSRP and higher-layers indicate certain SS/PBCH blocks for performing SS-RSRP measurements, then SS-RSRP is measured only from the indicated set of SS/PBCH block(s). For frequency range 1, the reference point for the SS-RSRP shall be the antenna connector of the UE. For frequency range 2, SS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SS-RSRP value shall not be lower than the corresponding SS-RSRP of any of the individual receiver branches. |
| --- | --- |

RAN1 impacts: the UE autonomous OD-SSB measurement outside of SMTC will also result in multiple RAN1 specification changes in TS 38.215 [9]. One example of required spec change can be found highlighted:

- Misaligned understanding with NW: the UE autonomous OD-SSB measurement outside of SMTC may lead to misalignment on actually measured SSB between UE and NW, especially when OD-SSB is partially overlapped or has small timing gap with SMTC.

- UE power saving: it mandates the UE to perform L3 measurement on OD-SSB outside of SMTC, which is bad for UE power saving.

Based on above analysis, we can summarize them as Observation 3-Observation 6.

Observation 3: To support UE autonomous OD-SSB measurement outside of SMTC, it essentially challenges the per-frequency L3 RRM framework and will introduce significant RAN2 specification impacts:

  - Allow UE to perform L3 measurement based on OD-SSB outside SMTC in Section 5.5.2.10 of TS 38.331.

  - As existing section 5.5 of 38.331 is drafted with assumption on per-frequency L3 measurement configuration, spec changes are needed for per-cell measurement configuration.

  - Spec changes are needed to link per-cell measurement configuration with one report configuration.

Observation 4: The UE autonomous OD-SSB measurement outside of SMTC will lead to significant RAN4 specification changes because all L3 measurement requirements are specified with SMTC configuration.

Observation 5: The UE autonomous OD-SSB measurement outside of SMTC will also result in multiple RAN1 specification changes in TS 38.215.

Observation 6: The UE autonomous OD-SSB measurement outside of SMTC is bad for UE power saving and may lead to misalignment on actually measured SSB between UE and NW, especially when OD-SSB is partially overlapped or has small timing gap with SMTC.

Thus, we propose to keep legacy SMTC based L3 RRM framework.

Proposal 5: Keep SMTC based L3 RRM framework for OD-SSB case 1 and case 2 (i.e. not allow UE autonomous SSB measurement outside of SMTC).

#### Whether multiple configurations are needed

In legacy L3 RRM based on SSB, the NW need to configure UE SMTC in MeasObjectiveNR. Furthermore, according to Section 5.5.2.10 of TS 38.331 [5], the UE shall not consider SSB outside the SMTC occasion for RRM measurements. SMTC is a periodical window because it is assumed that SSB is always transmitted in periodical way. However, one single SMTC may not cover all the required SSB(s) to perform L3 measurements because the assumption of periodic SSB transmission is NOT valid anymore for OD-SSB in case 2.

We provide an example illustrated in Figure.4, where the UE is configured with one SMTC whose window covers the always-on SSB. However, when OD-SSB is triggered after T1, the configured SMTC window can’t cover OD-SSB. Thus, although the UE may know when OD-SSB starts transmission via activation MAC-CE, it can’t perform L3 measurements towards OD-SSB because OD-SSB is not covered within its configured SMTC. One may argue that NW implementation may resolve this issue (e.g., via proper configuration of SMTC by considering OD-SSB pattern). However, please note that SMTC is configured per frequency rather than per cell, which makes NW difficulty to configure one single SMTC window to cover all SSBs without causing negative impacts to legacy UE. For example, NW may configure a SMTC window with short periodicity to cover both always-on SSB and OD-SSB, but it will lead to UE extra power consumption, especially for the legacy UEs.

Figure.3 Illustration of issue in case 2: one single SMTC can only cover always-on SSB but can’t cover OD-SSB. So, UE can’t perform L3 measurements towards OD-SSB, although it knows when OD-SSB is triggered.

Observation 7: As SMTC is configured per frequency rather than per cell, it is difficulty for NW to configure one single SMTC window to cover both OD-SSB and always-on SSB without causing negative impacts to legacy UE (e.g. extra power consumption if NW configures a dense SMTC to cover both types of SSBs).

In legacy, SMTC can only be reconfigured by RRC signaling. Because OD-SSB can be activated / deactivated via MAC-CE, we think one single SMTC can’t quickly align with the dynamic change of OD-SSB periodicity. Thus, it is necessary to allow the UE to dynamically adapt between different SMTC configurations depending on whether OD-SSB is activated / deactivated.

On detailed RRC signaling specification change, we think the following two solutions can be further discussed:

- Alt-1: OD-SSB dedicated SMTC configuration is included in the same MO of always-on SSB

- Alt-2: OD-SSB dedicated SMTC configuration is included in an OD-SSB dedicated MO.

Proposal 6: For case 2, introduce dynamic adaptation between different SMTC configurations depending on whether OD-SSB is activated / deactivated. FFS how to configure OD-SSB dedicated SMTC configuration (e.g. in the same MO of always-on SSB or in an OD-SSB dedicated MO).

During online discussion of RAN2#127b [3], it was raised the question if adaptation of SMTC will lead to RAN4 requirement changes. We want to clarify two aspects:

- There seems misunderstanding that SMTC adaptation needs to activate multiple SMTCs simultaneously. But we understand that only one SMTC is activated at one time (e.g., either legacy SMTC when only always-on SSB is transmitted, or OD-SSB dedicated SMTC when OD-SSB is activated).

- RAN2 had already introduced different features with multiple SMTCs (e.g. smtc2, smtc2-LP, smtc3list and smtc4list) as highlighted in below copy of Section 5.5.2.10 of TS 38.331 [5]. And RAN4 had specified their requirements in TS 38.133 [7]. So, we think it is “business as usual”.

5.5.2.10	Reference signal measurement timing configuration

…omit unrelated part...

If smtc2 is present, for cells indicated in the pci-List parameter in smtc2 in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2 configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition.

If smtc2-LP is present, for cells indicated in the pci-List parameter in smtc2-LP in the same frequency (for intra frequency cell reselection) or different frequency (for inter frequency cell reselection), the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received periodicity parameter in the smtc2-LP configuration and use the Offset (derived from parameter periodicityAndOffset) and duration parameter from the smtc configuration for that frequency. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell or serving cell (for cell reselection) meeting the above condition.

If smtc3list is present, for cells indicated in the pci-List parameter in each SSB-MTC3 element of the list in the same MeasObjectNR, the IAB-MT shall setup an additional SS block measurement timing configuration in accordance with the received periodicityAndOffset parameter (using same condition as smtc1 to identify the SFN and the subframe for SMTC occasion) in each SSB-MTC3 configuration and use the duration and ssb-ToMeasure parameters from each SSB-MTC3 configuration.

If smtc4list is present, for cells indicated in the pci-List parameter in each SSB-MTC4 element of the list in the same MeasObjectNR, the UE shall setup an additional SS/PBCH block measurement timing configuration (SMTC) in accordance with the received offset parameter in each SSB-MTC4 configuration and use the duration parameter and periodicity (derived from parameter periodicityAndOffset) from the smtc1 configuration. The first subframe of each SMTC occasion occurs at an SFN and subframe of the NR SpCell meeting the above condition.

Observation 8: Only one SMTC is activated at one time (e.g., either legacy SMTC when only always-on SSB is transmitted, or OD-SSB dedicated SMTC when OD-SSB is activated).

Observation 9: RAN2 had already introduced different features with multiple SMTCs (e.g. smtc2, smtc2-LP, smtc3list and smtc4list). And RAN4 had specified their requirements in TS 38.133. Thus, it is “business as usual”.

### 2.2.3 How to support L3 measurement in case 1

In RAN2#127 [2], it was agreed that the UE doesn’t expect to measure SSB when OD-SSB is deactivated, i.e. the UE measures SSB only when on-demand SSB is activated.

Agreements on OD-SSB in RAN2#127

- For Case #1, the UE does not expect to measure SSB when on-demand SSB transmission is deactivated. In other words, the UE expects to measure SSB when on-demand SSB transmission is activated.

From Section 5.3.3.1 of TS 38.331

omit unrelated part

The UE shall:

1>	whenever the UE has a measConfig, perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured as follows:

2>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO:

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

4>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

omit unrelated part

From L3 measurement perspective, one key difference of case 1 from case 2 is to allow the UE not to perform measurement for servingCellMO when OD-SSB transmission is deactivated. According to Section 5.3.3.1 of TS 38.331 [5], the UE is mandated to perform serving cell measurement when servingCellMO is configured (even if no ReportConfigNR is linked with the servingCellMO):

This is a basic principle ever since from LTE L3 RRM which means that the dynamic activation/deactivation for serving cell measurement is not allowed. It seems to be conflicted with above RAN2#127 agreement.

Observation 10: In legacy RRC, the UE is mandated to perform serving cell measurement when servingCellMO is configured (even if no ReportConfigNR is linked with the servingCellMO).

To resolve this issue, we think the same solution of case 2 in Proposal 6 can be reused. Please note that the UE doesn’t perform L3 measurements based on SSB outside SMTC according to TS 38.331 [5]. Thus, RAN2#127 agreement can be implemented by keeping absence of the SMTC configuration for legacy always-on SSB because the UE will regard all SSBs are outside SMTC window and thereby not perform L3 measurement. Thus, we propose:

Proposal 7: For case 1, reuse the same SMTC configuration adaptation solution of case 2 (e.g. the SMTC configuration for legacy always-on SSB is absent).

As Proposal 5/6/7 are related to RAN4 question to RAN2 in LS (R4-2416911) [8], we think they should be included in the reply LS to inform RAN4.

However, L3 measurement requirement in RAN4 is defined based on SMTC configuration rather than SSB configuration in legacy. RAN4 would like to ask RAN2 that whether there is any SMTC impact due to SSB adaptation.

Proposal 8: Send LS to inform RAN4 on Proposal 5, Proposal 6 and Proposal 7.

We believe that this is the unified solution with the least specification impacts, and we also provide an example in Figure. 4 to help illustration.

Figure.4 Illustration of unified solution for case 1 and case 2

# 3 Conclusion

In this contribution, we further discuss RAN2 specification impacts of on-demand SSB for SCell. Our observations are:

Observation 1: RAN1 agreed to include periodicity of OD-SSB in MAC-CE. RAN1 also agreed deactivation status of OD-SSB as one option (i.e. option 1) for further study, and other parameters were not precluded.

Observation 2: If OD-SSB and always-on SSB can have different SSB carrier frequencies for one serving cell, it will introduce significant spec changes in RAN2 spec, including TS 38.331 and TS 38.300:

- It challenges the basic UE assumption that SSBs in two MOs are regarded as different cells and definition of intra/inter-frequency SSB measurement captured in Section 9.2.4 of TS 38.300.

- It challenges the basic L3 RRM framework which is per-frequency configured. Essentially, it requires either per-cell configuration with two ssbFrequency, or one cell can be included in two different MOs.

- It challenges the basic loop structure of independent processing of L3 measurements for each measId in Section 5.5.3 / 5.5.4 / 5.5.5 of TS 38.331. Coupled processing across different measId is needed to derive L3 cell measurement result with SSBs across different measId.

Observation 3: To support UE autonomous OD-SSB measurement outside of SMTC, it essentially challenges the per-frequency L3 RRM framework and will introduce significant RAN2 specification impacts:

  - Allow UE to perform L3 measurement based on OD-SSB outside SMTC in Section 5.5.2.10 of TS 38.331.

  - As existing section 5.5 of 38.331 is drafted with assumption on per-frequency L3 measurement configuration, spec changes are needed for per-cell measurement configuration.

  - Spec changes are needed to link per-cell measurement configuration with one report configuration.

Observation 4: The UE autonomous OD-SSB measurement outside of SMTC will lead to significant RAN4 specification changes because all L3 measurement requirements are specified with SMTC configuration.

Observation 5: The UE autonomous OD-SSB measurement outside of SMTC will also result in multiple RAN1 specification changes in TS 38.215.

Observation 6: The UE autonomous OD-SSB measurement outside of SMTC is bad for UE power saving and may lead to misalignment on actually measured SSB between UE and NW, especially when OD-SSB is partially overlapped or has small timing gap with SMTC.

Observation 7: As SMTC is configured per frequency rather than per cell, it is difficulty for NW to configure one single SMTC window to cover both OD-SSB and always-on SSB without causing negative impacts to legacy UE (e.g. extra power consumption if NW configures a dense SMTC to cover both types of SSBs).

Observation 8: Only one SMTC is activated at one time (e.g., either legacy SMTC when only always-on SSB is transmitted, or OD-SSB dedicated SMTC when OD-SSB is activated).

Observation 9: RAN2 had already introduced different features with multiple SMTCs (e.g. smtc2, smtc2-LP, smtc3list and smtc4list). And RAN4 had specified their requirements in TS 38.133. Thus, it is “business as usual”.

Observation 10: In legacy RRC, the UE is mandated to perform serving cell measurement when servingCellMO is configured (even if no ReportConfigNR is linked with the servingCellMO).

Based on above observations, our proposals are:

MAC-CE format

Proposal 1: RAN2 postpone the discussion on detailed MAC-CE format design until RAN1 make conclusion on all the required OD-SSB configurations in the MAC-CE.

L3 measurement in NES case 2

Proposal 2: RAN2 leave it to RAN4 to conclude whether always-on SSB and/or OD-SSB are measured when both are transmitted in OD-SSB case 2.

Proposal 3: In OD-SSB case 2, RAN2 agree that if OD-SSB and always-on SSB have different SSB carrier frequencies for one serving cell, it will introduce significant RAN2 specification changes in TS 38.300 and TS 38.331. Thus, it is not preferred at late stage of 5G.

Proposal 4: Send LS to inform RAN4 on Proposal 2 and Proposal 3.

L3 RRM framework

Proposal 5: Keep SMTC based L3 RRM framework for OD-SSB case 1 and case 2 (i.e. not allow UE autonomous SSB measurement outside of SMTC).

Proposal 6: For case 2, introduce dynamic adaptation between different SMTC configurations depending on whether OD-SSB is activated / deactivated. FFS how to configure OD-SSB dedicated SMTC configuration (e.g. in the same MO of always-on SSB or in an OD-SSB dedicated MO).

L3 measurement in NES case 1

Proposal 7: For case 1, reuse the same SMTC configuration adaptation solution of case 2 (e.g. the SMTC configuration for legacy always-on SSB is absent).

Proposal 8: Send LS to inform RAN4 on Proposal 5, Proposal 6 and Proposal 7.

# 4 References

[1] RP-242354, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#127, Chair Notes.

[3] RAN2#127b, Chair Notes.

[4] R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson.

[5] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[6] TS 38.300-v18.0.0, NR and NG-RAN Overall Description; Stage 2, 2023-12.

[7] TS 38.133-v18.4.0, Requirements for support of radio resource management, 2024-1.

[8] TS 38.215-v18.4.0, Physical layer measurements, 2024-1.

[9] R4-2416911, LS on SSB adaptation,