3GPP TSG-RAN WG2 Meeting #128	R2-2409955

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.7.3

Source:	Apple

Title:	Views on Enhancements Relating to RRM Measurement Gaps

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

For Rel-19 XR, we have the following objective [1]:

-	Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4]

-	Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

In RAN2 #127bis, we have reached the following agreements [2]:

RAN2 #127bis Agreements: RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution

This paper presents some of our views on the following issues:

- The need of RRC-based solution for Measurement Gap skipping control.

- The impacts on DSR due to Measurement Gap skipping.

- The impacts on DRX due to Measurement Gap skipping.

# Discussions

## Need of RRC-based Control for MG Skipping

It was agreed in RAN2 #127bis that, we can further evaluate if RRC-based control MG skipping is also needed, in addition to the working assumption of DCI-based control that has been made by RAN1.

We would like to point out that, many XR traffics including video and audio streams are all periodic, and the network can have a good knowledge about when the packets would arrive based on the traffic periodicity. Such nature of XR use cases has motivated 3GPP to specify the features such as DRX for non-integer periodicity and Multi-PUSCH configured grants in Rel-18. Furthermore, RAN2 has also defined the UAI for the UE to indicate UL traffic information including periodicity and burst arrival times. These features have clearly shown that the arrival of XR packets should be predictable by the gNB.

Observation 1: Typical XR traffics are periodic, and hence the packet arrival times are predictable by the gNB.

On the other hand, we have noted that, if the UE can only know that a MG will be skipped based on DCI that could be signalled not long before the MG, there may be some additional restrictions and complications for implementation for certain L2 procedures such as the UTO-UCI mechanisms specified in Rel-18 XR. According to TS 38.321, when UTO-UCI is configured for a configured grant (CG) configuration, the MAC layer should determine if a number of consecutive subsequent CG occasions are to be used or unused, and then indicate such decisions to the gNB via the UTO-UCI. Since this is specified as a L2 functionality, we are not sure if RAN1 has considered this aspect in their discussions.

TS 38.321 V18.2.0: For a configured grant configured with UTO-UCI, the MAC entity determines if a configured uplink grant which is within the subsequent nrofBitsInUTO-UCI valid occasions of its associated configured grant configuration is going to be used for PUSCH transmission by considering at least the amount of buffered data that can be transmitted on the available occasions of the associated configured grant and other available UL-SCH resources. Upon this determination, the MAC entity sends an indication to lower layers, for use in the procedure for reporting UTO-UCI.

If the UE does not know that an upcoming MG will be skipped early enough, it may directly determine any CG occasions that overlap with this MG as “unavailable/unused”, because by default the UE is not allowed to transmit on UL-SCH resources during a MG. Thus, UTO-UCI may be derived in an inappropriate manner, as it may fail to take MG skipping into account (e.g. a UL-SCH resource that has been considered unavailable is actually available because of MG skipping). Moreover, the UE cannot reverse/update its decision for a CG occasion, if it is already indicated as “unused”. Consequently, the UE cannot use the CG occasion(s) even if the overlapping MG is skipped, which may result in undesirable latency for uplink transmission.

One could argue that such problem can be avoided, if the UE only derives UTO-UCI after it has confirmed that whether a MG will be skipped or not (e.g. after receiving a DCI that instructs the UE to skip a MG). However, this obviously creates a lot of constraints for UE implementation in terms of UTO-UCI derivation. The lead time of the DCI before the MG could be as short as 5ms, but the time required by the UE to derive UTO-UCI could be much longer than that (especially if the value of nrofBitsInUTO-UCI is large, e.g. 8). Ideally, we should not couple the processing timeline for UTO-UCI and MG skipping, as it would increase a lot of UE implementation complexity.

Observation 2: Solely relying on DCI for MG skipping may create some UE implementation problems for some L2 functionalities such as UTO-UCI determination, that may have been overlooked by RAN1.

In our understandings, the key motivation for MG skipping is to make sure that transmission/reception of packets from specific XR traffic flows will not be blocked by measurement gaps, in order to reduce the latency and thereby maximize XR user experience. Since the gNB can anyway predict when the packets are arriving based on their periodicities (based on Observation 1), we think it is reasonable for the gNB to pre-configure a pattern of time-windows that indicates when the MG can be skipped. This is illustrated in Figure 1:

Figure 1 MG Skipping based on RRC-configured time window pattern.

Since the UE is able to know which MG occasions will be skipped much earlier based on such RRC configuration, this is advantageous in terms of UE implementation. In particular, the UE can derive UTO-UCI by directly taking the MG skipping into account, without abandoning a CG occasion unnecessarily.

In general, we think both DCI-based and RRC-based signalling for MG skipping should be supported. The former allows the gNB to cancel a MG based on the spontaneous need, while the latter allows the UE to implement certain features (e.g. UTO-UCI) in a more sensible way with the knowledge of traffic periodicity and packet arrival times.

Proposal 1: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported.

## Impacts on DSR

During the discussions in RAN2 #127bis, some issues related to DSR triggering and MG skipping have been discussed. In particular, the DSR may be triggered during an active MG, and therefore DSR may not be immediately transmitted. Furthermore, some companies have also suggested that the definition of delay-critical data may take the timing of MG into account.

In our opinions, the mechanisms for both MG skipping and DSR triggering (at least for Rel-19 DSR where multiple remaining time thresholds are configured for an LCG) are still under discussions, and so it is too early for RAN2 to evaluate the potential impacts. Thus, we think RAN2 should not consider or discuss any potential enhancement relating to DSR for MG skipping, until both DSR triggering and MG skipping become clear later on.

Proposal 2: RAN2 does not discuss any enhancement based on impacts to DSR from MG skipping, until both DSR triggering and MG skipping mechanisms become clear in Rel-19.

## Impacts on DRX

Another issue that has been brought up is relating to inter-working between DRX and MG skipping. In particular, some companies think that if the network has instructed the UE to skip a MG, we must also ensure that the UE is in DRX ON-Duration, and hence it is able to receive the scheduling DCI during the interval of skipped MG.

Since both MG skipping and DRX configurations are controlled by the gNB, we think this can be entirely resolved via proper network implementation. For instance, if the gNB knows that the UE will not monitor PDCCH during a measurement gap, then the gNB would refrain from instructing the UE to skip such measurement gap for the purpose of dynamic scheduling. Therefore, we do not see the need for such optimization.

Proposal 3: RAN2 does not pursue the enhancement relating to inter-working between DRX and MG skipping.

# Conclusions

In this paper, we have provided some of our views about measurement gaps skipping, including the following observations and proposals:

Observation 1: Typical XR traffics are periodic, and hence the packet arrival times are predictable by the gNB.

Observation 2: Solely relying on DCI for MG skipping may create some UE implementation problems for some L2 functionalities such as UTO-UCI determination, that may have been overlooked by RAN1.

Proposal 1: In addition to DCI-based MG skipping control, RRC-based MG skipping control should also be supported.

Proposal 2: RAN2 does not discuss any enhancement based on impacts to DSR from MG skipping, until both DSR triggering and MG skipping mechanisms become clear in Rel-19.

Proposal 3: RAN2 does not pursue the enhancement relating to inter-working between DRX and MG skipping.

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.

- 3GPP RAN2 #127bis Chair’s Notes, Hefei, China, October 2024.