# 3GPP TSG RAN WG2 Meeting #128      	                           							R2-2409966
Orlando, USA, November 18-22, 2024

Agenda item:	8.2.5

Source:	Apple

Title:	Remaining open issues in Topology 2

WID/SID:	FS_Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#127-bis[1] meeting, the following agreements have been made for Topology 2 support:

RAN2#127-bis agreements on Topology 2 - Dedicated resource configuration is only given to the UE reader via dedicated signalling. Mechanisms for shared resource pool amongst readers are not considered in this release. - The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control. FFS how valid resource is determined - we will consider temporary out of connection scenarios. FFS on resource validity across multiple cells. - The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

In this contribution we provide our views on several outstanding technical issues in Topology 2.

# 2   	Discussion

## 2.1 Resource validity

RAN2#127-bis agreed

- The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

Naturally, when a UE reader is in RRC_CONNECTED and under full control of an A-IoT enabled gNB, the A-IoT radio resources are valid as long as they are configured.

Furthermore, the case when the network sends the UE reader to RRC_INACTIVE or RRC_IDLE needs to be discussed separately (see next section).

Therefore, there are two questions to discuss with regards to A-IoT resource validity:

- What happens during HO

- What happens during an RLF

In general, during HO the UE reader should follow network control. It is reasonable to assume that the source cell would know whether the target cell is A-IoT enabled or not and can decide to instruct the UE to keep the configured A-IoT resources, or release them, or follow the target cell control. All these are stage-3 details that can be discussed in the normative phase.

During RLF, a regular UE:

- selects a suitable cell and then initiates RRC re-establishment;

- enters RRC_IDLE if a suitable cell was not found within a certain time after RLF was declared.

For the case when a suitable cell is found, if it is an A-IoT enabled cell, the UE should follow its instructions – this is essentially similar to what should happen during a HO and likewise such details can be discussed in the normative phase.

Therefore, the only case we need to address is when the UE reader enters RRC_IDLE. Considering the previous agreement to support “temporary out of coverage”, we think the easiest way is to define a validity timer for A-IoT radio resources, which would start when a UE reader experiences RLF. As long as the timer is running the configured A-IoT radio resources remain valid and when the timer expires, the resources are released. Note that this timer may even last longer after UE entering IDLE state or UE becomes out of coverage. So, it may provide some temporary “validity” for UE operate in A-IoT air interface while in IDLE state. On the other hand, if we want to strictly prohibit any IDLE/OOC UE operation, then this timer may be realized with existing timers T310+T311.

Proposal 1 	RAN2 to discuss whether to define a validity timer for A-IoT radio resources; the timer is started when RLF occurs, when the timer expires the UE reader releases the configured A-IoT radio resources.

## 2.2 RRC State of UE reader

There are some controversial discussions in RAN2#1256 about whether a UE reader can operate A-IoT air interface in IDLE/INACTIVE RRC state or out of coverage. Basically, RAN2 has agreed in RAN2#127 that NW need to control the resource allocation in A-IoT air interface. Thus, UE reader in RRC_CONNENCTED state shall be supported by default as UE reader can also be directly controlled by gNB in such a state. Nonetheless, such resource control could be semi-static.  Also, given that there should be very light traffic for A-IoT communication in most cases, MO-SDT and MT-SDT mechanisms introduced for 3GPP Rel-17/Rel-18 for IoT systems can be reused as it seems a perfect a match for A-IoT UE reader operation.

Proposal 2 	For in-coverage operation, besides RRC_CONNECTED sate, RRC_INACTIVE UE reader can be supported with SDT.

One issue that must be addressed to support RRC_INACTIVE is how the UE reader handles the case when it reselects a new cell while in RRC_INACTIVE. If the new cell does not support to continue A-IoT, we propose the UE simply stop A-IoT communications with A-IoT devices and releases the A-IoT configuration. But if the new cell also supports A-IoT and SDT, we can assume the UE will follow legacy SDT solution for cell-reselection case (e.g., SDT procedure unsuccessfully completed upon cell re-selection, etc).

Proposal 3  	Upon cell reselection in RRC_INACTIVE, if the target cell doesn’t support A-IoT, stops A-IoT communications with A-IoT devices and releases the A-IoT configuration.

In regard to IDLE state operation for A-IoT UE reader, there is a concern that the A-IoT enabled UE cannot solely rely on SIB configuration, w/o any UE-specific signalling transmitted in Uu interface. This is because UE(s) relying on common resource configuration need contend for the resource usage in the same Uu channel, and channel sensing mechanism are to be supported by UE and A-IoT device to mitigate contention and interference problems among the UE and devices using the PDRCH/PRDCH. From this perspective, the UE reader and A-IoT device will be more complex in Topology 2 than a gNB reader in Topology 1 regards of A-IoT air interface support. Moreover, the UE reader may need to communicate with A-IoT enabled gNB in Uu interface anyway, to report inventory results. Therefore, the A-IOT enabled UE cannot remain in RRC_IDLE state to complete necessary A-IOT functionalities.  Similarly, UE readers cannot solely rely on pre-configuration for the same reasons. Thus, OOC UE reader are not supported, either.

Proposal 4 	RRC_IDLE  state UE reader and OOC UE reader is not supported in Rel-19 A-IoT.

Finally, regarding the agreement on “temporary out of coverage” connection below in RAN2#127:

  - Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

This agreement means that we don’t do anything specific for OOC in this release. At any rate, a UE reader should obviously have a well-defined standardized way for determining the resource to be used during RLF and HO cases, as we discussed in Section 2.1

In our view, this “extensibility” in the above-mentioned agreement to “temporarily OOC” case can only be considered based on the assumption that A-IoT air interface resource allocation for RLF and HOF scenarios are simply reused for this new case. That is the only reasonable way for an OOC UE can still be “with full NW control of resource”, w/o adding UE complexity. Any other new mechanism to pre-configure A-IOT air interface radio resource for this case is not to be considered in Rel-19. Therefore, we propose:

Proposal 5 	For the extensibility to future temporarily out-of-coverage case, RAN2 assumes that resource allocation for RLF/HOF is reused w/o any further enhancements in Rel-19.

## 2.3 Resource Allocation in A-IoT Air Interface for Topology 2

The A-IoT air interface has some resemblance with the Sidelink interface. So, it is not surprised that there are company views that SL resource allocation mode 1 and mode 2 shall be both considered for Rel-19 A-IoT design. However, we think there are also some important differences between A-IoT and Sidelink use cases, which are highlighted below:

- Sidelink communication aims to support OOC operation by default as the D2D communication between Sidelink UEs are supposed to be terminated at the Sidelink UE. But for Ambient IoT use cases, the reader is supposed to relay the collected device data to NW, so the UE reader must be in NW control.

- Sidelink communications are transmitted/received by a UEs with a capacity to support complex communication techniques and algorithms. Those Sidelink UEs are either smartphones or smart vehicles and able to maintain an accurate clock. But for A-IoT communication, the A-IoT devices are supposed to be low-power, low-cost and low-complexity, and subject to time drifting errors.

- Sidelink traffic are modelled similarly to Uu QoS flows with considerable amount of data throughput and a variety of QoS requirements. However, for A-IoT, RAN2 assumes that no per-packet QoS and no per-QoS flow is supported at AS level (for both UL/DL).  FFS how to handle the general QoS requirements from SA2.

Based on the above observed differences, we think there is no need to reuse all SL resource allocation methods for A-IoT. To be more specific, we discuss D2R and R2D cases respectively.

D2R resource allocation

It has been agreed that:

  - A-IoT air interface  in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

Then, from the device perspective, it is always the reader allocates D2R resource in PRDCH transmission, as RAN1 agreed.  There is no need for the device to know which resource are allocated by gNB in Topology 2. It is also true that any signalling appearing in PDRCH for resource allocation are to be transmitted by UE reader w/o any Topology 2 specific enhancements.

Proposal 6 	D2R resource indication signalling in Topology 2 reuse exactly the same design for Topology 1. Whether this signalling is from UE reader or gNB reader is transparent to A-IoT device.

Thus, from this perspective, whether there is autonomous D2R resource selection in A-IoT device or not in Topology 2 is hinged on whether there is such a mechanism to be supported in Topology 1. So far, we can see the “autonomy” is very limited in Topology 1 design because the timing of D2R resource transmission depends on the timing of last R2D transmission, as shown in both options in Figure 3 below, which are under RAN1 discussion.

Figure 2: Two Options for D2R scheduling in A-IoT air interface

Logically, this has already ruled out Sidelink mode 2 for device side, because there is no (pre-)configured common resource pool for devices to select. Instead, if it is CFRA, the dedicated resource will be provided by the reader. For CBRA, the set of resource to perform slotted aloha is also provided (via DL R2D transmission), which are not to be timed based on R2D transmission (paging/trigger message).

Proposal 7 	Mode 2 resource allocation (autonomous RA) is not applicable to A-IoT device in Topology 1 and Topology 2. Devices in Topology 2 use either dedicated resource indicated by the UE reader or perform slotted aloha among resources indicated by the UE reader.

Then, a question more essential to Topology 2 is that how UE reader determines the set of those allocated D2R resources which are to be conveyed in the D2R resource indication signalling. This would be different from the gNB reader case in Topology 1, which is up to gNB implementation.

Under the assumption that NW is in control of resource allocation, there are still three options:

- Option 1: NW-configured shared pool: gNB allocates a “shared” periodic common D2R resource set for all UE readers in its coverage, and each UE reader autonomously select D2R resource for scheduling D2R transmission.

- Option 2: NW-configured reader-specific pool: gNB allocates a dedicated periodic D2R resource set specific to one UE reader and the UE reader is free to select D2R resource among them to schedule D2R transmission.

- Option 3: NW-scheduled resource on-demand: gNB does not allocate periodic D2R resource set for UE reader to use by itself. Instead, the D2R resource(s) are allocated dynamically by gNB for a specific UE reader based on UE reader’s request, and both R2D and D2R transmissions are to be scheduled together by gNB.

Among all those three options, we think Option 3 is most efficient because any pre-allocated D2R resource may be wasted if there is no R2D transmission resource to be used to convey the scheduling information for D2R transmission. Also, this option will also make A-IoT enabled gNB behave similar to gNB reader in terms of determining A-IoT air interface resource allocation.

Proposal 8 	For D2R transmission in Topology 2, D2R resource(s) are allocated dynamically by gNB for a specific UE reader based on UE reader’s request, and D2R to be scheduled with corresponding triggering R2D transmission altogether by the gNB.

R2D resource allocation

For R2D resource to be used by UE reader, Sidelink mode 1 RA can be simply reused for A-IoT enabled UE. For instance, DCI-based and RRC-based CG can be considered, and DCI-based dynamic grant can also be supported. Those allocated A-IoT grants are supposed only used by a particular UE reader, and not to be shared with other UE readers.

Proposal 9 	Sidelink Mode 1 resource allocation method can be used as a baseline to design NW-allocated resource control for R2D resource allocation to A-IoT enabled UE.

To adopt Sidelink Mode 2 RA design for Ambient IoT, a common shared R2D resource pool in PRDCH has to be defined and broadcasted in SIB or configured with dedicated RRC signalling. Then, each UE reader autonomously select its R2D transmission resource, with random selection or based on sensing.

The biggest problem with such a RA configuration is performance, as UE reader’s transmission with mode 2 RA may collide with other UE because the selected resource is a shared common pool, especially when random resource selection is used. But if sensing is to be introduced, then some complicated L1/L2 design has to be added in UE reader. The benefit for Mode 2 RA approach is less gNB involvement in resource assignment and less Uu signalling overhead. Given the pro and con of mode 2 resource allocation, we think there is no strong justification to support this in Rel-19, especially when Rel-19 has much less TU to spend for normative work of Ambient IoT.

Proposal 10 	Mode 2 resource allocation method (common resource pool) is not considered in A-IoT R2D resource allocation for UE Reader in Rel-19.

## 2.4 UE reader Assistance information to A-IoT enabled gNB

RAN3 has also agreed the following signalling flow diagram for inventory procedure in Topology 2, as shown in Figure 3.

Figure 3: Message flow for A-IoT Inventory in Topology 2 (if NAS/UP based solution is used)

In the above diagram, the CN “inventory request” signalling for inventory procedure will trigger the corresponding AS layer procedure (e.g. Paging and Random Access) in A-IoT UE reader , as shown in Step 3, and the inventory results will be eventually reported back to A-IoT CN by the A-IoT UE reader. For command procedure, there will be similar interaction between CN and UE reader. However, the high-level signalling flow in Figure 2 has omitted necessary messages interaction with gNB, which may occur right ahead of step 3, or even earlier to solicit proper configuration for UE reader’s operation in A-IoT air interface.

Resource Configuration assistance

It is agreed that NW is in control of resources to be used by AIoT air interface. To be more specific, the resources are supposed to be configured by the serving gNB of UE reader. To make this configuration be effective and accurate, some assistance information from UE reader needs to be shared with A-IoT enabled gNB, For example, if gNB allocated dedicated R2D rand D2R resource (periodically) to the UE reader, the UE reader need report the expected A-IoT traffic amount in both directions and the number of intended devices (e.g., to participate in slotted aloha contention), so that the gNB can dimension the resource correspondingly.

CW node assistance

For example, to make step 3 work, the A-IoT UE reader need coordinate CW generation to support backscattering (for device Type 1/Type 2a) if the UE reader itself is not a CW node.  The CW node is usually co-located/managed by gNB (e.g., in-topology or out-of-topology). So, it is necessary for the UE reader to send an indication of the expected A-IoT air interface operation to allow NW to configure CW support. Note that even if UE itself is a CW node (in Topology 2), it may have some internal conflict for UE to generate CW in CW band and transmit/receive A-IoT air interface at the same time due to RF constraint, so NW assistance for CW generation from additional CW nodes may still be desired in this case.

To make the above information available to the gNB, we think it is important for UE to send an UL RRC message, which can be either UEAssistanceInformation or a new RRC message dedicated for A-IoT use case.

Proposal 11	Use UAI or a new RRC message for UE reader to indicate the resource configuration assistance information and/or CW assistance information.

There could be additional information by UE reader to NW, such as energy status, mobility pattern and UE location, which can be helpful for the NW selection of UE reader(s). However, we believe UE reader selection is to be discussed by SA2 and or RAN3 first. Whether UE should report that information in RRC message or upper-layer signalling (e.g., NAS) is to be determined based on conclusion from other WGs on UE reader selection process.

Proposal 12	Wait for other WG’s progress on UE reader selection process to determine whether/how UE reader to share its location, mobility pattern, energy status assistance information to NW.

## 2.5 SIB Enhancements for Topology 2

One thing is to consider SIB indication of the support of this function by A-IoT-enable gNB. This will help mobile UE reader choose a suitable cell for the reader operation, as we can’t assume all the gNBs will be upgraded to support A-IoT in Topology 2 scenarios. It is also evident that this indication is not equal to the “reader function” support in Topology 1, because an AIoT-enabled gNB may not be able to act as a reader in Topology 1. So far, there is no obvious need for a SIB change to indicate a “reader gNB”. Furthermore, such indication would be needed to support mobility in RRC_INACTIVE (if supported).

Proposal 13	Consider SIB indication of Topology 2 support by A-IoT enabled gNB, which does not necessarily support reader function.

## 2.6 Uu MAC Enhancements for Topology 2

Based on the discussion in Section 2.4, NW (A-IoT enabled gNB) may allocate R2D and/or D2R resources for the UE reader operation in Topology 2. Besides using dedicated RRC signalling to configure A-IoT grants, L1/L2 indication of those A-IoT resources in Uu interface are also possible and should be considered.  From the UE reader perspective, BSR-like procedure may be needed for solicit A-IoT grants. This is because the A-IoT traffic initiated by a CN A-IoT function may be transparent to gNB and serving gNB is not aware of actual traffic situation in the UE reader, in general. This is similar to the Sidelink BSR design [2] where the grants of another radio interface are solicited by a BSR procedure.

Proposal 14	Define new BSR MAC CEs in Uu to support A-IoT reporting buffered R2D/D2R data.

It is also very likely that some fine-tuning of UE reader operation regarding certain radio parameters are needed. For example, Tx power control  in A-IoT air interface can be done via a DL MAC CE in Uu interface to regulate the potential interference issue for gNB’s Uu operation . Also, if a UE reader is also acting as a CW node, then some ON/OFF switch of CW generation may be considered. We think this has to be first discussed in RAN1, then RAN2 can decide how to support with DL MAC CE(s) based on RAN1 agreements.

Proposal 15	RAN2 to study the support of DL MAC CE enhancements based on RAN1 agreements.

## 2.7 RAN2 impact of “NGAP + RRC” approach

There are views in SA2 that a “NGAP+ RRC” approach is to be considered for the transport of A-IoT data for Topology 2. It is important for RAN2 to have a clear idea on the potential RAN2 impact. It is preferred to limit the RRC impact as much as possible for supporting this approach. For example, the RRC Container shall be used to encapsulate A-IoT upper layer data. However, regarding the visibility information, such as “estimated response message size” or “estimated number of devices” or “service type” information, it is unclear whether they will be encoded as new RRC message in ASN.1 format or can also be included in RRC container. So, we think RAN2 need to clarify this in the SI phase.

Proposal 16 	RAN2 to clarify whether there is ASN.1 impact (e.g., new RRC message or new IE in existing RRC messages) to be introduced to support “NGAP + RRC message” for Topology 2.

# 3	Conclusions and Proposals

In this paper, we have discussed the remaining issues on Topology 2 and have the following proposals:

Proposal 1 	RAN2 to discuss whether to define a validity timer for A-IoT radio resources; the timer is started when RLF occurs, when the timer expires the UE reader releases the configured A-IoT radio resources.

Proposal 2 	For in-coverage operation, besides RRC_CONNECTED sate, RRC_INACTIVE UE reader can be supported with SDT.

Proposal 3  	Upon cell reselection in RRC_INACTIVE, if the target cell doesn’t support A-IoT, stops A-IoT communications with A-IoT devices and releases the A-IoT configuration.

Proposal 4 	RRC_IDLE  state UE reader and OOC UE reader is not supported in Rel-19 A-IoT.

Proposal 5 	For the extensibility to future temporarily out-of-coverage case, RAN2 assumes that resource allocation for RLF/HOF is reused w/o any further enhancements in Rel-19.

Proposal 6 	D2R resource indication signalling in Topology 2 reuse exactly the same design for Topology 1. Whether this signalling is from UE reader or gNB reader is transparent to A-IoT device.

Proposal 7 	Mode 2 resource allocation (autonomous RA) is not applicable to A-IoT device in Topology 1 and Topology 2. Devices in Topology 2 use either dedicated resource indicated by the UE reader or perform slotted aloha among resources indicated by the UE reader.

Proposal 8 	For D2R transmission in Topology 2, D2R resource(s) are allocated dynamically by gNB for a specific UE reader based on UE reader’s request, and D2R to be scheduled with corresponding triggering R2D transmission altogether by the gNB.

Proposal 9 	Sidelink Mode 1 resource allocation method can be used as a baseline to design NW-allocated resource control for R2D resource allocation to A-IoT enabled UE.

Proposal 10 	Mode 2 resource allocation method (common resource pool) is not considered in A-IoT R2D resource allocation for UE Reader in Rel-19.

Proposal 11	Use UAI or a new RRC message for UE reader to indicate the resource configuration assistance information and/or CW assistance information.

Proposal 12	Wait for other WG’s progress on UE reader selection process to determine whether/how UE reader to share its location, mobility pattern, energy status assistance information to NW.

Proposal 13	Consider SIB indication of Topology 2 support by A-IoT enabled gNB, which does not necessarily support reader function.

Proposal 14	Define new BSR MAC CEs in Uu to support A-IoT reporting buffered R2D/D2R data.

Proposal 15	RAN2 to study the support of DL MAC CE enhancements based on RAN1 agreements.

Proposal 16 	RAN2 to clarify whether there is ASN.1 impact (e.g., new RRC message or new IE in existing RRC messages) to be introduced to support “NGAP + RRC message” for Topology 2.

# 4	References

[1]	Chairman’s Notes, RAN2#127-bis

[2] TS 38.321 v 18.3.0