# 3GPP TSG RAN WG2 Meeting #128      	                           R2-2409968
Orlando, USA, November 18-22, 2024

Agenda item:	8.13.3

Source:	Apple

Title:	Discussion on End-to-End Connection Setup Approaches for Multi-hop UE-to-NW Relay

WID/SID:	NR_SL_relay_enh2– Release 19

Document for:	Discussion and Decision

# 1 Introduction

In this paper, we discuss the control plane design for setup end-to-end path for Layer-2 Multi-hop UE-to-NW relay.

# 2 Discussion

A major controversial issue in CP design is “FFS what RRC states are supported for the intermediate relay UE”[1], a. k. a., whether the “intermediate relay UE(s)” needs to be in RRC_CONNECTED state for end-to-end traffic delivery between the remote UE and gNB, or they can be in IDLE/INACTIVE. For this issue, we provide our analysis as below.

### 2.1 Connection Setup with Recursive U2N (mandating CONNECTED relays)

In “Recursive U2N” approach, as shown in the example diagram with one extra-hop scenario below in Figure 1, from the remote UE’s perspective, its serving relay UE is the first intermediate relay UE (labelled as “relay_1st"), and this first intermediate UE handles all the equivalent functions as Rel-17 U2N Relay UE. Similarly, the first intermediate relay UE models its own access as a single-hop U2N relay via Relay_Last UE. Basically, this is a recursive scheme where each intermediate relay UE plays a “double role” as “Remote U2N UE + Relay U2N UE”.

Figure 1: Recursive U2N approach for 2-hop U2N Relay

For the sake of clarity, as multiple intermediate relays may be involved in some of the supported topologies in MH case, we use “upstream” to describe the direction from remote UE to gNB, and “downstream” for the reverse direction. For example, a downstream relay node will be the relay node which is 1-hop closer to the remote UE than the current relay.

With this modelling and follow Rel-17 legacy U2N design, every intermediate relay UE has the following characteristics in regards of supporting end-to-end traffic:

- Each intermediate relay UE, once being triggered by its downstream node (e.g., first Uu RRC message from the downstream node), is triggered to create its own first RRC message to establish its own Uu RRC connection and will stay RRC_CONNECTED to support end-to-end data forwarding.

- Each intermediate relay UE cannot enter RRC_CONNECTED state until its next-hop upstream relay UE enters RRC_CONNECTED first. This is based on the legacy U2N relay principle.

- Each intermediate relay UE need to be allocated a local ID, so its e2e Uu RRC signalling can be delivered via a relay path. Corresponding SRAP mapping for SRB1 also needs to be provided. But each intermediate relay UE may not have its own Uu DRB traffic, so SRAP mapping for intermediate relay UE’s Uu DRB is not needed.

Then, for this topology to enable end-to-end user plane traffic along the “remote UE - first intermediate relay UE - last relay UE- gNB” path needs to be setup with CP signalling. We have illustrated the control plane signalling exchange sequence in Figure 2.

Note that the signalling diagram has not include all possible RRC messages, but focus on key essential RRC signalling needed for enable basic relay-related configurations. For example, “Security Model Command” are not included/depicted  for each CONNECTED remote/relay UE.

Figure 2: End-to-End RRC Connection Setup for MH (1 extra hop) case with all UE started in RRC_IDLE state and ended in RRC_CONNECTED state

A few points worth noting in the above signalling diagrams:

- The Uu link between the last relay UE and gNB need to be reconfigured multiple times. In addition to configuring remote UE, gNB need also configure the underlying L2 relay path to each of the intermediate relay UE (by treating them as a U2N Remote UE with Local ID/BEAER ID usage) to deliver the control message to this relay UE.  Each of such a “remote UE”(virtual or physical) added in CONNECTED, NW need to provide SRAP related configurations for this new “remote UE”. This results a linear O(n) increase of Uu hop transmissions with the increasing number of extra hops;

- Similarly, between each intermediate relay UE and gNB, the “virtual” RRC connection needs to be reconfigured multiple times, each for which for every extra “remote UE” (virtual or physical) added in the path, to provide SRAP related configurations for this new “remote UE”. Since this RRC connection itself spans crossing multiple hops, this results a non-linear O(n2) increase of per-hop transmissions with the increasing number of extra hops;

- The PC5 Relay RLC channels between remote UE and intermediate relay UE and PC5 Relay RLC channels between relay UEs need to be established and reconfigured at least twice (per each direction): One for the support of SRB-transport (e.g., for RRC signalling), once for support of remote UE Uu DRBs end-to-end. This results a linear O(n) increase of PC5 hop transmissions with the increasing number of extra intermediate relays. Here, we assume the first PC5 Relay RLC channel(s) established for Uu SRB transport can be reused whenever a new downstream relay node is added by NW reconfiguration. Therefore, this is a minimum number of PC5 RRC signalling to be generated. Some gNB configuration may require UEs to configure a new PC5 Relay RLC channel for the Uu SRB(s) of the newly added downstream intermediate relay(s), thereby requiring more PC5-RRC signalling invovled.

With all above considered, we can tally the minimum number of messages for the signalling diagram in Figure 2 as:

- 33 RRC messages exchanged (21 Uu RRC messages and 12 PC5-RRC message) among the UE(s) and gNB.

- There is total 52 per-hop control plane message transmissions (w/o retransmission) for all RRC signalling, and 40 of them are used for Uu RRC message transmissions.

Note that the above statistics does not include any Sidelink relay discovery message, or any PC5-S signalling exchanges to setup PC5 links among remote UE and relay UEs. We have only counted necessary signalling for the end-to-end connection setup triggered by remote UE’s RRCSetupRequest.

A mathematical analysis can be used to provide additional statistics for the scenarios with additional hops for multi-hop U2N relay. The results are summarized in the table below (with the corresponding mathematical formula):

| Number of extra Hops | 1 | 2 | 3 | 4 | N extra hops |
| --- | --- | --- | --- | --- | --- |
| Uu RRC Signalling | 21 | 30 | 39 | 48 | 12+9N |
| Per-hop transmissions of Uu RRC signalling | 40 | 71 | 111 | 160 | 18+17.5N+4.5N2 |
| PC5-RRC Signalling (RRCReconfigurationSL) | 12 | 20 | 28 | 36 | 4+8N |
| Total # of RRC signalling | 33 | 50 | 67 | 84 | 16+17N |
| Total # of RRC control related transmissions in air interface (Uu/PC5) | 52 | 91 | 139 | 196 | 22+25.5N+4.5N2 |

Table 1: Statistics of Minimum RRC signalling related transmissions for Recursive U2N Connection Setup

The very most concerning fact indicated by Table 1 is that the control plane signalling transmission’s exponential growth by O(n2) with the increasing number of extra hops for this approach. That means this solution is not scalable and extensible to multi-hop scenario in general. The signalling overhead for hundreds of messages transmission can easily outweigh the number of UP transmissions needed for upper layer data communication in public safety scenarios, where sometimes only a few emergency text messages need to be forwarded in user plane.

Observation 1	Control plane signalling overhead for set-up and configure CONNECTED state intermediate relays increase by O(n2) with the number of extra hops.

Regarding with the overall latency of the whole end-to-end connection setup procedure, we can also see this metric is also unscalable. From the RRC setup perspective, we can see the Uu RRC signalling transmissions (per-hop) increase by O(n2) with the increasing number of extra hops. More important, those signalling has to occur sequentially because each intermediate relay UE cannot really start is own RRC connection establishment unless its upstream relay node to enter CONNECTED state first. Only after this relay node is dedicatedly configured by gNB with SRAP configurations for the relay path, the intermediate relay UE’s own RRC signalling can be moved upstream. Therefore, this cascading effect for UEs entering RRC_CONNECTED state will increase the latency significantly. It is worth noting that the increased delay will add uncertainly and unreliability of the whole multi-hop setup. Because relay UEs and remote UEs may move during this long period of time, the relay path setup could fail, and the UP traffic will not be able to get delivered.

Observation 2	Latency in connection setup will scale non-linearly with the increasing number of hops, as each relay UE need wait for its upstream relay node to enter CONNECTED via its own multi-hop connection establishment/configuration procedure.

There are some other drawbacks for mandating intermediate relay UE(s) in CONNECTED state for Layer-2 multi-hop U2N scenarios. As a virtual remote UE (as playing “remote-relay” double role), intermediate relay UE can only “connect” to a single serving cell. Then, it will not be able to help other remote UEs to connected to a different cell via a different path (e.g., via a different last relay UE). This restriction will negatively affect the utilization of the relay UEs in public safety scenarios. For example, the cross-path scenarios [2] as shown in Figure 3 cannot be supported with this approach.

Figure 3. Cross-path NH Scenario (figure copied from R2-2407102[2])

Observation 3	CONNECTED Intermediate relay UE in Recursive U2N approach are locked to a single path towards a gNB will reduce its usefulness in support different remote UEs trying to reach different gNBs (i.e., cross-path setup).

Also, the requirement for intermediate relay UE staying CONNECTED in this approach would also mandate the intermediate relay UE’s own UL traffic to always follow the upstream path as the remote UE’s. This unnecessarily constrains the available of intermediate relay UEs. For example, if an intermediate relay UE camps in a cell which does not support SL, it cannot even send its own Uu traffic towards this cell directly but has to use the detour path to reaching the “CONNECTED” gNB several hops away. For this reason, there is much less motivation for the intermediate relay UE to help a remote UE while hurting its own Uu traffic delivery.

Observation 4	CONNECTED Intermediate relay UE in Recursive U2N approach are forced to send its own Uu traffic in a detour path even if a direct path is available.

On the other side, there is some benefit for requiring all intermediate relay UEs into the CONNECTED state. One benefit is that since the relay UE is in RRC_CONNECTED state, it could conduct measurements on PC5-link according to dedicated NW configuration and report the measurement results to the serving gNB. This may help gNB to make QoS split decisions.

Observation 5  CONNECTED intermediate relay UE can report its PC5 link measurement results to gNB.

### 2.2 Connection Setup while keeping intermediate relay IDLE/INACTIVE

In this alternative approach, the intermediate relay UE is neither behave like a Rel-17 L2 U2N Remote UE nor a Rel-17 L2 U2N Relay UE. Instead, its operating mechanism is only to facilitate the data/signalling transport between a remote UE and the corresponding last relay UE via PC5 hop(s). From this perspective, the intermediate relay UE does not need to enter RRC_CONNECTED state and does not need to be controlled by NW via dedicated control signalling. What an intermediate relay UE is required to manage is mainly the PC5 Relay RLC channels and related SRAP configurations to map end-to-end traffic between remote UE and gNB into those channels.

Based on this understanding, the end-to-end RRC connection setup, largely realized by Uu RRC signalling between gNB and remote UE, can be supported by intermediate relay UE(s) staying in RRC_IDLE or RRC_INACTIVE with default PC5 Relay RLC channel configurations, which is similar to SL-RLC0/SL-RLC1 introduced in Rel-17 Layer 2 U2N relay design. This has multiple benefits. In additional to the evident advantage of avoiding the cumbersome RRC connection establishment procedures for intermediate relay UE(s), the delivery of can also be streamlined and latency can be reduced.

Observation 6  	Intermediate relay UE can rely on default PC5 Relay RLC channel(s) for the transport/forwarding of e2e RRC signalling during end-to-end connection setup procedure.

Then, only after end-to-end Uu DRB is established/configured by NW, and QoS parameters are available, the intermediate relay UEs can derive PC5 relay RLC channel configurations and add/modify PC5 Relay RLC channel accordingly. In Rel-18 U2U work, such a mechanism is deemed feasible and can be reused here for Rel-19 multi-hop relays. Although multi-hop Layer 2 U2U relay is not in the scope of Rel-19 WI, this does not mean the R18 U2U mechanism cannot be adopted in this Rel-19 Multi-hop U2N relay work. For example, some of U2U relay discovery mechanism used in Rel-18 U2U relay have been reused for Rel-19 Multi-hop U2N relay discovery. So, there is no problem to also consider reusing the L2 U2U relay UE’s mechanism to manage PC5 Relay RLC channel(s) and corresponding SRAP mappings.

Therefore, without dedicated NW configuration, it is feasible and legitimate to allow IDLE/INACTIBE relay UEs to manage PC5 relay RLC channels and SRAP mappings, based on SIB or pre-configuration. With this approach,  the end-to-end user plane traffic can still be supported via intermediate relay UEs with self-derived SRAP mappings.

Observation 7 	Intermediate relay UE can derive PC5 Relay RLC channel configurations and SRAP mapping from SIB/Pre-configuration based on QoS requirements, as similar to R18 U2U design.

Then, for enabling the end-to-end traffic forwarding of UP data, we have illustrated the control plane signalling exchange sequence in Figure 4.

Similar to Figure 2, we have only included key essential RRC signalling needed for enable basic relay-related configurations. The highlights of this signalling diagrams are:

- Only the last relay UE and the remote UE has entered RRC_CONNECTED state.

- The first Uu RRC message sent by the remote UE will be immediately forwarded by intermediate relay UE(s) to reach the last relay UE.

- There is no dedicated NW SRAP configuration and PC5 Relay RLC channel configurations for Uu SRBs of intermediate relay UEs, as those UEs are not in CONNECTED state. No local ID allocations for intermediate UEs, either.

- Correspondingly, there is no need of PC5-RRC reconfiguration procedure to establish/modify PC5 Relay RLC channels to deliver any end-to-end traffic (e.g., Uu SRB) towards any intermediate relay UE.

Figure 4: End-to-End RRC Connection Setup for MH (1 extra hop) case with all UE started in RRC_IDLE state and only Remote UE and last Relay UE entering RRC_CONNECTED state

Note that what has been drawn in the above signalling diagram is the minimum number of message(s)/signalling. For example, we have assumed that after NW provide its initial SRAP configuration and Uu/PC5 relay RLC channels configurations to the last relay UE to support the remote UE’s CP/UP traffic, it does not need to reconfigure them even after receiving SUI message(s) from the remote UE later. As NW can always reconfigure Uu Relay RLC channel(s) and/or PC5 Relay RLC channel(s) based on QoS requirements on remote UE’s DRB anytime. Also, regarding the relay UE deriving QoS requirements and sharing QoS (e.g., remaining PDB) with the next-hop relay nodes, this is also assumed to be carried in existing RRCReconfigurationSidelink, which are used for PC5 relay RLC Channel configurations. No new PC5-RRC exchanges are dedicated for QoS-related operations. So, what we have depicted above is a minimum set of signalling exchange, as a fair comparison with the first approach illustrated in Figure 2.

Similarly, a mathematical analysis can be used to provide statistics for the scenarios with any number of hops for multi-hop U2N relay. The results are summarized in the table below (with the corresponding mathematical formula):

| Number of extra Hops | 1 | 2 | 3 | 4 | N extra hops |
| --- | --- | --- | --- | --- | --- |
| Uu RRC Signalling | 12 | 12 | 12 | 12 | 12 |
| Per-hop transmissions of Uu RRC signalling | 24 | 30 | 36 | 42 | 18+6N |
| PC5-RRC Signalling (RRCReconfigurationSL) | 8 | 12 | 16 | 20 | 4+4N |
| Total # of RRC signalling | 20 | 24 | 28 | 32 | 16+4N |
| Total # of RRC control related transmissions in air interface (Uu/PC5) | 32 | 42 | 52 | 62 | 22+10N |

Table 2: Statistics of Minimum RRC signalling related transmissions for e2e Connection Setup while keeping intermediate relay UEs in IDLE/INACTIVE

The most striking observation is that the number of Uu RRC signalling involved with this approach is a constant number. This is because no matter how many intermediate relay UEs are in the topology, only remote UE and last relay UE need send Uu RRC signalling. The number of PC5-RRC exchange needed scales by O(n) with the number of extra hops, which is expected. In overall, the total number of per-hop transmission scales with O(n).

Observation 8	The overall control plane signalling overhead for end-to-end set-up w/o requiring intermediate relays entering CONNECTED state scales linearly with the increasing number of extra hops.

Then, the latency for end-to-end connection setup is also reduced significantly. For example, there is no buffering of the initial RRC connection establishment requests in the intermediate relay UEs. Moreover, the cascading effect of RRC connection setup from each relay node disappears, which will largely reduce all the signalling-caused delays along the path. As a result, the remote UE will be expected to receive its response message (e.g., RRCSetup) much earlier than the first approach.

Observation 9	Much faster end-to-end connection setup is possible by enabling intermediate Relay UE operating in IDLE/INACTIVE.

Another benefit for allowing IDLE/INACTIVE intermediate relay UE is the improvement of topology diversity. This is especially good for out-of-coverage UEs. Since they are not really associated with any serving cell, they are out of the rigid control of the NW. Accordingly, it can have freedom to acting as intermediately relay for different paths, serving different remote UEs to reaching different cells. Moreover, the intermediate relay UE has its own freedom to decide how to deliver its own Uu traffic if it is in-coverage, w/o need of taking into the remote UE’s path setup into account. This will allow more intermediate relay UEs to be selected for establishing multi-hop SL relay scenarios.

Even if intermediate relay UE has its own independent path towards a gNB, we do not think this is a multi-path scenario. Because even a intermediate relay UE has the potential of multiple path to reach different cells (intra-gNB or inter-gNB), there is only at most a single path used for its own traffic, so this is not as same as the Rel-18 multi-path scenario.

Observation 10	OOC IDLE/INACTIVE intermediate relay UEs can serving multiple different paths towards different cells, which will improve topology diversity and overall connectivity for public safety use cases.

One drawback of IDLE/INATIVE sidelink UE is that it does not provide PC5 link measurements (e. g., RSRP) to the serving gNB. For this multi-hop case, that means NW may have no clue on the radio link quality of some PC5 hop(s). For example, if we assume link quality is reciprocal, the PC5 link quality between two intermediate relay UEs are not reported to the NW in the scenario with 2 extra hops. For the scenario with only 1 extra hop, the PC5 link quality may be still reported by remote UE or the last relay UE.

Nonetheless, if the SL UE-to-NW relay design principle moves away from absolute centralized control to a more distributed manner, it is reasonable to assume that the gNB does not need to know everything and control everything. If the QoS split for certain PC5 hops are to be done by the relay UE itself, maybe it is fine to let relay UE to decide based on its own PC5 RSRP measurements, w/o reporting them to NW.

Observation 11:	gNB is not aware of the link quality between IDLE/INACTVE intermediate relay UEs, which requires alternative design for multi-hop end-to-end QoS.

### 2.3 Comparison & proposed Way-forward

Based on the analysis of pros and cons of each approach above, we summarize the comparison in Table 3:

|  | Approach with CONNECTED intermediate relay | Approach with IDLE/INACTIVE intermediate relay | Comment |
| --- | --- | --- | --- |
| Control plane complexity and overhead | ✗ | ✓ | O(n2) complexity/overhead (Approach 1) with O(n) complexity (Approach 2) |
| Latency in end-to-end connection setup | ✗ | ✓ | Each relay UE need wait for downstream relay node to enter CONNECTED, adding latency and uncertainty of the path setup. |
| Flexibility to support different topologies | ✗ | ✓ | Support cross-path scenarios if OOC relay UE is not controlled by a single gNB |
| Availability of intermediate relay UE | ✗ | ✓ | For Approach 2, intermediate relay UEs in any RRC state and OOC case (including UEs connected to a cell not supporting SL) |
| gNB control of QoS based on radio link measurement reports | ✓ | ✗ |  |

Table 3: Comparison of pros and cons of two approaches

From the control plane perspective, the most important aspect is the complexity and associated overhead. The major concern of the Recursive U2N approach is that the control procedure and its overhand will be very heavy, as shown by our analysis with O(n2) complexity. This means the approach is not scalable for longer paths with more hops. It is not scalable from the latency perspective either, because the UP end-to-end traffic needs to wait for a large number of CP signalling transmissions to finish first. As any one of them may have transmission failure, this will result more retransmissions and even fail the signalling delivery, thereby endangering the whole end-to-end path setup procedure.

It is also worth considering the rigid constrains of the first approach which puts unnecessary “path sharing” burden for intermediate relay UEs. This limits the availability of suitable intermediate UE(s), as any UEs unwilling or unable to connected to the same cell as the selected last relay UE will be excluded by the relay selection process. This would hurt the chances of providing the needed connectivity for remote UEs demanding multi-hop access to NW.

Regarding ensuring end-to-end QoS, a centralized approach could be easier, but that is more of a secondary concern, compared to the primary concern of CP signalling overhead and complexity. Also, even for the QoS control itself, there is a trade-off between the increased latency and signalling overhead vs. the performance gain. For instance, without centralized NW control, a default simple PDB split scheme with PDB evenly distributed among hops for multi-hop relay traffic can also work with less optimized performance.

For the consideration of long-term design for diverse MH scenarios and future extensibility, using the centralized approach is short-sighted as this kind of approach is fundamentally unscalable.

Based on the above observations and considerations, we think the correct way-forward is to adopt the 2nd approach for control plane design.

Proposal 1	For CP design, adopt the approach of allowing intermediate relay UEs to stay IDLE/INACTIVE.

# 3 Conclusion

In this contribution, we discuss the CP design for Multi-hop SL relay,

For the approach of recursive U2N, we have the following observations:

Observation 1	Control plane signalling overhead for set-up and configure CONNECTED state intermediate relays increase by O(n2) with the number of extra hops.

Observation 2	Latency in connection setup will scale non-linearly with the increasing number of hops, as each relay UE need wait for its upstream relay node to enter CONNECTED via its own multi-hop connection establishment/configuration procedure.

Observation 3	CONNECTED Intermediate relay UE in Recursive U2N approach are locked to a single path towards a gNB will reduce its usefulness in support different remote UEs trying to reach different gNBs (i.e., cross-path setup).

Observation 4	CONNECTED Intermediate relay UE in Recursive U2N approach are forced to send its own Uu traffic in a detour path even if a direct path is available.

Observation 5  CONNECTED intermediate relay UE can report its PC5 link measurement results to gNB.

For the approach of supporting IDLE/INACTIVE intermediate relay UEs, we have the following observations:

Observation 6  	Intermediate relay UE can rely on default PC5 Relay RLC channel(s) for the transport/forwarding of e2e RRC signalling during end-to-end connection setup procedure.

Observation 7 	Intermediate relay UE can derive PC5 Relay RLC channel configurations and SRAP mapping from SIB/Pre-configuration based on QoS requirements, as similar to R18 U2U design.

Observation 8	The overall control plane signalling overhead for end-to-end set-up w/o requiring intermediate relays entering CONNECTED state scales linearly with the increasing number of extra hops.

Observation 9	Much faster end-to-end connection setup is possible by enabling intermediate Relay UE operating in IDLE/INACTIVE.

Observation 10	OOC IDLE/INACTIVE intermediate relay UEs can serving multiple different paths towards different cells, which will improve topology diversity and overall connectivity for public safety use cases.

Observation 11	gNB is not aware of the link quality between IDLE/INACTVE intermediate relay UEs, which requires alternative design for multi-hop end-to-end QoS.

Based on the discussion, we propose:

Proposal 1	For CP design, adopt the approach of allowing intermediate relay UEs to stay IDLE/INACTIVE.

# 4 References

[1] Chairman’s Notes, RAN2#127

[2] R2-2407102, Control procedure for multi-hop L2 based U2N relay , Qualcomm Incorporated