3GPP TSG-RAN WG2 Meeting #128	R2-2409985

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.18

Source:	Nokia

Title:	Support of ProSe in NPN (CT1 LS R2-2409504/C1-245500)

WID/SID:	TEI19 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

CT1 is working on TEI19_ProSe_NPN, which introduces the ProSe support in NPN. RAN2 has received an LS from CT1 (R2-2409504/C1-245500) requesting RAN2 to investigate if this has any RAN2 impacts and perform the necessary specification updates if necessary.

This paper investigates the potential RAN2 impacts of this CT1 work item.

# 2	Discussion

CT1 is working on TEI19_ProSe_NPN, which introduces the ProSe support in NPN. The related SA2 WID can be found in SP-240122. The CT WID (CP-242106) contains the following objectives for CT1 to Support Proximity-based Services in 5GS in NPN scenarios:

-	extending the existing specification to support 5G ProSe in subscribed SNPN in non-roaming scenarios (e.g. updating PLMN specific terminologies/identifiers to also cover SNPN);

-	5G ProSe layer-2 UE-to-network relay in conjunction with CAG in PNI-NPN; and

-	potential update to support NPN selection triggered by 5G ProSe communication to cover particular interactions between Proximity-based Services in 5GS and NAS functions related to MS in idle mode.

Based on these objectives the RAN2 specifications should be checked whether the network identification part of the RAN2 specifications contains any limitation that prevents the use of SL communication including layer 2 relays in NPNs.

In the next subsections the impact of the introduction of ProSe support in NPNs is investigated in RAN2 specifications.

## 2.1	Stage 2 (38.300) specification impacts

In TS 38.300 Section 16.6 contains the stage 2 specification for SNPNs, and 16.7 contains the stage 2 specification for PNI-NPNs. None of the sections contains any reference to the use of ProSe in NPNs. Therefore, nothing prevents based on these sections to support ProSe in NPNs.

Observation 1: No change is needed in SNPN and PNI-NPN stage 2 specifications to enable ProSe support in NPNs.

In TS 38.300 Section 16.9 contains the stage 2 specification for NR sidelink communication and 16.12 contains the stage 2 specification for sidelink relays. None of the sections contains any reference or limitation on to the use of ProSe in NPNs. However, in 16.12.4 (Relay Selection/Reselection) there is a sentence that may limit the use of SL relays in SNPNs:

“For L2 U2N Relay (re)selection, the PLMN ID and cell ID can be used as additional AS criteria.”

Observation 2: A minor rewording in section 16.12.4 is needed to avoid any limitation of using SL relays in SNPNs.

## 2.2	RRC (38.331) specification impacts

SIB1 contains the network identifiers including CAG and SNPN identifiers supporting the cell. The only SL related parameter in this SIB is n3c-Support, which is not PLMN specific. Therefore, we think the support of sidelink in NPNs has no impact to SIB1. SIB12 contains NR sidelink communication/discovery configuration. As this is common for all network supporting the cell, this contains no direct or indirect reference to the network (PLMN) identifier. The situation is similar with SIB23, which contains NR sidelink Positioning configuration for dedicated SL-PRS resource pool.

Observation 3: No change is needed in SIB definitions to enable ProSe support in NPNs.

In PC5 sidelink information elements are specified in clause 6.3.5 and the PC5 sidelink messages are specified in clause 6.6. No direct or indirect reference to the network (PLMN) identifier has been found in these sections, except that the SL-MeasResultRelay-r17, which contains CellAccessRelatedInfo. As this CellAccessRelatedInfo already can contain the NPN related information, no need to change anything has been identified.

Observation 4: No change is needed in PC5 specification to enable ProSe support in NPNs.

The sidelink configuration parameters that can be found in SL-ConfigDedicatedNR and in SL-PreconfigurationNR are also network (PLMN) agnostic parameters, therefore the support of NPNs has no impact to the specification.

Observation 5: No change is needed in SL configuration specification to enable ProSe support in NPNs.

As the CT1 LS highlighted in the sidelink related procedure descriptions (clause 5.8) there is one point in clause 5.8.2 where the specification is limited to PLMNs:

1>	if the UE's serving cell is suitable (RRC_IDLE or RRC_INACTIVE or RRC_CONNECTED); and if either the selected cell on the frequency used for NR sidelink communication/discovery/positioning operation belongs to the registered or equivalent PLMN as specified in TS 24.587 [57] or TS 24.554 [72] or the UE is out of coverage on the frequency used for NR sidelink communication/discovery/positioning operation as defined in TS 38.304 [20] and TS 36.304 [27]; or

Observation 6: A minor rewording in section 5.8.2 is needed to avoid any limitation of using SL communication in SNPNs.

## 2.3	Other specification impacts

The lower layer specifications (including SRAP) are agnostic to the network identification. The idle state mobility specification (TS 38.304) already supports NPNs, and sidelink capabilities are not considered during cell selection and reselection.

Observation 7: No other specification impact is expected to enable ProSe support in NPNs.

# 3	Way forward proposals

Based on above observations the support of ProSe in NPNs can be enabled in RAN2 specifications by minor changes in TS 38.300 (see observation 2) and 3TS 8.331 (see observation 6).

Proposal: RAN2 to agree that RAN2 specifications will be updated based on observation 2 (38.300 CR) and 6 (38.331 CR) to enable the use of ProSe in NPNs in Rel-19.

If this proposal is accepted Nokia is ready to provide the CRs for the next meeting. Our view is that if this is agreed then no reply LS to CT1 is needed.

Observation 8: Reply LS to CT1 is only needed if RAN2 cannot update the specifications to enable ProSe support in NPNs.

# 4	Conclusion

This document has made the following observations:

Observation 1: No change is needed in SNPN and PNI-NPN stage 2 specifications to enable ProSe support in NPNs.

Observation 2: A minor rewording in section 16.12.4 is needed to avoid any limitation of using SL relays in SNPNs.

Observation 3: No change is needed in SIB definitions to enable ProSe support in NPNs.

Observation 4: No change is needed in PC5 specification to enable ProSe support in NPNs.

Observation 5: No change is needed in SL configuration specification to enable ProSe support in NPNs.

Observation 6: A minor rewording in section 5.8.2 is needed to avoid any limitation of using SL communication in SNPNs.

Observation 7: No other specification impact is expected to enable ProSe support in NPNs.

Observation 8: Reply LS to CT1 is only needed if RAN2 cannot update the specifications to enable ProSe support in NPNs.

This document has made the following proposal:

Proposal: RAN2 to agree that RAN2 specifications will be updated based on observation 2 (38.300 CR) and 6 (38.331 CR) to enable the use of ProSe in NPNs in Rel-19.