3GPP TSG-RAN WG2 Meeting #128	R2-2410015

Orlando, US 18th-22nd Nov. 2024

Agenda Item:	8.5.2

Source: 	Xiaomi

Title:  	Discussion on on-demand SSB

Document for:	Discussion and Decision

1	Introduction

In last RAN2 meeting, we had some further disccussion on OD-SSB and achieved the following agreement.

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

- NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

In this contribution, we would like to further discuss the procedures and signalling to support on-demand SSB and have corresponding observations and proposals.

2	Discussion

2.1 Activation/deactivation MAC CE

Also according to RAN1 discussion, deactivation of OD-SSB transmission is supported and there are the following alternatives to indicate the deactivation of OD-SSB transmission explicitly or implicitly.

Agreement - For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options

In addition, RAN2 agreed to include OD-SSB indication for multiple SCells simultaneously in the same MAC CE. Generally, there are two options as listed below:

- Altenative1: bitmap to indicate the activation/deactivation status of OD-SSB per SCell

One byte/four bytes bitmap can be used to indicate the activation/deactivation status of SSB transmission per SCell. Regarding the definition of each bit, similar as the SCell activation/deactivation MAC CE, each bit can be related to a SCell with SCellIndex i in no matter whether this SCell is OD-SSB SCell or not and the bit corresponding to normal SCell should be set to a specific value. Another alternative is each bit related to an OD-SSB SCell, but to keep the MAC CE simple and unified, the bitmap should still remain as one byte or 4 bytes with all the remaining bits as reserved bits.

- Alternative2: SCell index to indicate the activation of OD-SSB per SCell

For this option, SCell index is explicitly indicated in the MAC CE. Only index related to an OD-SSB SCell of which the OD-SSB transmission is activated can be included.

In our understanding, if OD-SSB deactivation is indicated explicitly, alternative 1 can be adopted while if OD-SSB is implicitly deactivated, alternative 2 is more suitable. Considering whether to explicitly or implicitly deactivate OD-SSB is still under RAN1 discussion, how to design the indication should be postponed until there is clear conclusion from RAN1.

Observation 1: How to design the indication in MAC CE depends on the conclusion of implicit deactivation or explicit deactivation of OD-SSB which is still under discussion in RAN1.

In addition, RAN1 has agreed to indicate at least the applicable periodicity with MAC CE, therefore, additional information associated with the OD-SSB transmission should be included in the MAC CE.  If only few parameters to be additionally indicated, these parameters can be explicated included while if there are multiple parameters, an ID may be used to associate with a set of applicable parameters and including the ID is sufficient. However, since the other parameters are still pending RAN1 discussion, how to indicate the additional information can be addressed later when RAN1 has final conclusion.

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

Observation 2: How to design the additional information field in MAC CE depends on the conclusion of parameters (e.g., periodicity) that can be indicated by MAC CE which is still under discussion in RAN1.

Therefore, we think the detailed design of the MAC CE should be further discussed once there is more progress in RAN1.

Proposal 1: RAN2 postpone the detailed design of the MAC CE until there is more progress from RAN1 on implicit/explicit deactivation and parameters to be indicated by MAC CE.

2.2 RRC impact

No matter of which scenario and which trigger to be supported, UE should be informed of whether a SCell is a normal SCell or on-demand SSB SCell. This is because without this information, UE may derive incorrect measurement results with the assumption that the SCell is normal SCell and SSB is transmitted. It may cause problems on RRM procedure as the accuracy of measurement may be significantly degraded. Therefore, we think upon SCell addition, NW should indicates whether the SSB transmission on the SCell is on-demand manner.

Proposal 2: Upon SCell addition, NW indicates whether the SSB transmission on this SCell is on-demand manner via RRC signalling.

2.3 L3 RRM measurement

2.3.1 Impact on SMTC

In last RAN2 meeting, we discussed the impact of OD-SSB on L3 RRM measurement, there are two candidate solutions as listed below. In addition, RAN4 sent LS [3] to request RAN2 to discuss the impact on SMTC due to SSB adaptation, which is quite similar as case-2 of OD-SSB.

- Option1: Based on different SMTC configuration when OD-SSB is transmitted

- Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted

For option 1, the RRM measurement is still based on SMTC, but a list of SMTC is configured/applied to OD-SSB with each SMTC associated with a periodicity.  However, considering the OD-SSB can be activated quite dynamically with different periodicities, to adjust the SMCT based on the indicated periodicity and to perform measurement based on adjusted SMTC would increase UE complexity especially if the periodicity is allowed to be updated after the activation of OD-SSB and whether there will be any delay on measurement due to this SMTC adjustment is not clear. In addition, with the introduction of the per periodicity SMTC, the measurement requirement defined in RAN4 may be impacted.

Observation 3: if OD-SSB is measured based on different SMTC, to perform measurement based on adjusted SMTC would increase UE complexity and may introduce delay on measurement as well as impact on RAN4.

Regarding option 2, considering the OD-SSB can be activated quite dynamically, it seems reasonable to not limit the measurement to a determined window, however, according to RAN4 specification, the requirement on RRM measurement is based on SMTC framework, if the measurement of OD-SSB is based on the transmission pattern, RAN4 needs to define totally new RRM measurement requirement not based on SMTC for OD-SSB, which would introduce significant impact on RAN4. Therefore, we think the feasibility/impact of this kind of fundamental change should be further checked with RAN4.

Observation 4: if OD-SSB is measured based on OD-SSB pattern ignoring SMTC, there would be significant impact on RRM measurement requirement defined in RAN4.

We think even in legacy, this issue may exist, since the SMTC is configured per frequency while the SSB transmission is configured per cell. When the transmission periodicity of SSB does not match with the SMTC window, some of the SSB transmission may fall outside of the SMTC window. For those SSB that are not covered by the SMTC window, UE shall not perform L3 measurement.

Observation 5: In legacy, due to the mismatch between the SMTC window and SSB transmission periodicity, some SSB may fall outside of the SMTC window.

With OD-SSB, which is triggered more dynamically, even this issue may happen more frequently but we can still rely on NW implementation to handle, since NW configures the SMTC and triggers the transmission of OD-SSB. One possible implementation is to configure a suitable SMTC value e.g., with relatively small periodicity/offset and relatively large duration, to cover most of the always-on SSB and OD-SSB. For the OD-SSB outside of the SMTC, UE just ignores, as in legacy.

SSB-MTC information element

-- ASN1START

-- TAG-SSB-MTC-START

SSB-MTC ::=                             SEQUENCE {

periodicityAndOffset                    CHOICE {

sf5                                 INTEGER (0..4),

sf10                                    INTEGER (0..9),

sf20                                    INTEGER (0..19),

sf40                                    INTEGER (0..39),

sf80                                    INTEGER (0..79),

sf160                                   INTEGER (0..159)

},

duration                                ENUMERATED { sf1, sf2, sf3, sf4, sf5 }

}

Observation 6: NW implementation may avoid this issue by configuring suitable SMTC value to cover most of the always-on SSB and OD-SSB, e.g., relatively small periodicity/offset and relatively large duration.

Based on the above analysis, we think from RAN2 point of view, we can assume the measurement of OD-SSB is based on SMTC framework but single SMTC as in legacy is sufficient.

Proposal 3: For L3 measurement, RAN2 assumes the OD-SSB on the serving cell is measured based on legacy SMTC. It is up to NW to configure a suitable SMTC to cover most of OD-SSB and always-on SSB, e.g., relatively small periodicity/offset and relatively large duration.

2.3.2 Which SSB to use for measurement

Another issue is which SSB to consider during RRM measurement for case 2 and two options were discussed last meeting.

- Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB

- Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

Actually, we think this issue is out of RAN2 scope and should be discussed by RAN4, actually based on the progress in RAN4, they already discussed about this issue.

Issue 1-4-5: Which type of SSB is used during SCell activation - Recommended WF - Based on moderator’s understanding, this issue can be summarized with the following options. - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, - Option 1: the same as Case 1 - Option 2: UE needs to use both OD-SSB and always-on SSB for Scell activation. - Option 3: UE is assumed to use the SSB with the smaller periodicity.

Proposal 4: For case 2, it is up to RAN4 to discuss the SSB to be used for RRM measurement on SCell configured with OD-SSB.

2.3.3 Located frequency of OD-SSB

In addition, during the discussion, an issue on the frequency relationship between OD-SSB and always-on SSB was raised. RAN4 also sent LS to RAN1 and CC RAN2 to ask this question [4]. According to the RRC specification, for L3 measurement, for all SSB based measurements there is at most one measurement object with the same ssbFrequency, therefore if the OD-SSB and always-on SSB are on different frequencies, the network shall configure separate MO for the same SCell which would add complexity. In addition, this issue is more or less related to the discussion on which type of SSB is used for case 2, if both OD-SSB and always-on SSB are used but located on different frequencies, whether it is allowed and how to derive the per cell result based on measurement of SSB on different frequencies needs to be further discussed by RAN4.

The network applies the procedure as follows: - to ensure that, whenever the UE has a measConfig associated with a CG, it includes a measObject for the SpCell and for each NR SCell of the CG to be measured; - to configure at most one measurement identity across all CGs using a reporting configuration with the reportType set to reportCGI; - to configure at most one measurement identity per the node hosting PDCP entity using a reporting configuration with the ul-DelayValueConfig; - to configure at most one measurement identity per the node hosting PDCP entity using a reporting configuration with the ul-ExcessDelayConfig; - to ensure that, in the measConfig associated with a CG: - for all SSB based measurements there is at most one measurement object with the same ssbFrequency; - an smtc1 included in any measurement object with the same ssbFrequency has the same value and that an smtc2 included in any measurement object with the same ssbFrequency has the same value and that an smtc3list included in any measurement object with the same ssbFrequency has the same value and that an smtc4list included in any measurement object with the same ssbFrequency has the same value;

Considering the motivation to support OD-SSB and always-on SSB on different frequencies it not clear, we think from RAN2 perspective, we can conclude RAN2 assumes that OD-SSB and always-on SSB are on the same frequency, i.e., frequency of the associated SCell. Since RAN2 is also in the CC of RAN4 LS, we can send LS to RAN4/RAN1 to inform them this RAN2 assumption.

Proposal 5: For case2, RAN2 assumes the OD-SSB and always-on SSB are on the same frequency, i.e., frequency of associated SCell. Send LS to RAN1/RAN4 to inform this conclusion.

2.3.4 Neighbour cell measurement

For neighbour cells measurements, we think according to the WID, the impact on neighbour cell measurement with OD-SSB is out of scope. From RAN2 perspective, there is no need to discuss this aspect and we can assume the SSB status of neighbour cell is transparent to UE and how to handle the reported neighbour cell measurement is up to gNB implementation.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

Proposal 6: There is no impact on neighbour cell measurement with OD-SSB, e.g., the OD-SSB status of neighbour cell is transparent to UE.

2.4 UE based trigger

As captured in the WID, there are three potential triggering methods for on-demand SSB:

- Method 1: Cell on/off indication via backhaul

- Method 2: Scell activation/deactivation signalling

- Method 3: UE uplink wake-up-signal

Both method 1 and method 2 are up to gNB to trigger the transmission of SSB without considering the requirement of UE while method 3 is based on UE demand. As agreed by RAN1, NW trigger is supported in Scenario#2/2A.

However, network-only mechanism cannot perfectly fit the requirement or conditions at UE side. For example, after SCell being activated (Scenario#3A/B), it is up to UE implementation to use SSB for DL sync when necessary. Some company may assume periodically transmitted SSB after SCell activation, however, this of course will reduce the NW power saving gain, which is not aligned with the WID.

Observation 7: Network-only mechanism cannot perfectly fit the requirement or conditions at UE side. UE based trigger is a better trade-off between energy saving and QoS.

Given that, it is reasonable to allow UE to send wake-up-signal for requesting SSB so as a better trade-off between energy saving and QoS. Therefore, we think both NW trigger and UE trigger should be supported.

Proposal 7: RAN2 to agree to support UE-based trigger of on-demand SSB transmission.

3	Conclusion

In this contribution, we discussed the procedures and signalling to support on-demand SSB and have corresponding observations and proposals:

Observation 1: How to design the indication in MAC CE depends on the conclusion of implicit deactivation or explicit deactivation of OD-SSB which is still under discussion in RAN1.

Observation 2: How to design the additional information field in MAC CE depends on the conclusion of parameters (e.g., periodicity) that can be indicated by MAC CE which is still under discussion in RAN1.

Observation 3: if OD-SSB is measured based on different SMTC, to perform measurement based on adjusted SMTC would increase UE complexity and may introduce delay on measurement as well as impact on RAN4.

Observation 4: if OD-SSB is measured based on OD-SSB pattern ignoring SMTC, there would be significant impact on RRM measurement requirement defined in RAN4.

Observation 5: In legacy, due to the mismatch between the SMTC window and SSB transmission periodicity, some SSB may fall outside of the SMTC window.

Observation 6: NW implementation may avoid this issue by configuring suitable SMTC value to cover most of the always-on SSB and OD-SSB, e.g., relatively small periodicity/offset and relatively large duration.

Observation 7: Network-only mechanism cannot perfectly fit the requirement or conditions at UE side. UE based trigger is a better trade-off between energy saving and QoS.

Activation/deactivation MAC CE

Proposal 1: RAN2 postpone the detailed design of the MAC CE until there is more progress from RAN1 on implicit/explicit deactivation and parameters to be indicated by MAC CE.

RRC impact

Proposal 2: Upon SCell addition, NW indicates whether the SSB transmission on this SCell is on-demand manner via RRC signalling.

L3 measurement

Proposal 3: For L3 measurement, RAN2 assumes the OD-SSB on the serving cell is measured based on legacy SMTC. It is up to NW to configure a suitable SMTC to cover most of OD-SSB and always-on SSB, e.g., relatively small periodicity/offset and relatively large duration.

Proposal 4: For case 2, it is up to RAN4 to discuss the SSB to be used for RRM measurement on SCell configured with OD-SSB.

Proposal 5: For case2, RAN2 assumes the OD-SSB and always-on SSB are on the same frequency, i.e., frequency of associated SCell. Send LS to RAN1/RAN4 to inform this conclusion.

Proposal 6: There is no impact on neighbour cell measurement with OD-SSB, e.g., the OD-SSB status of neighbour cell is transparent to UE.

UE based trigger

Proposal 7: RAN2 to agree to support UE-based trigger of on-demand SSB transmission.

4	Reference

[1] RAN1#118bis Chair notes.

[2] RAN2#127bis meeting minutes

[3] R4-2416911

[4] R4-2416913