3GPP TSG-RAN WG2 Meeting #128	R2-2410022

Orlando, USA, Nov. 18th – 22nd , 2024

Source: 	Xiaomi

Title:  	Discussion on conditional LTM

Agenda Item:	8.6.4

Document for:	Discussion and Decision

Introduction

A Revised WID on NR mobility enhancements Phase 4 [1] has been approved. In the WID, one of the objectives is to specify support for conditional Intra-CU LTM, as shown below.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this contribution, we provide our understandings on conditional intra-CU LTM.

Discussion

  - Stage-2 signalling procedure

The signalling procedure of Rel-18 intra-CU LTM is divided into four key components, which are LTM preparation, Early synchronization, LTM cell switch execution and LTM cell switch completion, as specified in TS 38.300 [2].

To design the signalling procedure of intra-CU conditional LTM, the flow chart of Rel-18 LTM can be taken as a baseline. In essence, the main difference between LTM and conditional LTM is the entity to trigger the decision on LTM cell switch. In LTM, the gNB-DU decides whether to execute LTM and which LTM candidate cell to be triggered based on the L1 measurement report from UE. In conditional LTM, the UE decides to execute the LTM cell switch based on the evaluation for conditions pre-configured by the network.

Hence, based on the signalling procedure of Rel-18 intra-CU LTM, we modify the LTM cell switch execution phase, including removing steps for the L1 measurement report and the transmission of LTM cell switch command MAC CE, and adding the step for conditional LTM evaluation in the UE. The signalling procedure for intra-CU conditional LTM, as illustrated in the Figure 1 below, can be considered.

Figure 1. Signalling procedure for intra-CU conditional LTM

Proposal 1: For Rel-19 intra-CU conditional LTM in SA case, the signalling procedure of Rel-18 intra-CU LTM is taken as the baseline. The signalling procedure for intra-CU conditional LTM, as illustrated in Figure 1, can be considered with the following modifications compared to the Rel-18 intra-CU LTM:

- Remove the step: L1 measurement report

- Remove the step: LTM cell switch command (MAC CE)

- Add a new step: Evaluate the conditions of conditional LTM in the UE

  - Conditions for triggering LTM

In RAN2#127bis meeting, RAN2 has agreed event LTM3-like and LTM5-like are used as the conditional LTM execution condition. Whether to reuse CHO conditions as the conditional LTM execution condition is an open issue.

The cell-level measurement result is very important for the cell-level mobility performance and only considering the beam-level measurement result to trigger the mobility may increase handover frequency and the probability of ping-pong. Furthermore, LTM and L1 measurement are decoupled, and Rel-18 LTM can be triggered based on Layer 3 measurements by the network. Therefore, the L3 events CondEventA3 and CondEventA5 (i.e., CHO conditions), which are associated with the L3 cell-level measurement results, can be reused as the LTM execution conditions.

RAN2 has agreed DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. For conditional LTM, as in CHO, it is CU to generate the L3 execution condition.

Proposal 2: CondEventA3 and CondEventA5 (i.e., CHO conditions) can be reused as the LTM execution conditions.

Proposal 3: It is CU to generate the L3 execution condition for conditional LTM.

For the event-triggered L1 measurement reporting, MAC layer handles the event evaluation and measurement report triggering. Hence, for L1 events (LTM3-like and LTM5-like) based L1 conditions, MAC layer handles the evaluation of L1 execution condition and determines to trigger the cell switch for conditional LTM. For L3 events (CondEventA3 and CondEventA5) based L3 conditions, as in the legacy CHO, RRC layer handles the evaluation of L3 execution condition and determines to trigger the cell switch for conditional LTM.

Proposal 4: For intra-CU conditional LTM, MAC layer handles the evaluation of L1 execution condition and RRC layer handles the evaluation of L3 execution condition.

In the WID, the objective aims to support conditional LTM including subsequent LTM. Thus, how to determine the execution condition associated with each candidate cell for the initial and subsequent conditional LTM shall be discussed. After executing the initial conditional LTM, the serving cell is changed. Considering whether the condition (based on the event LTM3-like, event LTM5-like, CondEventA3 or CondEventA5) is fulfilled depends on the measurement results of current serving cell and candidate cells, similar to the L3 subsequent CPAC, the execution condition should be configurable for different pairs of serving cell and candidate cell. And RAN2 has clarified source cell and each candidate cell provides its own execution condition for conditional LTM. Therefore, the CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell. In addition, the CLTM configuration of each candidate cell shall include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the subsequent CLTM for other candidate cells when the candidate cell becomes a serving cell.

We provide the Figure 2 as an illustration:

In Figure 2, there are three CLTM configurations. Each CLTM configuration includes the initial execution condition (blue font) and a list of subsequent execution conditions (green font). Taking CLTM configuration 1 as an example, when the candidate cell 1 fulfils the Condition 1, UE performs CLTM and cell 1 becomes the serving cell. Then UE evaluates the Condition 1-2 for the candidate cell 2 and the Condition 1-3 for the candidate cell 3 for subsequent conditional LTM.

Figure 2. RRC structure for the configuration of execution conditions

A potential ASN.1 structure is provided in Annex.

Proposal 5: To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell shall include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

  - L2 handling

In Rel-18 intra-CU LTM, security keys are maintained upon an LTM cell switch. And whether to perform L2 reset is based on the Rel-18 NoResetID. i.e., when performing the LTM cell switch procedure, if the value of field ltm-NoResetID of the target cell is not equal to the value of ltm-ServingCellNoResetID of the serving cell, the UE performs PDCP data recovery and RLC re-establishment. The L2 handling procedure for Rel-18 intra-CU LTM can be reused for intra-CU conditional LTM.

Proposal 6：For intra-CU conditional LTM, L2 handling of Rel-18 LTM can be reused and security keys are maintained upon a conditional LTM cell switch.

  - Early sync

For the early UL synchronization procedure of conditional LTM, both UE based TA measurement mechanism and PDCCH ordered early TA acquisition are supported for conditional LTM. For UE based TA measurement, the mechanism of Rel-18 LTM is sufficient.

However, in the legacy procedure PDCCH ordered early RACH, UE only transmits a preamble towards the candidate cell and the TA value of the candidate cell is included in the LTM cell switch command MAC CE. As we described in Chapter 2.1, the UE decides to execute the cell switch of conditional LTM based on the evaluation of conditions without the LTM cell switch command MAC CE. Therefore, how to provide the TA value of candidate cell(s) to UE needs to be discussed and there are two options can be considered:

- Option 1: RAR-based TA acquisition can be considered, and the TA value is provided by the candidate cell in RAR

- Option 2: As in Rel-18 LTM, the TA value is also provided by the serving cell. But, for conditional LTM, the serving cell shall provide the TA value of candidate cells to UE before conditional LTM execution.

For option 1, the RAR from the candidate cell as in the legacy RACH procedure can be reused to transmit the TA value, which results in the additional data interruption for the serving cell link, due to RAR monitoring. For option 2, it might be necessary to design a new MAC CE to transmit the TA value of candidate cell(s) to UE by the serving cell.

Proposal 7: For early TA acquisition, the following options can be considered to provide the TA value of the candidate cell to UE:

- Option 1: the TA value of the candidate cell is provided by the candidate cell in RAR

- Option 2: the TA value of the candidate cell is provided by the serving cell (e.g., via a MAC CE)

In previous meeting, how to determine candidate cells on which to perform early TA acquisition has been discussed. Allowing the network implementation to decide is a straightforward approach. The network can determine candidate cells on which to perform early TA acquisition based on the L3 measurement reporting or LTM L1 measurement reporting, if any.

Proposal 8: It is up to network implementation to determine candidate cells on which to perform early TA acquisition.

Moreover, because the TA value of candidate cell needs to be provided to UE before the execution of CLTM, how to maintain the TA value at UE side is the other open issue for PDCCH ordered early TA acquisition. A simple method is to reuse the legacy TA maintenance mechanism and introduce a TAT-like timer for candidate cells. TAT-like timer of candidate cell is maintained at UE side to maintain the TA value of candidate cell. The UE starts the TAT-like timer of candidate cell when upon receiving its TA value. If the TAT-like timer of candidate cell expires, the TA value of the candidate cell becomes invalid.

Proposal 9: A TAT-like timer of candidate cell can be introduced to maintain the TA value of candidate cell at UE side.

  - Conditional LTM execution

In the network triggered LTM, the network sends cell switch command MAC CE to trigger cell switch. The cell switch command indicates the target cell (via target configuration ID) and the target beam (via the TCI state). For RACH-less LTM, the UE accesses the target cell using either a configured grant or a dynamic grant. The configured grant is provided in the LTM candidate configuration, and the UE selects the configured grant occasion associated with the beam indicated in the cell switch command. Upon initiation of LTM cell switch to the target cell, the UE starts to monitor PDCCH on the target cell for dynamic scheduling [2].

However, for conditional LTM, there is no LTM cell switch command MAC CE to indicate the target cell and target beam for the CLTM execution and the UE triggers conditional LTM cell switch autonomously when the condition of candidate cells is fulfilled. To support the RACH-less conditional LTM, the target cell and target beam can be determined by UE based on the condition evaluation and the UE selects the candidate cell/candidate beam which fulfils the execution condition.

For the conditional LTM triggered by the L1 execution condition, the UE shall select the candidate beam which fulfils the associated L1 execution condition as the target beam for the CLTM execution.

For the conditional LTM triggered by the L3 execution condition (if P2 is supported) based on L3 cell-level measurement results, the UE can select the best candidate beam of the candidate cell which fulfils the associated L3 execution condition as the target beam for the CLTM execution.

Proposal 10: The UE selects the target beam based on condition evaluation, and the following items can be considered for different types of conditions：

- For the L1 execution condition, the UE selects the beam which fulfils the associated L1 condition as the target beam.

- For the L3 execution condition, the UE selects the best beam of the candidate cell which fulfils the associated L3 condition as the target beam.

For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. But how to select the configured grant occasion for conditional LTM is an open issue. In the Rel-18 RACH-less LTM, the UE selects the configured grant occasion associated with the beam indicated in the cell switch command. Similarly, The UE uses the CG resource associated with the UE-selected beam for first UL transmission for CG-based RACH-less conditional LTM.

Proposal 11: The UE uses the CG resource associated with the UE-selected beam for CG-based RACH-less conditional LTM.

Whether to support the DG-based RACH-less is FFS for conditional LTM. One concern in supporting the DG-based RACH-less is how the network can schedule a dynamic grant to UE for conditional cell switch execution, because the network cannot know when the UE performs the cell switch and which target cell and target beam are selected by the UE to transmit first UL data. Nevertheless, the issue can be solved by the network implementation. Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM, and the network can only send the dynamic scheduling information on the activated beam, which associated with the activated TCI State. In addition, PDCCH ordered early TA acquisition is supported for conditional LTM, thus, only candidate cells whose TA is valid need to send the dynamic scheduling information to UE. According to the activated TCI states, the candidate cell with a valid TA and the L1/L3 measurement reporting from UE, the network can estimate which beams the UE might select and start dynamic scheduling on these beams for the UE, based on the network implementation. Hence, DG-based RACH-less conditional LTM can be supported.

Proposal 12: DG-based RACH-less conditional LTM can be supported.

Conclusions

According to the analysis given above, we have the following proposals:

2.1	Stage-2 signalling procedure

Proposal 1: For Rel-19 intra-CU conditional LTM in SA case, the signalling procedure of Rel-18 intra-CU LTM is taken as the baseline. The signalling procedure for intra-CU conditional LTM, as illustrated in Figure 1, can be considered with the following modifications compared to the Rel-18 intra-CU LTM:

- Remove the step: L1 measurement report

- Remove the step: LTM cell switch command (MAC CE)

- Add a new step: Evaluate the conditions of conditional LTM in the UE

2.2	Conditions for triggering LTM

Proposal 2: CondEventA3 and CondEventA5 (i.e., CHO conditions) can be reused as the LTM execution conditions.

Proposal 3: It is CU to generate the L3 execution condition for conditional LTM.

Proposal 4: For intra-CU conditional LTM, MAC layer handles the evaluation of L1 execution condition and RRC layer handles the evaluation of L3 execution condition.

Proposal 5: To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell shall include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

2.3	L2 handling

Proposal 6：For intra-CU conditional LTM, L2 handling of Rel-18 LTM can be reused and security keys are maintained upon a conditional LTM cell switch.

2.4	Early sync

Proposal 7: For early TA acquisition, the following options can be considered to provide the TA value of the candidate cell to UE:

- Option 1: the TA value of the candidate cell is provided by the candidate cell in RAR

- Option 2: the TA value of the candidate cell is provided by the serving cell (e.g., via a MAC CE)

Proposal 8: It is up to network implementation to determine candidate cells on which to perform early TA acquisition.

Proposal 9: A TAT-like timer of candidate cell can be introduced to maintain the TA value of candidate cell at UE side.

2.5	Conditional LTM execution

Proposal 10: The UE selects the target beam based on condition evaluation, and the following items can be considered for different types of conditions：

- For the L1 execution condition, the UE selects the beam which fulfils the associated L1 condition as the target beam.

- For the L3 execution condition, the UE selects the best beam of the candidate cell which fulfils the associated L3 condition as the target beam.

Proposal 11: The UE uses the CG resource associated with the UE-selected beam for CG-based RACH-less conditional LTM.

Proposal 12: DG-based RACH-less conditional LTM can be supported.

Reference

[1] RP-242356, “Revised WID: NR mobility enhancements Phase 4”, Apple Inc, China Telecom

[2] 3GPP TS 38.300 V18.3.0 (2024-09), “NR and NG-RAN Overall Description (Stage 2)”

- Annex

The field ltm-condExecutionCond indicates the initial execution condition for initial conditional LTM

The field ltm-subsequentCondReconfig contains the execution conditions that need to be fulfilled in order to trigger the execution of a subsequent conditional LTM. The subsequent execution condition is used for conditional LTM evaluation for other candidate cells when the RRCReconfiguration message contained in ltm-CandidateConfig has been applied.

LTM-Candidate-r18 ::= SEQUENCE { ltm-CandidateId-r18 LTM-CandidateId-r18, ltm-CandidatePCI-r18 PhysCellId OPTIONAL, -- Need M ltm-SSB-Config-r18 LTM-SSB-Config-r18 OPTIONAL, -- Need M ltm-CandidateConfig-r18 OCTET STRING (CONTAINING RRCReconfiguration) OPTIONAL, -- Need M ltm-ConfigComplete-r18 ENUMERATED {true} OPTIONAL, -- Need R ltm-EarlyUL-SyncConfig-r18 OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18) OPTIONAL, -- Need R ltm-EarlyUL-SyncConfigSUL-r18 OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18) OPTIONAL, -- Need R ltm-TCI-Info-r18 LTM-TCI-Info-r18 OPTIONAL, -- Need M ltm-NoResetID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ltm-UE-MeasuredTA-ID-r18 INTEGER (1..maxNrofLTM-Configs-plus1-r18) OPTIONAL, -- Need M ..., [[ ltm-condExecutionCond-r19 LTM-CondExecutionCond-r19 OPTIONAL, -- Need M ltm-subsequentCondReconfig-r19 LTM-SubsequentCondReconfig-r19 OPTIONAL, -- Need M ]] } LTM-SubsequentCondReconfig-r19 ::= SEQUENCE { ltm-condExecutionCondToReleaseList-r19 LTM-CondExecutionCondToReleaseList-r19 OPTIONAL, -- Need N ltm-condExecutionCondToAddModList-r19 LTM-CondExecutionCondToAddModList-r19 OPTIONAL, -- Need N ... } LTM-CondExecutionCondToAddModList-r19 ::= SEQUENCE (SIZE (1.. maxNrofLTM-Configs-r18)) OF LTM-CondExecutionCondToAddMod-r19 LTM-CondExecutionCondToAddMod-r19 ::= SEQUENCE { ltm-subsequentCondReconfigId-r19 LTM-CandidateId-r18, ltm-subsequentCondExecutionCond-r19 LTM-CondExecutionCond-r19 OPTIONAL, -- Need M ... } LTM-CondExecutionCondToReleaseList-r19 ::= SEQUENCE (SIZE (1.. maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18