3GPP TSG-RAN WG2 Meeting #128	R2-2410039

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.7.6

Source:	Nokia, Nokia Shanghai Bell

Title:	XR rate control

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#127bis:

FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc. RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - Extend the Bit Rate field - Define a new bit rate table to provide sufficient granularity for XR traffic - Introduce new values for the bitRateMultiplier Send LS to SA4 asking about range/granularity which is required

In this contribution, we are discussing the identified open issues related to XR rate control.

# 2	Discussion

## 2.1	Indication granularity: per DRB vs. per QoS flow

The current recommended bit rate (RBR) procedure can be considered as being specified at the DRB level because the RBR MAC CE indicates the LCID and the LCH is mapped to a single DRB. Additionally, RAN operates with DRBs making it easier to manage each DRB independently while ensuring to meet the QoS requirement regardless of how many QoS flows are mapped to one DRB. Of course, according to the specifications, multiple QoS flows can be mapped to a single DRB, but in practice, each QoS flow is likely to be mapped to its own DRB thanks to e.g., enough number of IDs available. Therefore, in most cases, indicating congestion at DRB level should be sufficient. Even if multiple QoS flows are mapped to a single DRB, the UE seems to have enough information to decide which QoS flow to apply the rate control. Thus, one option could be that the UE, by implementation, decides to which QoS flow the rate control is applied by considering the received bit rate.

Observation 1: With 1-to-1 mapping between the QoS flow to DRB or UE implementation in case of N-to-1 mapping, congestion indication at the DRB level still allows rate control applied to each QoS flow.

Therefore, we think that the congestion indication can be per DRB and LCID can be used for this purpose.

Proposal 1: XR rate control signalling is adopted at DRB level.

## 2.2	Applicability of the XR Rate Control

It is clearly not a right assumption that all QoS flows of all types of XR applications will have the capability of adjusting their codec bit rate and encoding parameters. For example, a video flow may have an adaptive codec which can be adjusted based on the congestion in the network, but the same may not be true for another XR application. Therefore, it becomes important to identify which DRB(s) have the possibility to adjust their bit rate. At a high level, two options can be explored further in RAN2:

-	Option 1: UE based approach. For example, UE informs gNB of the QoS flow(s)/DRB(s) that are with the capability of adjusting their bit rate.

-	Option 2: CN based approach. For example, gNB gets the information from CN entity (e.g. SMF) of which QoS flow(s) are with the capability of adjusting their bitrate when provided with congestion information.

Both options are feasible, while considering potential UE impact, Option 2 is slightly preferred.

Proposal 2: For XR rate control, RAN2 discuss different options how the gNB identifies which QoS flows are subject to adaptive bit rate.

## 2.3	Refined bit rate values for XR

In TS 38.321, for the Recommended Bit Rate procedure, a fixed bit rate table is defined with an index mapping to a bit rate value, allowing indication of the bit rate values not listed in the table using the bit rate multiplier (x1, x40, x70, x100, x200). The combination of the bit rate index and the multiplier results in many potential bit rate values from 0 Kbps to around 1.6 Gbps. While we are awaiting a reply from SA4, the minimum and the maximum values in the table could be generally sufficient for XR use case according to TR 38.835.

This table was initially designed for voice services, and the single generic bit rate table may not be efficiently support the multi-modal services in XR as different QoS flows may require varied ranges and granularities of the bit rates. One might suggest increasing the number of multiplier values to achieve finer granularity. However, the multiplier approach does provide different granularities for the high and the low bit rate ranges with the quantization error between two indexes remained the same. In our view, the most effective way to achieve consistent granularity across all bit rate ranges is by designing the table with the proper values.

Given these considerations, configuring bit rate tables with distinct bit rate ranges for each QoS flow/DRB would enable more precise rate control without requiring excessive effort to create a highly generalized bit rate range applicable to all types of QoS flows from different XR applications. We therefore suggest that RAN2 consider introducing a configurable table for XR rate control.

Observation 2: A fixed table with generalized bit rate ranges and a multiplier may not be efficient in defining the bit rate range needed for different QoS flows and providing consistently good granularity across the entire bit rate ranges.

Proposal 3: RAN2 to consider whether to allow defining/configuring different bit rate tables for XR rate control.

# 3	Conclusion

In this contribution, we discuss the details of signalling aspect for XR rate control, and provide the followings observations and proposals:

Observation 1: With 1-to-1 mapping between the QoS flow to DRB or UE implementation in case of N-to-1 mapping, congestion indication at the DRB level still allows rate control applied to each QoS flow.

Proposal 1: XR rate control signalling is adopted at DRB level.

Proposal 2: For XR rate control, RAN2 discuss different options how the gNB identifies which QoS flows are subject to adaptive bit rate.

Observation 2: A fixed table with generalized bit rate ranges and a multiplier may not be efficient in defining the bit rate range needed for different QoS flows and providing consistently good granularity across the entire bit rate ranges.

Proposal 3: RAN2 to consider whether to allow defining/configuring different bit rate tables for XR rate control.