3GPP TSG RAN WG2#128	R2-2410044

Orlando, US, 18th - 22th November 2024

Agenda Item:	8.6.4

Source:	Lekha Wireless Solutions

Title:	Discussion on Conditional Intra CU LTM

Document for:	Discussion, Decision

Introduction

At last meeting, we discussed Conditional intra CU LTM and made the following agreements [1]:

- Agreements on Conditional intra CU LTM #RAN2 127bis: - Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions. - Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions. - Source cell and each candidate cell provides its own execution condition for conditional LTM. - It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used. - RACH-less Conditional intra-CU LTM is supported. - RACH based conditional intra-CU LTM is supported. - UE based TA measurement mechanism is supported for conditional intra-CU LTM. - PDCCH ordered early TA acquisition is supported for conditional LTM. - Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM. - For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach. - 11.The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

This contribution discussed different aspect to support conditional intra CU mobility design.

Discussion on Inter CU LTM

  - 2.1 Subsequent Conditional LTM

- Observation 1: In RAN#105 meeting, the objective related to conditional LTM of Rel-19 Mobility enhancements was agreed, following is the related description [2]:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

- In order to support subsequent LTM, the following conclusions are derived based on Rel-16 CHO and Rel-18 LTM.

- In Rel-16 Conditional Handover (CHO) procedure, the User Equipment (UE) releases stored CHO configurations after the successful completion of the RRC handover process.

- However, for Rel-18 Layer 1/2 Triggered Mobility (LTM), the UE retains the LTM configuration after completing an LTM cell switch unless instructed by the network to release it. This retention supports subsequent LTM operations.

- Similarly, to enable further conditional LTM, the UE must not release the configuration following a conditional LTM cell switch completion.

- Proposal 1: In order to support subsequent LTM, Rel-18 LTM procedure is considered as a baseline.

- Proposal 2: To facilitate subsequent conditional LTM, UE should retain the configuration even after the completion of a conditional LTM cell switch.

- Proposal 3: The source cell consolidates conditional LTM candidate configurations along with their execution conditions and shares them with potential candidate cells. This enables candidate cells to be aware of the UE's potential entry and supports subsequent conditional LTM.

  - 2.2 Conditional LTM Fast recovery

Observation 2: In TR 38.300 [3, section 9.2.3], the procedure to handle CHO failure is given as follows

Mobility in RRC-Connected: - When initial CHO execution attempt fails or HO fails, the UE performs cell selection, and if the selected cell is a CHO candidate and if network configured the UE to try CHO after handover/CHO failure, then the UE attempts CHO execution once, otherwise re-establishment is performed.

Based on the Rel-16 CHO failure procedure , Conditional intra CU LTM recovery can be handled similarly.

Proposal 4: The Rel-16 CHO failure recovery is considered the baseline for conditional intra-CU LTM failure. Additionally, conditional LTM fast recovery should be supported in Rel-19.

Proposal 5: When initial conditional LTM execution attempt fails, UE can select another intra CU conditional LTM cell from the available candidate list. If the second attempt also fails, an RLF occurs and UE needs to re-establish the initial connection.

  - 2.3 Conditional LTM Evaluation

In the last meeting, the following agreement has been made.

RAN2#127bis meeting agreements on execute condition: - It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

With these agreement, UE evaluates L1 execution condition of candidate cells and apply triggering mechanism. If UE fails to attach in the target cell, radio link failure occurs. In order to avoid conditional LTM failure, UE should evaluate both L1 and L3 based execution condition. CU can generate the execution condition for each candidate cell based on L3 measurements.

Regarding the evaluation of execution condition for conditional LTM, the UE should start evaluating the execution condition(s) upon receiving the conditional LTM configuration, and stop evaluating the execution condition(s) once a handover (e.g. including L3 HO, LTM, conditional LTM) is executed, i.e. similar to CHO evaluation

Proposal 6: To enhance robustness in conditional LTM, the UE should evaluate execution conditions based on both L1 and L3 measurements.

Proposal 7: CU generates the execution condition based on L3 measurements and shared it to UE.

Proposal 8: UE begins assessing the execution condition(s) as soon as it receives the conditional LTM configuration and stops the assessment once a handover (such as L3 handover, LTM, or conditional LTM) is carried out.

  - 2.4 DG based RACH-less Conditional LTM

Observation 3:

In the last meeting, the following agreement has been made.

RAN2#127bis meeting agreements on RACH Less Procedure: - For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

In conditional LTM, the UE itself triggers the LTM cell switch, meaning the source node is unaware of when the UE detaches from the source cell or which target cell the UE selects. Consequently, the target node lacks information on when to begin dynamic scheduling for the UE.

In the current Rel-18 Downlink Group-based (DG-based) Random Access Channel (RACH)-less LTM, upon initiation of an LTM cell switch to the target cell, the UE monitors the Physical Downlink Control Channel (PDCCH) of the designated Transmission Configuration Indicator (TCI) state on the target cell for dynamic scheduling. Additionally, after sending the LTM cell switch command to the UE, the source node informs the target node of the target cell and the TCI state, allowing the target node to start dynamic scheduling for the UE. However, this setup does not work for conditional LTM, where the source node has no prior knowledge of the UE's switch timing or selected target cell.

Proposal 9: Rel-18 DG based RACH-less procedure for LTM is not applicable to conditional intra CU LTM scenarios.

Proposal 10: If the UE informs the serving cell of the cell switch timing, a DG-based RACH-less procedure is possible. However, in conditional LTM, the UE does not inform the serving cell about the cell switch, so the DG-based RACH-less procedure is not supported for conditional intra-CU LTM.

Conclusion

In this contribution, the following proposals are made:

- Proposal 1: In order to support subsequent LTM, Rel-18 LTM procedure is considered as a baseline.

- Proposal 2: To facilitate subsequent conditional LTM, UE should retain the configuration even after the completion of a conditional LTM cell switch.

- Proposal 3: The source cell consolidates conditional LTM candidate configurations along with their execution conditions and shares them with potential candidate cells. This enables candidate cells to be aware of the UE's potential entry and supports subsequent conditional LTM.

Proposal 4: The Rel-16 CHO failure recovery is considered the baseline for conditional intra-CU LTM failure. Additionally, conditional LTM fast recovery should be supported in Rel-19.

Proposal 5: When initial conditional LTM execution attempt fails, UE can select another intra CU conditional LTM cell from the available candidate list. If the second attempt also fails, an RLF occurs and UE needs to re-establish the initial connection.

Proposal 6: To enhance robustness in conditional LTM, the UE should evaluate execution conditions based on both L1 and L3 measurements.

Proposal 7: CU generates the execution condition based on L3 measurements and shared it to UE.

Proposal 8: UE begins assessing the execution condition(s) as soon as it receives the conditional LTM configuration and stops the assessment once a handover (such as L3 handover, LTM, or conditional LTM) is carried out.

Proposal 9: Rel-18 DG based RACH-less procedure for LTM is not applicable to conditional intra CU LTM scenarios.

Proposal 10: If the UE informs the serving cell of the cell switch timing, a DG-based RACH-less procedure is possible. However, in conditional LTM, the UE does not inform the serving cell about the cell switch, so the DG-based RACH-less procedure is not supported for conditional intra-CU LTM.

References

- R2-2407571 “Report from session on V2X/SL, R18/19 MOB, and R19 NES”, Vice Chairman (Samsung)

- RP-234036  New WID: NR mobility enhancements Phase 4

- 38.300, NR and NG-RAN Overall Description; Stage 2, Release 16