3GPP TSG-RAN WG2 Meeting #128                                    	R2-2410046

Orlando, USA, Nov 18th – 22th, 2024

Agenda item:	8.8.4

Source:	Qualcomm Incorporated

Title:	Signaling of MBS broadcast service area information

Document for:	Discussion and Decision

# Introduction

In RAN#127, following agreements were made on MBS broadcast service.

Agreements:

1.	The intended broadcast service area is defined by a geographical area represented by a (set of) referenceLocation and radius or by a (set of) polygon(s).

2.	RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

3.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

4.	For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

In RAN2#127bis, following agreement was made.

Agreements:

1.	For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

In this document, we provide signaling details for MBS broadcast service area information.

# Discussion

Signaling of MBS broadcast service area

Similar to the list of TN coverage areas in SIB25, the list of MBS broadcast service areas can be broadcast. It may be possible to broadcast just a list of areas where the MBS broadcast service is not intended. For this the existing SIB25 can be extended, or a new SIB can be introduced. The SIB20 can be extended as it provides MBS information for serving cell. It is also possible to extend the existing SIB21 as it provides the information for service continuity such as neighbor frequency information.

-- ASN1START

-- TAG-SIB21-START

SIB21-r17 ::= SEQUENCE {

mbs-FSAI-IntraFreq-r17                   MBS-FSAI-List-r17                OPTIONAL,  -- Need R

mbs-FSAI-InterFreqList-r17               MBS-FSAI-InterFreqList-r17       OPTIONAL,  -- Need R

lateNonCriticalExtension                 OCTET STRING                     OPTIONAL,

... ,

[[

mbs-CoverageAreaInfoList-r19             MBS-CoverageAreaInfoList-r19       OPTIONAL,  -- Need R

]]

}

MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17

MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17

MBS-FSAI-InterFreq-r17 ::= SEQUENCE {

dl-CarrierFreq-r17         ARFCN-ValueNR,

mbs-FSAI-List-r17          MBS-FSAI-List-r17

}

MBS-FSAI-r17 ::= OCTET STRING (SIZE (3))

MBS-CoverageAreaInfoList-r19 ::=   SEQUENCE (SIZE (1..maxMBS-AreaInfo-r19)) OF MBS-CoverageAreaInfo-r19

MBS-CoverageAreaInfo-r19 ::=       SEQUENCE {

mbs-SessionAreaId-r19		MBS-SessionAreaId-r19,

mbs-FSAI-List-r19           MBS-FSAI-List-r17, OPTIONAL,  -- Need R

TMGI-r19					TMGI-r19,   OPTIONAL,  -- Need R

coverageAreaInfo-r19 ::=    CHOICE {

circularAreaInfo-r19		circularAreaInfo-r19,

polygonAreaInfo-r19			polygonAreaInfo-r19

}

-- TAG-SIB21-STOP

-- ASN1STOP

In our view, it may be simple just to extend the SIB25 or SIB21 and there is no need to add a new SIB. The UEs not supporting intended service area do not have to acquire the unnecessary part, i.e., coordinates, from MCCH channel or SIB20. Since SIB21 already provides the association of FSAI and frequencies, for service continuity it may be better we extend the SIB21 to provide list of MBS geographical service areas and make its association with MBS frequency/FSAI simpler as shown in text proposal above. This can also be used for providing serving cell MBS broadcast area information. We think the whole list of Area information (i.e., list of area coordinates) should not be included inside MCCH channel due to large signaling overhead.

It is possible to use the Area ID in MCCH for serving cell MBS broadcast information and use Area ID in SIB21 for service continuity, it may be better just to extend the SIB25 to include the list of area coordinates commonly for both serving cell and service continuity. The list of area coordinates provided in SIB25 can also be used for other purposes in future.

List of area coordinates is not included in mbs-SessionInfoList-r17. Decide whether to extend existing SIB25 or SIB21 for broadcasting the list of MBS broadcast service area coordinates.

Mapping

One solution for mapping the serving cell MBS broadcast service with the geographical area information is that the mbs-SessionInfoList-r17 is extended to include list of geographic service area IDs for each session.

One potential issue with this approach is that when the UE is not in the intended area, it might have to check MCCH if it is interested in MBS broadcast to know if any change in geographical area info has changed. As RAN2 has made following agreements, we could also leave this to UE.

- RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

The other solution is that all the mapping information such as mbs-SessionInfo-17 and g-RNTI-r17 are included within each geographical area information in SIB21 or SIB25. In this case, the UE will only need to track the changes of SIB and does not need to monitor MCCH to determine whether intended service area has changed.

MBS-CoverageAreaInfo-r19 ::=       SEQUENCE {

mbs-SessionAreaId-r19		MBS-SessionAreaId-r19,

mbs-FSAI-List-r19           MBS-FSAI-List-r17, OPTIONAL,  -- Need R

TMGI-r19					TMGI-r19,   OPTIONAL,  -- Need R

coverageAreaInfo-r19 ::=    CHOICE {

circularAreaInfo-r19		circularAreaInfo-r19,

polygonAreaInfo-r19			polygonAreaInfo-r19

}

In our view, this could also add complexity on SIB change when there is change in MBS session information. Therefore, it could be ok to add the service area information index or Area ID in the mbs-SessionInfoList-r17 as what has already been agreed.

- For each MBS service we include one or more intended service area IDs into MCCH.

To include one or more intended service area IDs into MCCH, mbs-SessionInfoList-r17 is extended.

Service continuity

In our view, UE can already map the MBS broadcast frequency to the associated MBS service through USD (User Service Description) or SIB21. It is to note that the UE needs to be aware of the frequency of the service it is interested in before it can receive the MBS broadcast service. The UE knows the interested frequency either from SIB21 or from USD. Therefore, it may be simple just to map the provided geographical area to one or more MBS broadcast service frequency.

The existing procedure (e.g., via SIB21 or USD) is already sufficient for UE to map Frequency and MBS broadcast service(s).

MBS FSAI (Frequency Selection Area Identity) is another way how the intended service area can be associated with MBS session. SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS FSAI. The UE can obtain the FSAI of the MBS service from USD. A frequency can cover multiple MBS service Areas and the MBS FSAI could be different for the different service area. In this case, the service area can be associated with the MBS session simply by FSAI.

Similar to mapping with serving cell MBS broadcast service area, we think it is simpler to extend the SIB21 to include the list of applicable geographical service area IDs under each inter-frequency information.

For service continuity, extend SIB21 to include the list of applicable geographical service area IDs under each inter-frequency information.

Coordinates format

It was agreed that service area can be in the form of circles or polygons. For circle, we can reuse the format defined for TN area coverage in SIB25, i.e., reference location and radius.

However, for the polygon, we suggest reusing the format defined in TS 23.032. It is to note that location coordinates type used by LPP also uses the same polygon format as follows. We think same format used as polygon type in location coordinates in LPP protocol should be used to describe polygon shape of MBS service area.

#### –	Polygon

The IE Polygon is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

Polygon ::= SEQUENCE (SIZE (3..15)) OF PolygonPoints

PolygonPoints ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

Same format used as polygon type in location coordinates in LPP protocol (TS 37.355) is used to describe polygon shape of MBS service area.

# Conclusion

Following proposals are made.

Proposal 1	List of area coordinates is not included in mbs-SessionInfoList-r17. Decide whether to extend existing SIB25 or SIB21 for broadcasting the list of MBS broadcast service area coordinates.

Proposal 2	To include one or more intended service area IDs into MCCH, mbs-SessionInfoList-r17 is extended.

Observation 1.	The existing procedure (e.g., via SIB21 or USD) is already sufficient for UE to map Frequency and MBS broadcast service(s).

Proposal 3	For service continuity, extend SIB21 to include the list of applicable geographical service area IDs under each inter-frequency information.

Proposal 4	Same format used as polygon type in location coordinates in LPP protocol (TS 37.355) is used to describe polygon shape of MBS service area.