3GPP TSG-RAN WG2 Meeting #128                                    	R2-2410050

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.9.3

Source:	Qualcomm Incorporated

Title:	CB-Msg3 and Msg4 enhancements

Document for:	Discussion and Decision

- Introduction

In RAN2#127bis, following agreements were made.

Agreements:

1.	RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19.

2.	RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed.

3.	At least system Information is used to provide CB-msg3 EDT cell-specific PUSCH resources for Msg3 transmission. FFS on signalling details.

4.	CB-msg3 EDT cell specific PUSCH resources for Msg3 transmission are provided per CE level (FFS whether we have a CE level specific configuration for DSA)

5.	RAN2 assumes that CB-msg3 EDT cell specific PUSCH resources are associated with number of repetitions, RSRP selection threshold to determine the CE level and largest TBS for Msg3 transmission, but this has to be confirmed by RAN1. FFS if there is an RSRP threshold that determines whether CB-msg3 EDT cannot be used (the UE will have to use 4-step RA)

6.	At least in the cases confirmed by RAN1/RAN4, a running TAT is not needed to initiate a CB-msg3 EDT transmission

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

8.	CB-msg3 EDT procedures and any Msg4 enhancement are only introduced for IoT NTN.

In this document, we provide details for DSA solution.

- Discussion

## 1. CB-Msg3 transmission procedure

The primary goal of the EDT enhancement is to improve the random-access procedure. Currently, the existing preamble transmission and RAR reception are inefficient because collisions are detected by the UE only after receiving DL response message (i.e., Msg4) including contention resolution MAC CE that does not match with UE’s contention resolution ID. Restarting the entire RACH procedure is extremely costly in NTN due to the very long RTT. One example of enhancement is the transmission of Msg3 without the need for Msg1 or the Random Access Response (RAR).

This improvement is particularly relevant in NTN, as the UEs are GNSS-capable, meaning they know their location before accessing the NTN cell, and they also know the satellite’s location from the ephemeris information broadcasted in SIB. Consequently, these NTN UEs have a valid timing advance based on their location and the satellite’s location, making a PRACH preamble transmission unnecessary.

Currently EDT is a four-step random access procedure, i.e., EDT specific Msg1, RAR, RRCEarlyDataRequest message (Msg3) and RRCEarlyDataComplete message (Msg4). In our understanding the objective is to reduce the EDT procedure from 4 steps to 2 steps, i.e., UE directly transmitting RRCEarlyDataRequest message (Msg3) and receiving RRCEarlyDataComplete message (Msg4). And this is without Msg1. i.e., preamble, therefore, it is not 2 step RACH procedure.

Figure 1 An example of EDT without Msg1 and RAR

We think in NTN, preamble transmission and RAR reception not only adds significant delay (up to one extra RTT, which is approximately 540 ms in GEO, per random access attempt) but also  consumes significant downlink resources, e.g., a PDCCH scheduling a RAR and the RAR PDSCH itself, which scales (in terms of DL resources necessary to transmit the PDSCH) approximately with the number of UEs attempting to transmit in the uplink. Specially in GEO, this is major bottleneck for supporting large number of UEs. Therefore, preamble-less EDT is very beneficial in NTN.

It is agreed to continue work on contention based Msg3 (CB-Msg3) transmission mechanisms where a pool of PUSCH resources for Msg3 is provided to UEs and UEs will choose a resource randomly. RAN2 has also agreed to introduce DSA (Diversity Slotted Aloha) mechanism for CG-Msg3 transmission where more than one replica of the Msg3 transmission can be transmitted.

Diversity Slotted Aloha (DSA)

How DSA works is explained in [1]. For DSA, the network should configure a number of replicas (i.e., K) allowed for transmission. There could be N EDT occasions configured for UE to choose randomly and UE selects randomly K+1 occasions (i.e., a Msg3 transmission and K replicas) out of N occasions.

If K = 1, then UE will choose randomly 2 PUSCH resources in different time occasions and transmit one original msg3 transmission and one replica of the Msg3 transmission as shown in figure 2 below. It is to note that at a time occasion, a UE may randomly choose different frequency domain or OCC domain resources, for example in figure 2, the frequency domain resources at subframe 2 and 6 can be different.

Figure 2 DSA operation with K = 1

When K > 0, it should be clear how the Msg4 transmission is performed. One simple option is that there is no grouping, i.e., difference from UE’s perspective whether K = 0 or K > 0. In this case, the UE will start the contention resolution timer associated with each Msg3 transmission. Then the RNTI to be monitored will be different for each contention resolution window. Once a Msg4 indicating success is received in response to any of the CB-Msg3 transmission, the UE can stop all the contention resolution timers and consider the Msg3 transmission as successful. If the UE supports DSA with K > 0, then we can assume that UE should also support monitoring multiple RNTIs. This option works for UEs that do not support DSA as they will perform only one msg3 transmission and monitor only one RNTI while the contention resolution timer is running. This option also works for the case network configures K =2 but the UE supports only K =  1 as network would not need to know UE’s DSA capability.

However, this approach of operation without grouping has some issues:

- As shown in figure 2, if the UE receives rescheduling of the first Msg3 transmission, then the benefit of DSA is not achieved as the successful Msg4-2 response may collide with or may be scheduled after the retransmission grant of the first Msg3 retransmission scheduling.

- If UE has transmitted multiple copies of Msg3 transmissions and network is able to decode only one copy of the Msg3 transmission, then the network can transmit the Msg4 response corresponding to the only one Msg3 transmission that is successfully decoded. However, the other Msg3 transmission copy or replicas may not be decoded, and network may unnecessarily schedule the retransmission of the replica Msg3 transmission that was not decoded successfully. This results in unnecessary resource wastage.

To address the above scenarios, we think a separate resource for DSA can be configured. The DSA resource includes a group of N PUSCH occasions defined by the parameter #K which is number of NPUSCH transmission copies the UE is allowed to transmit. If the K = 0, then it is simply non-DSA CB-Msg3 transmission. When K > 0, it is DSA CB-Msg3 transmission. When K = 1, the UE is allowed to transmit the first original Msg3 transmission and one additional replica of the Msg3 transmission.

The set of first Msg3 transmission occasion location is configured to UE (i.e., similar to determination of configured grant occasion). The subsequent Msg3 transmission occasions in the group are determined from the configured time offset from the first Msg3 transmission occasion.

Additionally, a mechanism can be defined for the network to identify the location of the other copy if it is able to decode a Msg3 transmission successfully to e.g. avoid unnecessary retransmissions.

Benefits of N grouping

For resolving issue#1 above, the network can wait until the resource occasion N (i.e., last transmission occasion of the group) to decide on retransmission. Then UE will always receive the successful response first.

For resolving the second issue#2 above, at occasion N, the network may also be able to figure out whether retransmission is needed based on the RSRP level. If the second copy is decoded, then if the network knows resource for first copy(say resource A), the network can decide to not issue a retransmission for resource A if it can determine the decoding failure of the first copy was likely due to poor RSRP or not due to contention.

The other benefit of grouping is that network can take maximum benefit from the Msg4 multicast enhancements as it will have a group RNTI and network can also wait until the last occasion of the group.

Therefore, we propose followings for DSA solution.

For DSA solution, a separate dedicated resource is configured via system information.

Grouping of Msg3 transmission occasions in time domain is defined.

Number of occasions for CB-Msg3 transmission group, i.e., N and optionally K occasions out of N allowed for replica transmission, is configured via system information.

The number of occasions, i.e., N > 0 indicates the size of group of Msg3 transmission occasions in time domain.

The starting point of the first transmission occasion of the group is configured. The transmission occasions for replicas within the group are defined w.r.t. the first transmission occasion.

When a UE transmits a copy of the CB-Msg3 in a transmission occasion, a mechanism is defined to help network determine the location of the other copy(ies). FFS on details of how to do it.

The RNTI derived should still be based on the time/frequency/occ resource of the N group, for example, based on N*f*O. Even in case of grouping of Msg3-based transmission occasions, the Msg3 retransmission scheduling can still be based on the transmission resource (time/frequency/OCC).

For DSA, RNTI(s) is also derived based on the time/frequency/occ resource associated with CB-msg3 transmission occasion(s). FFS on details.

The contention resolution can still be handled in the same existing way, i.e., by the use of contention resolution timer and contention resolution MAC CE in Msg4. There can be two options to maintain contention resolution timer.

Option#1: maintain a separate CR timer for each Msg3 transmission

Option#2: keep restarting the same CR timer after each CB-Msg3 transmission as shown in figure 2.

In our view, there is no need for maintaining multiple CR timers as the purpose is to decide how long the UE needs to monitor the PDCCH.

For Msg4, existing contention resolution procedure can be used, i.e., contention resolution timer and contention resolution MAC CE.

The contention resolution timer is started or restarted after the last repetition of the transmission of a msg3 transmission plus UE-eNB RTT.

Based on the configuration of DSA resource UE knows the network’s capability. Based on the use of the DSA resource, network knows the UE’s capability to support DSA.

Based on the configuration of DSA resource UE knows the network’s capability. Based on the use of the DSA resource, network knows the UE’s capability to support DSA.

Initiating procedure

Currently EDT and PUR have different sections for initiating procedure. However, same as EDT, the contention-based common PUSCH resource needs to be cell specific and needs to be provided per CE level. Similar to EDT, multiple TBS concept of EDT can also be considered as UEs may have different size of small UL data transmissions. It is also simple to consider the UL data transmission to be considered as EDT session from higher layer perspective to minimize the impact to other working groups such as SA2 and CT1.

Though this new procedure can have a separate section, the UL data transmission initiating procedure can be same as the EDT initiating procedure. Currently EDT initiating procedure does not allow mo-signaling and mo-VoiceCall, we think same as in NR, the mo-signaling should be allowed, i.e., UE can also send TAU update message via CB-Msg3. The CB-Msg3 transmision procedure can be initiated for a voice call as quick start (i.e., without Msg1 and Msg2) and network can bring the UE in RRC_CONNECTED. Therefore, the mo-VoiceCall can also be allowed.

Allow the CB-Msg3 establishment cause to be mo-Data or mo-Signalling or mo-ExceptionData or delayTolerantAccess or mo-VoiceCall.

Aspects related to RAN1

Similar to what was done in EDT, the introduction of CB-Msg3 may include some small specification updates in RAN1 specifications. For instance, the following should be considered by RAN1:

- Design of search spaces for response.

- Aspects related to retransmission of msg3-CB (e.g. DCI design)

- Usage of multiple TBSs

- Any other feedback on msg3-CB transmission (e.g. DMRS design, scrambling of NPUSCH, etc.)

Send LS to RAN1 informing potential alignment work associated with the CB-Msg3 transmission for DSA and non-DSA.

## 2. CB-Msg3 transmission attempt

It should be discussed how to handle the fallback scenario. It may not be efficient to keep trying only DSA resource or CB-Msg3 transmission which is based on PUSCH resource. Since the existing RSRP threshold is to define the initial PRACH CE level, same threshold may not be suitable for determining the CE level for the RSRP threshold. The network should always configure the lowest RSRP threshold to use the CB-Msg3 transmission for any CE level.

A new RSRP threshold to define the CE level for CB-Msg3 transmission is defined.

A lowest RSRP threshold is defined for each CE level.

The UE uses the CE level with the highest RSRP threshold among those CE levels for which the measured RSRP is higher that threshold. If none of the RSRP thresholds are met, the UE does not use CB-msg3.

There should also be a limit on how many attempts the UE is allowed for the CB-Msg3 transmission if the UE declares the contention resolution failure or contention resolution timer expires. After the limit, the UE can fallback to normal 4 step RACH-based procedure, e.g., RACH or EDT. We think this can be configured by network via system information.

If CB-Msg3 transmission fails, network configures via system information the maximum number of re-attempts allowed for CB-Msg3 transmission before falling back to 4 step RACH-based procedure.

It is also possible that the UL data is larger than the TBS size configured for the CB-Msg3 transmission. In general use case, the UL data.  In our view, it may be better to further study on allowing segmentation or transmission of multiple UL packets or defining total data volume as condition for the CB-Msg3 transmission or retransmission similar to subsequent SDT defined in NR NTN.

Consider subsequent CB-msg3 transmission based on total UL data volume similar to subsequent SDT defined for NR NTN.

If the total UL data volume is greater than the data volume threshold defined for CB-Msg3 transmissions, the UE does not use CB-Msg3 procedure.

After CB-Msg3 transmission, if the network wants UE to move to RRC_CONNECTED mode, the same RNTI cannot converted to the C-RNTI. It is because the RNTI is derived from the CB-Msg3 resource and should not be reused in RRC_CONNECTED. Therefore, the network should provide a new C-RNTI (e.g. by RRC message) to be used when moved to RRC_CONNECTED.

After CB-Msg3 transmission, if a UE is moved to RRC_CONNECTED, a new C-RNTI is provided to the UE.

Also, it is possible that network wants UEs to take back off or fall back to the 4 RACH procedure. We this option should also be possible for the network.

Support network indication of backoff or fallback to 4 step RACH procedure.

## 3. Msg4 enhancements

The second objective of the EDT enhancement is to reduce the congestion/overhead for transmission of Msg4 to several UEs. For example, many UEs may initiate EDT and network may exhaust the PDCCH or PDSCH resources while scheduling Msg4 (which comprises a PDCCH and at least one PDSCH) to each individual UE. There may be cases where network may not have any downlink response data and network would need to schedule Msg4 just including RRCEarlyDataComplete message (without DL NAS PDU) and contention resolution MAC CE of multiple UEs.

Instead of sending individual Msg4’s in such instances, a “multiplexed Msg4” to several UEs may be considered. An example procedure where multiplexed transmissions to several UEs using a single PDSCH are performed exists in case of Random-Access response. As shown in figure 2, network can schedule multiple RARs in a single MAC PDU.

Figure 3 TS 36.321 Figure 6.1.5-4: Example of MAC PDU consisting of a MAC header and MAC RARs

In this case there could be several possible options for the enhancements, some examples being:

- Reusing a PUR-like L1 ACK procedure

  - The L1 ACK may also be used to resolve contention between UEs

  - The L1 ACK can move UE back to RRC_IDLE

- Reusing concept of RAR-like multiplexing (e.g., of multiple users RAPIDs in a single RAR)

  - A single PDU can multiplex contention resolution MAC CEs (or Msg4) of more than one UE.

- Others such as existing Multi-TB scheduling for Msg4 could also be studied for multi-user scenario.

The second objective can be on the top of existing 4 steps EDT procedure as shown in figure 3, or applicable for the case of CB-msg3 / DSA as described for the first objective.

For the second objective, a PUR-like L1 ACK concept, as a response to the EDT transmission, can be considered.

For the second objective, multicast Msg4 (multi-user Msg4 multiplexing, similar to RAR or NR 2-step RACH response) or multi-user Msg4 scheduled by a single DCI (as in multi-TB Msg4 scheduling) can be studied as a solution.

The enhancements done for Msg4 transmission can be applicable to normal 4 step RACH or CB-Msg3 or DSA transmission.

Figure 4 Msg4 enhancement for EDT

Currently when a UE receives the Msg4 response, but the contention resolution ID included in the Msg4 does not match with it, the UE declares the contention resolution failure. We think for DSA solution, it is not necessary for UE to declare failure due to multiple replicas in a group as described in the previous section. When network detects collision, the network may decode one Msg3 transmission from a UE successfully. Then the network may want to respond the UE with Msg4 as well as scheduling Msg4 retransmission or fallback indication for the other UEs whose Msg3 transmissions were not successfully decoded. The UE can ignore the Msg4 if the contention resolution ID does not match. Then the UE can continue to monitor for potential reception of intended Msg4.

RAN2 study the mechanism that upon detection of collision, e.g., based on signal level, the network may respond with Msg4 for the UE whose Msg3 is decoded and scheduling Msg4 retransmission or fallback indication for the other UEs whose Msg3 transmissions were not successfully decoded.

- Conclusion

Following proposals are made:

Proposal 1	For DSA solution, a separate dedicated resource is configured via system information.

Proposal 2	Grouping of Msg3 transmission occasions in time domain is defined.

Proposal 3	Number of occasions for CB-Msg3 transmission group, i.e., N and optionally K occasions out of N allowed for replica transmission, is configured via system information.

Proposal 4	The number of occasions, i.e., N > 0 indicates the size of group of Msg3 transmission occasions in time domain.

Proposal 5	The starting point of the first transmission occasion of the group is configured. The transmission occasions for replicas within the group are defined w.r.t. the first transmission occasion.

Proposal 6	When a UE transmits a copy of the CB-Msg3 in a transmission occasion, a mechanism is defined to help network determine the location of the other copy(ies). FFS on details of how to do it.

Proposal 7	For DSA, RNTI(s) is also derived based on the time/frequency/occ resource associated with CB-msg3 transmission occasion(s). FFS on details.

Proposal 8	For Msg4, existing contention resolution procedure can be used, i.e., contention resolution timer and contention resolution MAC CE.

Proposal 9	The contention resolution timer is started or restarted after the last repetition of the transmission of a msg3 transmission plus UE-eNB RTT.

Proposal 10	Based on the configuration of DSA resource UE knows the network’s capability. Based on the use of the DSA resource, network knows the UE’s capability to support DSA.

Proposal 11	Allow the CB-Msg3 establishment cause to be mo-Data or mo-Signalling or mo-ExceptionData or delayTolerantAccess or mo-VoiceCall.

Proposal 12	Send LS to RAN1 informing potential alignment work associated with the CB-Msg3 transmission for DSA and non-DSA.

Proposal 13	A new RSRP threshold to define the CE level for CB-Msg3 transmission is defined.

Proposal 14	A lowest RSRP threshold is defined for each CE level.

Proposal 15	The UE uses the CE level with the highest RSRP threshold among those CE levels for which the measured RSRP is higher that threshold. If none of the RSRP thresholds are met, the UE does not use CB-msg3.

Proposal 16	If CB-Msg3 transmission fails, network configures via system information the maximum number of re-attempts allowed for CB-Msg3 transmission before falling back to 4 step RACH-based procedure.

Proposal 17	If the total UL data volume is greater than the data volume threshold defined for CB-Msg3 transmissions, the UE does not use CB-Msg3 procedure.

Proposal 18	After CB-Msg3 transmission, if a UE is moved to RRC_CONNECTED, a new C-RNTI is provided to the UE.

Proposal 19	Support network indication of backoff or fallback to 4 step RACH procedure.

Proposal 20	For the second objective, a PUR-like L1 ACK concept, as a response to the EDT transmission, can be considered.

Proposal 21	For the second objective, multicast Msg4 (multi-user Msg4 multiplexing, similar to RAR or NR 2-step RACH response) or multi-user Msg4 scheduled by a single DCI (as in multi-TB Msg4 scheduling) can be studied as a solution.

Proposal 22	The enhancements done for Msg4 transmission can be applicable to normal 4 step RACH or CB-Msg3 or DSA transmission.

Proposal 23	RAN2 study the mechanism that upon detection of collision, e.g., based on signal level, the network may respond with Msg4 for the UE whose Msg3 is decoded and scheduling Msg4 retransmission or fallback indication for the other UEs whose Msg3 transmissions were not successfully decoded.

- References

[1] R2-2408896, “Discussion on EDT enhancements”, Hefei, China, Oct 14th – 18th, 2024.