3GPP TSG-RAN WG2 Meeting #128	R2-2410083

Orlando, USA, Nov. 18th – 22nd, 2024

Agenda Item:	8.2.5 Topology 2 considerations

Source:	NEC

Title:	Validity of resources and reader behavior in Topology 2

Document for:	Discussion, Decision

Introduction

Rel-19 Ambient IoT (A-IoT) SI was approved at RAN#102. On deployment scenario with Topology 2 and UE as intermediate node, under network control, RAN2#127bis meeting achieved following agreements [1]:

Agreements 1 Dedicated resource configuration is only given to the UE reader via dedicated signalling. Mechanisms for shared resource pool amongst readers are not considered in this release. 2 The UE may perform the A-IoT procedure on A-IoT interface between the reader and the device only if the resource configuration is valid in the cell under network control. FFS how valid resource is determined – we will consider temporary out of connection scenarios. FFS on resource validity across multiple cells. 3 The A-IoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

This contribution further discusses validity of resources and reader behavior in Topology 2.

Discussion

Since FDMA and/or CDMA are still under RAN1 discussion, here we focus on TDMA case in which validity of resources” can also be interpreted as “validity duration of resources”.

A “validity duration of resources” indicates the time duration during which the UE reader can perform the A-IoT procedure on A-IoT interface between the reader and the device. To define such a “validity duration of resources”, the initiation and termination of the validity duration are essential. Since the A-IoT device is out of NW control (i.e., under the UE reader control), the UE reader behavior upon initiation and termination of the validity duration are observed below:

- Upon initiation of the validity duration, the UE reader is allowed to send R2D message and receive D2R message over configured/assigned A-IoT R2D and D2R resources

- Upon termination of the validity duration, the UE reader shall terminate R2D message transmission and may continue to receive D2R message over configured/assigned A-IoT D2R resources

Upon initiation of the validity duration, the UE reader is allowed to send R2D message and receive D2R message over configured/assigned A-IoT R2D and D2R resources.

Upon termination of the validity duration, the UE reader shall terminate R2D message transmission and may continue to receive D2R message over configured/assigned A-IoT D2R resources.

Hereafter, we will discuss initiation and termination of the validity duration for RRC connection scenario in 2.1. The temporary out of connection scenarios will be discussed in 2.2.

RRC connection scenario

Initiation of the validity duration

To support Topology 2, RAN3 agreed to study the following 3 solutions for conveying A-IoT upper layer information and capture them in the TR:

-	RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

-	NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

-	UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

RAN2#127bis agreed that dedicated resource configuration is only given to the reader via dedicated signaling. From the dedicated resource configuration procedure point of view, one point that NAS/UP based solution differs from the RRC based solution is that the resource configuration must be triggered by a request from the UE reader. While in RRC based solution, the resource configuration can be proceeded by the gNB itself without the request from the UE reader.

Regardless of which solution is applied, the dedicated resource configuration is prior to the validity duration. Therefore, one option of the initiation of the validity duration is upon receiving the dedicated signaling containing the dedicated A-IoT resource configuration, the UE reader initiates the validity of resources immediately. It is obviously that such an option is appropriate for NAS/UP based solutions since the configuration is provided upon the UE reader request. However, for RRC based solution, there might be a time interval between resource configuration and initiation of the validity duration, i.e., provide dedicated resource configuration via dedicated signaling in advance, then explicitly initiate the valid duration upon reception of A-IoT upper layer information from A-IoT CN over XXAP signaling. Therefore, it would be beneficial to provide the A-IoT resource configuration status (e.g., activated or deactivated) together with dedicated resource configuration.

RAN2 agree to provide explicit A-IoT resource configuration status (e.g., activated or deactivated) together with dedicated resource configuration.

When explicit A-IoT resource configuration status is configured as deactivated, it is obviously that a trigger for initiating the validity duration is needed. Basically, there are two options:

- Initiating the validity duration via a new MAC CE transmitted from gNB to the UE reader

- Initiating the validity duration via a new DCI transmitted from gNB to the UE reader

Based on above analyses, on the UE reader behavior of initiation of the validity duration, we have following options:

- Option 1: The UE reader initiates the validity of resources immediately upon receiving the dedicated signaling containing the dedicated A-IoT resource configuration and/or the explicit resource configuration status (e.g., activated).

- Option 2: After receiving the dedicated signaling containing the dedicated A-IoT resource configuration, the UE reader initiates the validity of resources upon receiving the MAC CE for initiating the validity duration.

- Option 3: After receiving the dedicated signaling containing the dedicated A-IoT resource configuration, the UE reader initiates the validity of resources upon receiving the DCI for initiating the validity duration.

On the UE reader behavior of initiation of the validity duration, RAN2 agree to capture following options to the TR: 
- Option 1: The UE reader initiates the validity of resources immediately upon receiving the dedicated signaling containing the dedicated A-IoT resource configuration and/or the explicit resource configuration status (e.g., activated).
- Option 2: After receiving the dedicated signaling containing the dedicated A-IoT resource configuration, the UE reader initiates the validity of resources upon receiving the MAC CE for initiating the validity duration.
- Option 3: After receiving the dedicated signaling containing the dedicated A-IoT resource configuration, the UE reader initiates the validity of resources upon receiving the DCI for initiating the validity duration.

Termination of the validity duration

So far, A-IoT resource allocation granularity has not been discussed/agreed at RAN2. There are several candidates, i.e., per UE reader or per A-IoT procedure/sub-procedure/message performed by a certain UE reader (for example, per D2R and R2D transmission pair or per access round).

The first candidate, i.e., per UE reader, means that how to use the assigned resource to proceed different A-IoT procedure/sub-procedure/message is determined by the UE reader. While for the second candidate, per A-IoT procedure/sub-procedure/message, the NW (gNB) may need to understand the A-IoT procedure/sub-procedure/message and/or grasp progress/situation of them at UE reader side which may result in a complicated network node implementation and/or additional information exchange between the UE reader and the NW (gNB).

RAN2 agree to assign/configure A-IoT resources to be used by A-IoT air interface per UE reader as baseline.

If proposal 3 is agreeable, then the termination of the validity duration may be variable since many uncertain factors (e.g., contention resolution during random access, re-access for handling failure cases, etc.) exist over the A-IoT radio interface between the UE reader and A-IoT devices. To properly terminate the validity duration, some assistance information (e.g., the A-IoT resource termination indication) from the UE reader to the NW is needed since only the UE reader knows details of the progress/situation of A-IoT operation over A-IoT air interface. Upon receiving the assistance information on termination of the validity duration, the NW terminates the resource validity via dedicated signaling/MAC CE/DCI.

To properly terminate the validity duration, RAN2 agree to further study necessary assistance information (e.g., the A-IoT resource termination indication) from the UE reader to the NW.

On the UE reader behavior of termination of the validity duration, there are two options:

-	Option 1: The UE reader terminates the validity of resources immediately upon sending the assistance information (e.g., the A-IoT resource termination indication) to the network.

-	Option 2: The UE reader terminates the validity of resources upon receiving the termination command of the resource validity duration via dedicated signaling/MAC CE/DCI.

As per our understanding, Option 2 is much aligned with the RAN2#127 agreement “NW is in control of resources to be used by AIoT air interface.”

On the UE reader behavior of termination of the validity duration, RAN2 agree to capture the following option to the TR:
The UE reader terminates the validity of resources upon receiving the termination command of the resource validity via dedicated signaling/MAC CE/DCI.

Temporary out of connection scenarios

Initiation of the validity duration

As RAN2#127bis agreed that dedicated resource configuration is only given to the UE reader via dedicated signaling, we think it is common understanding that initiation of the validity duration is limited to RRC connection status, i.e., there is no impact on initiation of the validity duration from temporary out of connection scenarios.

Termination of the validity duration

Different from RRC connection scenario, when the UE reader is in temporary out of connection, it may be impossible to receive the termination command of the resource validity duration via dedicated signaling/MAC CE/DCI from current serving cells. Therefore, termination of the validity duration for temporary out of connection needs to be discussed separately.

Two cases, RLF and handover, may cause temporary out of connection. Hereafter, we discuss these two cases one by one.

A simple procedure of RLF is illustrated in Fig.1.

Fig.1 A simple procedure of RLF

Firstly, clarification on the “temporary out of connection” may be needed. In our understanding, “temporary out of connection” related discussion is targeting the duration while the UE still stays at RRC_CONNECTED state as shown in Fig.1, i.e., before going back to idle.

The “temporary out of connection” related discussion in RAN2 is targeting the duration while the UE still stays at RRC_CONNECTED state, i.e., before going back to idle.

Then on the UE reader behavior of termination of the validity duration, we have following options:

-	Option 1: The UE reader terminates the validity of resources upon detecting radio problem, i.e., upon t310 starting.

-	Option 2: The UE reader terminates the validity of resources upon initiating the RRC connection re-establishment procedure, i.e., upon t311 starting

- Option 3: The UE reader terminates the validity of resources upon going back to idle, i.e., upon t311 expiring

Similar to RLF, for analyzing “temporary out of connection” caused by handover, a simple procedure of handover is illustrated in Fig.2.

Fig.2 A simple procedure of handover

Then on the UE reader behavior of termination of the validity duration, we have following options:

-	Option 1: The UE reader terminates the validity of resources upon reception of RRCReconfiguration message including reconfigurationWithSync., i.e., upon t304 starting.

-	Option 2: The UE reader terminates the validity of resources upon initiating the RRC connection re-establishment procedure, i.e., upon t311 starting

- Option 3: The UE reader terminates the validity of resources upon going back to idle, i.e., upon t311 expiring

It is obviously that Option 2/3 are the same as those for RLF case. As per our understanding, Option 2 and Option 3 are much aligned with the intention of RAN2#127 agreement “we will consider temporary out of connection scenarios”.

On the UE reader behavior of termination of the validity duration in temporary out of connection, RAN2 agree to capture following options to the TR: 
Option 1: The UE reader terminates the validity of resources upon initiating the RRC connection re-establishment procedure, i.e., upon t311 starting
Option 2: The UE reader terminates the validity of resources upon going back to idle, i.e., upon t311 expiring

Summary

This contribution further discussed validity of resources and reader behavior in Topology 2.

Observation 1	Upon initiation of the validity duration, the UE reader is allowed to send R2D message and receive D2R message over configured/assigned A-IoT R2D and D2R resources.

Observation 2	Upon termination of the validity duration, the UE reader shall terminate R2D message transmission and may continue to receive D2R message over configured/assigned A-IoT D2R resources.

Observation 3	The “temporary out of connection” related discussion in RAN2 is targeting the duration while the UE still stays at RRC_CONNECTED state, i.e., before going back to idle.

Proposal 1: RAN2 agree to provide explicit A-IoT resource configuration status (e.g., activated or deactivated) together with dedicated resource configuration.

Proposal 2: On the UE reader behavior of initiation of the validity duration, RAN2 agree to capture following options to the TR:

- Option 1: The UE reader initiates the validity of resources immediately upon receiving the dedicated signaling containing the dedicated A-IoT resource configuration and/or the explicit resource configuration status (e.g., activated).

- Option 2: After receiving the dedicated signaling containing the dedicated A-IoT resource configuration, the UE reader initiates the validity of resources upon receiving the MAC CE for initiating the validity duration.

- Option 3: After receiving the dedicated signaling containing the dedicated A-IoT resource configuration, the UE reader initiates the validity of resources upon receiving the DCI for initiating the validity duration.

Proposal 3: RAN2 agree to assign/configure A-IoT resources to be used by A-IoT air interface per UE reader as baseline.

Proposal 4: To properly terminate the validity duration, RAN2 agree to further study necessary assistance information (e.g., the A-IoT resource termination indication) from the UE reader to the NW.

Proposal 5: On the UE reader behavior of termination of the validity duration, RAN2 agree to capture the following option to the TR:

The UE reader terminates the validity of resources upon receiving the termination command of the resource validity via dedicated signaling/MAC CE/DCI.

Proposal 6: On the UE reader behavior of termination of the validity duration in temporary out of connection, RAN2 agree to capture following options to the TR:

Option 1: The UE reader terminates the validity of resources upon initiating the RRC connection re-establishment procedure, i.e., upon t311 starting.

Option 2: The UE reader terminates the validity of resources upon going back to idle, i.e., upon t311 expiring.

- References

[1] RAN2#127bis chair note