3GPP TSG-RAN2 Meeting #128	R2-2410087

Orlando, US, 18 - 22 November 2024	Revision of R2-2409060

Agenda Item:	8.4.4 Procedures for LP-WUS in RRC_CONNECTED

Source: 	Ericsson

Title:  	LP-WUS in Connected

Document for:	Discussion and Decision

# Introduction

In this contribution LP-WUS in connected mode is discussed further.

For the RAN1 and RAN2 agreements related the reader is referred to the Annex.

# Discussion

## Overview

The following options are discussed in RAN1 and RAN2:

Option 1-1 (DCP like):

- UE monitors LP-WUS before the drx-OnDurationTimer.

- When LP-WUS is detected the UE starts drx-OnDurationTimer and enters Active Time.

- UE can be configured to report periodic CSI/ L1-RSRP when LP-WUS is not detected.

Option 1-2:

- UE monitors LP-WUS periodically outside Active Time.

- When LP-WUS is detected the UE starts a new timer and enters Active Time.

- UE can be configured to report periodic CSI/ L1-RSRP during the OnDuration period when LP-WUS is not detected.

Option 1-3:

- UE monitors LP-WUS periodically inside Active Time.

- MAC is not aware of the LP-WUS monitoring in L1.

- FFS details and interaction with PDCCH skipping/adaptation, if any.

## Option 1-2

RAN1 agreed that option 1-2 can be configured without option 1-1, but it is FFS whether option 1-1 and 1-2 can be configured simultaneous or whether they can be combined. RAN1 agreed that the LP-WUS periodicity is not larger than the C-DRX cycle.

RAN2 agreed that a new timer is used with Option 1-2 and that the new timer is started when LP-WUS is detected. When the new timer is started the UE enters Active Time and starts monitoring PDCCH. When the new timer expires the UE goes back to LP-WUS monitoring. Other DRX parameters are not affected by option 1-2, e.g. when there is new UL/DL transmission the UE starts drx-InactivityTimer. It is FSS if the legacy drx-onDurationTimer is started or not if the new timer is configured with Option 1-2.

RAN1 already agreed that LP-WUS is only configured when legacy C-DRX is configured. The legacy drx-onDurationTimer is mandatory present in drx-Config, i.e. when the new timer is configured then the legacy drx-onDurationTimer is also configured. Thus the question is not whether they are configured together, but whether they can be used together. We think there is no need to use them together, which would only complicate matters (e.g. when one timer is started the other timer has to be stopped, etc). Nevertheless in the context of LP-WUS activation/deactivation/fallback it can be discussed further how the UE starts and stops using the new and legacy timer:

When the UE monitors LP-WUS outside Active Time the UE does not start the drx-onDurationTimer. FFS how the UE starts/stops monitoring LP-WUS outside Active Time in case LP-WUS activation/deactivation/fallback is supported.

The LP-WUS configuration, including the new timer, can be included in a new IE in MAC-CellGroupConfig IE in the RRCReconfiguration message:

The LP-WUS configuration is included in MAC-CellGroupConfig IE in RRCReconfiguration message. FFS whether LP-WUS configuration can be included in the initial configuration in RRCSetup/RRCResume message based on early measurements.

LP-WUS should probably not be used during handover, but this can be left to gNB implementation.

RAN1 agreed that with option 1-1 there is no impact on RRM/RLM/BFD measurement requirements, and that similar as with DCP, the UE can be configured to report periodic CSI/L1-RSRP when LP-WUS is not detected.

This means that while there is no traffic, i.e. the UE is outside Active Time and LP-WUS is not detected, then the UE still has to measure RLM/BFD every C-DRX cycle to detect RLF/BFD. Thus during idle periods the UE still have to wake-up the MR every C-DRX cycle. The power saving gains of option 1-1 are mainly/only when the UE is not configured to report periodic CSI/L1-RSRP, i.e. UL transmissions are avoided during idle periods.

Option 1-1 provides power saving gains while option 1-2 provides DL latency reduction for the initial DL packets, i.e. there is a benefit to support both options. In case the LP-WUS periodicity equals the C-DRX cycle length, then option 1-2 equals option 1-1:

Both option 1-1 and 1-2 are supported, i.e. the LP-WUS periodicity with option 1-2 can be equal to the C-DRX cycle length. The LP-WUS periodicity is larger or equal to the time offset. Option 1-1 and 1-2 are not configured simultaneously.

With option 1-2 the UE can be configured to report periodic CSI/L1-RSRP during the OnDuration period when LP-WUS is not detected.

During Rel-16 the support of DCP with short DRX was discussed in both RAN1 and RAN2. RAN1 did not have time to finish their work and DCP is not supported with short DRX. There is no RAN2 impact to support LP-WUS with short DRX:

From RAN2 perspective LP-WUS can be configured with short DRX.

With DCP the UE is required to start the drx-OnDurationTimer when the UE is not able to monitor the DCP, e.g. when the time between the end of the Active Time is shorter than the DCP time offset. Or when the UE cannot monitor DCP because it has to perform other activities (e.g. during BWP switching, measurement gaps or when monitoring CFRA response during beam failure recovery). The same principle should be applied with option 1-1. However further discussion is needed with option 1-2, because when the LP-WUS periodicity is very short, and the UE is not able to monitor LP-WUS in one occasion then the UE will be able very soon to monitor LP-WUS in the next occasion:

When the LP-WUS periodicity equals the C-DRX cycle length and the UE is not able to monitor the LP-WUS occasion then the UE starts the new timer. FFS when a smaller LP-WUS periodicity is configured and the UE is not able to monitor the LP-WUS occasion.

Similar as with DCP for improved reliability and robustness:

The UE can be configured to start the new timer when the UE is able to monitor LP-WUS but LP-WUS is not detected.

## Minimum time gap and preferred time offset

RAN1 agreed that the UE can indicate the minimum time gap that is requires between LP-WUS reception and the start of PDCCH monitoring by MR in the UE capability. RAN1 also agreed that the UE can indicate a preferred time offset using UAI signalling. The gNB configures a single time offset, which is equal or longer than the minimum gap supported by the UE.

With DCP the UE can signal a minimum time gap (in slots) per SCS supported in FR1, FR2-1 and FR2-2 in licensed and unlicensed spectrum. With LP-WUS the time offset refers to the time between LP-WUS reception by the LR and start of MR PDCCH monitoring by the MR:

The gNB configures a single time offset which is equal or longer than the minimum time gap signaled in the LP-WUS UE capability.

The motivation to signal a preferred time offset via UAI is to enable a different sleep state in the UE e.g. light or deep sleep:

|  | Relative Power (unit) | Transition time (ms) | Transition energy (ms) |
| --- | --- | --- | --- |
| Micro sleep | 45 | 0 | 0 |
| Light Sleep | 20 | 6 | 100 |
| Deep Sleep | 1 | 20 | 450 |

If configured the UE can report a preferred time offset, i.e. try to change the configured offset:

If configured, the UE can signal a preferred time offset via UAI signalling.

RAN1 indicated that for Idle and Inactive the UE may not support LP-WUS on all the supported bands [3]. It is assumed that the same considerations may apply for LP-WUS support in Connected:

In the LP-WUS connected mode capability the UE can indicate in which bands LP-WUS is supported in supportedBandListNR IE.

## CA, secondary DRX and DC

RAN1 agreed that LP-WUS is supported with CA and DC. It is FFS for which cells LP-WUS is applicable. RAN plenary confirmed that FR2 is within the scope of the LP-WUS WID [1,2]. This implies that LP-WUS with secondary DRX is within scope.

The UE is typically configured with CA to provide the highest throughput and shortest latency. Based on RRM measurement reporting and load balancing the CA configuration may change using RRCReconfiguration, i.e. carriers can be added or removed. Based on traffic demand and to save power SCells may be deactivated or enter dormant state (dormant BWP is used).

The DRX is common for all the cells of the Cell Group (CA/DC) or DRX group (secondary DRX), i.e. all the cells are in the same DRX state (e.g. Active Time) and there is a single DRX parameter, e.g. drx-OnDurationTimer running per cell/DRX group. The scheduler uses the full capacity provided by the cell/DRX group to schedule traffic.

With DC and secondary DRX the Cell/DRX groups can be in different DRX states e.g. MCG is inside Active State and SCG is outside Active State. With DC the timing is not aligned, i.e. the DRX cycles including drx-OnDurationTimer start/stop at different times. With secondary DRX the timing is aligned between the DRX group, i.e. the DRX cycles in the DRX groups start/stop at the same time.

With secondary DRX a shorter drx-OnDurationTimer and drx-InactivityTimer for FR2 can be configured to enable the secondary DRX group on FR2 to go into sleep more quickly and save power:

Cross carrier scheduling is not supported with secondary DRX. A trigger for aperiodic CSI and PUSCH reporting is on the same carrier e.g. in PCell of the default DRX group, but the cell for which CSI is reported may be in the same or other DRX group (frequency range). The network does not trigger aperiodic for the other DRX group, when the other DRX group is outside Active Time, i.e. the UE is only required to measure during Active Time.

DCP can be configured with CA/DC, i.e. the primary cell in the cell group can be configured with DCP. When DCP is triggered then the drx-OnDurationTimer is started and all the serving cells of the cell group start monitoring PDCCH and enter Active Time. This is not applicable for cells that are deactivated or in dormant state, i.e. the UE does not monitor PDCCH on those cells. DCP is not supported with secondary DRX, because secondary DRX was introduced under TEI it was agreed to keep it simple.

The UE indicates LP-WUS support per band i.e. the UE may not support LP-WUS on all the supported bands. This implies that the UE may not support LP-WUS on some of the SCells when CA/DC or secondary DRX is configured. In the following it is assumed that the UE supports LP-WUS on PCell (CA) and PSCell (DC) carrier/band, otherwise LP-WUS cannot be configured unless the PCell/PSCell can be reconfigured on a carrier the UE supports LP-WUS.

The UE may support LP-WUS using one or multiple LRs e.g. each LR supports a limited number of bands/frequencies to simplify the implementation. Or the UE may support LP-WUS only in one DRX group (e.g. FR1 only) or only in one Cell group.

In the following it is assumed that option 1-2 as described in section 2.2 is supported. Thus dependent on the configuration also option 1-1 is supported.

LP-WUS should be supported with CA and DC on the primary cell of the cell group, similar as DCP:

LP-WUS can be configured with CA and DC on the primary cell of the cell group. When LP-WUS is detected and the new timer is started then all the serving cells of the cell group enter Active Time.

LP-WUS should also be supported with secondary DRX and configured per DRX group:

LP-WUS can be configured with secondary DRX, i.e. one LP-WUS on the PCell of the default DRX group and another LP-WUS on a serving cell of the secondary DRX group. When LP-WUS is detected the new timer of the DRX group is started and all the serving cells of the DRX group enter Active Time.

One of the limitations of secondary DRX without DCP was that when a new traffic burst arrives and FR2 is sleeping, then data transmissions on FR2 can only start at the end of the DRX cycle, i.e. when drx-OnDurationTimer on FR2 is started. This limitation, i.e. delay is avoided with option 1-2 where the UE monitors LP-WUS periodically, and no cross-carrier wake-up is needed.

## Activation/deactivation

RAN1 and RAN2 agreed that LP-WUS monitoring is enabled/disabled via RRC signalling. It is FFS whether there is additional L1/L2 signalling, timers or conditions to activate/deactivate LP-WUS monitoring when LP-WUS is configured.

Based on existing RRM/CSI measurement reporting the gNB can configure or de-configure LP-WUS via RRCReconfiguration, e.g. when the UE is inside/outside LP-WUS coverage. When LP-WUS is configured the gNB uses LP-WUS to wake-up the UE irrespective whether the UE is inside or outside LP-WUS coverage. To avoid ping-pong, i.e. frequent reconfigurations, the gNB can decide to configure LP-WUS after a certain time period when the UE has reported good coverage and immediately de-configure LP-WUS when the quality drops below a certain threshold. This can be left to gNB implementation.

It is important that LP-WUS it is reliable and robust, i.e. the UE and NW should be synchronized whether LP-WUS is used or not. For example when the UE is outside LP-WUS coverage and cannot receive LP-WUS then the UE should monitor PDCCH directly. It is possible that the UE suddenly loses LP-WUS coverage and there is delay in the RRM/CSI measurement reporting, then the UE may become temporarily unreachable until LP-WUS is de-configured. Typically RRM and CSI reporting is not configured periodically to avoid high signalling load, but event based (RRM) or aperiodic (CSI). Independent on how the reporting is configured, the UE performs RRM/RLM/BFD measurements much more often than the reporting, and a local decision by the UE on current measurements would take much quicker effect.

There is no inter-operability issue when LP-WUS configuration is delayed, i.e. UE has entered LP-WUS coverage but LP-WUS has not been configured yet. An entry condition would also requires signalling towards the network, i.e. increase the signalling which is not preferred.

To resolve the problem that the UE suddenly may lose LP-WUS coverage before LP-WUS is de-configured by the gNB an LR exit condition can be configured when LP-WUS is configured, similar when LP-WUS is configured in SIB for UEs in idle/inactive. In connected mode the UE is not required to measure LP-SS, i.e. an exit condition based on LP-SS measurements is not possible. But the exit condition can be based on RRM/RLM/BFD measurements, and RLM/BFD measurements are preferred because the is also required to measure RLM/BFD outside Active Time at least every C-DRX cycle:

An exit condition based on RLM measurements can be configured when LP-WUS is configured. When the exit condition is fulfilled the UE stops monitoring LP-WUS.

## Option 1-3

Option 1-3 concerns PDCCH monitoring triggered by LP-WUS during Active Time (except when the drx-InactivityTimer is running):

But there are existing features like PDCCH skipping, SSSG switching and cross slot scheduling that can be used effectively on this very short time scale where light/micro sleep is possible during Active Time. For example with PDCCH skipping the UE may enter micro or light sleep temporarily while sending and receiving data. The pdcch-SkippingDurations that can be configured are between {1, 2, 3, …, 20, 30, 40, 50, 60, 80, 100} ms.

RAN2 should not discuss enhancements on a very short time scale which do not impact MAC. Furthermore few companies see the benefit of option 1-3 in addition to existing L1 power saving methods:

Option 1-3 is not supported.

## Subgrouping

RAN1 agreed that subgrouping is also supported in connected mode, and that either a bit or codepoint in LP-WUS indicate that one or more UEs should start monitoring PDCCH.

The OnDuration of UEs in connected mode typically do not overlap, or an attempt is made to minimize overlap. During the OnDuration the gNB may want to reach a specific UE, and when the start of the OnDurations coincide, then the LP-WUS can wake-up multiple UEs and cause false alarms.

The number of “individual” LP-WUS occasions that can be configured in connected mode depends on:

- C-DRX cycle length

- Numerology, i.e. number of slot per C-DRX cycle

- LP-WUS duration (1 or more slots)

- LP-WUS bandwidth (up to 5 Mhz)

The number of required LP-WUS occasions depends on the number of UEs in connected mode, and penetration of the feature, i.e. whether the UE supports the feature.

Subgroup info can be included in the LP-WUS configuration. Wait for RAN1 progress to discuss further details.

## Paging PDCCH

In connected mode the UE does not receive Paging PDSCH, but the UE is required to monitor Paging PDCCH, e.g. to receive ETWS/CMAS warning messages. If LP-WUS is configured in SIB, then the UE in connected mode can use the idle/inactive LP-WUS to monitor Paging PDCCH:

If LP-WUS in idle/inactive mode is configured in SIB then the UE in connected mode may use LP-WUS monitoring as in idle/inactive mode to receive Paging PDCCH e.g. SI-change and ETWS/CMAS indication.

## Specification impact

DCP monitoring is specified in RAN1 38.213 and lower layers provide an indication to MAC when DCP is detected (38.321):

3>	if DCP indication associated with the current DRX cycle received from lower layer indicated to start drx-onDurationTimer, as specified in TS 38.213 [6]; or

With DCP there is an implicit assumption that lower layers are aware of the MAC Active Time, i.e. the UE is only required to monitor DCP outside Active Time before the OnDuration. It is proposed to follow the same principle with LP-WUS:

Lower layer indicates when LP-WUS is detected outside Active Time.

The difference between LP-WUS and DCP is that:

- Lower layer indication that LP-WUS is detected may happen during any LP-WUS occasion outside Active Time during a DRX cycle, not only before the LP-WUS occasion before the OnDuration.

- Dependent on the configuration and traffic it is possible that the new timer is started multiple times during a DRX cycle, however this is not considered a typical case.

In case LP-WUS is captured in a similar way as DCP in MAC, then the two cases described above are also captured implicitly. RAN1 will discuss the impact on 38.213, and hopefully the impact is similar as with DCP, i.e. the same principles apply but there are only more monitoring occasions.

# Summary

RAN2 is kindly asked to discuss LP-WUS in connected mode:

Proposal 1	When the UE monitors LP-WUS outside Active Time the UE does not start the drx-onDurationTimer. FFS how the UE starts/stops monitoring LP-WUS outside Active Time in case LP-WUS activation/deactivation/fallback is supported.

Proposal 2	The LP-WUS configuration is included in MAC-CellGroupConfig IE in RRCReconfiguration message. FFS whether LP-WUS configuration can be included in the initial configuration in RRCSetup/RRCResume message based on early measurements.

Proposal 3	Both option 1-1 and 1-2 are supported, i.e. the LP-WUS periodicity with option 1-2 can be equal to the C-DRX cycle length. The LP-WUS periodicity is larger or equal to the time offset. Option 1-1 and 1-2 are not configured simultaneously.

Proposal 4	With option 1-2 the UE can be configured to report periodic CSI/L1-RSRP during the OnDuration period when LP-WUS is not detected.

Proposal 5	From RAN2 perspective LP-WUS can be configured with short DRX.

Proposal 6	When the LP-WUS periodicity equals the C-DRX cycle length and the UE is not able to monitor the LP-WUS occasion then the UE starts the new timer. FFS when a smaller LP-WUS periodicity is configured and the UE is not able to monitor the LP-WUS occasion.

Proposal 7	The UE can be configured to start the new timer when the UE is able to monitor LP-WUS but LP-WUS is not detected.

Proposal 8	The gNB configures a single time offset which is equal or longer than the minimum time gap signaled in the LP-WUS UE capability.

Proposal 9	If configured, the UE can signal a preferred time offset via UAI signalling.

Proposal 10	In the LP-WUS connected mode capability the UE can indicate in which bands LP-WUS is supported in supportedBandListNR IE.

Proposal 11	LP-WUS can be configured with CA and DC on the primary cell of the cell group. When LP-WUS is detected and the new timer is started then all the serving cells of the cell group enter Active Time.

Proposal 12	LP-WUS can be configured with secondary DRX, i.e. one LP-WUS on the PCell of the default DRX group and another LP-WUS on a serving cell of the secondary DRX group. When LP-WUS is detected the new timer of the DRX group is started and all the serving cells of the DRX group enter Active Time.

Proposal 13	An exit condition based on RLM measurements can be configured when LP-WUS is configured. When the exit condition is fulfilled the UE stops monitoring LP-WUS.

Proposal 14	Option 1-3 is not supported.

Proposal 15	Subgroup info can be included in the LP-WUS configuration. Wait for RAN1 progress to discuss further details.

Proposal 16	If LP-WUS in idle/inactive mode is configured in SIB then the UE in connected mode may use LP-WUS monitoring as in idle/inactive mode to receive Paging PDCCH e.g. SI-change and ETWS/CMAS indication.

Proposal 17	Lower layer indicates when LP-WUS is detected outside Active Time.

# References

- RP-242361, Way forward on LPWUS for FR2, vivo, DISC, RAN#105

- RP-241824, Revised WID Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC, RAN#105

- R1-2407559, LS on LP-WUS operation in IDLE/INACTIVE mode, LS out, RAN1, To: RAN2, RAN4, RAN1#118

# Annex: Agreements

Option 1-1 as a specifial case of option 1-2 only

## RAN1 agreements (selection)

R1-2409010	FL summary #1 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

Agreement

- For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective.

  - LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer

  - UE is configured with legacy C-DRX configurations as Rel-18

  - UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal

  - No impact on RRM/RLM/BFD measurement requirements is assumed

  - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP)

    - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up

    - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not

      - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification

Agreement

Confirm following working assumption

Working Assumption

From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration

  - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

  - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

  - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Note: Above can be revisited considering RAN2 decisions.

R1-2409197	FL summary #2 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

Agreement

For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting.

- FFS: X

- FFS: definition of reported candidate value

- UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional.

- FFS: A single time offset is configured for a UE at a given time.

R1-2409237	FL summary #3 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

Agreement

LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode

- Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group

- FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

R1-2409265	FL summary #4 on LP-WUS operation in CONNECTED mode	Moderator (NTT DOCOMO)

Agreement

For LP-WUS CONNECTED mode operation, Option 1-2 can be configured separately from Option 1-1

- FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE

Agreement

Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective,

- Opt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions

- Introducing monitoring window is not precluded

## RAN2 agreements (selection)

Option 1-2 related

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2.

In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring.

When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.