3GPP TSG-RAN WG2 Meeting #128												R2-2410091

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	8.7.4.2

Source: 	Sharp

Title:  	Details of DSR Enhancements

Document for: 	Discussion

# Introduction

In RAN2#127bis meeting, RAN2 made the following agreements:

Agreements on DSR enhancements - We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

This document discusses open issues and detail of DSR enhancements for XR.

# Discussion

## Relation between Triggering Threshold and Reporting Threshold.

In RAN2#127bis meeting, there was a discussion whether the triggering threshold should depend on reporting threshold. Figure 1 illustrates an example that the triggering threshold is smaller than the reporting threshold. In the example, we assume only one reporting threshold for simplicity without loss of generality. The example assumes that the triggering threshold is 40ms, reporting threshold is 50ms, and the triggered DSR is transmitted 10ms after the triggering. Until the DSR triggering time (time t), an SDU arrives every 5ms. When the DSR is triggered, only one SDU has remaining time below 40ms. When the corresponding DSR is transmitted at time t+10ms, five SDUs have remaining time below the reporting threshold. The total amount of those five SDUs will be reported. There is no serious problem foreseen.

We can leave all flexibility up to the network, as no complexity nor problem is expected. It is possible that one of the reporting thresholds is the same as the triggering threshold. Also, the network may configure threshold value greater than remaining time threshold. It can mandate the UE to report remaining time and buffer size for non-delay critical data.

The triggering threshold can be different from any of reporting threshold.

Figure 1. An example of DSR trigging and contents for triggering threshold of 40ms and reporting threshold of 50ms.

## New DSR Format and Configuration

The next issue is how to change the DSR format to report multiple portions. Figure 2 depicts a possible DSR format for the enhancements. We see that R field in Rel-18 DSR format can be simply used to indicate the next pair of remaining time and buffer size is for the different portion of the same LCG (shown in E field in the figure). In this case, minimum change can be achieved without increasing MAC CE size unnecessarily. Remaining time field can be associated with one of portion reported in the DSR, so additional indication of the portion is not necessary.

R field in Rel-18 DSR MAC CE is changed to E field to indicate whether the next pair of remaining time and buffer size is for the different portion of the same LCG.

Figure 2. New DSR MAC CE Format with Multiple Portions

If the DSR MAC CE format shown in Figure 2 is agreeable, there is no size difference between old DSR and new DSR in case that only one portion is reported. Thus, it does not have any additional overhead even if the new DSR MAC CE is always used. Then, if a reporting threshold is separately configured, the UE can always use the new DSR MAC CE.

If a reporting threshold is separately configured, the UE can always use the new DSR MAC CE.

## Buffer Size of Non-Delay-Critical Data

When non-delay-critical data SDUs are interspersed with delay-critical data, reporting only delay-critical data volume in DSR might not be sufficient for gNB to schedule the exact amount of uplink resources that the reported delay-critical data are all transmitted. For this scenario, it would be good to allow buffer size information including non-delay-critical data which impacts to the data transmission of delay-critical data. The problem of insufficient information should be resolved. For network flexibility, we need to make it configurable.

gNB can configure whether a buffer size information indicated by the buffer size field includes non-delay-critical data.

# Conclusions

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

- The triggering threshold can be different from any of reporting threshold.

R field in Rel-18 DSR MAC CE is changed to E field to indicate whether the next pair of remaining time and buffer size is for the different portion of the same LCG.

If a reporting threshold is separately configured, the UE can always use the new DSR MAC CE.

gNB can configure whether a buffer size information indicated by the buffer size field includes non-delay-critical data.