3GPP TSG-RAN WG2 Meeting #128												R2-2410093

Orlando, USA, November 18th – 22nd, 2024

Agenda Item:	8.7.6

Source: 	Sharp

Title:  	XR Rate Control Enhancements

Document for: 	Discussion

# Introduction

In RAN2#127bis meeting, RAN2 made the following agreements:

Agreements on XR rate control - FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc. - RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - Extend the Bit Rate field - Define a new bit rate table to provide sufficient granularity for XR traffic - Introduce new values for the bitRateMultiplier - Send LS to SA4 asking about range/granularity which is required

This document discusses open issues on XR rate control enhancements.

# Discussion

In the MAC specification, a rate control signaling mechanism called a recommended bit rate procedure is defined. The recommended bit rate procedure consists of query and recommendation indication. Both of them use a common MAC CE format called recommended bit rate MAC CE as shown in Figure 1.

Figure 1. Recommended bit rate MAC CE in TS 38.321

The current rate control signaling, i.e., MAC CE, is per logical channel. The reason was that the recommended bit rate MAC CE was introduced since LTE where the basic traffic granularity is logical channel. NR MAC specification reused the LTE feature with minimum change. However, in 5G, each traffic flow is mapped to a QoS flow. Also, one QoS flow may serve multiple traffic flows with similar QoS characteristics. Hence, a logical channel may serve a number of QoS flows and a number of actual traffic flow, especially when a UE serves multi-modal traffic or multiple traffic flows in tethering scenarios. In those scenarios, the legacy rate control siganling per LCH is not appropriate.

One could argue that one-to-one mapping between traffic flow and logical channel may enable per flow control. However, in the scenarios described above, a large number of logical channels (more than 16) are required for a MAC entity, which may not be supported under the current UE. As a result, the rate control siganling mechanism should be extended to per traffic signaling.

In case of multi-modal traffic or multi traffic flows served in tethering scenario, one-to-one mapping between logical channel and traffic flow may not be possible due to the number of supported logical channels.

Recommended bit rate signaling per QoS flow is introduced.

In RAN2#127bis meeting, RAN2 sent an LS to SA4 to ask range/granularity of bit rate required for XR traffic. RAN2 expects that SA4 will provide some information on value range and granularity of bit rate. We see that RAN2 can discuss how to enhance the field information to be reported in the MAC CE, based on SA4 response.

SA4 will provide some information on value range and granularity of bit rate to be reported for XR.

Enhancements on bit rate field/table and multiplier can be discussed after SA4’s reply LS is received.

Currently, triggering of the recommended bit rate query is based on the upper layer request. However, the allowed bit rate (i.e., codec rate) highly depends on RAN status, e.g. congestion and uplink scheduling. The application layer may not be sure when and how often the UE can be updated with the recommended bit rate from the gNB. It may lead unnecessarily frequent queries to solicit the recommendation, however the network does not want to change the codec rate so frequently. Except for the prohibit timer which may prohibit the necessary query, there is no mechanism for the network to control the query. It may bring a situation that different UEs may have different triggering mechanisms. For efficient and feasible use of rate adaptation of XR applications, it would be better to define standardized triggering conditions which can be under network control.

Currently, triggering of the recommended bit rate query is based on the upper layer request. It is not under network control.

Triggering condition of recommended bit rate query based on available information in UE AS layer is introduced.

We see that the current recommended bit rate query is not suitable for XR application which requires timely transmission with reliability. The recommended bit rate query is transmitted in a best effort manner due to the following reasons:

- The recommended bit rate MAC CE has lower priority than any other MAC CEs and data. In congestion scenario, the recommended bit rate MAC CE will not be transmitted until all other data is completely transmitted.

- The recommended bit rate query is transmitted as a MAC CE. Its successful transmission is not known by the UE. In case that the MAC CE is lost, the rate adaptation will be significantly delayed.

The main reason for those problems is that the existing recommended bit rate procedure was designed as an optional assistance information of voice traffic. This is not adequate for XR application. Thus, RAN2 should consider those problems and discuss how to resolve them.

The current recommended bit rate query is transmitted in a best effort manner. It can be delayed significantly, and successful delivery is not guaranteed.

RAN2 to discuss how a recommended bit rate query can be timely transmitted.

# Conclusions

Based on the discussion given above, RAN2 is requested to discuss and agree the following proposals:

- Recommended bit rate signaling per QoS flow is introduced.

- Enhancements on bit rate field/table and multiplier can be discussed after SA4’s reply LS is received.

Triggering condition of recommended bit rate query based on available information in UE AS layer is introduced.

RAN2 to discuss how a recommended bit rate query can be timely transmitted.