3GPP TSG-RAN WG2 Meeting #128	R2-2410137

Orlando, USA, Nov 18th – 22nd, 2024

Agenda item:	8.2.2

Source:	Spreadtrum,UNISOC

Title:	Discussion on the functionalities required for Ambient IoT

Document for:	Discussion and Decision

1 Introduction

In RAN2#127bis meeting, RAN2 discussed the functionalities required for A-IoT and achieved the following agreements [1]:

Agreements on AS ID RAN2 assumes that if “AS ID” is defined it is used at least for purpose of D2R scheduling and R2D reception. Up to RAN1 to decide whether a “AS ID” is defined. RAN2 assumes this “AS ID” should be a short AS layer ID, rather than the full upper layer device ID. FFS on the length. FFS if AS ID can be based on partial upper layer device ID. From RAN2 perspective, following options are possible for “AS ID”: • Option 1: a random ID if used in Msg1 can be reused; • Option 2: reader assigns this “AS ID”. FFS via which R2D message. RAN2 will aim to define one common design for all RA procedures (if technically possible) Agreements on energy status reporting Capture in the TR the option: the device may include energy status indication in D2R messages (e.g. MSG1, MSG3 and Command Response message), if the device can. 1 bit indication can be captured in the TR. We will capture the use case in the TR. FFS whether it is reader controlled. Agreements on message size indication From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size. Two options can be captured: 1) from the CN and 2) from the device (simple message size indication). Capture the advantages/disadvantages. The D2R message size would be beneficial but it is not essential. Ask SA2 if it is possible to provide the expected (e.g. approximate, estimate, exact, max) future response D2R message size. Is it always available, sometimes, or never. Agreements on segmentation RAN2 will study segmentation and RAN2 will need to give some guidance once there is more clarity on solution, on the minimum TB size. Study the functionality of segmentation in the MAC. We focus on D2R and can consider the support for R2D later depending on SA2 and RAN1. The solution should aim to be applicable to both direction (i.e. unified solution), if we decide to support on both directions. Do not support sequence number for A-IoT segmentation functionalities. Do not support segment number and the number of segments for A-IoT segmentation functionalities. Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment. FFS the details on how this is designed and number of bits (one or two). RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s). Agreements on DoA From RAN2 perspective, at least the A-IoT paging is an aspect/part of the current design which is not sufficient for the DO-A use case. From RAN2 perspective, DO-A use case cannot be supported with the current design discussed in the study item.

In this contribution, we will continue discussion the functionalities required for A-IoT.

2 Discussion

## 2.1 The protocol stack of A-IoT

In RAN2 meeting, it agreed that SDAP, PDCP and RLC layer are not needed for A-IoT. RAN2 assumes that RRC layer is not necessary between the reader and the device, RAN2 later discusses whether we will have a new AS protocol on top of A-IoT MAC layer or A-IoT MAC.

In our view, to meet the compact protocol stack system design objective for A-IoT, a new AS protocol on the top of A-IoT MAC is not needed. The necessary function A-IoT device required may done in MAC layer, we should define a new MAC layer for A-IoT.

Proposal 1: No a new AS protocol on top of A-IoT MAC layer.

Only MAC and PHY protocol layers are needed in the protocol stack for A-IoT, the compact protocol stack for A-IoT as following Figure 1.

Figure 1 A-IoT protocol stack for topology 1

Proposal 2: For A-IoT topology 1, RAN2 agrees the protocol stack in Figure 1.

## 2.2 Information visible to reader

In 127 meeting, RAN2 agreed that the service type of A-IoT message, the approximate number of target devices and targeted for one or more devices are useful information for reader to obtained from the CN. But whether command type (such as read/write/disable) is needed is still FFS.

At least the following information are considered useful to be visible to the reader from CN 1) The service type of A-IoT (e.g. inventory, command). FFS if more information on command type (e.g. read/write/disable) is useful 2) targeted for one or more than one devices; 3) approximate number of target devices (if available). FFS on mandatory/optional

For A-IoT radio between device and reader, it is the network to allocate RA resource and data transmission resource to A-IoT radio. For different kinds of procedure, the resource allocation can be different. For example, if read command is triggered, the acknowledgement can be 1000bits at most; if write command is included in the AIOT paging message, the D2R feedback can be one or limit number of bits. For effective resource scheduling, the reader at least needs to understand the type of the command from CN, e.g. read/write/disable to assistant resource allocation.

In addition, if CN can provide the service type (e.g. inventory, command (read/write/disable)) of A-IoT message, the approximate number of target devices and targeted for one or more devices are useful information to reader, we expect that CN always provides these useful information to reader. Since it is useful for reader to allocate resource.

Proposal 3: The command type (e.g., read/write/disable) should be visible to reader.

Proposal 4: From RAN2 perspective, we expect that CN always provides these useful information to reader.

3 Conclusion

In this contribution we discuss the functionalities required for A-IoT, with the following proposals:

Proposal 1: No a new AS protocol on top of A-IoT MAC layer.

Proposal 2: For A-IoT topology 1, RAN2 agrees the protocol stack in Figure 1.

Proposal 3: The command type (e.g., read/write/disable) is visible to reader.

Proposal 4: From RAN2 perspective, we expect that CN always provides these useful information to reader.

- References

- Draft report of 3GPP TSG RAN WG2 #127bis, RAN2 chair (interDigtal), Hefei, China, Oct 14th – 18th, 2024.