3GPP TSG-RAN WG2 Meeting #128 	R2-2410141

Orlando, USA, 18th – 22th Nov, 2024

Agenda Item:	8.2.5

Source:	Spreadtrum, UNISOC

Title:	Discussion on Topology 2 issues

Document for:	Discussion and Decision

Introduction

In the SID [1], topologies 1 and 2 are the scope of AIoT. And the overall objective is to study a harmonized air interface design with minimized differences for different topologies.

RP-240826 Revised SID [1] General Scope The definitions provided in TR 38.848 are taken into this SI, and the following are the exclusive general scope: - The overall objective shall be to study a harmonized air interface design with minimized differences (where necessary) for Ambient IoT to enable the following devices: - ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally. - ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm,DL and/or UL amplification in the device. The device’s UL transmission may be generated internally by the device, or be backscattered on a carrier wave provided externally. - X is to be decided in WGs. - Coverage design target: Maximum distance of 10-50 m with device indoors as per TR 38.848: “…a range that WGs can sub-select within”. - For Topologies 1 & 2 (UE as intermediate node under NW control) per TR 38.848, with no RRC states, no mobility (i.e. at least no cell selection/re-selection -like function), no HARQ, no ARQ. RAN1-led: …… For Topology 2, no difference in physical layer design from Topology 1.

This Tdoc mainly focuses on topology 2 related aspects between gNB and reader. In this contribution, we discuss the issues related to topology 2. And we provide our opinions.

Discussion on Topology 2 related issues

2.1 Uu link spec impact

For topology 2, the AIoT link is no difference in physical layer, RAN2 protocol stack from topology 1. For topology 2, since the UE acts as an intermediate node, the link between intermediate UE and gNB would reuse the legacy Uu interface as much as possible, with minimum improvements if deemed necessary.

Proposal 1: In topology 2, reuse the legacy Uu interface for the interface between the intermediate UE and gNB with minimum improvements.

2.1.1 Resource allocation for AIoT link

RAN2 #127bis agreements: 2 The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control. FFS how valid resource is determined – we will consider temporary out of connection scenarios. FFS on resource validity across multiple cells. 3 The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

For resource allocation for AIoT link, the above agreements are achieved. FFS how valid resource is determined. Valid area and valid timer are proposed in last meeting in some contributions. In our understanding, if resource is valid across multiple cells, we needs to define valid area. The coordination between the gNB is complicated. If resource is valid within a duration of time, it hard to determine the duration of validity timer. So we think the resource is valid in the (serving) cell from which UE receives the configuration. When the UE reselect or handover to the other cell, the allocated resource by source cell is invalid. Or the allocated resource is invalid when UE reader receives the RRC Release message.

Proposal 2: The resource is valid only in the (serving) cell from which UE received the configuration in R19.

For temporary out of connection scenarios, e.g., RLF, HO, upon RLF/HO is detected, before UE reader (re)select or handover to other cell, the resource is still valid. It means that the resource is valid after the resource configured and becomes invalid when the UE reader reselects or switches to a new cell.

Proposal 3: The resource is still valid during temporary out of connection within same cell.

2.1.2 Traffic differentiate in Uu link

For topology 2, the traffic on intermediate UE mainly contains signalling for inventory and command, and the DL/UL data. the AIoT R2D traffic, when the intermediate UE reader receive the inventory/command request, it will trigger AIoT related procedure, such as paging, random access, data communication procedure. After RAN-related procedure, intermediate UE will forward AIoT link response data to gNB, such as device ID or command response.

According to SA2 progress, there are three types of data transmission solutions: 1) upper layer data transmission via user plane solutions; 2) upper layer data transmission via control plane solutions; 3) RRC layer based data transmission.

For upper layer data transmission via user plane solutions, the A-IoT messages is transmitted as user plane data. And gNB is unaware of the contents of the AIoT message. And upper layer data transmission via control plane solutions, AIoT messages is transmitted as NAS PDU. And gNB also is unaware of the contents of the AIoT messages. And for RRC-based solutions, AIoT message is transmitted as RRC signalling. If those AIoT messages are seen as normal data/NAS/RRC messages of intermediate UE, it will incur frequent BSR or SR report to gNB to request UL resources, especially for inventory response (amount of devices response the device ID to CN node). So the intermediate UE needs to differentiate the AIoT traffic from its own traffic based on the configuration from the gNB, for example, gNB configures different LCID/LCG ID/SR configuration for relayed AIoT traffic and the traffic of the intermediate node.

Proposal 4: gNB configures different LCID/LCG ID/SR configuration for relayed AIoT traffic and the traffic of the intermediate node.

2.1.3 The state of intermediate UE

Another issue that needs to be discussed is: what is the RRC status of the intermediate node UE?

The intermediate UE needs to receive the AIoT request from gNB and transmit the AIoT response to gNB. So the intermediate UE can work normally when the UE is in the RRC_Connected state during inventory or command procedure. Furthermore, if the UE is in the RRC_Inactive state, it needs to be warked up and moved to RRC_Connected state. If the intermediate UE is in the RRC_Idle state, when there is AIoT data, the intermediate UE need to transfer to the RRC_Connected state. So we think when selecting the UE reader, UE reader in any RRC states can be selected. But during AIoT procedure, especially for inventory service, inventory response are frequently transmit to network, so UE reader should be in RRC_Connected when there is AIoT data transmission procedure. UE reader should be in RRC_Connected state. The spec impact is minimum.

Proposal 5: In R19, the intermediate UE should be in RRC_Connected state during AIoT procedure.

2.1.4 The selection of the intermediate UE

The selection of the intermediate UE can be performed based on device specific information (e.g. geographic location of the AIoT devices, the identity of the intermediate node from AF, AIoT device capability and the UE reader capability etc.) by CN or by RAN [2]. For the selection by CN option, the selected UE readers can be informed to gNB. There is no RAN2 spec impact. For the selection by RAN option, the assistance information from CN node may be needed to select UE reader. So RAN2 can wait for SA2 and RAN3 progress to discuss whether/how to select the intermediate UE.

Proposal 6: Whether/how to select the intermediate UE is up to SA2/RAN3.

Conclusions

In this contribution, we discuss the Rel-19 Topology 2 related enhancements. And the proposals are given as follows:

Proposal 1: In topology 2, reuse the legacy Uu interface for the interface between the intermediate UE and gNB with minimum improvements.

Proposal 2: The resource is valid in the (serving) cell from which UE receives the configuration.

Proposal 3: The resource is still valid during temporary out of connection within same cell.

Proposal 4: gNB configures different LCID/LCG ID/SR configuration for relayed AIoT traffic and the traffic of the intermediate node.

Proposal 5: In R19, the intermediate UE should be in RRC_Connected state during inventory/command procedures.

Proposal 6: Whether/how to select the intermediate UE is up to SA2/RAN3.

References

[1] RP-240826, Revised Rel-19 Ambient IoT SID, RAN #103 meeting.

[2] TR 23.700-13, Study on Architecture support of Ambient power-enabled Internet of Things (Release 19)