3GPP TSG-RAN WG2 Meeting # 128	R2-2410150

# Orlando, USA,  November 18th - 22th 2024

Agenda Item:	8.13.2

Source:	Ericsson

Title:	Discussion on relay discovery and relay (re)selection

Document for:	Discussion, Decision

# 1	Introduction

For the WI on NR sidelink multiple-hop relay, the below objectives on relay discovery and relay (re)selection have been defined in the WID [1].

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

.

We discuss aspects on relay discovery and relay (re)selection in this paper. All discussions refer to L2 U2N Remote UE or L2 U2N Relay UE unless otherwise stated.

# 2	Discussion

In this section, we discuss aspects regarding Relay discovery including discovery model, RRC state, discovery configuration, Uu RSRP threshold, resource pool, resource allocation mode, power control and security.

In the legacy, standalone discovery model (i.e., Model A and Model B discovery model) are supported for single hop U2N relay. It is natural to support both of these models same as in the legacy for multi-hop relay.

Same as in legacy, support standalone discovery model (i.e., Model A and Model B discovery model) for multi-hop U2N relay.

In the legacy, a U2N remote UE can be configured with a Uu RSRP threshold, based on which the U2N remote UE determines if it can transmit discovery messages. Similarly, a U2N relay UE can be configured with a maximum Uu RSRP threshold, a minimum Uu RSRP threshold, or both, based on which the U2N relay UE determines if it can transmit discovery messages. It is reasonable to reuse these thresholds for Remote UE and U2N Relay UE in multi-hop relay scenario. If the gNB supports both single hop U2N relay and multi-hop U2N relay, it is reasonable for the gNB to configure the same Uu RSRP thresholds. In other words, the gNB doesn’t need to distinguish between single hop Remote UEs and multi-hop Remote UEs.

Reuse the same Uu RSRP thresholds as in the legacy (i.e., single hop L2 U2N relay) for L2 Remote UE and L2 U2N Relay UE in multi-hop U2N relay.

Regarding resource pools, same as in the legacy, a dedicated resource pool and shared resource pools can be (pre)configured for discovery in multi-hop relay. Same as Uu RSRP threshold, the gNB can configure the same discovery resource pool(s) for both single hop U2N relay and multi-hop U2N relay.

Reuse the same discovery resource pool(s) and configurations as in the legacy (i.e., single hop L2 U2N relay) in multi-hop U2N relay.

## 2.2 Relay (re)selection

In legacy, the U2N Remote UE triggers U2N Relay selection in following cases:

-	Direct Uu signal strength of current serving cell of the U2N Remote UE is below a configured signal strength threshold;

-	Indicated by upper layer of the U2N Remote UE.

The U2N Remote UE may trigger U2N Relay reselection in following cases:

1)	PC5 signal strength of current U2N Relay UE is below a (pre)configured signal strength threshold;

2)	Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by U2N Relay UE via PC5-RRC signalling;

3)	When U2N Remote UE receives a PC5-S link release message from U2N Relay UE;

4)	When U2N Remote UE detects PC5 RLF;

5)	Indicated by upper layer.

For Relay selection, the existing conditions can be directly reused.

Therefore, we would like to make the below proposal.

Supports the same trigger conditions of relay selection for L2 U2N Remote UE as in legacy.

For Relay reselection,

The trigger conditions 1) is applicable to both L2 Intermediate Relay UE and L2 U2N Relay UE. For the latter, FFS on how the latter is informed to L2 U2N Remote UE.

The condition 3) is only applicable to L2 Intermediate Relay UE.

the legacy trigger conditions including 4) and 5) can be directly reused for L2 U2N Remote UE.

We think the condition 2) is only applicable for L2 U2N Relay UE. In addition, upon detection of any such event, the Relay UE would need to inform Remote UE of the event. RAN2 can further discuss the signaling details. In legacy, PC5-RRC signalling is exchanged between UEs in a per hop manner, which would not be applicable for the Relay UE which is not directly connected to Remote UE.

In addition, an L2 Intermediate Relay UE may detect PC5 RLF towards L2 U2N Relay UE, in this case, the Intermediate Relay UE can use PC5-RRC message to inform L2 U2N Remote UE of the detected PC5 RLF. Based on which, L2 U2N Remote UE may trigger relay reselection.

Therefore, we would like to make the below proposal.

Similar as in legacy, L2 U2N Remote UE may trigger relay reselection in following cases:

  - PC5 signal strength of current L2 Intermediate Relay UE or current L2 U2N last Relay UE is below a (pre)configured signal strength threshold. FFS on how the latter is informed to L2 U2N Remote UE.

  - Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by L2 U2N last Relay UE. FFS on signaling details.

  - When L2 U2N Remote UE receives a PC5-S link release message from L2 Intermediate Relay UE

  - When L2 U2N Remote UE detects PC5 RLF towards Intermediate Relay UE

  - PC5 RLF towards L2 U2N last Relay UE has been indicated by L2 Intermediate Relay UE

  - Indicated by upper layer.

In Rel-17 single hop U2N relay, the PLMN ID and cell ID can be used as additional AS criteria. Similarly, Remote UE may need to consider both PLMN ID and cell ID in the relay (re)selection procedure. In the procedure, Remote UE may first select a U2N Relay UE considering its PLMN ID and cell ID. After that, Remote UE selects one or multiple intermediate Relay UE by checking if they are authorized to access the cell of U2N Relay UE. Therefore, each Relay UE candidate can include the PLMN info and cell ID in discovery messages as a container (which is transmitted or forwarded). In legacy, the discovery message can already contain PLMN ID and cell ID for single Relay UE. For multiple U2N relay, the discovery message can be extended to include PLMN ID and cell ID for each Relay UE in the path. Upon reception of discovery messages/responses, Remote UE considers received PLMN info and cell ID info in relay (re)selection procedure.

Same as in legacy, PLMN info and cell ID info of each Relay UE may be considered as additional AS criteria in relay (re)selection.

Each Relay UE candidate can include PLMN info and cell ID in discovery messages (which is transmitted or forwarded).

In addition, in Solution #1(TR 23.700-03), SA2 has discussed how to use Hop-Count value in relay (re)selection procedure. In an example, Remote UE selects the Relay UE announcing the least Hop-Count value to build the path with shortest hop numbers. We think it is not necessary to disclose Hop-Count value into AS, due to below reasons

1) Hop-count is included in Discovery or DCR/DCA messages, which are not seen by AS by nature.

2) It is sufficient to categorize such criteria as Upper layer criteria, and leave them to be handled by Upper layers. Disclose them to AS may introduce double control/usage of these criteria at both Upper layer and AS layer, which increases additional complexity.

Therefore, we propose that

Hop-Count can be categorized as an upper layer criterion and left to be handled by upper layer, and therefore, no need to be disclosed to AS layer.

In Rel-18, U2U Relay UE may consider PC5 RSRP threshold when transmitting or forwarding discovery messages. We think such mechanisms can be reused by an Intermediate Relay UE. A UE can determine whether itself is capable to operate as an Intermediate Relay UE or a U2N Relay UE depending on e.g., whether the UE has direct connection to the network.

A UE can determine whether itself is capable to operate as an Intermediate Relay UE or a U2N Relay UE depending on e.g., whether the UE has direct connection to the network.

An Intermediate Relay UE can consider PC5 RSRP threshold when transmitting or forwarding discovery messages.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	A UE can determine whether itself is capable to operate as an Intermediate Relay UE or a U2N Relay UE depending on e.g., whether the UE has direct connection to the network.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Same as in legacy, support standalone discovery model (i.e., Model A and Model B discovery model) for multi-hop U2N relay.

Proposal 2	Reuse the same Uu RSRP thresholds as in the legacy (i.e., single hop L2 U2N relay) for L2 Remote UE and L2 U2N Relay UE in multi-hop U2N relay.

Proposal 3	Reuse the same discovery resource pool(s) and configurations as in the legacy (i.e., single hop L2 U2N relay) in multi-hop U2N relay.

Proposal 4	Supports the same trigger conditions of relay selection for L2 U2N Remote UE as in legacy.

Proposal 5	Similar as in legacy, L2 U2N Remote UE may trigger relay reselection in following cases:

1)	PC5 signal strength of current L2 Intermediate Relay UE or current L2 U2N last Relay UE is below a (pre)configured signal strength threshold. FFS on how the latter is informed to L2 U2N Remote UE.

2)	Cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure has been indicated by L2 U2N last Relay UE. FFS on signaling details.

3)	When L2 U2N Remote UE receives a PC5-S link release message from L2 Intermediate Relay UE

4)	When L2 U2N Remote UE detects PC5 RLF towards Intermediate Relay UE

5)	PC5 RLF towards L2 U2N last Relay UE has been indicated by L2 Intermediate Relay UE

6)	Indicated by upper layer.

Proposal 6	Same as in legacy, PLMN info and cell ID info of each Relay UE may be considered as additional AS criteria in relay (re)selection.

Proposal 7	Each Relay UE candidate can include PLMN info and cell ID in discovery messages (which is transmitted or forwarded).

Proposal 8	Hop-Count can be categorized as an upper layer criterion and left to be handled by upper layer, and therefore, no need to be disclosed to AS layer.

Proposal 9	An Intermediate Relay UE can consider PC5 RSRP threshold when transmitting or forwarding discovery messages.

# 4	References

[1] RP-241609, “New WID on NR sidelink multi-hop relay”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17 – 20 June 2024.