3GPP TSG-RAN WG2 #128	R2-2410151

Orlando, USA, 18th – 22nd November 2024

Agenda Item:	8.2.5

Source:	Ericsson

Title:	Topology 2 for Ambient IoT

Document for:	Discussion/Decision

# Introduction

Topology 2 considerations were discussed in RAN2#127 with some agreements below being made:

Agreements

- RAN2 is responsible for the interface between intermediate node (i.e. Reader) and RAN for topology 2.

- A-IoT air interface in topology 1 between A-IoT device and reader is fully reused in topology 2, i.e. topology is transparent to the A-IoT device and there is no impact on A-IoT device.

- RAN2 assumes that intermediate UE authorization is performed by upper layers.  RAN2 impact can be studied after further SA/RAN3 progress.

- RAN2 will study the impacts to RAN2 based on the SA2 identified architecture options (i.e. no new AS layer architecture options will be studied)

- NW is in control of resources to be used by A-IoT air interface.

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

And recent RAN2#127bis agreements

Agreements

- Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

- The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

- The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

This contribution discusses in detail some open aspects of A-IoT with a focus on Topology 2 considerations, DO-A assessment.

# 2	Discussion

## 2.1	Topology 2 aspects

### 2.1.1	Protocol stack options

Current TR38.769 captures three stack options to be studied, i.e., RRC based solution, NAS based solution, and UP based solution. RAN2 impact from the three options are different. The differences among the three options mainly depend on which sublayer of the Intermediate UE handles the A-IoT procedure. In other words, it is a matter of whether A-IoT data transfer is visible to serving gNB.

Note, that RAN3 also introduced an Editorial Note stating, that it needs to be studied for all these options it needs to be studied how (AIoT) radio resource control can be enabled.

RAN2 impact from Topology 2 depends on which sublayer of intermediate UE (RRC, NAS, or UP) handles A-IoT communication.

Excerpt from TR 38.769 §6.4.2.1:

- RRC based solution. With this solution, A-IoT CN applies A-IoT upper layer information explicitly over XXAP signaling. A-IoT upper layer information is then relayed by gNB explicitly to/from the A-IoT-enabled UE via NR Uu RRC.

- NAS based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted over A-IoT enabled UE's NAS.

- UP based solution. With this solution, there is no explicit termination of A-IoT upper layer information at A-IoT-enabled gNB. A-IoT upper layer information is transmitted as A-IoT-enabled UE's user plane data.

Editor’s note 1: How to enable radio resource control, i.e. trigger A-IoT RAN node functions for above solutions is FFS.

For UP based solution, the A-IoT data is delivered over PDU session user plane of the Intermediate UE, which are transparent towards NG-RAN. That is, relying on the exchange of A-IoT application level data at inventory and command alone does not allow the NG-RAN to control A-IoT radio resources. This is especially true if one considers a setup of A-IoT RAN as a network of multiple readers (irrespective of whether topology 1 or 2 is deployed) covering a large geographical area where AIoT service is provided, which calls for central point of coordination at the AIoT enabled gNB.

The observation above is also applicable to the NAS based solution where the A-IoT data transmission is only aware by the Intermediate UE and CN-CP (AMF/AIoTF). In both UP based and NAS based solutions, NG-RAN, however, needs to be aware of the A-IoT procedure with relevant control information in order to control radio resources used by A-IoT. The control information can be provided together with UE NAS messages in NAS based option. However, for UP based solution, how control information related to a A-IoT procedure can be provided to NG-RAN needs to be addressed, e.g., either by Intermediate UE or CN or both. This requires coordination with other WGs (RAN3, SA3).

For UP based solution and NAS based solution, exchange of A-IoT data between intermediate UE and CN is transparent towards NG-RAN. NG-RAN just needs to be aware of relevant control information of A-IoT procedure to control radio resources.

For UP based solution, control information related to A-IoT procedure can be provided to the serving NG-RAN either by intermediate UE or CN.

From a RAN2’s perspective, for UP based solution, one open aspect related to handling A-IoT data over Uu is whether it is necessary to treat A-IoT data differently from the legacy DRB data. For example, as detailed in section 2.1.3, there would need some changes in handling of temporary out of connection scenario for an ongoing A-IoT procedure compared to legacy NR, it would be beneficial to have a DRB dedicated for A-IoT data delivery. Details about L2/L1 functions and configurations for such a A-IoT DRB can be further studied in WI phase, if UP based solution is adopted.

For UP based solution, how to distinguish A-IoT data delivery from legacy DRB handling, e.g., how to map A-IoT data onto separate DRB(s), L2 functions is one RAN2 impact and captured in the TR.

We think NAS based solution has minimum RAN2 impact. This solution has mainly SA and CT impacts. We assume A-IoT data is transferred as a Intermediate UE NAS message which is piggybacked in an RRC message over a signaling radio bearer. A RAN2 question is whether a new SRB dedicated for A-IoT needs to be defined, for example, considering the possible different handling of this SRB data in case of e.g., temporary out of connection scenario. This question is relevant to RRC based solution as well.

For CP based solution (both NAS and RRC based), how to distinguish A-IoT data delivery over RRC signaling from legacy SRB handling, e.g., whether and how to define a new SRB for A-IoT data is one RAN2 impact and captured in the TR.

Between RRC based solution and NAS based solution, system architecture wise, there are no significant differences between them. We assume that from CN point of view, the only difference is on whether the device message delivery:

- In NAS based solution, the device message is embeded within AIOT UE NAS message. The protocol stack can be depicted as below:

Figure 1: Example of NAS based protocol stack

As one can see, this stack figure only shows the transport of AIoT application data via diverse NAS layers, but it does not show how AIoT RAN is triggered to performe AoT radio resource control, a detail to be studied by RAN3.

- In RRC based solution, the device message is delivered over NGAP towards NG-RAN, and then over RRC towards Intermediate UE. The protocol stack can be depicted as below:

Figure 2: Example of RRC based protocol stack

Comparing those two options, RRC based solution brings more impacts to NG-RAN, but results in less overhead from two aspects:

- AMF is not involved in communicating AIoT application data to the AIoT device (AMF could be involved when paging of the Intermediate UE if needed).

- Therefore no UE NAS overhead when delivering device message.

- Most likely alignment with topology 1.

From an SA2 and RAN3 point of view, it might look as if the RRC based solution is more efficient.

In addition, given that resource allocation is taken care by the RRC sublayer in legacy NR, we think RRC based protocol stack would naturally allow serving gNB to control radio resources to be used by A-IoT communication.

RRC based protocol stack option would naturally allow serving gNB to control radio resources to be used by A-IoT air interface.

Based on the above description, we have proposed a TP (in clause 5) on RAN2 impact due to different protocol stack options,

Adopt the TP on protocol stack options in clause 5.

### 2.1.2	UE reader selection and A-IoT resource allocation

It is already agreed in RAN2#127 that network is in control of resources to be used by A-IoT air interface. In addition to resource allocation, we think RAN2 should study how serving gNB control Intermediate UE(s) in terms of reader selection and handling the RRC state and temporary out of connection irrespective of protocol stack option. We discuss our views on some of the aspects in this and subsequent sections.

As for Intermediate UE (re)selection, we assume the UE is authorized to be a reader by CN however, RAN should be involved in selecting what Intermediate UEs to reach out for A-IoT communication based on some RAN metrics such as Uu channel quality, which RRC state the UE is currently in, or mobility status.

Serving gNB determines the intermediate UE readers.

As for radio resource allocation for AIoT services, there are some aspects to study, for example how (based on what) a serving gNB can allocate resources, how to define radio resources for A-IoT services, signaling for provisioning resources.

First, for radio resource allocation, serving gNB cannot allocate radio resource blindly. It needs to be aware of e.g., what type of upcoming/ongoing A-IoT Service Request (inventory or command or disabling), approximate number of target devices (to allocate radio resources accordingly), etc. It is beneficial for the Intermediate UE to provide some scheduling request/assistance information concerning the need of A-IoT resources to the serving gNB. Detailed assistance information can be further studied, including the resource parameters defined by RAN1 e.g., time, frequency, chip duration, transmission round, etc.

Serving gNB allocates radio resources for A-IoT based on control information associated with the service request provided by CN. In addition, intermediate UE may provide further assistance information to serving gNB for A-IoT resource allocation refinement.

Regarding resource structure and definition, A-IoT radio resource can be defined as Intermediate UE specific/dedicated resources or a common resource pool that multiple Intermediate UEs can share. Allocation granularity is another aspect to consider, i.e., serving gNB may allocate resource for every single transmission/message over A-IoT air interface, or every pair of transmissions, or for whole access/inventory round. Especially when the gNB schedules dedicated resources per transmission pair(s) or access round(s), the Intermediate UE would need to autonomously select resources for each R2D and D2R transmission among the dedicated resources. We think all levels of allocation granularity should be studied and captured in TR38.769.

Capture in the TR all the options for resource allocation granularity, i.e., per transmission, per D2R and R2D transmission pair, and per access round.

### 2.1.3	RRC state

An Intermediate UE can be in any RRC state, the gNB needs ensure that the Intermediate UE is in RRC_CONNECTED when performing A-IoT procedures. This is because A-IoT procedures such as inventory, command, disabling are expected to require relatively long-duration communication between Intermediate UE and serving gNB to accomplish, which is not suitable for Intermediate UE in RRC_IDLE/RRC_INACTIVE. In addition, in order for the gNB to assign resources to Intermediate UE for A-IoT traffic using the legacy scheduling procedure (e.g., dynamic scheduling or configured scheduling), Intermediate UE needs to be in RRC_CONNECTED. Hence, in case the Intermediate UE is in RRC_IDLE or RRC_INACTIVE, gNB needs to transition it to RRC_CONNECTED, e.g., by means of legacy paging.

In addition to RRC state handling, for radio resource management and coordination among Intermediate UEs, the legacy RRC signaling framework (interface between Intermediate UE and serving gNB) may need to incorporate new signaling procedures/messages e.g., to indicate and schedule a particular Intermediate UE to perform A-IoT procedures, or to avoid a particular Intermediate UE occupy air interface too long due to serving A-IoT devices.

A-IoT procedures are expected to require relatively long-duration communication between intermediate UE and serving gNB, which is not suitable for intermediate UE in RRC_IDLE/RRC_INACTIVE.

Intermediate UE stays in RRC_CONNECTED during an A-IoT procedure.

Same as in legacy, intermediate UE can be moved to RRC_IDLE or RRC_INACTIVE if there is no on-going A-IoT procedure.

### 2.1.4	Temporary out of connection

- Will support in-coverage and study cases for reader “temporarily” out of connection (e.g. RLF, HO).  May consider extendibility to future “temporarily” out of coverage case with full NW control of resources (if possible).

In our understanding, “Temporarily Out of Connection” in general means the connection to the serving cell becomes unstable or UE may lose/release RRC connection to serving cell soon but still stays in RRC connected. More specifically, it may include the following cases/events:

- The UE detects physical layer problems and the RLF timer (i.e., T310) is still running. In this case, the number of out of sync instances has reached a configured number (e.g., N310).

- The UE is performing (conditional) HO and the HO timer (e.g., T304) is still running.

- The UE is performing RRC reestablishment and the reestablishment timer (i.e., T301 or T311) is still running.

Given the UE reader is still in RRC_CONNECTED when temporarily out of connection occurs, RAN2 can study this case and develop corresponding solutions.

“Temporarily out of connection” in general means the connection to the serving cell becomes unstable or intermediate UE may lose/release RRC connection to serving cell soon but still stays in RRC connected.

Temporarily Out of Connection” means the connection to the serving cell becomes unstable or the intermediate UE may lose/release RRC connection to serving cell soon but still stays in RRC connected. Examples of temporarily out of connection include (but not limited to) following cases/events:

  - The intermediate UE detects physical layer problems and the RLF timer (i.e., T310) is still running. In this case, the number of out of sync instances has reached a configured number (e.g., N310).

  - The intermediate UE is performing (conditional) HO and the HO timer (e.g., T304) is still running.

  - The intermediate UE is performing RRC reestablishment and the reestablishment timer (i.e., T301 or T311) is still running.

When an intermediate UE detects temporary out of connection, its RRC connection to the serving gNB is not stable and even may be released. The Intermediate UE’s allocated resources for A-IoT data transmissions and receptions over legacy Uu interface and A-IoT interface are not available accordingly to the legacy, i.e., this is because usage of these resources may create interference to other UEs which are of decent connections to the gNB. Thus, there would be interruption for ongoing A-IoT procedure/transactions (if there is any) performed via the Intermediate UE.

It is reasonable to follow the legacy procedures to handle interruption. In legacy, when a UE experiences an interruption due to handover, or RLF, the UE stops using the dedicated resources since the connection is not in stable state. It would be beneficial to avoid generating interferences to transmissions of other UEs whose connections are in stable state. People may argue that SL V2X supports exceptional resources while in out of connection. However, that feature was designed to improve service continuity purpose, which is not applicable to A-IoT services, due to the fact that A-IoT services are not associated with QoS requirements.

In legacy, when a UE experiences an interruption due to handover, or RLF, the UE stops using the dedicated resources since the connection is not in stable state. It would be beneficial to avoid generating interferences to transmissions of other UEs whose connections are in stable state.

In legacy, SL V2X supports exceptional resources while in out of connection. However, that feature was designed to improve service continuity purpose, which is not applicable to A-IoT services, due to the fact that A-IoT services are not associated with QoS requirements.

Therefore, it is reasonable to follow the legacy rules that devices and the reader should stop using resources allocated for A-IoT services while temporary out of connection occurs for the intermediate UE. In this case, the previous allocated A-IoT resources are considered as invalid. Thus, any on-going A-IoT session/procedure should be suspended. A suspended A-IoT session/procedure can be resumed after the connection is restored, using the previous resources, or resources re-configured by the gNB. In case the connection cannot be restored, any suspended A-IoT session/procedure should be terminated. In addition, the intermediate UE releases all A-IoT resources allocated by the gNB, same as in legacy.

In addition, the intermediate UE indicates to target devices to perform relevant actions accordingly. For example, if the ongoing inventory procedure is terminated due to out of connection, the indication from the intermediate UE may instruct the devices which have already responded to the A-IoT paging message to reset their inventory status and consider inventory unsuccessful. It can be further studied later on (e.g., during WI phase) the indication can be common for different procedures.

In addition, we see no need to introduce any additional validity conditions for A-IoT resources based on the above analyses.

Same as in legacy, the resources allocated to an Intermediate UE are invalid when the intermediate UE experiences temporary out of connection.

On-going A-IoT sessions/procedures are suspended when the Intermediate UE experiences temporary out of connection.

Suspended A-IoT sessions/procedures can be resumed after the Intermediate UE has restored its connection.

Suspended A-IoT sessions/procedures are terminated if the Intermediate UE has failed to restore its connection.

The Intermediate UE releases resources for A-IoT allocated by the gNB if the Intermediate UE has failed to restore its connection.

Target AIoT devices are indicated/instructed how to behave when intermediate UE experiences temporary out of connection. FFS on signaling details and device behaviors.

For the below agreement made at RAN2#127bis,

The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios.    FFS on resource validity across multiple cells.

As we discussed in the above, we think it makes sense to fully follow the legacy rules. The resources allocated for the Intermediate UE before are considered as invalid while temporary out of connection occurs. In addition, regarding FFS on resource validity across multiple cells, we think it is unnecessary either to introduce any resource validity condition across multiple cells, due to below reasons

- Multiple cells/connectivity scenario of the Intermediate UE is not in the scope of the SI. RAN2 shall focus on the single connectivity scenario to complete the study.

- There is neither QoS requirement nor service continuity requirement defined in the scope of the SI.

A-IoT resources are valid when the Intermediate UE has a stable connection to the gNB, i.e., no temporary out of connection has occurred.

A-IoT Resource validity across multiple cells is not pursued.

In addition, we think further clarification is needed for the below RAN2 agreement

The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

It is of course true that the gNB can reconfigure the AIoT resources at any time when the connection of the Intermediate UE is stable, i.e., no radio failure is being detected. However, the time period during a handover procedure is typically considered as an interruption time for the UE, as in the legacy. If we strictly follow the legacy rules, during the handover interruption period, the resources allocated to the UE before the handover is considered as invalid. The UE cannot apply resources obtained from the target cell due to that the UE has no valid UL synchronization towards the target cell yet. The UE has no resources to use until the handover procedure is completed. Therefore, it is not correct for the Intermediate UE to apply A-IoT radio resources reconfigured by the network during HO procedure.

In legacy, the time period during the handover procedure is considered as an interruption time for UP data. The resources allocated to the UE before the handover is considered as invalid. The UE cannot apply resources obtained from the target cell due to that the UE has no valid UL synchronization towards the target cell yet.

Therefore, the above RAN2 agreement needs to be corrected.

Correct the RAN2 agreement as in below.

  - “The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc)”.

We think it is important to have network control over behavior of Intermediate UE and A-IoT devices during out of connection duration. First, network can select another Intermediate UE to serve A-IoT services, if available. In this case, serving Intermediate UE is indicated to stop serving A-IoT devices.

Network can select a new intermediate UE prior to occurrence of temporary out of connection.

Based on the above description, we have proposed a TP (in clause 5) on temporary out of connection,

Adopt the TP on temporary out of connection in clause 5.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 impact from Topology 2 depends on which sublayer of intermediate UE (RRC, NAS, or UP) handles A-IoT communication.

Observation 2	For UP based solution and NAS based solution, exchange of A-IoT data between intermediate UE and CN is transparent towards NG-RAN. NG-RAN just needs to be aware of relevant control information of A-IoT procedure to control radio resources.

Observation 3	For UP based solution, control information related to A-IoT procedure can be provided to the serving NG-RAN either by intermediate UE or CN.

Observation 4	RRC based protocol stack option would naturally allow serving gNB to control radio resources to be used by A-IoT air interface.

Observation 5	A-IoT procedures are expected to require relatively long-duration communication between intermediate UE and serving gNB, which is not suitable for intermediate UE in RRC_IDLE/RRC_INACTIVE.

Observation 6	Same as in legacy, intermediate UE can be moved to RRC_IDLE or RRC_INACTIVE if there is no on-going A-IoT procedure.

Observation 7	“Temporarily out of connection” in general means the connection to the serving cell becomes unstable or intermediate UE may lose/release RRC connection to serving cell soon but still stays in RRC connected.

Observation 8	In legacy, when a UE experiences an interruption due to handover, or RLF, the UE stops using the dedicated resources since the connection is not in stable state. It would be beneficial to avoid generating interferences to transmissions of other UEs whoseconnections are in stable state.

Observation 9	In legacy, SL V2X supports exceptional resources while in out of connection. However, that feature was designed to improve service continuity purpose, which is not applicable to A-IoT services, due to the fact that A-IoT services are not associated with oS requirements.

Observation 10	In legacy, the time period during the handover procedure is considered as an interruption time for UP data. The resources allocated to the UE before the handover is considered as invalid. The UE cannot apply resources obtained from the target cell due to that the UE has no valid UL synchronization towards the target cell yet.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For UP based solution, how to distinguish A-IoT data delivery from legacy DRB handling, e.g., how to map A-IoT data onto separate DRB(s), L2 functions is one RAN2 impact and captured in the TR.

Proposal 2	For CP based solution (both NAS and RRC based), how to distinguish A-IoT data delivery over RRC signaling from legacy SRB handling, e.g., whether and how to define a new SRB for A-IoT data is one RAN2 impact and captured in the TR.

Proposal 3	Adopt the TP on protocol stack options in clause 5.

Proposal 4	Serving gNB determines the intermediate UE readers.

Proposal 5	Serving gNB allocates radio resources for A-IoT based on control information associated with the service request provided by CN. In addition, intermediate UE may provide further assistance information to serving gNB for A-IoT resource allocation refinement.

Proposal 6	Capture in the TR all the options for resource allocation granularity, i.e., per transmission, per D2R and R2D transmission pair, and per access round.

Proposal 7	Intermediate UE stays in RRC_CONNECTED during an A-IoT procedure.

Proposal 8	Temporarily Out of Connection” means the connection to the serving cell becomes unstable or the intermediate UE may lose/release RRC connection to serving cell soon but still stays in RRC connected. Examples of temporarily out of connection include (but not limited to) following cases/events:

a.	The intermediate UE detects physical layer problems and the RLF timer (i.e., T310) is still running. In this case, the number of out of sync instances has reached a configured number (e.g., N310).

b.	The intermediate UE is performing (conditional) HO and the HO timer (e.g., T304) is still running.

c.	The intermediate UE is performing RRC reestablishment and the reestablishment timer (i.e., T301 or T311) is still running.

Proposal 9	Same as in legacy, the resources allocated to an Intermediate UE are invalid when the intermediate UE experiences temporary out of connection.

Proposal 10	On-going A-IoT sessions/procedures are suspended when the Intermediate UE experiences temporary out of connection.

Proposal 11	Suspended A-IoT sessions/procedures can be resumed after the Intermediate UE has restored its connection.

Proposal 12	Suspended A-IoT sessions/procedures are terminated if the Intermediate UE has failed to restore its connection.

Proposal 13	The Intermediate UE releases resources for A-IoT allocated by the gNB if the Intermediate UE has failed to restore its connection.

Proposal 14	Target AIoT devices are indicated/instructed how to behave when intermediate UE experiences temporary out of connection. FFS on signaling details and device behaviors.

Proposal 15	A-IoT resources are valid when the Intermediate UE has a stable connection to the gNB, i.e., no temporary out of connection has occurred.

Proposal 16	A-IoT Resource validity across multiple cells is not pursued.

Proposal 17	Correct the RAN2 agreement as in below.

a.	“The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc)”.

Proposal 18	Network can select a new intermediate UE prior to occurrence of temporary out of connection.

Proposal 19	Adopt the TP on temporary out of connection in clause 5.

# 4	References

RP-240826, “Revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, Huawei, T-Mobile USA, Maastricht, Netherlands, March 18-21, 2024

R3-244387, “On A-IoT RAN functional and logical architecture”, Ericsson, RAN3#125.

TR22.840, “Study on Ambient power-enabled Internet of Things”.

R2-2409812, “TP for TR 38.769 update”, Huawei, CMCC, T-Mobile USA, RAN2#128.

# 5	Text Proposal

In this clause, the proposed texts are captured in yellow based on the TP [4]. Changes cover only proposals reflecting controversial issues.

### 6.3.6	Topology 2 aspects on the interface between UE reader and RAN

For Topology 2, the architecture/protocol stack options in [R2-1] are studied (also corresponding to the studies in clause 6.4.2.1), while no new AS layer architecture/protocol stack options will be studied:

-	RRC based solution: A-IoT upper layer information is explicitly forwarded via NR Uu RRC message.

-	NAS based solution: A-IoT upper layer information is transmitted over UE reader's NAS message.

-	UP based solution: A-IoT upper layer information is transmitted as UE reader's user plane data.

It is assumed that the intermediate UE authorization is performed by upper layers, according to [R2-1] and [R2-2].

The radio resources used by A-IoT radio interface between the A-IoT device(s) and UE reader are controlled by the network. The radio resources, which are dedicated for a UE reader, are only configured to the UE reader via dedicated signalling. The mechanisms for shared resource pool amongst UE readers are not considered in this release.

The UE reader in coverage of BS scenario is supported. The UE reader may perform the A-IoT procedure on A-IoT radio interface between the reader and the device(s), only if the radio resource configuration is valid in the cell, which is under network control.

It can be further discussed how the UE reader determines this A-IoT radio resource validity in its temporary out of connection scenarios (e.g., RLF and handover cases). Temporarily out of connection means the RRC connection of the UE reader to the serving cell becomes unstable or the UE reader may lose/release RRC connection to serving cell soon but still stays in RRC_CONNECTED. Examples of temporarily out of connection include (but not limited to) following cases/events:

- The UE reader detects physical layer problems and the RLF timer (i.e., T310) is still running. In this case, the number of out of sync instances has reached a configured number (e.g., N310).

- The UE reader is performing (conditional) HO and the HO timer (e.g., T304) is still running.

- The UE reader is performing RRC reestablishment and the reestablishment timer (i.e., T301 or T311) is still running.

The A-IoT radio resource can be (re)configured by UE reader’s RRC reconfiguration (including during handover procedure, after re-establishment of the UE reader, etc.). It can be further discussed on the resource validity across multiple cells.

In case of temporary out of connection is detected by the UE reader, any on-going A-IoT session/procedure shall be suspended. The radio resources allocated for the UE reader is considered invalid during temporary out of connection period. In addition, resource validity across multiple cells is not supported.

A suspended A-IoT session/procedure can be resumed after the RRC connection of the UE reader is restored, using the previous resources, or resources re-configured by the gNB.

In case the RRC connection of the UE reader cannot be restored, all suspended A-IoT session(s)/procedure(s) shall be terminated. In addition, the UE reader releases all A-IoT resources allocated by the gNB.

Target A-IoT devices are indicated by the UE reader how to behave before suspension of the RRC connection.

For UP based solution, A-IoT data delivery needs to be distinguished from legacy DRB handling. A-IoT data is mapped to separate DRB(s) in Uu interface.

For CP based solution (both NAS and RRC based), A-IoT data delivery is delivered on a separate SRB from legacy SRBs.

Editor’s Note x: it is to further study whether to define a new SRB for A-IoT data forwarding between the UE reader and CN in case of CP based solution.

Editor’s Note xx: Signalling details on how the UE reader indicates suspension/resume/termination of A-IoT session(s)/procedure(s) to devices is FFS.

Editor’s Note xxx: device behaviours upon suspension/resume/termination of A-IoT session(s)/procedure(s) are FFS.