3GPP TSG-RAN WG2 Meeting #128	R2-2410153

Orlando, USA, 18th - 22nd November, 2024

Agenda Item:	 8.6.3

Source:	 Huawei, HiSilicon

Title:	 Discussion on L1 event-triggered measurement reporting

Document for:  Discussion and Decision

1	Introduction

LTM (L1/L2 Triggered Mobility) was introduced in Rel-18. The network receives L1 measurement report to determine the LTM target cell and target beam. Three types of report are defined, namely, periodic, aperiodic and semi-persistent L1 report.

For R19 mobility enhancements, one of the goals outlined in the WID is to define the required improvements for enabling event-triggered L1 measurements.

RP-242356, Revised Work Item: NR mobility enhancements Phase 4 - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1]

In this contribution, we present a detailed analysis of event-triggered L1 measurement reports. This includes the event triggering condition, the event reporting procedures and measurement and reporting of LTM events.

2	Discussion

2.1	Event triggering condition

For the previous RAN2 discussion, the following agreements have been reached.

Agreements on measurement enhancements for LTM 6. For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

In RAN2 #126 meeting, TTT mechanism should be applied for the LTM L1 event evaluation procedure.

Agreements on L1 MR event evaluation 2. Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. 3. Any beam in candidate RS configuration can be used for LTM event evaluation.

2.1.1	Beam in serving cell

At the last meeting, it was agreed that the beam of the serving cell is the beam corresponding to the indicated TCI state, referred to as the "current beam", which is used for event evaluation in L1 measurement reporting. The current beam may change due to the beam management processes so it is necessary to consider whether the TTT should be restarted when the current beam changes. We consider two options to address this:

- Option-1: If the current beam changes and the entering condition is still met with the new current beam, the TTT continues to run without being restarted.

- Option-2: If the current beam changes regardless whether the new current beam satisfied the entering condition or not, the TTT will be restarted.

The beam management process is expected to select the best beam, so the new current beam is better than the old current beam. The entering condition of the events for L1 event-triggered reporting is that the current beam is below some absolute or some relative level, so if such a condition is met with the new current beam, it is also met with the previous current beam. Restarting the TTT will postpone the event triggering time, resulting in a delayed reception of the measurement results by the network. Therefore, Option-1 is appropriate and there is no reason to restart the TTT like in Option-2.

Proposal 1: TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

2.1.2	Beam in the candidate cell

Similarly, we need to discuss whether the TTT should be restarted when the beam (any beam) used for LTM event evaluation changes. Two options are identified to address this:

Start the TTT

LTM resource set {SSB1, SSB2. SSB3}  √means the beam satisfies the event condition

SSB3√

SSB2√

SSB1√

SSB1√

- Option-1: If the beam (any beam) used for LTM event evaluation changes and the entering condition is still met with the new beam, the TTT continues to run without being restarted.

Restart the TTT

Start the TTT

LTM resource set {SSB1, SSB2. SSB3}  √means the beam satisfies the event condition

SSB2√

SSB2√

SSB1√

SSB1√

- Option-2: If the beam (any beam) used for LTM event evaluation changes, like the UE selects SSB1 for event evaluation and then selects SSB2 for that, the TTT is restarted.

Unlike the previous case, it is possible that the entering condition is not met anymore with the previous beam of the candidate cell. However, if the event would be triggered based on cell results and the number of beams used to derive the cell results would be 1, the TTT would continue running, so we think that it should be the same here i.e., Option-1.

Proposal 2: TTT timer should not be restarted if the beam in the candidate cell used for event evaluation changes and the new beam still meets the entering condition.

2.1.3	TTT maintenance

Agreements on L1 event triggered MR 9. For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

In the R19 LTM framework, it has been agreed that R18 LTM way is reused: an LTM resource set is associated with an LTM report Config and an event is often associated with an LTM report Config as well. There are also three options for the application of the TTT.

- Option-1: TTT operation can be per beam: When TTT expires, MR (measurement report) MAC CE is triggered for a certain beam

- Option-2: TTT operation can be per cell: When TTT expires, MR (measurement report) MAC CE is triggered for the cell

- Option-3: TTT operation can be per LTM report Config: when TTT expires, MR MAC CE is triggered for the report configuration

For example, if 64 beams are configured within the resource set and the TTT operates on a per-beam basis, this could result in 64 TTT processes running simultaneously. Such an approach would significantly increase the complexity for the UE, which may not be appropriate. Thus, Option-1 can be precluded. Besides, one advantage of option-3 is that for TTT per LTM report Config, the RS/beam can belong to either a single cell or multiple cells. This allowed for greater flexibility, as compared to applying TTT per cell. Additionally, in the legacy RRC event evaluation procedure, the TTT is configured per report Config. The TTT per report Config offers a more simplified design for specification, and it is also aligned with the L3 meas report since for L3 report, the event evaluation is performed per MeasID.

Proposal 3: TTT timer is maintained per report Config.

2.2	Event reporting procedures

2.2.1	Report contents

According to the agreements in last meeting, it has been agreed that the beam information, beam quality and triggered event information should be contained in the measurement report MAC CE.

Agreements on L1 event triggered MR 13. Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW.

2.1.1.1	Candidate cell beams

For beam information, a decision is required on whether to use SSBRI/CRI of candidate beams or the combination of (LTM configuration ID + SSB/CSI-RS ID) for beam indication information. If the latter approach (LTM configuration ID + SSB/CSI-RS ID) is selected, it needs specifying which of the eight LTM candidate cells are represented in the report. Besides, it also includes identifying the number of beams reported per candidate cell and the RS ID within each LTM candidate, resulting in a higher bit overhead compared to the use of SSBRI/CRI. Therefore, using SSBRI/CRI will save signalling overhead.

Observation 1: The use of SSBRI/CRI will save signalling overhead compared to the combination of (LTM configuration ID + SSB/CSI-RS ID).

Additionally, in legacy R18 LTM, the CSI reporting framework for SSB is shown below. In the last RAN1 meeting of LTM, it has also agreed that this reporting format can be extended to CSI-RS by replacing the SSBRI with CRI. Thus, choosing SSBRI/CRI to indicate the beam information is also aligned with RAN1’s agreement. Apart from that, we also need to discuss whether to introduce differential RSRP as implemented in legacy Rel-18 LTM.

In order to align with R18 LTM, it will be beneficial to reuse the reporting format as much as possible. We propose that RAN2 agree the differential reporting format for the measurement report MAC CE, consistent with the approach outlined above. In addition, the report can include maximum of 4 SSB/CSI-RSs per LTM candidate cell(M) and maximum pf 4 LTM candidate cells(L).

Proposal 4: Include the following contents in the measurement report MAC CE:

- SSBRI/CRI of N beams.

- Differential reporting format.

- Maximum number of beams to be reported (16): L cells x M beams, where L is configured by RRC parameter nrOfReportedCells-r18 (4) and M is configured by nrOfReportedRS-PerCell-r18 (4).

In the previous meeting, it was agreed that the MR MAC CE could support reporting up to N beams, with FFS on whether the reported beam should satisfy the event condition or not. For these N beams, where the maximum value of N is set to 16, two options are under consideration: reporting both candidate and serving beams, or reporting only candidate beams. A separate analysis is required for each option.

- For example, if the NW configures the UE to report up to N beams, where N=L ×M as specified above, and the number of beams that meet the event condition is K. Reporting up to N beams implies that the UE should report at maximum of L cells, with up to M beams per cell. There are two cases here:Case-1: If K ≥ N, UE should report N beams. This scenario poses no issues, as N serves as an upper limit on the number of reported beams.

- Case-2: If K < N, there is a question of whether UE still need to report N beams or only K beams. If UE still need to report N beams, the MR MAC CE will be generated with fixed size, which seems to lose the flexibility benefit of using MAC CE.. However, if UE only report K beams (those that meet the event condition), will reduce the size of MAC CE compared with previous method, and thereby enhancing signalling efficiency. This approach takes advantage of the flexibility of MAC CE, optimizing signalling overhead by adjusting the report size according to the number of event-satisfied beams.

Therefore, MR MAC CE should only report beam which satisfy the event condition.

Proposal 5: Measurement Report MAC CE can include up to K beams which should satisfy the event, where K is configurable by NW.

2.1.1.2	Serving beams

For serving cell scenario, it is essential to discuss whether the serving cell should be included in the measurement report. In legacy R18 LTM, the serving cell can be added into a candidate configuration for subsequent cell switch. In such cases, the network can configure the UE to measure the beams of the serving cell and include them in the event-triggered L1 measurement report. This approach aligns with the existing R18 LTM configuration and offers no apparent drawbacks, thus supporting to reuse it. The maximum number of beams that can be reported per cell is 4, hence, up to four beams from the serving cell should be included in the report.

Proposal 6: Multiple serving beams can be reported in the measurement report MAC CE.

2.2.2	Transmitted MAC CE

According the agreements in previous meeting, RAN2 has already agreed to use MAC CE. The LTM MR can be applied to both SSB and CSI-RS in multiple candidate cells, thus it is essential to discuss the details of the reporting format for the MAC CE at first. Since the UL grant may be limited, it may not be possible to include the entire set of measurement reports within a single UL grant. Therefore, a part of the reports can be transmitted within the current UL grant while the remainder can be deferred to a subsequent UL grant. Thus, it would be beneficial to consider the introduction of a truncated MAC CE, which would allow for measurement reports can be transmitted separately across different MAC CE.

Proposal 7: Support the truncated measurement report MAC CE.

Moreover, it is feasible that multiple event triggering conditions are met within the period of TTT simultaneously. In such cases, generating a separate measurement report for each event will lead to frequent triggering of SR for MR MAC CE.. Therefore, only one MAC CE should be transmitted at this time. This measurement report can carry all measurement results and triggering event information to save the signalling overhead.

Proposal 8: When multiple events are triggered simultaneously, they can be reported within a single MR MAC CE.

When the UE sends a measurement report to the gNB triggered by an event, there may be cases where the gNB does not make a handover decision after receiving the report. In such scenarios, the conditions that triggered the measurement report (MR) may persist, leading to the transmission of reports with identical measurement results frequently. For instance, for example, if a resource set is configured to trigger the LTM3 event at a high threshold value, and then and the same resource set triggers the LTM3 event at a lower threshold a bit later, the MR may contain the same RS with the same RSRP results.

In the current framework, this situation can result in the transmission of two MRs with the same contents, which do not provide any additional information to the network but contribute to unnecessary signalling overhead. Therefore, we should introduce a new prohibit timer to prevent the frequent transmission of the MR MAC CE.

MR MAC CE

MR MAC CE

Prohibit timer

Proposal 9: Introduce a prohibit timer for the transmission of the measurement report MAC CE.

2.3	Measurement and reporting of LTM events

Based on previous meetings, LTM2~LTM5 has been agreed so far, while RAN2 has not agreed LTM1 with the main justification that LTM1 may not be necessary for LTM. In legacy L3 measurement, the A1 event was approved for use in legacy L3 measurements, particularly in conjunction with the A2 event. For instance, if the serving cell signal is not good enough, which prompts the network to consider a handover, this event justifies the introduction of the A2. The A1 event is used to inform the network that the serving cell quality is sufficient, thereby indicating that a handover is unnecessary. Consequently, introducing A1 can help prevent the network from making unnecessary handover decisions and for the UE to stop the unnecessary inter-frequency measurement, which interrupts UE’s services. The advantage of event-triggered lower layer report is also beneficial since the MR MAC CE can be transmitted more quickly than the higher layer measurement report by RRC. Hence, we think that  similar situation applies in LTM, where support for both LTM2 and LTM1 is recommended to achieve benefits.

Proposal 10: RAN2 should agree on supporting the event LTM1.

# 3	Conclusion

In this paper, we have addressed the objective regarding improvements in event-triggering reports for LTM. There are the following proposals:

Event triggering condition

Proposal 1: TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

Proposal 2: TTT timer should not be restarted if the beam in the candidate cell used for event evaluation changes and the new beam still meets the entering condition.

Proposal 3: TTT timer is maintained per report Config.

Event reporting procedures

Proposal 4: Include the following contents in the measurement report MAC CE:

- SSBRI/CRI of N beams.

- Differential reporting format.

- Maximum number of beams to be reported (16): L cells x M beams, where L is configured by RRC parameter nrOfReportedCells-r18 (4) and M is configured by nrOfReportedRS-PerCell-r18 (4).

Proposal 5: Measurement Report MAC CE can include up to K beams which should satisfy the event, where K is configurable by NW.

Proposal 6: Multiple serving beams can be reported in the measurement report MAC CE.

Proposal 7: Support the truncated measurement report MAC CE.

Proposal 8: When multiple events are triggered simultaneously, they can be reported within a single MR MAC CE.

Proposal 9: Introduce a prohibit timer for the transmission of the measurement report MAC CE.

LTM event type

Proposal 10: RAN2 should agree on supporting the event LTM1.

# 4	Reference

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4;

- RAN2 #127bis chairman notes;

- TS 3GPP 38.331 V18.3.0

- TS 3GPP 38.212 V18.3.0