3GPP TSG-RAN WG2 Meeting #128	R2-2410155

Orlando, USA, 18th Nov – 22nd, 2024

Agenda Item:	8.7.5

Source:	Huawei, HiSilicon

Title:	Discussion on RLC AM enhancements

Document for: Discussion and Decision

# 1	Introduction

In the Rel-19 XR WID approved in RAN#103 [1], one of the objectives is about the RLC re-transmission enhancements for RLC AM. In this contribution, we will firstly discuss how to avoid unnecessary RLC retransmission, and then provide our views on the timely RLC retransmission.

# 2	Discussion on avoiding unnecessary transmission

From RAN2#127-bis agreements [2], to avoid unnecessary RLC AM retransmission for the outdated RLC SDU (segment), RAN2 agreed to adopt the combined approach to stop the transmission of unnecessary RLC SDUs, as shown below:

- RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

## 2.1	Maintenance of Local Timer for Rx RLC entity

In such approach, the Rx RLC entity can stop receiving the RLC SDU (segment) based on a local timer, which can be maintained per RLC SDU or per Rx RLC entity. To achieve this, we believe that at least two possible timer based options have been observed:

- 2.1.1 Option 1 Timer maintained for each RLC SDU

When there is an RLC SN gap, e.g. RX_Next_Highest > RX_Next + 1, the Rx RLC entity can start a Timer for each missing RLC SDU.  Upon the expiry of the Timer, the Rx RLC entity considers the associated missing RLC SDU as outdated and updates the receiving window and the Status Report accordingly.

Figure 1 gives the example for local timer maintained for each RLC SDU. In Figure 1, the RLC SDUs labelled with SN = 1,2,3 and 4 are transmitted, where the RLC SDUs of SN = 1, 2 and 3 are not successfully received. As a consequence, the resultant RX_Next_Highest > RX_Next + 1 allows the Rx RLC entity to identify the missing RLC SDUs, hence starting their timers, respectively. Moreover, after receiving the RLC Status Report, the Tx RLC entity performs retransmission of RLC SDUs of SN = 1, 2 and 3 together with initial transmission of RLC SDUs of SN = 5 and 6 to the Rx RLC entity, where the RLC SDUs of SN = 2 and 6 are successfully received, the RLC SDUs of SN = 1 and 3 are still received incompletely, and the RLC SDU of SN = 5 is missing. Then, the Rx RLC entity stops the timer of RLC SDU of SN = 2, and starts the timer of RLC SDU of SN = 5. After a while, the timer of RLC SDUs with SN = 1 and 3 are expired, the Rx RLC entity will stop waiting for RLC SDUs of SN = 1 and 3, and may update the variable RX_Next accordingly.

Figure 1 Timer maintained per RLC SDU for the Rx initiated RLC AM retransmission enhancement

- 2.1.2 Option 2 Timer maintained for the Rx RLC entity

When there is an RLC SN gap, e.g. RX_Next_Highest > RX_Next + 1, the Rx RLC entity can start a Timer.  Upon the expiry of the Timer, the Rx RLC entity considers the missing RLC SDU(s) whose SN falls within RX_Next and RX_T as outdated and updates the receiving window and the Status Report accordingly, where RX_T can be the variable to record the SN triggering the Timer.

Figure 2 illustrates example for a single local timer maintained for the Rx RLC entity. Instead of maintaining a timer for each RLC SDU, a timer similar to t-reassembly, referred as T below, can be maintained for the Rx RLC entity. Meanwhile, a new variable RX_T can be introduced at the Rx RLC entity to determine the outdated RLC SDUs. When the timer T expires, the Rx RLC entity will consider the unsuccessfully received RLC SDUs with SN between RX_Next and RX_T as outdated, and then updates RX_Next and RX_T accordingly.

For example, the RLC SDUs with SN = 1,2,3 and 4 are transmitted, where the RLC SDUs of SN = 1, 2 and 3 are not successfully received. The resultant SN gap drives the Rx RLC entity to start the new timer T and meanwhile to set the new variable RX_T to RX_Next_Highest, i.e. SN = 5, as shown in Figure 2 (a).  Moreover, after receiving the RLC Status Report, the Tx RLC entity performs retransmission of RLC SDUs of SN = 1, 2 and 3 together with initial transmission of RLC SDUs of SN = 5 and 6 to the Rx RLC entity, where the RLC SDUs of SN = 2 and 6 are successfully received, and the RLC SDUs of SN = 1, 3 and 5 are unsuccessfully received, as shown in Figure 2(b). After a while, as the timer T is expired, the Rx RLC entity considers the RLC SDUs with SN between RX_Next and RX_T, i.e. SN = 1 and 3, as outdated, and hence stops receiving their RLC SDUs. As the consequence depicted in Figure 2(c), the Rx RLC entity updates RX_Next to SN = 5. Since there remains the SN gap, i.e. RX_Next_Highest > RX_Next + 1, the timer will be started again, and RX_T is set to SN = 7.



(a)

(b)

(c)

Figure 2 Timer maintained per entity for the Rx initiated RLC AM retransmission enhancement

It is evident that Option 1 provides a more accurate discarding in comparison to Option 2, at the expense of more processing power utilization. As there can be hundreds of PDUs within a PDU set, the processing cost for maintaining these timers will be significant. However, we believe that considering the unaffordable processing cost at the Rx RLC entity, the benefit of such precise discarding offered by Option 1 is marginal. Since the PDUs within a PDU set are expected to be transmitted within the same PSDB, a single timer with maximum running time larger than the PSDB can avoid inaccurate discarding. Moreover, since the Tx RLC entity can stop retransmitting the outdated RLC SDUs, there will be no capacity loss even if the Rx RLC entity keeps sending NACK for the RLC SDUs whose remaining time for transmission has run out.

Proposal 1:	When there is an RLC SN gap, e.g. RX_Next < RX_Next_Highest + 1, the Rx RLC entity starts a new timer.

- 2.1.3 UE procedure upon local timer expiry

Moreover, RAN2 # 127-bis [2] also confirms that the receiving window will be updated before the transmitting window, as given below:

- RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does.

To update the reassembly window in advance to update the transmitting window, we believe that the RLC entity should be able to update the reassembly window once it decides to abandon the missing RLC SDU (s), e.g. update the variable RX_Next to the SN of the first RLC SDU with SN >= RX_Next for which RLC SDU is not abandoned and not all bytes have been received.

Proposal 2:	When the timer expires, the Rx RLC entity should update the reassembly window accordingly, e.g. set RX_Next to the SN of the first RLC SDU with SN >= RX_Next for which RLC SDU is not abandoned and not all bytes have been received.

Furthermore, another issue is on the reuse of the Status Report by the following RAN2 agreement:

- Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified

With the introduction of a local timer and the agreement above on the transmission of Status Report from the Rx RLC entity to the Tx RLC entity, we propose the following on the triggering of RLC Status Report.

Proposal 3: When the local timer expires, RLC Status Report is triggered.

## 2.2	Reliability Deterioration of Enhanced RLC AM

- 2.2.1 Discarding of Important XR Data

In the combined approach, the Tx RLC entity has the knowledge of the outdated RLC SDU based on their remaining time and hence determines whether to stop the retransmission. However, according to TS 26.522[3], the RLC SDU should not be randomly discarded, as given below:

In general, whenever the RAN needs to discard packets (e.g., under congestion situations), it is better to discard packets of lower importance rather than discarding packets randomly. If a discarded packet is critical for the media stream, the QoE may be severely degraded. For this reason, the PDU Set Importance (PSI) field can be used to mark PDU Sets with their importance level. The PSI field can then be used by the RAN to discard PDU sets. The PSI field can then be used by the RAN to discard PDU sets. In case of congestion, PDU Sets with higher PSI values are more likely to be discarded. PDU Sets that contain audio data should be assigned a lower PSI value (i.e., higher importance) compared with PDU Sets that contain other media types. PDU Sets that contains the reference frames present in the video bitstream should be assigned a lower PSI value compared with PDU Sets that contain non-reference frames.

In other words, we can conclude that some frames can significantly contribute to the decoding of their successive frames that have dependency on them. In such case, their successive frames cannot be successfully decoded and displayed even if they are transmitted correctly. Since the local timer starts later than the time point of actual transmission and it usually is configured with sufficient value for avoiding mistaken discarding, there remain chances for retransmission of the high importance RLC SDU, which is capable of securing the reliability for the entire XR traffic. Therefore, we believe that it is beneficial that the Tx RLC entity keeps the retransmission of the high PSI RLC SDU (segment) regardless of its remaining time, until its associated acknowledgement is received.

Observation 1: Discarding critical packets for the media stream will severely degrade the QoE.

Proposal 4: Tx RLC entity can stop retransmissions of an outdated RLC SDU according to its importance (e.g. stop retransmission of low importance RLC SDU).

- 2.2.2 Discarding of PDCP control PDU

As the Rx RLC entity has no knowledge of outdated RLC SDU to be abandoned, the PDCP control PDU may be discarded mistakenly. However, based on our analysis, the majority of PDCP control PDUs, e.g. ROHC feedback, PDCP SN gap report, can be applied with the UM DRB, which means that they are robust to the transmission loss. For example, the PDCP receiving entity can update the receiving window based on t-reordering timer if the PDCP SN gap report is abandoned by the Rx RLC entity. Therefore, we believe that the loss of PDCP control PDU will not be an issue in the enhanced RLC AM configured with the retransmission termination mechanism.

Proposal 5:  No enhancement is needed to handle missing PDCP control PDUs for avoiding unnecessary retransmission.

## 2.3	Indication from Tx to Rx

In the combined approach, when the Tx RLC entity decides to stop retransmitting the outdated SDUs, it can maintain the transmitting window unchanged until the indication, e.g. Status Report, sent from the Rx RLC entity is received.

Instead of endlessly waiting for the retransmission in legacy, the enhanced Rx RLC entity is able to stop receiving the outdated RLC SDU and send the associated report to the Tx RLC entity before the RLC SDU is literally received. Thus, the transmitting window can be updated more promptly in comparison to the legacy Rx RLC entity, and the latency of window update should be therefore mitigated.

Moreover, as confirmed in RAN2 127-bis [3] below, the receiving window will advance before the transmitting window updates, from our analysis, the window update procedure can be maintained as similar to the legacy window update procedure, and there is no out-of-synchronization between the two windows.

- RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does.

By contrast, the benefit of signalling introduced that may enable a faster receiving window update remains unclear. Therefore, we believe that introducing the indication from the Tx RLC entity to Rx RLC entity when Tx RLC stops the retransmission brings no benefit.

Proposal 6: RAN2 not to introduce the indication from Tx to Rx RLC entity when the Tx side stops the transmissions of an outdated SDU.

# 3. Discussion on timely RLC retransmission enhancement

An agreement on analysing solutions to ensure timely RLC retransmission has been reached in RAN2 127-bis [2] meeting, as given below:

- Exclude enhanced status reporting. - Focus the discussion on autonomous retransmission and polling enhancements, e.g. we need to understand how each option affects the capacity and packet delay

The major motivation on timely RLC retransmission is to reduce RLC ARQ retransmission latency, and there are two remaining options:

- Alt1 Autonomous retransmission is to offer the Tx RLC entity a sort of capability to autonomously retransmit an RLC SDU (segment) even if the corresponding NACK has not been received. To be more specific, to enable autonomous RLC retransmission, an RLC SDU (segment) can be considered for retransmission when the specific condition(s) is met. For example, the corresponding remaining time drops below a threshold before the associated feedback is received, as shown in Figure 3 (a). Alternatively, autonomous RLC retransmission can be triggered when the associated feedback is not received at Tx RLC entity within a duration since its last transmission, as illustrated in Figure 3 (b). Moreover, the autonomous retransmission can be limited to be triggered based on the channel quality, e.g. RSRP, where the RLC SDU (segment) can be retransmitted when the channel quality is deteriorated, to ensure the transmission reliability of the associated RLC SDU.

(a) Retransmitting SDU#1 when its remaining time below a threshold and no associated feedback is received



(b)  Retransmitting SDU#1 when no any associated feedback received within a certain period Figure 3 potential methods for autonomous RLC retransmission

- Alt2 Enhanced polling based retransmission tends to reuse the existing Status Report based RLC retransmission mechanism, but applies a more proactive mechanism to request a frequent Status Report to enable the RLC retransmission latency reduction. One potential enhancement is to consider the remaining time of the data as an additional factor for polling triggering. For example, when there is delay-critical RLC SDU, or when the data volume of the delay-critical SDUs exceeds a threshold, the polling can be triggered. Upon the reception of the polling, the Rx RLC entity can trigger Status Report. Similar as above, the frequent polling needs to work together with a shorter t-StatusProhibit.

Alt2 intends to request a fast Status Report and hence an in-time RLC retransmission based on the Status Report. To reduce the latency of polling triggering, the remaining time based new triggering condition was proposed. In such solution, the polling can be triggered once there is presence of delay-urgent data, e.g. remaining time of an RLC SDU becomes relatively low. However, it is well acknowledged that there remains the delay between the time point of polling triggering and its transmission, as there can be a scarcity of the grant resource.  Moreover, in addition to the triggering introduced latency, we believe that the latency imposed by the polling based solution can also be affected by the Status Report triggering procedure after the polling is received. For example, the Status Report may be delayed to avoid including premature NACK in the Status Report. As a result, the resource used for conveying polling SDU may be waste and hence negatively impact the system capacity. Additionally, we believe that an appropriate configuration of the polling triggering condition, e.g. based on pollByte/pollPDU, is able to ensure that the polling can be triggered and transmitted timely. Therefore, from our analysis, enhancing polling may bring no evident benefits for timely RLC retransmission.

Observation 2: There exists the evident latency between polling triggering and Status Report reception.

Based on the observation, there is no need to discuss polling based enhancements.

Proposal 7: RAN2 does not specify enhanced polling-based mechanism for the timely RLC retransmission, as network implementation can achieve the same effect.

As discussed above, the RLC retransmission latency can be reduced by configuring an appropriate value for the parameters relevant to the Status Report triggering. However, the Status Report based retransmission is not one hundred percent reliable, and the Tx RLC entity cannot retransmit the RLC SDU (segment) until the Status Report is received, even if it has the limited remaining time. For example, the Status Report that has been sent may experience numerous HARQ retransmissions before it is received. Moreover, the RLC SDU (segment) determined to be autonomously retransmitted can be sent out immediately as long as there is available resource, significantly reducing the retransmission latency. As for the concern on the potential capacity degradation, we believe that the appropriate configuration of the parameter values related to the Status Report can avoid majority of resource waste, where only few RLC SDUs (segment) demand for autonomous retransmission, which enables Alt1 to be a complementary solution to achieve the timely RLC retransmission. Additionally, the capacity loss can be further reduced by considering the time point of last (re)transmission of the RLC SDU (segment), as illustrated in Figure 3 (b), allowing the RLC SDU to experience a complete HARQ procedure. In such scenario, a timer can be introduced and started once an RLC SDU (segment) is transmitted. The autonomous retransmission can be triggered upon the expiry of the timer and the remaining time of the RLC SDU, which avoids the potential redundant retransmission between the HARQ and ARQ.

Observation 3: The capacity loss imposed by the autonomous retransmission can be mitigated if it is performed after its complete HARQ procedure is experienced.

Alternatively, the channel quality, e.g. RSRP, based autonomous retransmission triggering is beneficial to reduce the capacity loss, where the RLC SDU (segment) can be retransmitted when the channel quality is deteriorated, to ensure the transmission reliability of the associated RLC SDU.

Observation 4: The capacity loss imposed by the autonomous retransmission can be mitigated if there is degradation in channel quality.

Proposal 8:	RAN2 works on the autonomous RLC retransmission, where the capacity loss can be mitigated by introducing a transmission gap or considering the channel quality.

# 4. Discussion on SA2 LS on AL-FEC awareness

SA2 has investigated on AL-FEC awareness during the previous meetings. In S2-2410999 [4], SA2 introduced the latest progress and further ask RAN2 to provide some feedback on the feasibility and applicability for DL discard based on the FEC content ratio.

As introduced in S2-2410999, the potential RAN impacts for AL-FEC is that, the SMF can provide the content ratio to NG-RAN when establishing/modifying a DL QoS flow, which is defined as the minimum ratio of PDUs of a PDU Set in order for the UE to recover the PDU set content. NG-RAN may use the content ratio information for DL discarding in case of congestion. The SA2 agreements and principles for AL-FEC awareness are excerpted below.

Normative work for AL-FEC awareness at NG-RAN will be based on the following principles: - When requesting or updating QoS for a DL flow, an AF may provide to PCF/NEF - together with PDU Set QoS parameters - the content ratio for the flow. An AF may provide either PSIHI or content ratio for a flow. - If a PCF receives the content ratio from an AF, then the PCF may include the content ratio in PCC rules that it provides to the SMF. - The SMF provides content ratio to NG-RAN when establishing/modifying a QoS flow. - If NG-RAN supports PDU Set content ratio awareness and has received content ratio information for a QoS flow, then NG-RAN may consider content ratio information as follows: - If NG-RAN needs to discard PDUs of the QoS flow during congestion, then NG-RAN may use the content ratio information for discarding. NOTE 1: The principles concluded for AL-FEC awareness at RAN (as above) is subject to RAN-level enhancements of RLC Acknowledged Mode (RLC AM) for XR as per RP-240791 and further feasibility analyses and feedback from RAN WG2. RLC UM is not intended to be supported in this release due to concerns shared so far from RAN WG2 (see R2-2405781), pending further status update from RAN WG2. NOTE 2: RAN can, e.g. primarily discard obsolete AL-FEC PDUs of each PDU Set. NOTE 3: There is no difference in terms of how many PDUs NG-RAN will be discarded for a given QoS flow. In other words, NG-RAN will not discard more PDUs if content ratio information is provided for a QoS flow compared to when no content ratio information is provided for the same QoS flow. This scheme merely allows NG-RAN to select the PDUs to be discarded to ensure that the original content can still be recovered at the receiver. NOTE 4: Since there is no difference how many PDUs will be discarded, there are no impacts to existing congestion control algorithms.

Based on the principles, SA2 asks RAN2 to answer the following two questions:

Related to this agreement, SA2 would like to ask the following questions: Question 1: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC AM mode based on the above principles. Please also provide feedback on applicability of RLC AM for XR services, also considering the RAN-level enhancements of RLC Acknowledged Mode (RLC AM) with small packet delay budget as per RP-240791. With respect to RLC UM, RAN indicated in R2-2405781 that "RAN2 does not think it is possible for NG-RAN to determine reliably whether a PDU has been successfully delivered over an unacknowledged-mode data bearer." After sending the previous LS in S2-2405625, SA2 clarified the principles for potential use of content ratio information for discarding of PDUs during congestion as listed above. Question 2: Please provide feedback on feasibility of use of content ratio information for discarding of DL PDUs during congestion for RLC UM based on the above principles.

  - gNB procedure for PDU discard with FEC

It can be seen that question 1 asks about the feasibility and applicability for RLC AM mode, while question 2 concerns RLC UM mode. Before answering the questions, we think RAN2 should first consider the potential RAN solutions for FEC-based discard since the questions are related to the solution. From our point of view, there are two alternatives for FEC-based discard at NG-RAN, as listed below.

- Alternative 1: The gNB discards the remaining PDUs of the PDU set after it determines that the ratio of the successfully delivered PDUs is above the threshold.

The solution is developed based on the definition of the content ratio. As SA2 explained, the UE is able to reconstruct the original content when it receives the ratio of PDUs of the PDU set, which means the other PDUs of the PDU set are not needed by the application. If there are remaining PDUs waiting for transmission when the gNB finds that UE has already received enough PDUs, the gNB can discard the remaining PDUs to save the network resource.

- Alternative 2: The gNB discards PDUs of the PDU set according to the content ratio without confirming the successful delivery of the PDUs.

According to SA2’s principles, DL FEC-based discard is used during congestion, so another idea similar to R18 PSI-based discard can be considered. With the content ratio provided, the gNB is aware that there are partial PDUs redundant for the application. When the network is congested, the gNB can proactively discard the redundant PDUs to leave more resource for the transmission of the necessary PDUs (similar to discarding of low importance PDUs). Based on the congestion level, the gNB can decide to discard less or more redundant PDUs, which is aligned to the NOTE 3 listed in SA2’s principles.

Proposal 9: RAN2 reply to SA2 there are two potential directions to enhance DL discard at RAN based on the FEC content ratio information.

      - Alternative 1: The gNB discards the remaining PDUs of the PDU set after it determines that the ratio of the successfully delivered PDUs is above the threshold.

      - Alternative 2: The gNB discards PDUs of the PDU set according to the content ratio without confirming the successful delivery of the PDUs.

  - Analysis on the two alternatives

Considering the above two directions for FEC-based discard, RAN2 may evaluate the questions asked by SA2 differently. We provide our analysis on both directions as following.

- Analysis on alternative 1

For alternative 1, the key point is whether and how the gNB can determine the successful delivery of enough PDUs in the PDU set. For RLC AM mode, the determination can be directly performed based on the RLC status report from the UE. When there are sufficient PDUs been acknowledged for a PDU set, the gNB can discard the remaining PDUs. For RLC UM, things are a bit more complex since there is no status report. Previously, RAN2 has agreed that gNB cannot reliably determine the whether transmission of a PDU mapped to UM DRB is successful or not. The gNB can potentially derive this information based on the HARQ feedback, but this will cause additional implementation complexity as the gNB will have to remember the mapping between the PDUs and the TBs.

Observation 35: For both RLC AM and RLC UM, the gNB can determine whether enough PDUs been successfully delivered to the UE for a PDU set.

However, determining the successful delivery of a PDU based on the ARQ or HARQ feedback requires a period of time. For the ARQ process, the transmission of the status report is limited by t-StatusProhibit and t-reassembly, while for the HARQ process, the feedback is received at least after the RTT time. It may be not timely for the gNB to perform DL discard with the purpose of saving resource.  Considering the stringent delay budget for DL XR services, it is very possible that all the PDUs or most of the PDUs in the PDU set are transmitted before the successful delivery of sufficient PDUs is identified, which means the benefit of discarding is quite limited. For example, assuming there are 100 PDUs within the PDU set, and the content ratio is 80%. When the network is congested, the gNB may want to discard 20 redundant PDUs for the PDU set. But it has to confirm the successful delivery of at least 80 PDUs first. After that, there may be only 5 PDUs haven’t been transmitted by the gNB, and discarding the remaining PDUs is not enough to mitigate the congestion.

Observation 46: For alternative 1, the benefit is limited since the gNB may not be able to determine the successful delivery of sufficient PDUs for a PDU set timely.

Proposal 10: RAN2 reply to SA2 the use of content ratio information for discarding of DL PDUs based on alternative 1 is feasible, but the benefit is limited since the gNB may not be able to determine the successful delivery of sufficient PDUs for a PDU set timely for both RLC AM and RLC UM.

- Analysis on alternative 2

Considering the disadvantages of direction 1, direction 2 provides a more simple and useful solution. The gNB does not need to identify whether the UE has received enough PDUs to reconstruct the original content for the PDU set before discarding PDUs of the PDU set. Similar to R18 PSI-based discard, the gNB considers part of the PDUs within the PDU set as “low important” in case of congestion since the UE is still possible to reconstruct the content without these PDUs. To save the resource for mitigating the congestion, the gNB can discard those unnecessary PDUs proactively, which can be decided based on the FEC content ratio and the congestion level. For example, when there are 100 PDUs within the PDU set and the content ratio is 80%, the gNB may at most discard 20 PDUs proactively. If the congestion is not that heavy, less PDUs can be discarded with the principle mentioned by SA2 in NOTE 3 in the LS.

With such solution, there is no need to check the successful delivery of a PDU, which makes the gNB implementation easier. Besides, the gNB does not need to wait for the result of the transmissions of the other PDUs before discarding the redundant PDUs, which works faster and can provide an effective way to mitigate the network congestion in time. The solution can be applied for both RLC AM and RLC UM equally.

Proposal 11: RAN2 reply to SA2 the use of content ratio information for discarding of DL PDUs based on alternative 2 is feasible and beneficial for both RLC AM and RLC UM.

Another question asked by SA2 in question 1 is whether RLC AM is applicable for XR services. As per RP-240791 [5], RAN2 is working on enhancements for RLC AM in R19 to reduce the latency caused by ARQ retransmissions to make RLC AM better fit for XR services. So it should be confirmed that RLC AM can be used for XR services, especially with the R19 enhancements.

Proposal 12: RAN2 reply to SA2 that R19 RLC AM enhancement is applicable for AL-FEC.

# 5. Conclusion

This contribution makes the following proposals:

Avoiding unnecessary transmission

Proposal 1:	When there is an RLC SN gap (e.g. RX_Next < Rx_Next_Highest + 1), the Rx RLC entity starts a new timer.

Proposal 2:	When the timer expires, the Rx RLC entity should update the reassembly window accordingly (e.g. set RX_Next to the SN of the first RLC SDU with SN >= RX_Next for which RLC SDU is not abandoned and not all bytes have been received).

Proposal 3: When the local timer expires, RLC Status Report can be triggered.

Proposal 4: Tx RLC entity can stop retransmissions of an outdated RLC SDU according to its importance (e.g. stop retransmission of low importance RLC SDU).

Proposal 5:  No enhancement is needed to handle missing PDCP control PDUs for avoiding unnecessary retransmission.

Proposal 6: RAN2 not to introduce the indication from Tx to Rx RLC entity when the Tx side stops the transmissions of an outdated SDU.

Timely RLC retransmission enhancement

Proposal 7: RAN2 does not specify enhanced polling-based mechanism for the timely RLC retransmission, as network implementation can achieve the same effect.

Proposal 8:	RAN2 works on the autonomous RLC retransmission, where the capacity loss can be mitigated by introducing a transmission gap or considering the channel quality.

SA2 LS on AL-FEC awareness

Proposal 9: RAN2 reply to SA2 there are two potential directions to enhance DL discard at RAN based on the FEC content ratio information.

      - Alternative 1: The gNB discards the remaining PDUs of the PDU set after it determines that the ratio of the successfully delivered PDUs is above the threshold.

      - Alternative 2: The gNB discards PDUs of the PDU set according to the content ratio without confirming the successful delivery of the PDUs.

Proposal 10: RAN2 reply to SA2 the use of content ratio information for discarding of DL PDUs based on alternative 1 is feasible, but the benefit is limited since the gNB may not be able to determine the successful delivery of sufficient PDUs for a PDU set timely for both RLC AM and RLC UM.

Proposal 11: RAN2 reply to SA2 the use of content ratio information for discarding of DL PDUs based on alternative 2 is feasible and beneficial for both RLC AM and RLC UM.

Proposal 12: RAN2 reply to SA2 that R19 RLC AM enhancement is applicable for AL-FEC.

# 6. Reference

[1] RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3, 3GPP TSG RAN Meeting #103

[2] Chair note of RAN2#127-bis, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_127bis/Inbox/Chair_Notes/R2_127bis%20R18%20MBS%2C%20R18%20QoE%20and%20R19%20XR%20session%20notes%20(Dawid)_EOM.docx

[3] 3GPP TS 26.522

[4] S2-2410999

[5] RP-240791