3GPP TSG-RAN WG2 Meeting #128										R2-2410168

Orlando, USA, 14 - 18 Nov, 2024                                            	Revision of R2-2408009

Agenda item:		8.4.4

Title: 	Procedures for LP-WUS in RRC_CONNECTED

Source: 			ZTE Corporation, Sanechips

Document for: 	Discussion and Decision

- Introduction

In RAN#102 meeting, a new WID on Low-power wake-up signal and receiver for NR (LP-WUS/WUR) was approved. In RAN#103 meeting, the WID was further refined in [1] with the following objective for LP-WUS in connected:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

  - Check in RAN#105 for potential TU adjustment in RAN2

  - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

In RAN2#126 meeting, the following agreements have been made for Procedures for LP-WUS in RRC_CONNECTED state [3]:

- In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. - For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. - RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. - The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

In RAN2#127 meeting, the following agreements have been made for Procedures for LP-WUS in RRC_CONNECTED [4]:

- For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

In RAN2#127bis meeting, the following agreements have been made for Procedures for LP-WUS in RRC_CONNECTED [5]:

- For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. - In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. - When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

In this contribution, we will further discuss the RAN2 related issues on procedure of LP-WUS for RRC_CONNECTED state and give our proposals.

- Discussion

2.1 Activation/deactivation for LP-WUS monitoring in RRC_CONNECTED state

It has been agreed that the LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message. Considering that the coverage of LP-WUS may be less than or equal to the cell coverage, even the LP-WUS related configuration is provided, the LP-WUS may not be monitored, e.g. when the UE is at the cell edge. The LP-WUS monitoring activation/deactivation should be considered.

Usually, there are two solutions for LP-WUS monitoring activation/deactivation:

- Solution 1: similar as RRC_IDLE/RRC_INACTIVE state, a cell edge condition (e.g. RSRP threshold) is configured to UE, and UE monitors LP-WUS in RRC_CONNECTED state when it is not at the cell edge. For this solution, Network cannot know whether the UE is monitoring the LP-WUS and should always send LP-WUS, whichever the UE monitors it or not. In this case, for Option 1-1, Network couldn’t know whether UE starts the drx-onDurationTimer.

- Solution 2: Network activate or deactivate the LP-WUS explicitly, e.g. with RRC message, DCI or MAC CE. For this solution, UE need monitor LP-WUS only when it is activated by network, and network can activate or deactivate the LP-WUS based on the CQI info or UE assistance information reported by UE. So that network can determine whether and when to activate or deactivate the LP-WUS monitoring in RRC_CONNECTED state, LP-WUS monitoring entry and exit condition(e.g. RSRP and optional RSRQ threshold) should be configured to UE(e.g. by RRCReconfiguration message), and UE can report LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message .

To keep alignment between UE LP-WUS monitoring and gNB LP-WUS transmission, solution 2 is preferred. For light signaling load and not impact RAN1 on DCI design, MAC CE is suggested to be used for network to activate or deactivate the LP-WUS explicitly.

Proposal 1: LP-WUS for RRC_CONNECTED is activated or deactivated by MAC CE.

Proposal 1a: Network configures LP-WUS monitoring entry and exit condition (e.g. configure RSRP and optional RSRQ threshold by RRCReconfiguration message) to UE in RRC_CONNECTED state.

Proposal 1b: UE reports LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message.

## 2.2 LP-WUS monitoring schemes with C-DRX configured

RAN1 has identified the following 3 options for the LP-WUS monitoring with C-DRX configured, and has made the following working assumptions for Option 1-1 and Option 1-2:

Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions.

The working assumption in RAN1 aligns with RAN2 agreement, e.g.

RAN2 #126 Agreement: - For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. - For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 #127bis Agreement: - For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. - In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring.

Proposal 2: Confirm the RAN1 working assumption that both Option 1-1 and Option 1-2 are supported:

- Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

- Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

In the current specification, secondary DRX Group can be configured for UE power saving when both FR1 and FR2 cells are configured in one cell group, in which case the UE power consumption can be reduced if a separate drx-InactivityTimer and drx-onDurationTimer in secondary DRX Group is configured for the FR2 cells enabling FR2 to go to sleep more quickly. Since both secondary DRX Group and LP-WUS are used for UE power saving, LP-WUS is used to trigger the UE enters into DRX active time, and DRX active time is started per DRX group, it should support to configure LP-WUS and secondary DRX Group simultaneously for UE power saving.

Proposal 3a: The LP-WUS can be configured with secondary DRX group simultaneously.

When LP-WUS is configured with secondary DRX Group simultaneously, how to correlate LP-WUS with DRX group should be considered. One of the following options can be considered:

- Option 1: one LP-WUS resource is configured per DRX group.

- Option 2: one LP-WUS resource is configured per cell group, with indication in LP-WUS to indicate which DRX group is triggered.

Since Option 2 needs less signaling overhead than Option 1, but Option2 depends on RAN1 design for the LP_WUS.

Proposal 3b: When LP-WUS is configured with secondary DRX Group simultaneously, down select from the following options with an LS to RAN1 to inform our decision:

- Option 1: one LP-WUS resource is configured per DRX group.

- Option 2: one LP-WUS resource is configured per cell group, with indication in LP-WUS to indicate which DRX group is triggered

In RAN1#118bis meeting, it has agreed that LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode, as follows:

Agreement LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable

Based on the current specification [6], “In MR-DC, separate DRX configurations are provided for MCG and SCG. A secondary DRX group can be configured in MR-DC for a cell group that includes cells in different Frequency Ranges”. Since LP-WUS is used to trigger UE to enter DRX active time, LP-WUS should be configured for MCG and LCG separately.

Proposal 4a: In NR-DC, LP-WUS is configured for MCG and LCG separately.

Since C-DRX can be configured for MCG in NE-DC case and can be configured for SCG in EN-DC case, and LP-WUS is used to trigger UE to enter DRX active time, LP-WUS can be configured for MCG in NE-DC case, and LP-WUS can be configured for SCG in EN-DC case.

Proposal 4b: LP-WUS can be configured for MCG in NE-DC case, i.e. the MN is a gNB, and LP-WUS can be configured for SCG in EN-DC case, i.e. the SN is a gNB.

Considering that legacy, in case DCP is monitored but not detected, the legacy drx-onDurationTimer is not started and UE does not enter into active time. Same mechanism can be reused for LP-WUS in RRC_CONNECTED, e.g. If LP-WUS is monitored but not detected, the legacy drx-onDurationTimer is not started and the UE does not enter into active time.

Proposal 5: For UE in RRC_CONNECTED state, if LP-WUS is monitored but not detected, the legacy drx-onDurationTimer is not started for option 1-1 and the newly defined timer (e.g. drx-onDurationTimer-r19) is not started for option 1-2.

For Option 1-2, as ‘UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration)’ in RAN1, UE can determine the LP-WUS monitoring occasion using the similar formula as that to start legacy drx-onDurationTimer. Considering that LP-WUS monitoring consumes much less UE power than PDCCH monitoring, and smaller LP-WUS cycle can decrease service delay, one LP-WUS cycle can be configured, which is less than the C-DRX cycle(e.g. less than the long DRX cycle and the short DRX cycle, if configured) and not differentiate long LP-WUS cycle and short LP-WUS cycle.

Proposal 6: For Option 1-2, RAN2 confirms that one LP-WUS cycle is defined to determine the LP-WUS monitoring occasion, which does not differentiate long LP-WUS cycle and short LP-WUS cycle.

In Rel-18, non-integer C-DRX cycle is defined to match the non-integer traffic periodicity (e.g. XR periodicity), considering the low power consumption, small granularity of LP-WUS cycle (e.g. value of 1ms) can be used to match the non-integer C-DRX periodicity, and no non-integer LP-WUS cycle is defined.

Proposal 6a: For Option 1-2, RAN2 confirms that small granularity of LP-WUS cycle (e.g. value of 1ms) is defined to match the non-integer traffic periodicity, and no non-integer LP-WUS cycle is defined.

In RAN2#117 meeting, it has been agreed that “For Option 1-2,The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring.”, that means that a timeOffset should be defined for the GAP between LP-WUS monitoring and the start of newly defined timer (e.g. drx-onDurationTimer-r19). Since legacy drx-startOffset is defined to determine the PDCCH monitoring start occasion, drx-startOffset - timeOffset can be used as the LP-WUS startOffset to determine the LP-WUS monitoring occasion.

Proposal 7:  For Option 1-2, RAN2 confirms that drx-startOffset - timeOffset is used as the LP-WUS startOffset to determine the LP-WUS monitoring occasion. Wherein the timeOffset is the time GAP between the LP-WUS reception and the start of newly defined timer (e.g. the new timer triggered by LPWUS).

Similar as the drx-onDurationTimer start occasion, For Option 1-2, the LP-WUS monitoring occasion can be determined with the LP-WUS cycle and LP-WUS startoffset(e.g. drx-startOffset - timeOffset) as follows: if LP-WUS is configured and activated, the UE monitors the LP-WUS at the SFN and subframe determined by:

[(SFN × 10) + subframe number -( drx-StartOffset - timeOffset)] modulo (LP-WUS-cycle)) =0:

Proposal 8: For Option 1-2, the LP-WUS monitoring occasion can be determined using the similar formula as that to determine the start of drx-onDurationTimer, e.g. For UE in RRC_CONNECTED state, if LP-WUS is activated, the UE monitors LP-WUS if:

[(SFN × 10) + subframe number-(drx-StartOffset - timeOffset)] modulo (LP-WUS-cycle)) = 0.

With this method, if the length of the newly defined timer (e.g. drx-onDurationTimer-r19) is set to the same value as the legacy drx-onDurationTimer, and the DRX cycle is a multiple of the LP-WUS-cycle, then Option 1-2 can cover the option 1-1 for PDCCH monitoring; and if the length of the newly defined timer (e.g. drx-onDurationTimer-r19) is set to the same value as the legacy drx-onDurationTimer, and LP-WUS-cycle is set to the same value as the DRX cycle, then option 1-2 is the same as option 1-1. .

Proposal 9: With proposal 8, option 1-1 can be considered as a special case of option 1-2 with the LP-WUS-cycle set to the same value as the DRX cycle.

About the Option 1-3 in RAN1 agreement for LP-WUS monitoring for RRC_CONNECTED state (LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring): This option can be used to decrease the UE power when intermittent DL data burst arrives at gNB, e.g. in the C-DRX active time, only if LP-WUS is detected, UE monitors PDCCH. Since LP-WUS consumes little UE power, a small C-DRX cycle, large drx-OndurationTimer and/or large drx-inactiveTimer can be configured to keep the UE always in the DRX active time. This solution can also reduce the UE power consumption, but the following issue should be considered: Whether the LP-WUS will be monitored when the HARQ timer is running. If LP-WUS will be monitored when the HARQ timer is running, the UE performance should be evaluated, e.g. whether UE can monitor LP-WUS and perform main radio Tx/Rx simultaneously; if LP-WUS will be monitored only after the HARQ timer expires, the LP-WUS monitoring may lead PDCCH monitoring delay, and further lead traffic transmission delay, which depends on the RAN1 design for LP-WUS, e.g. the time GAP between LP-WUS monitoring and PDCCH monitoring.

Observation 1: Option 1-3 can also reduce UE power consumption, meanwhile keep small data transmission latency, but the UE performance and the PDCCH monitoring delay caused for LP-WUS monitoring should be evaluated by RAN1.

Proposal 10: Option 1-3 is not supported, e.g. UE does not monitor LP-WUS during the active time of C-DRX cycle.

- Conclusion

Proposal 1: LP-WUS for RRC_CONNECTED is activated or deactivated by MAC CE.

Proposal 1a: Network configures LP-WUS monitoring entry and exit condition (e.g. configure RSRP and optional RSRQ threshold by RRCReconfiguration message) to UE in RRC_CONNECTED state.

Proposal 1b: UE reports LP-WUS activation/deactivation assistance information to network, e.g. by UEAssistanceInformation message.

Proposal 2: Confirm the RAN1 working assumption that both Option 1-1 and Option 1-2 are supported:

- Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer.

- Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring.

Proposal 3a: The LP-WUS can be configured with secondary DRX group simultaneously.

Proposal 3b: When LP-WUS is configured with secondary DRX Group simultaneously, down select from the following options with an LS to RAN1 to inform our decision:

- Option 1: one LP-WUS resource is configured per DRX group.

- Option 2: one LP-WUS resource is configured per cell group, with indication in LP-WUS to indicate which DRX group is triggered

Proposal 4a: In NR-DC, LP-WUS is configured for MCG and LCG separately.

Proposal 4b: LP-WUS can be configured for MCG in NE-DC case, i.e. the MN is a gNB, and LP-WUS can be configured for SCG in EN-DC case, i.e. the SN is a gNB.

Proposal 5: For UE in RRC_CONNECTED state, if LP-WUS is monitored but not detected, the legacy drx-onDurationTimer is not started for option 1-1 and the newly defined timer (e.g. drx-onDurationTimer-r19) is not started for option 1-2.

Proposal 6: For Option 1-2, RAN2 confirms that one LP-WUS cycle is defined to determine the LP-WUS monitoring occasion, which should be less than the C-DRX cycle, and does not differentiate long LP-WUS cycle and short LP-WUS cycle.

Proposal 6a: For Option 1-2, RAN2 confirms that small granularity of LP-WUS cycle (e.g. value of 1ms) is defined to match the non-integer traffic periodicity, and no non-integer LP-WUS cycle is defined.

Proposal 7: For Option 1-2, RAN2 confirms that drx-startOffset - timeOffset is used as the LP-WUS startOffset to determine the LP-WUS monitoring occasion. Wherein the timeOffset is the time GAP between the LP-WUS reception and the start of newly defined timer (e.g. the new timer triggered by LPWUS).

Proposal 8: For Option 1-2, The LP-WUS monitoring occasion can be determined using the similar formula as that to determine the start of drx-onDurationTimer, e.g. For UE in RRC_CONNECTED state, if LP-WUS is activated, the UE monitors LP-WUS if:

[(SFN × 10) + subframe number-(drx-StartOffset - timeOffset)] modulo (LP-WUS-cycle)) = 0.

Proposal 9: With proposal 8, option 1-1 can be considered a special case of option 1-2 with the LP-WUS-cycle set to the same value as the DRX cycle.

Observation 1: Option 1-3 can also reduce UE power consumption, meanwhile keep small data transmission latency, but the UE performance and the PDCCH monitoring delay caused for LP-WUS monitoring should be evaluated by RAN1.

Proposal 10: Option 1-3 is not supported, e.g. UE does not monitor LP-WUS during the active time of C-DRX cycle.

- References

- RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, RAN#103, March, 2024

- Draft_Minutes_report_RAN1#116b_v030, RAN1#116bis, April, 2024

- Draft_R2_126_Meeting_Report_v1, RAN2#126, May, 2024

- Draft_RAN2_127_Meeting_Report_v1

- Draft_RAN2_127b_Meeting_Report_v1

- 3GPP TS 37.340 V18.3.0 (2024-09), Multi-connectivity; Overall Description; Stage 2

- Annex

RAN1#116 meeting agreements:

R1-2401698 FL summary #1 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-3: LP-WUS monitoring inside C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded. Agreement For RRC CONNECTED mode, maximum number of LP-WUS information bits is up to X bits - FFS value X, which is no more than [8 or 16] Agreement For RRC CONNECTED mode, minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following - LP-WUS processing time - MR transition time for ramp up - Time/frequency synchronization of MR - FFS whether UE can report supported minimum time gap from candidate values FFS: Whether the minimum time gap values can be more than one R1-2401836 FL summary #2 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

RAN1#116bis meeting agreements:

R1-2403517 FL summary #1 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) R1-2403713 FL summary #2 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement Update the following agreement in RAN1#116 in red: Agreement - For RRC CONNECTED mode, from RAN1 perspective, further study following LP-WUS procedures to trigger PDCCH monitoring: - Case 1: PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - This option may replace DCP functionality - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-1-2: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - Option 1-3: LP-WUS monitoring inside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - Case 2: PDCCH monitoring is triggered by LP-WUS without C-DRX configuration. LP-WUS can be monitored at any time according to the LP-WUS monitoring configuration - FFS duty-cycled and/or continuous LP-WUS monitoring - Combination of options in Case 1 and combination of options in Case 1 and Case 2 are not precluded should be considered. - RAN1 does not discuss C-DRX related timers other than drx-onDurationTimer, this topic is up to RAN2 - Note: Above does not preclude to support fallback mechanism to trigger PDCCH monitoring, if any

RAN1#117 meeting agreements:

R1-2405471 FL summary #1 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring. - FFS: exact value(s) of the minimum time gap - FFS: support of multiple minimum time gaps - FFS whether the reported value includes the duration for time/frequency synchronization of MR R1-2405592 FL summary #2 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement For LP-WUS monitoring in RRC CONNECTED mode, a LP-WUS is QCLed with existing NR signal/channel/CORESET for the TCI state - FFS which existing NR signal/channel/CORESET is the QCL source of LP-WUS - FFS exact definition of QCL relationship between LP-WUS and existing NR signal/channel/CORESET Agreement LP-WUS monitoring occasions (MOs) are configured by RRC, where UE can monitor for LP-WUS transmission in RRC CONNECTED mode. - FFS whether to define a time window for Mos - It is at least supported that a UE can monitor MOs with a periodicity. - FFS details of the periodicity, e.g. derived from DRX cycle, separately configured R1-2405659 FL summary #3 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement Study whether/how LP-WUS works when UE is configured with CA in RRC CONNECTED mode - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable - Option 1: one or more serving cells based on gNB indication/configuration - Option 2: all activated serving cells - Note: other options are not precluded Agreement For RRC CONNECTED mode, LP-WUS can be configured without following existing features. - Rel-16 DCP - Rel-17 PDCCH skipping - Rel-17 SSSG switching - Rel-18 cell DTX Further study whether/how LP-WUS works with following existing features (PDCCH skipping, SSSG switching, cell DTX)

RAN1#118 meeting agreements:

R1-2407301 FL summary #1 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) R1-2407378 FL summary #2 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement - For option 1-2 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - UE is configured with legacy C-DRX configurations as Rel-18 - UE is expected to be configured with LP-WUS monitoring configuration (periodicity and offset can be different from those from C-DRX configuration) - FFS potential restriction for LP-WUS configuration in relation with C-DRX configuration - LP-WUS triggers the start of a timer during which UE monitors PDCCH - FFS the timer is existing timer and/or new timer - UE PDCCH monitoring behaviors related to other legacy DRX timers are not affected - drx-InactivityTimer, drx-RetransmissionTimerDL, drx-RetransmissionTimerUL, drx-HARQ-RTT-TimerDL, drx-HARQ-RTT-TimerUL - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP and option 1-1) - Periodic CSI/L1-RSRP is not reported if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported regardless if UE is indicated to wake-up or not - UE PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS Working Assumption - From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE - Note: Above can be revisited considering RAN2 decisions. - R1-2407516 FL summary #3 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) - Agreement - Select one of the following alternatives in RAN1#118bis Alt 1: For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value Alt2: For RRC CONNECTED mode, UE reports multiple values for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported value - Different minimum time gaps correspond to different sleep states - Companies are encouraged to details on how the reported values are to be used by the network - Agreement - LP-WUS is at least supported for the case where a UE is configured with CA in RRC CONNECTED mode - FFS: DC Agreement For LP-WUS monitoring in RRC CONNECTED mode, SSB and/or CSI-RS can be the QCL source of LP-WUS - FFS applicable QCL type(s)

RAN1#118bis meeting agreements:

R1-2409010 FL summary #1 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer - UE is configured with legacy C-DRX configurations as Rel-18 - UE behaviors related to CDRX active time triggered by all legacy DRX timers are not affected unless included in this proposal - No impact on RRM/RLM/BFD measurement requirements is assumed - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP) - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification Agreement Confirm following working assumption Working Assumption From RAN1 perspective, for RRC CONNECTED mode, PDCCH monitoring is triggered by LP-WUS with C-DRX configuration - Support Option 1-1: LP-WUS monitoring according to the LP-WUS monitoring configuration before drx-onDurationTimer to trigger the starting of the drx-onDurationTimer. - Support Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE Note: Above can be revisited considering RAN2 decisions. R1-2409197 FL summary #2 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting. - FFS: X - FFS: definition of reported candidate value - UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional. - FFS: A single time offset is configured for a UE at a given time R1-2409237 FL summary #3 on LP-WUS operation in CONNECTED mode Moderator (NTT DOCOMO) Agreement LP-WUS is supported when UE is configured with NR-DC in RRC CONNECTED mode - Above is supported for the case PDCCH monitoring is triggered by LP-WUS in the same cell group - FFS: The cell(s) where PDCCH monitoring triggered by a LP-WUS is applicable Agreement - For LP-WUS CONNECTED mode operation, Option 1-2 can be configured separately from Option 1-1 - FFS whether/how to support both Option 1-1 and Option 1-2 simultaneously configured for the same UE ZTE: remove supporting option 1-2-2, and leave it to RAN2. - Option 1-2: LP-WUS monitoring outside at least legacy C-DRX active time according to the LP-WUS monitoring configuration to trigger PDCCH monitoring. - PDCCH monitoring possibly irrespective of drx-onDurationTimer - Option 1-2-1: PDCCH monitoring may be additionally triggered based on legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS - If this is adopted, it should be configured together with Option 1-1 to achieve power saving gain compared to legacy C-DRX - Option 1-2-2: PDCCH monitoring is not triggered by legacy C-DRX cycle and drx-onDurationTimer when monitoring LP-WUS Agreement - Regarding the potential restriction for LP-WUS configuration in relation with C-DRX configuration for option 1-2 of LP-WUS CONNECTED mode operation, from RAN1 perspective, - AltOpt2: At least LP-WUS periodicity is no larger than long C-DRX cycle, FFS other restrictions - Introducing monitoring window is not precluded