3GPP TSG-RAN WG2 Meeting #128	R2-2410179

Orlando, USA, Nov. 18th – 22nd, 2024	Revision of R2-2408357

Agenda Item:	8.2.5

Source:			ASUSTeK

Title:	Consideration on information and scenario in Topology 2

Document for: 	Discussion and Decision

# Introduction

In RAN2#127bis meeting [1], the the following agreements on Ambient IoT topology 2 were made:

Agreements

- Dedicated resource configuration is only given to the UE reader via dedicated signalling.  Mechanisms for shared resource pool amongst readers are not considered in this release.

- The UE may perform the AIoT procedure on AIoT interface between the reader and the device only if the resource configuration is valid in the cell under network control.    FFS how valid resource is determined – we will consider temporary out of connection scenarios. FFS on resource validity across multiple cells.

- The AIoT radio resource can be (re)configured during RRC reconfiguration (including during HO procedure, after re-establishment, etc).

And in RAN1#116bis meeting [2], several scenarios for topology 2 are defined as following:

| Scenario | CW Inside/outside topology | Diagram of the scenario | Description of the scenario | Device 1/2a/2b | CW spectrum | D2R spectrum |
| --- | --- | --- | --- | --- | --- | --- |
| D2T2-A1 | CW inside topology |  | - CW node inside topology 2 - ‘CW’ in CW2D and ‘R2’ in D2R are different - ‘CW’ in CW2D and ‘R1’ in R2D are same - ‘R1’ in R2D and ‘R2’ in D2R are different - BS communicates with R1 and R2 | Device 1, 2a | Case 2-2 (inside topology, UL) | Same as CW |
| D2T2-A2 |  |  | - CW node inside topology 2 - same ‘CW’ and ‘R’ node for CW2D, D2R and R2D - BS communicates with R |  | Same as D2T2-A1 | Same as CW |
| D2T2-B | CW outside topology |  | - CW node outside topology 2 - ‘CW’ in CW2D and ‘R’ in D2R are different - ‘CW’ in CW2D and ‘R’ in R2D are different - ‘R’ in R2D and ‘R’ in D2R are same - BS communicates with R |  | Case 2-3 (outside topology, DL) Case 2-4 (outside topology, UL) | Same as CW |
| D2T2-C | No CW |  | - No CW Node. - BS communicates with R | Device 2b | N/A | FFS |

In this contribution, we would like to discuss the information required in Topology 2 and study different scenarios.

# Discussion

- UE assistance information

In the previous discussion in RAN2, most of the companies had the similar view that the UE needed to be in RRC Connected mode at least when the reader configured resource for AIoT interface, considering the network authorization issue. By the same token, the network should be responsible for the initiation of intermediate node functionality, including the determination of whether a UE can perform as an intermediate node.

For a UE to become an intermediate node, it should be useful for the UE to provide some information to the gNB. For example, the UE should at least report its capability to perform functionalities of intermediate node before being configured. For another example, the UE could provide its location information to the network, and the network would configure the UE to become an intermediate node if there are Ambient IoT devices nearby. The information could assist the network to determine whether a UE could become an intermediate node.

Proposal 1: 	The UE should be able to provide information to network to assist determining whether the UE could become an intermediate node.

- Scenarios in Topology 2

On the other hand, different scenarios in Topology 2 have been discussed by RAN1. When the carrier wave is inside the topology, D2T2-A1 refers to the case that the intermediate node for D2R transmission and the one for R2D transmission are different. On the contrary, D2T2-A2 stands for the case that the intermediate node for D2R transmission and the one for R2D transmission are the same.

Observation 1: 	Based on RAN1’s agreement, the intermediate node for R2D transmission and D2R transmission could be the same or different.

Since the intermediate node (e.g. a UE) is assigned by the gNB (or the network), the operation and configuration should be under network control. For the case where there are two intermediate nodes to serve a device, there should be alignment for the gNB to make. For example, the resource used between gNB and each intermediate node, and the resource used between each intermediate node and the devices should be allocated without collision or interference. The gNB provides scheduling information for R2D transmission to R2D intermediate node. And the gNB provides scheduling information for D2R transmission to R2D intermediate node (e.g. to include it in the PRDCH) and D2R intermediate node (e.g. to know where to receive the D2R transmission).

Additionally, the required information used to assist corresponding R2D transmission should be aware to the R2D intermediate node after receiving at the D2R intermediate node. As a result, RAN2 could study the impact to procedures with different intermediate nodes for R2D and D2R transmissions.

Proposal 2:	For the case where there are two intermediate nodes in Topology 2, the gNB provides scheduling information for R2D transmission to R2D intermediate node. In addition, the gNB provides scheduling information for D2R transmission to R2D intermediate node and D2R intermediate node.

# Conclusion

Proposal 1: 	The UE should be able to provide information to network to assist determining whether the UE could become an intermediate node.

Observation 1: Based on RAN1’s agreement, the intermediate node for R2D transmission and D2R transmission could be the same or different.

Proposal 2:	For the case where there are two intermediate nodes in Topology 2, the gNB provides scheduling information for R2D transmission to R2D intermediate node. In addition, the gNB provides scheduling information for D2R transmission to R2D intermediate node and D2R intermediate node.

# Reference

[1] Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China

[2] RAN1#116bis meeting minute