3GPP TSG-RAN WG2 #128 meeting	R2-2410191

# Orlando, the USA, 18th Nov – 22nd Nov, 2024

Source:		NEC

Agenda item:		8.7.6	XR rate control

Title:		Uplink rate control for XR

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#127bis meeting, agreements [1] on XR rate control were made as below:

- FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc. - RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - - Extend the Bit Rate field - - Define a new bit rate table to provide sufficient granularity for XR traffic - - Introduce new values for the bitRateMultiplier - Send LS to SA4 asking about range/granularity which is required

In this contribution, we will further discuss the potential enhancements on XR rate control.

# 2. Discussion

2.1 per QoS flow/DRB recommended bit rate.

To assist the UE to select or adapt to a codec rate for MMTEL voice or MMTEL video, the current specification provides a RAN-assisted codec adaptation mechanism, which allows gNB to provide a recommended bit rate to UE via the Recommended bit rate MAC CE.

- gNB may send the MAC CE to the UE to indicate the recommended bit rate for a specific LCH and a specific direction (UL or DL).

- To check whether a bit rate recommended by its peer can be provided by the gNB, UE may send a recommended bit rate query (via the MAC CE) for a specific LCH and a specific direction to gNB. This may be triggered because the UE receives a bit rate request from its peer UE via the application layer.

The objective of XR rate control is to enable faster source rate adaption to uplink congestion over downlink. In this sense, the extension should apply to uplink bit rate control via downlink signalling, that is, gNB indicates a recommended bit rate for uplink transmission to UE. However, the existing recommended bit rate framework also supports

- Bit rate recommendation query via uplink signalling.

- Downlink bit rate control via downlink signalling.

Since these two procedures are not mentioned in the objective, we propose RAN2 to discuss whether to support them. From our perspective, at least bit rate recommendation query should also be supported.

Proposal 1: RAN2 to discuss whether the enhancements for XR rate control are only applicable to uplink bit rate recommendation, or also applicable to

- Bit rate recommendation query.

- Downlink bit rate recommendation.

In the RAN2 #127bis meeting, there was a discussion on whether the ranges or granularities of the recommended bit rate are sufficient for XR traffic, and an LS was sent to SA4. RAN2 can await SA4's reply for further discussion of this aspect.

Proposal 2: RAN2 can wait for SA4's reply to further discuss the ranges or granularities of the recommended bit rate for XR traffic.

Current Recommended bit rate MAC CE only supports per-LCH recommended bit rate indication. To achieve the objective of bit rate control on a per QoS flow/DRB basis on the MAC layer, we will first evaluate the feasibility and pros/cons of these two indication granularities.

QoS flow level

For downlink, the QoS profile, including QoS flow level QoS parameters, is provided by 5GC to gNB, which can be used by gNB to determine the treatment on the radio interface. In CU-DU split architecture, CU will further forward these QoS flow level QoS parameters to DU, together with QoS flow to DRB mapping information (refer to TS38.413). In conclusion, gNB has full knowledge of QoS flow and network conditions, and is able to provide per-QoS flow recommended bit rate to UE. Upon reception of the indication by the MAC entity of UE, it can indicate to upper layer the recommended bit rate for the indicated QoS flow.

For uplink, UE can determine the mapping between uplink User Plane traffic and QoS flows based on the QoS rule(s) provided by 5GC (or NAS reflective QoS). However, the MAC layer is unaware of QoS flow, so it may rely on the upper layer to provide the QFI when the MAC entity intends to send a recommended bit rate query for a specific QoS flow to gNB.

Hence, we think it is feasible to support per QoS flow rate control in the MAC layer. The most straightforward way for per-QoS flow bit rate recommendation is to carry the QFI (QoS Flow ID) in a new recommended bit rate MAC CE.

Proposal 3: per-QoS flow bit rate indication can be implemented by indicating the QFI in a new recommended bit rate MAC CE, for which the QFI is provided by the upper layer.

DRB level

There may be two approaches to indicate the recommended bit rate for a DRB:

- Similar to per-QoS flow indication, DRB ID can be explicitly indicated in a new recommended bit rate MAC CE.

- Since there is a direct mapping between DRB and LCHs, UE can derive the recommended bit rate for a DRB based on the existing per-LCH bit rate recommendation.

For the 2nd approach,

- If there is 1:1 mapping between DRB and LCH, the recommended bit rates for both are equivalent.

- If there is 1:N mapping between DRB and LCHs (e.g., during PDCP duplication or split bearer), the recommended bit rates for LCHs may need to be combined to determine the recommended bit rate for a DRB (see section 2.2).

Proposal 4: The following two approaches can be considered to indicate the recommended bit rate for a DRB:

- Explicitly indicate DRB ID in a new recommended bit rate MAC CE.

- Derive the recommended bit rate for a DRB based on the existing per-LCH bit rate recommendation.

According to the discussion above, both per-QoS flow and per-DRB bit rate recommendation are feasible from the technical view.

- With per QoS flow indication, it would avoid converting QoS level data rate adaptation ability/range into DRB level at gNB side, and avoid converting LCH/DRB level recommended bit rate back to QoS level for the UE application layer. But it may rely on the upper layer to indicate the QFI, and a new recommended bit rate MAC CE should be introduced.

- With per DRB indication, the existing per-LCH bit rate recommendation may be reused. However, it is required to derive the recommended bit rate of DRB based on that of LCHs. From our understanding, the application layer prefers the per-QoS recommended bit rate because it directly maps to a specific traffic. So UE may need to convert the DRB-level recommended bit rate into a QoS-level recommendation for the UE application layer. If multiple QoS flows are mapped to a DRB, conversion from DRB level to QoS level shall be challenging for obtaining an accurate recommended bit rate.

Proposal 5: Regarding per-QoS flow or per-DRB indication, RAN2 considers the impact of conversion from the DRB-level recommended bit rate to the QoS-level recommended bit rate.

It should be noted that not all services or QoS flows support source bit rate adaptation. When determining the recommended bit rate, only QoS flows that support bit rate adaptation should be considered. For uplink traffic, only the UE knows whether a specific service or QoS flow supports bit rate adaptation. Therefore, the network may rely on the UE to report this information, for example, via UAI. With this information, the network can properly configure or determine the recommended bit rate for uplink traffic.

Proposal 6: UE to report whether a QoS flow supports bit rate adaptation (e.g., via UAI).

2.2 recommended bit rate during CA/DC.

The data of a DRB or QoS flow may be transmitted over different carriers (or LCHs) belonging to the same or different MAC entities during PDCP duplication or split transmission. Due to varying congestion situations in different cells, UE may receive different recommended bit rate values for the same DRB or QoS flow from different cells or cell groups (e.g., during CA/DC). In this case, RAN2 should discuss how to apply the indicated recommended bit rate.

For example, UE may be provided with recommended bit rate 1 of a QoS flow/DRB from cell 1 and recommended bit rate 2 of the same QoS flow/DRB from cell 2.

- For PDCP duplication, the UE may not be expected to go beyond the recommended bit rate of min (recommended bit rate 1, recommended bit rate 2) for the DRB or the QoS flow.

- For split transmission, the recommended bit rate for a DRB or QoS flow may be applied to the cell or cell groups where the MAC CE is received. UE should guarantee that the bit rate of the DRB or QoS flow in each cell or cell group will not go beyond the recommended bit rate of the corresponding cell or cell groups.

The same issue arises in scenarios of per DRB bit rate recommendation with a 1:N mapping between DRBs and LCHs, but the solution is analogous, so we avoid redundancy by not repeating it.

Proposal 7: RAN2 to discuss how to apply the indicated recommended bit rate of a DRB or QoS flow when more than one indication is received from different cells or cell groups (e.g., during CA/DC).

# Summary

This contribution provides our analysis on the uplink control for XR, and has the following proposals:

Proposal 1: RAN2 to discuss whether the enhancements for XR rate control are only applicable to uplink bit rate recommendation, or also applicable to

- Bit rate recommendation query.

- Downlink bit rate recommendation.

Proposal 2: RAN2 can wait for SA4's reply to further discuss the ranges or granularities of the recommended bit rate for XR traffic.

Proposal 3: per-QoS flow bit rate indication can be implemented by indicating the QFI in a new recommended bit rate MAC CE, for which the QFI is provided by the upper layer.

Proposal 4: The following two approaches can be considered to indicate the recommended bit rate for a DRB:

- Explicitly indicate DRB ID in a new recommended bit rate MAC CE.

- Derive the recommended bit rate for a DRB based on the existing per-LCH bit rate recommendation.

Proposal 5: Regarding per-QoS flow or per-DRB indication, RAN2 considers the impact of conversion from the DRB-level recommended bit rate to the QoS-level recommended bit rate.

Proposal 6: UE to report whether a QoS flow supports bit rate adaptation (e.g., via UAI).

Proposal 7: RAN2 to discuss how to apply the indicated recommended bit rate of a DRB or QoS flow when more than one indication is received from different cells or cell groups (e.g., during CA/DC).

# References

[1] Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.3.0