3GPP TSG-RAN WG2 Meeting # 128	R2-2410192

# Orlando, USA,  18th Oct. – 22nd Oct. 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5147 | rev |  | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement to measurement report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For enhancement to measurement report, the RRC CR is agreed in R2-2406021 and UE capability CR is agreed in R2-2405994. Later on CR R2-2407783 is agreed to add the condition i.e. cellsMetLeavingCond to store cells into cellsMetLeavingCond. In the same section the stored cell(s) is removed from cellsMetLeavingCond: 2> if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event: 3> if enteringLeavingReport is configured for the corresponding reporting configuration: 4> store the concerned cell(s) in the cellsMetLeavingCond defined within the VarMeasReportList for this measId; 3> remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId; 3> if reportOnLeave is set to true for the corresponding reporting configuration: 4> if the corresponding reportConfig does not include numberOfTriggeringCells; or 4> if the corresponding reportConfig includes numberOfTriggeringCells and a measurement report was previously sent to the network for at least one of the concerned cell(s): 5> initiate the measurement reporting procedure, as specified in 5.5.5; 3> remove all the concerned cell(s) from cellsMetLeavingCond defined within the VarMeasReportList for this measId, if any; 3> if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty: 4> remove the measurement reporting entry within the VarMeasReportList for this measId; 4> stop the periodical reporting timer for this measId, if running; The concerned cell(s) should be stored until one measurement report is triggered due to meeting leaving condition. In current spec if reportOnLeave is not true, then the stored cell(s) in cellsMetLeavingCond will be removed directly without triggering any measurement report. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1: in section 5.5.4.1, it is clarified that when both reportOnLeave and enteringLeavingReport are configured as True, cell(s) meeting leaving condition will be stored in cellsMetLeavingCond. And those stored cell(s) will be deleted when a measurement report is transmitted due to meeting leaving condition Impact analysis Impacted 5G architecture options: NR SA, MRDC Impacted functionality: - Reporting of which cell(s) fulfilled the event leaving condition Inter-operability: - If the network is implemented according to the CR and the UE is not, inter-operability issue is not forseen. - If the UE is implemented according to the CR and the NW is not, inter-operability issue is not forseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, when concerned cell(s) are stored but reportOnLeave is not true, the stored cell(s) in cellsMetLeavingCond will be removed directly without reporting to network |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/***************************change 1***********************/

5.5.4	Measurement report triggering

5.5.4.1	General

/unchanged text is removed…/

2>	if the reportType is set to eventTriggered and if the leaving condition applicable for this event is fulfilled for one or more of the cells included in the cellsTriggeredList defined within the VarMeasReportList for this measId for all measurements after layer 3 filtering taken during timeToTrigger defined within the VarMeasConfig for this event:

3>	if reportOnLeave is set to true and enteringLeavingReport is configured for the corresponding reporting configuration:

4>	store the concerned cell(s) in the cellsMetLeavingCond defined within the VarMeasReportList for this measId;

3>	remove the concerned cell(s) in the cellsTriggeredList defined within the VarMeasReportList for this measId;

3>	if reportOnLeave is set to true for the corresponding reporting configuration:

4>	if the corresponding reportConfig does not include numberOfTriggeringCells; or

4>	if the corresponding reportConfig includes numberOfTriggeringCells and a measurement report was previously sent to the network for at least one of the concerned cell(s):

5>	initiate the measurement reporting procedure, as specified in 5.5.5;

3>	remove all the concerned cell(s) from cellsMetLeavingCond defined within the VarMeasReportList for this measId, if any;

3>	if the cellsTriggeredList defined within the VarMeasReportList for this measId is empty:

4>	remove the measurement reporting entry within the VarMeasReportList for this measId;

4>	stop the periodical reporting timer for this measId, if running;

/ unchanged text is removed…/

/***************************change 1***********************/