3GPP TSG-RAN WG2 #128 meeting	R2-2410194

# Orlando, the USA, 18th Nov – 22nd Nov, 2024

Source:		NEC

Agenda item:		8.7.4.1	LCP enhancements

Title:		Considerations on LCP enhancements for XR

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#127bis meeting, agreements [1] on LCP enhancements for XR were made as below:

As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold. For now, we will use “LCH priority-adjusted data”

In this contribution, we will provide our views on the enhancements for LCP procedure.

# 2. Discussion

2.1 LCP enhancements for priority-adjusted data.

In the RAN2#127bis meeting, 2 options were discussed concerning whether the additional LCH priority should be applied during the entire resource allocation procedure of LCP.

- Option 1: additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. - Option 2: additional LCH priority is applied for an LCH in 1st Round of resource allocation procedure in LCP, if the LCH has delay-critical data available for transmission when starting the 1st Round, and the additional LCH priority is applied for an LCH in 2nd Round of resource allocation procedure in LCP, if the LCH has residual delay-critical data available for transmission when starting 2nd Round.

And the following agreements and issues were raised.

As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation

Regarding the fairness issue, it should be noted that the fairness of LCP procedure is assessed over a relatively long period instead of a single LCP instance. And it has been guaranteed by the token-based LCP mechanism, that is, resources are allocated in the 1st ground according to the number of tokens an LCH possesses (i.e., Bj). In the 2nd round of LCP procedure, the remaining resources shall be allocated on a best-effort basis (regardless of the value of Bj) to maximize the transmission of high-priority data. From this perspective, the 2nd round is not concerned with fairness.

Observation 1: The 2nd round of the LCP procedure is not concerned with fairness, as it’s designed to maximize the transmission of high-priority data on a best-effort basis.

For adjusting the priority in the 2nd round, the concern about the extra delay and complexity is mainly related to checking any residual priority-adjusted data available for transmission at the start of the 2nd round. As UE has the knowledge of 1) the size of each RLC SDU; 2) whether an RLC SDU is priority-adjusted data; and 3) the total size of served MAC SDUs for an LCH after the 1st round, it can calculate the total size of priority-adjusted data for an LCH (e.g., by updating the value upon reception of priority-adjusted indication from upper layer). By subtracting the total size of served MAC SDUs, it can further identify whether there is residual priority-adjusted data for an LCH at the start of 2nd round.

Even though Option 2 is technically feasible, it will increase the complexity of UE implementation and entail reordering the priority of LCHs at the start of 2nd round. As LCP can only run once the PUSCH scheduling DCI is received and UL TB size is known, and TB preparation has to be finished at the PUSCH transmission time point, any enhancement which will increases TB preparation shall be avoid unless huge gain is justified.  Without priority re-adjustment between 1st and 2nd round. LCHs’ Priority maintenance can run before and parallel to LCP, and TB preparation time will not be impacted.

Observation 2: Adjust LCH priority at the start of 2nd round is technically feasible but will increase TB preparation time.

Consequently, we prefer Option 1, where the additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of the LCP procedure.

Proposal 1: The additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of the LCP procedure.

The MAC entity shall, when a new transmission is performed: 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. NOTE 1: The value of Bj can be negative.

In the legacy LCP process, logical channels with equal priority should be served equally. However, with the enhanced delay-awareness LCP, there's a need to discuss whether an LCH with priority-adjusted data should be served before one without such data when both have equal priority. To ensure the timely transmission of delay-critical data, we prefer to serve the LCH with priority-adjusted data first in this scenario.

Proposal 2: RAN2 to discuss whether an LCH with priority-adjusted data should be served before one without such data when both have equal priority.

The concept of PDU Set has been introduced in Rel-18, and all PDUs belonging to a PDU Set may be needed for the decoding of the corresponding information unit. In this situation, all PDUs belonging to a PDU Set shall be discarded as a whole if at least one PDU is discarded. Similarly, as all PDUs belonging to a PDU Set share the same PSDB, to avoid only part of a PDU Set being multiplexed in a MAC PDU but all PDUs are needed for decoding, which still brings extra delay for a PDU Set, RAN2 should consider maximizing the transmission of a PDU Set during delay-awareness LCP procedure.

Proposal 3: Maximize the transmission of a PDU Set during delay-awareness LCP procedure when all PDUs are needed for a PDU Set.

In the RAN2#125bis meeting, it was agreed to consider the potential impact of delay-awareness LCP on traffic from SRBs. From our understanding, we should discuss SRB 0 and SRBs other than SRB 0 separately.

RAN2 should consider potential impact on traffic from SRBs.

For SRB0 (i.e., data from UL-CCCH), it always has a higher priority than delay-critical data (i.e., data from any Logical Channel, except data from UL-CCCH), which has already been specified by the priority order of logical channels in current specification [2]. There is no need to break the rules.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first): - MAC CE for C-RNTI, or data from UL-CCCH; - MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation; … - MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment; - data from any Logical Channel, except data from UL-CCCH; … NOTE 2: Prioritization among MAC CEs of same priority is up to UE implementation.

For SRBs other than SRB 0, a priority value is associated with the corresponding logical channel, either by applying the default configuration or an explicit configuration. From our understanding, by configuring a proper priority value for the logical channel with delay-critical data, the network can control the impact and the priority order between logical channels with delay-critical data and logical channels for SRBs other than SRB 0. So it can left to the network’s implementation and there should be no specification impact.

Proposal 4: It can be left to the network’s implementation to configure the priorities of LCHs with delay-critical data and LCHs for SRBs other than SRB 0.

Regarding the configured UL grant originally for XR traffic, the delay-awareness LCP mechanism can be applied to multiplex the delay-critical data with a higher priority. However, there exist other types of periodic traffic (e.g., voice or URLLC service) that are configured with configured UL grant as well. For these services, if delay-awareness LCP is applied to the configured UL grants, delay-critical data of XR traffic may occupy the resources originally reserved for the periodic voice or URLLC traffic, which shall bring extra delay for these services and cause a bad user experience. For this, we propose that different configured UL grant configurations can apply different strategies of whether to apply the delay-awareness LCP mechanism.

Proposal 5: Different configured UL grant configurations can apply different strategies of whether to apply delay-awareness LCP mechanism.

2.2 Applicability of adjusted priority in other procedures.

LCH’s Priority is mainly but not only used for LCP. LCH priority is also used in intra-UE prioritization, BSR trigger and truncated BSR report procedures. There then comes a question: While an LCH’s priority is adjusted based on remaining delay, whether the adjusted priority or original priority should be used for these other procedures?

As shown in the cited example texts from MAC specification, we looked into these procedures one by one:

- Inter-UE prioritization:

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects a HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection among initial transmission and retransmission with equal priority, the UE shall prioritize retransmissions before initial transmissions.

When a configured uplink grant, dynamical uplink grant and/or SR transmissions overlap in time for a UE, the UE prioritizes the transmission based on the priorities of the LCHs that are multiplexed to transmit. When UE adjusts an LCH’s priority into a higher value, the eventual intention is to transmit these date to gNB with higher priority, to achieve this, LCH with priority adjusted date shall not only be prioritized during LCP procedure but also intra-UE prioritization procedure.

Observation 3: Applying adjusted priority to intra-UE prioritization is helpful to achieve the overall objective of avoiding too late transmission

- BSR trigger:

A BSR shall be triggered if any of the following events occur for activated cell group: - UL data, for a logical channel which belongs to an LCG, becomes available to the MAC entity; and either - this UL data belongs to a logical channel with higher priority than the priority of any logical channel containing available UL data which belong to any LCG; or - none of the logical channels which belong to an LCG contains any available UL data. in which case the BSR is referred below to as 'Regular BSR';

As above, a regular BSR will be triggered when new data arrives in an empty LCH and this LCH has higher priority than other LCHs having buffered data and of the same LCG. Typically, data that just arrived should not be priority-adjusted data, instead, data that has been buffered a while may become priority-adjusted data. A LCH#A having new data arrival may have higher priority than the original priority of another LCH#B but lower priority than the adjusted priority of the same LCH#B, in this situation, the regular BSR may not be triggered if adjusted priority is applied.

Observation 4: Applying adjusted priority to BSR trigger may eliminate regular BSR triggers

- Truncated (padding) BSR report:

For Padding BSR, the MAC entity for which logicalChannelGroupIAB-Ext is not configured by upper layers shall: 1> if the number of padding bits is equal to or larger than the size of the Short BSR plus its subheader but smaller than the size of the Long BSR plus its subheader: 2> if more than one LCG has data available for transmission when the BSR is to be built: 3> if the number of padding bits is equal to the size of the Short BSR plus its subheader: 4> report Short Truncated BSR of the LCG with the highest priority logical channel with data available for transmission. 3> else: 4> report Long Truncated BSR of the LCG(s) with the logical channels having data available for transmission following a decreasing order of the highest priority logical channel (with or without data available for transmission) in each of these LCG(s), and in case of equal priority, in increasing order of LCGID. 2> else: 3> report Short BSR.

As above, when there are padding bits but not enough for a full long BSR, UE will construct a (short or long) truncated BSR. In this scenario, UE will prioritize LCG which has an LCH with higher priority and with buffered data to report. In one hand, because truncated BSR is best effort approach, it won’t be a big issue if we do not use adjusted priority in our opinion. In the other hand, if there is priority-adjusted data in an LCH, it would be better to report this first.

Observation 5: Applying adjusted priority to truncated BSR report would be preferred but not a must-to-do.

In summary, adjusted priority should be also used in intra-UE prioritization in addition to LCP with the same motivation. On the other hand, it is not preferable to use it in regular BSR trigger but preferable to use it in truncated BSR report, however with DSR is configured, it won’t be a big issue if BSR is triggered less often or truncated BSR report this but not that LCG. The specification effort might be slightly higher if different procedures/places used different priority orders:

Proposal 6: Adjusted priority is applicable to intra-UE prioritization

Proposal 7: RAN2 to discuss whether adjusted priority is applicable to truncated BSR report, and or regular BSR trigger or not considering functional necessity and specification effort.

# Summary

This contribution provides our analysis on the enhancement of LCP procedure, and has the following proposals:

Observation 1: The 2nd round of the LCP procedure is not concerned with fairness, as it’s designed to maximize the transmission of high-priority data on a best-effort basis.

Observation 2: Adjust LCH priority at the start of 2nd round is technically feasible but will increase TB preparation time.

Proposal 1: The additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of the LCP procedure.

Proposal 2: RAN2 to discuss whether an LCH with priority-adjusted data should be served before one without such data when both have equal priority.

Proposal 3: Maximize the transmission of a PDU Set during delay-awareness LCP procedure when all PDUs are needed for a PDU Set.

Proposal 4: It can be left to the network’s implementation to configure the priorities of LCHs with delay-critical data and LCHs for SRBs other than SRB 0.

Observation 3: Applying adjusted priority to intra-UE prioritization is helpful to achieve the overall objective of avoiding too late transmission.

Observation 4: Applying adjusted priority to BSR trigger may eliminate regular BSR triggers.

Observation 5: Applying adjusted priority to truncated BSR report would be preferred but not a must-to-do.

Proposal 6: Adjusted priority is applicable to intra-UE prioritization

Proposal 7: RAN2 to discuss whether adjusted priority is applicable to truncated BSR report, and or regular BSR trigger or not considering functional necessity and specification effort.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China

[2] TS 38.321, Medium Access Control (MAC) protocol specification, v18.3.0