3GPP TSG-RAN WG2 Meeting #128	R2-2410200

# Orlando, USA, 18th – 22nd November 2024

Agenda Item:	8.7.6

Source:	Ericsson

Title:	More on XR Rate Control

Document for:	Discussion, Decision

# 1	Introduction

The following are the agreements from the last meeting on the XR rate control objective:

Agreements on XR rate control - FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc. - RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - Extend the Bit Rate field - Define a new bit rate table to provide sufficient granularity for XR traffic - Introduce new values for the bitRateMultiplier - Send LS to SA4 asking about range/granularity which is required

In this contribution, we discuss the agreement made above and provide our views.

# 2	Discussion

## 2.1	Flow-level vs DRB-level Granularity, Application knows best!

FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc.

From the discussion in the last meeting, companies have a different understanding on the QoS-flow vs DRB-level granularity. In our understanding, only the UE (application in the UE) is aware and in the best position to choose the data/IP flow that should be rate adapted for e.g., which video stream. In contrast (to the UE which has a lot of information), RAN only processes SDUs/PDUs in DRBs to/from the UE. It is not aware whether a particular data/IP flow is (or not) suitable to adapt, this information is not available at the RAN.

- As opposed to RAN, the UE is aware and in the best position to decide if/how to perform rate adaptation of the data flows.

The benefit of including the QFI (QoS-flow indicator) instead of the LCID (DRB) are not shown i.e., it is unclear how the QFI can assist or add more value than if LCID is provided. It is also not clear how the application will behave or adjust the bit rate of the distinct flows. All these aspects are outside the RAN’s scope.

- It is unclear how using the QFI instead of the LCID provides more value as the application adjusting the rate of the distinct flows is outsides RAN’s scope.

The application also does not directly understand QoS flows or the QFI-DRB mapping, and anything which is 3GPP specific. Thus, the AS-layer in the UE will always need to translate (filter) towards the application the information provided in the MAC CE.

- The AS in the UE cannot report the QoS flow information directly (unfiltered) as the application will not understand it. It would need to translate (filter) it into something that the application understands.

To assist in the UE in performing the translation (filtering), SA4 has already defined an Access Network Bit Rate Recommendation (ANBR) framework. The “ANBR-based approach” (as detailed in [1]) enables the application to access the bit rate recommendation provided in the RBR MAC CE from the network. For example, the following shows the ANBR-based approach for the 5G Media Streaming (5GMS) where the bit rate recommendation from the RAN is provided to the 5GMSd client. Similarly, an XR client can also use the ANBR-based approach to obtain the bit rate recommendation.

Figure 5.9.1-1: Downlink Network Assistance alternative approaches ### 5.9.3 ANBR-based downlink Network Assistance This approach for the downlink Network Assistance feature is based upon the exchange of the bit rate recommendation message and bit rate recommendation query message between the RAN and UE as described in TS 38.321 [9]. In this case the Network Assistance function in the 5GMSd Client utilises a UE-internal facility to access the RAN modem driver to access ANBR signalling operations and to receive corresponding responses from the network. Hence, this control interface is outside the scope of the 5GMS architecture. The RAN in this architecture contains control plane and user plane entities that interact with peer control plane and user plane entities in the UE. - The control plane interactions involve the sending by the RAN of downlink bit rate recommendations to the UE, which may occur either independently, or in response to a boost request from the UE, via the exchange of the bit rate recommendation message and bit rate recommendation query message between the RAN and UE as described in TS 38.321 [9]. This control interface is outside the scope of the 5GMS architecture. ### - The user plane interaction pertains to downlink streaming media transport over interface M4d.

- An XR client can reuse the ANBR-based approach to obtain the bit rate recommendation.

Thus, there is already a way for the application (XR client) to access the RAN modem to obtain the bit rate recommendation. Based on the bit rate recommendation, it is up to the application (XR client) to decide which of the data flows are adapted independent of mapping between QoS flows and DRBs. Thus, there is no need to provide additional information/granularity.

- It is already possible for the application (XR client) to access the RAN modem and obtain the bit rate recommendation, perform rate adaption independent of the mapping between QoS flow and DRBs.

There also limitations and challenges to make the AS-layer aware of QoS flow information as follows:

- QFI is only known and understood by the SDAP layer and is not carried across layers. Thus, MAC, RLC, and PDCP entities do not know the QFI of each packet. Thus, MAC cannot derive the QFI information.

- In UL, the QFI is derived from the QoS rules provided by core (based on packet filters) and the QFI to DRB mapping is configured by RRC. In addition, the SDAP layer may dynamically change the QFI-DRB mapping may using the RQI mechanism.

On the other hand, the DRB information is already known at the MAC level and is straight-forward to use in the MAC CE. Considering the challenges listed above, it can be concluded that adding the QFI in a MAC PDU is technically not feasible. Adding the QFI in the MAC would result in breaking of the protocol stack separation between the AS and NAS layer. This also may have impact in RAN3 as it is not immediately clear how F1 would be impacted for CU-DU split scenarios as the MAC is located on the DU.

- Cross-layer interactions to expose the QoS information at the MAC-layer (AS) breaks the protocol stack separation between AS and NAS, results in significant delays to trigger the XR rate control signaling and has large specification impact possibly on other working groups.

Considering the above observations, we believe it is sufficient to reuse the current DRB-level granularity as the bit rate recommendation is directly made available to the application.

Support DRB-level granularity for the MAC layer XR rate control signaling.

## 2.2	XR Rate Control Signaling

As LS has been sent to SA4 to understand the bit rate range/granularity required for such XR applications. This should be considered in the design of the MAC-layer signaling for e.g., whether the current recommended bit rate (RBR) MAC CE can be re-used, and/or pursue the other approaches as agreed in the previous meeting. RAN2 should wait for the LS reply from SA4 before proceeding.

Wait for LS reply from SA4 before delving into the design approaches to provide recommended bit rate values better fitting XR applications.

Although we believe it is sufficient to reuse the recommended bit rate MAC CE. Considering the LS sent to SA4, RAN2 should wait for their reply before discussing whether the RBR MAC CE can be reused.

Whether the recommended bit rate MAC CE can be reused should be discussed after the LS reply from SA4 is received.

# 3 	Conclusion

Based on the discussion in the previous sections, we have the following observations:

Observation 1	Compared to the RAN, the UE is aware and in the best position to decide if/how to perform rate adaptation of the data flows.

Observation 2	It is unclear how using the QFI instead of the LCID provides more value as the application adjusting the rate of the distinct flows is outsides RAN’s scope.

Observation 3	The AS in the UE cannot report the QoS flow information directly (unfiltered) as the application will not understand it. It would need to translate (filter) it into something that the application understands.

Observation 4	An XR client can reuse the ANBR-based approach to obtain the bit rate recommendation.

Observation 5	It is already possible for the application (XR client) to access the RAN modem and obtain the bit rate recommendation, perform rate adaption independent of the mapping between QoS flow and DRBs.

Observation 6	Cross-layer interactions to expose the QoS information at the MAC-layer (AS) is unprecedented, results in significant delays to trigger the XR rate control signaling and has large specification impact even on other working groups.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Support DRB-level granularity for the MAC layer XR rate control signaling.

Proposal 2	Wait for LS reply from SA4 before delving into the design approaches to provide recommended bit rate values better fitting XR applications.

Proposal 3	Whether the recommended bit rate MAC CE can be reused should be discussed after the LS reply from SA4 is received.

# 4 	References

- TS 26.501, “5G Media Streaming (5GMS); General description and architecture”, V16.5.0 (2020-10)