3GPP TSG-RAN WG2 Meeting #128	R2-2410210

Orlando, USA, 18 – 22 November 2024

Agenda item:	8.7.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	LCP Enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#127bis:

As a baseline, additional LCH priority is applied for an LCH in both 1st and 2nd Rounds of resource allocation procedure in LCP, as long as the LCH has delay-critical data available for transmission when starting the 1st Round. FFS if we can still change the priority for the 2nd round to ensure fairness, but we need to consider tight timeline of LCP procedure and UE complexity. Companies can also check whether we can leave this to UE implementation Introduce an independent per-LCH remaining time threshold for applying delay-critical priority. We do not introduce any setting restrictions of this new remaining time threshold with relation to DSR triggering threshold.

# 2	Discussion

## 2.1	LCH priority adjustments

In our views, the baseline agreed in RAN2#127bis (i.e., application of the additional LCH priority prior running each LCP procedure) represents the option with the least UE complexity because changing the priority in the second LCP round requires incorporating an evaluation to identify the presence/absence of LCH priority-adjusted data in each LCH prior running the second LCP.

How the UE would identify the presence of LCH priority-adjusted data after 1st round and if it is complex for the UE is up to debate. We can also understand that some UE vendors do not want to touch the legacy LCP procedure itself (other than the LCH priority adjustment before initiation of the LCP), hence the proposal of up to UE implementation. However, leaving it to UE implementation means it is not testable thus cannot be ensured. From NW point of view, we would prefer to have specified behavior other than leaving it to UE implementation.

Proposal 1: LCP should be specified behavior, i.e. not leave it to UE implementation.

It is true that the disadvantage of no adjustment within the LCP procedure is the possible unfairness that non-delay critical data might also benefit from adjusted priority. But it is still the case even if the data status and priority is rechecked at second round, since it does not ideally ensure only the delay critical data uses the higher priority while the non-delay critical data falls back to legacy as there could be delay critical data and non-delay critical data within the 1st or 2nd round. That is the consequence with the agreement of no intra-LCH prioritization and to keep the LCP procedure simple.

Besides, it should be noted that prior occurring the second round, all LCHs are served with resources to guarantee each PBR while the second round is best effort. For XR traffic with GBR, the first round should have already meet the QoS requirement. Thus, it makes less sense to optimize for the second round. Therefore, we consider that for Rel-19 RAN2 should consider only priority adjustment upon initiation of the LCP procedure.

Proposal 2: For simplicity, no additional priority adjustment within LCP procedure.

## 2.2	Other LCP impact

RAN#105 included UL congestion signaling in the Rel-19 XR Phase 3 objectives. Currently, XR rate control per DRB/per QoS flow is being discussed in RAN2. When the bit rate changes, the GBR for the DRB would also change, which means the PBR for the LCH should be reflected as well. If the PBR is not updated based on bit rate set by rate control, this may lead to a starvation problem. For example, a DRB/LCH is configured with a given PBR.  Later, rate control decreases the bit rate for the corresponding DRB/LCH. If the PBR is not correspondingly adjusted, MAC may allocate a relatively higher number of resources to the corresponding LCH as compared to lower priority LCHs. Then, the starvation becomes more critical as the network gets congested and less resources are available. RAN2 should discuss the impact of bit rate adjustment on PBR setting.

Proposal 3: RAN2 to discuss the impact of bit rate adjustment on PBR setting.

# 3	Conclusion

LCP enhancements are discussed in this contribution with the following proposals proposed:

Proposal 1: LCP should be specified behavior, i.e. not leave it to UE implementation.

Proposal 2: For simplicity, no additional priority adjustment within LCP procedure.

Proposal 3: RAN2 to discuss the impact of bit rate adjustment on PBR setting.